MULTIMEDIA-SYSTEM-CONTROL DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- Export all symbols

-- ===================================================================================

-- Top level Messages

-- ===================================================================================

MultimediaSystemControlMessage	::=CHOICE

{

request	RequestMessage,

response	ResponseMessage,

command	CommandMessage,

indication	IndicationMessage,

...

}

-- A RequestMessage results in action and requires an immediate response

RequestMessage	::=CHOICE

{

nonStandard	NonStandardMessage,

masterSlaveDetermination	MasterSlaveDetermination,

terminalCapabilitySet	TerminalCapabilitySet,

openLogicalChannel	OpenLogicalChannel,

closeLogicalChannel	CloseLogicalChannel,

requestChannelClose	RequestChannelClose,

multiplexEntrySend	MultiplexEntrySend,

requestMultiplexEntry	RequestMultiplexEntry,

requestMode	RequestMode,

roundTripDelayRequest	RoundTripDelayRequest,

maintenanceLoopRequest	MaintenanceLoopRequest,

...,

communicationModeRequest	CommunicationModeRequest,

conferenceRequest	ConferenceRequest,

multilinkRequest	MultilinkRequest

}

-- A ResponseMessage is the response to a request Message

ResponseMessage	::=CHOICE

{

nonStandard	NonStandardMessage,

masterSlaveDeterminationAck	MasterSlaveDeterminationAck,

masterSlaveDeterminationReject	MasterSlaveDeterminationReject,

terminalCapabilitySetAck	TerminalCapabilitySetAck,

terminalCapabilitySetReject	TerminalCapabilitySetReject,

openLogicalChannelAck	OpenLogicalChannelAck,

openLogicalChannelReject	OpenLogicalChannelReject,

closeLogicalChannelAck	CloseLogicalChannelAck,

requestChannelCloseAck	RequestChannelCloseAck,

requestChannelCloseReject	RequestChannelCloseReject,

multiplexEntrySendAck	MultiplexEntrySendAck,

multiplexEntrySendReject	MultiplexEntrySendReject,

requestMultiplexEntryAck	RequestMultiplexEntryAck,

requestMultiplexEntryReject	RequestMultiplexEntryReject,

requestModeAck	RequestModeAck,

requestModeReject	RequestModeReject,

roundTripDelayResponse	RoundTripDelayResponse,

maintenanceLoopAck	MaintenanceLoopAck,

maintenanceLoopReject	MaintenanceLoopReject,

...,

communicationModeResponse	CommunicationModeResponse,

conferenceResponse	ConferenceResponse,

multilinkResponse	MultilinkResponse

}

-- A CommandMessage requires action, but no explicit response

CommandMessage	::=CHOICE

{

nonStandard	NonStandardMessage,

maintenanceLoopOffCommand	MaintenanceLoopOffCommand,

sendTerminalCapabilitySet	SendTerminalCapabilitySet,

encryptionCommand	EncryptionCommand,

flowControlCommand	FlowControlCommand,

endSessionCommand	EndSessionCommand,

miscellaneousCommand	MiscellaneousCommand,

...,

communicationModeCommand	CommunicationModeCommand,

conferenceCommand	ConferenceCommand,

h223MultiplexReconfiguration	H223MultiplexReconfiguration,

newATMVCCommand	NewATMVCCommand

}

-- An IndicationMessage is information that does not require action or response

IndicationMessage	::=CHOICE

{

nonStandard	NonStandardMessage,

functionNotUnderstood	FunctionNotUnderstood,

masterSlaveDeterminationRelease	MasterSlaveDeterminationRelease,

terminalCapabilitySetRelease	TerminalCapabilitySetRelease,

openLogicalChannelConfirm	OpenLogicalChannelConfirm,

requestChannelCloseRelease	RequestChannelCloseRelease,

multiplexEntrySendRelease	MultiplexEntrySendRelease,

requestMultiplexEntryRelease	RequestMultiplexEntryRelease,

requestModeRelease	RequestModeRelease,

miscellaneousIndication	MiscellaneousIndication,

jitterIndication	JitterIndication,

h223SkewIndication	H223SkewIndication,

newATMVCIndication	NewATMVCIndication,

userInput	UserInputIndication,

...,

h2250MaximumSkewIndication	H2250MaximumSkewIndication,

mcLocationIndication	MCLocationIndication,

conferenceIndication	ConferenceIndication,

vendorIdentification	VendorIdentification,

functionNotSupported	FunctionNotSupported,

multilinkIndication	MultilinkIndication

}

-- SequenceNumber is defined here as it is used in a number of Messages

SequenceNumber	::=INTEGER (0..255)

-- ===================================================================================

-- Non standard Message definitions

-- ===================================================================================

NonStandardMessage	::=SEQUENCE

{

nonStandardData	NonStandardParameter,

...

}

NonStandardParameter	::=SEQUENCE

{

nonStandardIdentifier	NonStandardIdentifier,

data		OCTET STRING

}

NonStandardIdentifier	::=CHOICE

{

object		OBJECT IDENTIFIER,

h221NonStandard	SEQUENCE

{

t35CountryCode	INTEGER (0..255),	-- country, per T.35

t35Extension	INTEGER (0..255),	-- assigned nationally

manufacturerCode	INTEGER (0..65535)	-- assigned nationally

}

}

-- ===================================================================================

-- Master-slave determination definitions

-- ===================================================================================

MasterSlaveDetermination	::=SEQUENCE

{

terminalType	INTEGER (0..255),

statusDeterminationNumber	INTEGER (0..16777215),

...

}

MasterSlaveDeterminationAck	::=SEQUENCE

{

decision	CHOICE

{

master	NULL,

slave	NULL

},

...

}

MasterSlaveDeterminationReject	::=SEQUENCE

{

cause		CHOICE

{

identicalNumbers	NULL,

...

},

...

}

MasterSlaveDeterminationRelease	::=SEQUENCE

{

...

}

-- ===================================================================================

-- Capability exchange definitions

-- ===================================================================================

TerminalCapabilitySet	::=SEQUENCE

{

sequenceNumber	SequenceNumber,

protocolIdentifier	OBJECT IDENTIFIER,

-- shall be set to the value

-- {itu-t (0) recommendation (0) h (8) 245 version (0) 4}

multiplexCapability	MultiplexCapability OPTIONAL,

capabilityTable	SET SIZE (1..256) OF CapabilityTableEntry OPTIONAL,

capabilityDescriptors	SET SIZE (1..256) OF CapabilityDescriptor OPTIONAL,

...

}

CapabilityTableEntry	::=SEQUENCE

{

capabilityTableEntryNumber	CapabilityTableEntryNumber,

capability	Capability OPTIONAL

}

CapabilityDescriptor	::=SEQUENCE

{

capabilityDescriptorNumber	CapabilityDescriptorNumber,

simultaneousCapabilities	SET SIZE (1..256) OF AlternativeCapabilitySet OPTIONAL

}

AlternativeCapabilitySet	::=SEQUENCE SIZE (1..256) OF CapabilityTableEntryNumber

CapabilityTableEntryNumber	::=INTEGER (1..65535)

CapabilityDescriptorNumber	::=INTEGER (0..255)

TerminalCapabilitySetAck	::=SEQUENCE

{

sequenceNumber	SequenceNumber,

...

}

TerminalCapabilitySetReject	::=SEQUENCE

{

sequenceNumber	SequenceNumber,

cause		CHOICE

{

unspecified	NULL,

undefinedTableEntryUsed	NULL,

descriptorCapacityExceeded	NULL,

tableEntryCapacityExceeded	CHOICE

{

highestEntryNumberProcessed	CapabilityTableEntryNumber,

noneProcessed	NULL

},

...

},

...

}

TerminalCapabilitySetRelease	::=SEQUENCE

{

...

}

-- ===================================================================================

-- Capability exchange definitions: top level capability description

-- ===================================================================================

Capability		::=CHOICE

{

nonStandard	NonStandardParameter,

receiveVideoCapability	VideoCapability,

transmitVideoCapability	VideoCapability,

receiveAndTransmitVideoCapability	VideoCapability,

receiveAudioCapability	AudioCapability,

transmitAudioCapability	AudioCapability,

receiveAndTransmitAudioCapability	AudioCapability,

receiveDataApplicationCapability	DataApplicationCapability,

transmitDataApplicationCapability	DataApplicationCapability,

receiveAndTransmitDataApplicationCapability	DataApplicationCapability,

h233EncryptionTransmitCapability	BOOLEAN,

h233EncryptionReceiveCapability	SEQUENCE

{

h233IVResponseTime	INTEGER (0..255),	-- units milliseconds

...

},

...,

conferenceCapability	ConferenceCapability,

h235SecurityCapability	H235SecurityCapability,

maxPendingReplacementFor	INTEGER (0..255),

receiveUserInputCapability	UserInputCapability,

transmitUserInputCapability	UserInputCapability,

receiveAndTransmitUserInputCapability	UserInputCapability

}

H235SecurityCapability	::=SEQUENCE

{

encryptionAuthenticationAndIntegrity	EncryptionAuthenticationAndIntegrity,

mediaCapability	CapabilityTableEntryNumber,

-- Note: the mediaCapability shall refer to Capability Table Entries that do contain a transmit,

-- receive, or receiveAndTransmit AudioCapability, VideoCapability,

-- DataApplicationCapability, or similar capability indicated by a NonStandardParameter only

...

}

-- ===================================================================================

-- Capability exchange definitions: Multiplex capabilities

-- ===================================================================================

MultiplexCapability	::=CHOICE

{

nonStandard	NonStandardParameter,

h222Capability	H222Capability,

h223Capability	H223Capability,

v76Capability	V76Capability,

...,

h2250Capability	H2250Capability

}

H222Capability	::= SEQUENCE

{

numberOfVCs	INTEGER (1..256),

vcCapability	SET OF VCCapability,

...

}

VCCapability	::=SEQUENCE

{

aal1		SEQUENCE

{

nullClockRecovery	BOOLEAN,

srtsClockRecovery	BOOLEAN,

adaptiveClockRecovery	BOOLEAN,

nullErrorCorrection	BOOLEAN,

longInterleaver	BOOLEAN,

shortInterleaver	BOOLEAN,

errorCorrectionOnly	BOOLEAN,

structuredDataTransfer	BOOLEAN,

partiallyFilledCells	BOOLEAN,

...

} OPTIONAL,

aal5		SEQUENCE

{

forwardMaximumSDUSize	INTEGER (0..65535),	-- units octets

backwardMaximumSDUSize	INTEGER (0..65535),	-- units octets

...

} OPTIONAL,

transportStream	BOOLEAN,

programStream	BOOLEAN,

availableBitRates	SEQUENCE

{

type	CHOICE

{

singleBitRate	INTEGER (1..65535),	-- units 64 kbit/s

rangeOfBitRates	SEQUENCE

{

lowerBitRate	INTEGER (1..65535),	-- units 64 kbit/s

higherBitRate	INTEGER (1..65535)	-- units 64 kbit/s

}

},

...

},

...

aal1ViaGateway	SEQUENCE

{

gatewayAddress	SET SIZE(1..256) OF Q2931Address,

nullClockRecovery	BOOLEAN,

srtsClockRecovery	BOOLEAN,

adaptiveClockRecovery	BOOLEAN,

nullErrorCorrection	BOOLEAN,

longInterleaver	BOOLEAN,

shortInterleaver	BOOLEAN,

errorCorrectionOnly	BOOLEAN,

structuredDataTransfer	BOOLEAN,

partiallyFilledCells	BOOLEAN,

...

} OPTIONAL

}

H223Capability	::=SEQUENCE

{

transportWithI-frames	BOOLEAN,	-- I-frame transport of H.245

videoWithAL1	BOOLEAN,

videoWithAL2	BOOLEAN,

videoWithAL3	BOOLEAN,

audioWithAL1	BOOLEAN,

audioWithAL2	BOOLEAN,

audioWithAL3	BOOLEAN,

dataWithAL1	BOOLEAN,

dataWithAL2	BOOLEAN,

dataWithAL3	BOOLEAN,

maximumAl2SDUSize	INTEGER (0..65535),	-- units octets

maximumAl3SDUSize	INTEGER (0..65535),	-- units octets

maximumDelayJitter	INTEGER (0..1023),	-- units milliseconds

h223MultiplexTableCapability	CHOICE

{

basic	NULL,

enhanced	SEQUENCE

{

maximumNestingDepth	INTEGER (1..15),

maximumElementListSize	INTEGER (2..255),

maximumSubElementListSize	INTEGER (2..255),

...

}

},

...,

maxMUXPDUSizeCapability	BOOLEAN,

nsrpSupport	BOOLEAN,

mobileOperationTransmitCapability	SEQUENCE

{

modeChangeCapability	BOOLEAN,

h223AnnexA	BOOLEAN,

h223AnnexADoubleFlag	BOOLEAN,

h223AnnexB	BOOLEAN,

h223AnnexBwithHeader	BOOLEAN,

...

} OPTIONAL,

h223AnnexCCapability	H223AnnexCCapability OPTIONAL

}

H223AnnexCCapability	::= SEQUENCE

{

videoWithAL1M	BOOLEAN,

videoWithAL2M	BOOLEAN,

videoWithAL3M	BOOLEAN,

audioWithAL1M	BOOLEAN,

audioWithAL2M	BOOLEAN,

audioWithAL3M	BOOLEAN,

dataWithAL1M	BOOLEAN,

dataWithAL2M	BOOLEAN,

dataWithAL3M	BOOLEAN,

alpduInterleaving	BOOLEAN,

maximumAL1MPDUSize	INTEGER (0..65535),	-- units octets

maximumAL2MSDUSize	INTEGER (0..65535),	-- units octets

maximumAL3MSDUSize	INTEGER (0..65535),	-- units octets

...

}

V76Capability	::=SEQUENCE

{

suspendResumeCapabilitywAddress	BOOLEAN,

suspendResumeCapabilitywoAddress	BOOLEAN,

rejCapability	BOOLEAN,

sREJCapability	BOOLEAN,

mREJCapability	BOOLEAN,

crc8bitCapability	BOOLEAN,

crc16bitCapability	BOOLEAN,

crc32bitCapability	BOOLEAN,

uihCapability	BOOLEAN,

numOfDLCS	INTEGER (2..8191),

twoOctetAddressFieldCapability	BOOLEAN,

loopBackTestCapability	BOOLEAN,

n401Capability	INTEGER (1..4095),

maxWindowSizeCapability	INTEGER (1..127),

v75Capability	V75Capability,

...

}

V75Capability	::=SEQUENCE

{

audioHeader	BOOLEAN,

...

}

H2250Capability	::=SEQUENCE

{

maximumAudioDelayJitter	INTEGER(0..1023),	-- units in milliseconds

receiveMultipointCapability	MultipointCapability,

transmitMultipointCapability	MultipointCapability,

receiveAndTransmitMultipointCapability	MultipointCapability,

mcCapability	SEQUENCE

{

centralizedConferenceMC	BOOLEAN,

decentralizedConferenceMC	BOOLEAN,

...

},

rtcpVideoControlCapability	BOOLEAN,	-- FIR and NACK

mediaPacketizationCapability	MediaPacketizationCapability,

...,

transportCapability	TransportCapability OPTIONAL,

redundancyEncodingCapability	SEQUENCE SIZE(1..256) OF RedundancyEncodingCapability OPTIONAL,

logicalChannelSwitchingCapability	BOOLEAN,

t120DynamicPortCapability	BOOLEAN

}

MediaPacketizationCapability	::=SEQUENCE

{

h261aVideoPacketization	BOOLEAN,

...,

rtpPayloadType	SEQUENCE SIZE(1..256) OF RTPPayloadType OPTIONAL

}

RSVPParameters	::=SEQUENCE

{

qosMode	QOSMode OPTIONAL,

tokenRate	INTEGER (1..4294967295) OPTIONAL,

-- rate in bytes/sec

bucketSize	INTEGER (1..4294967295) OPTIONAL,        -- size in bytes

peakRate	INTEGER (1..4294967295) OPTIONAL,

-- peak bandwidth bytes/sec

minPoliced	INTEGER (1..4294967295) OPTIONAL,        --

maxPktSize	INTEGER (1..4294967295) OPTIONAL,        -- size in bytes

...

}

QOSMode		::=CHOICE

{

guaranteedQOS	NULL,

controlledLoad	NULL,

...

}

-- Ed. Replace with ITU-T ATM Transfer Capability Types

ATMParameters	::=SEQUENCE

{

maxNTUSize	INTEGER(0..65535),        --units in octets

atmUBR	BOOLEAN,                       --unspecified bit rate

atmrtVBR	BOOLEAN,                       --real time variable bit rate

atmnrtVBR	BOOLEAN,                       --non real time variable bit rate

atmABR	BOOLEAN,                       --available bit rate

atmCBR	BOOLEAN,                       --constant bit rate

...

}

QOSCapability	::=SEQUENCE

{

nonStandardData	NonStandardParameter OPTIONAL,

rsvpParameters	RSVPParameters OPTIONAL,

atmParameters	ATMParameters OPTIONAL,

...

}

MediaTransportType	::=CHOICE

{

ip-UDP		NULL,

ip-TCP		NULL,

atm-AAL5-UNIDIR	NULL, -- virtual circuits used as unidirectional

atm-AAL5-BIDIR	NULL, -- virtual circuits used as bidirectional

...

}

MediaChannelCapability	::=SEQUENCE

{

mediaTransport	MediaTransportType  OPTIONAL,

...

}

TransportCapability	::=SEQUENCE

{

nonStandard            	NonStandardParameter OPTIONAL,

qOSCapabilities	SEQUENCE SIZE(1..256) OF QOSCapability OPTIONAL,

mediaChannelCapabilities	SEQUENCE SIZE(1..256) OF MediaChannelCapability OPTIONAL,

...

}

RedundancyEncodingCapability	::=SEQUENCE

{

redundancyEncodingMethod	RedundancyEncodingMethod,

primaryEncoding	CapabilityTableEntryNumber,

secondaryEncoding	SEQUENCE SIZE(1..256) OF CapabilityTableEntryNumber OPTIONAL,

...

}

RedundancyEncodingMethod	::=CHOICE

{

nonStandard	NonStandardParameter,

rtpAudioRedundancyEncoding	NULL,

...,

rtpH263VideoRedundancyEncoding	RTPH263VideoRedundancyEncoding

}

RTPH263VideoRedundancyEncoding	::= SEQUENCE

{

numberOfThreads	INTEGER (1..16),

framesBetweenSyncPoints	INTEGER (1..256),

frameToThreadMapping	CHOICE

{

roundrobin	NULL,

custom	SEQUENCE SIZE(1..256) OF

RTPH263VideoRedundancyFrameMapping,

-- empty SEQUENCE for capability negotiation

-- meaningful contents only OpenLogicalChannel

...

},

containedThreads	SEQUENCE SIZE(1..256) OF INTEGER (0..15) OPTIONAL,

-- only used for opening of logical channels

...

}

RTPH263VideoRedundancyFrameMapping	::= SEQUENCE

{

threadNumber	INTEGER (0..15),

frameSequence	SEQUENCE SIZE(1..256) OF INTEGER (0..255),

...

}

MultipointCapability	::=SEQUENCE

{

multicastCapability	BOOLEAN,

multiUniCastConference	BOOLEAN,

mediaDistributionCapability	SEQUENCE OF MediaDistributionCapability,

...

}

MediaDistributionCapability	::=SEQUENCE

{

centralizedControl	BOOLEAN,

distributedControl	BOOLEAN,	-- for further study in H.323

centralizedAudio	BOOLEAN,

distributedAudio	BOOLEAN,

centralizedVideo	BOOLEAN,

distributedVideo	BOOLEAN,

centralizedData	SEQUENCE OF DataApplicationCapability OPTIONAL,

distributedData	SEQUENCE OF DataApplicationCapability OPTIONAL,

-- for further study in H.323

...

}

-- ===================================================================================

-- Capability exchange definitions: Video capabilities

-- ===================================================================================

VideoCapability	::=CHOICE

{

nonStandard	NonStandardParameter ,

h261VideoCapability	H261VideoCapability,

h262VideoCapability	H262VideoCapability,

h263VideoCapability	H263VideoCapability,

is11172VideoCapability	IS11172VideoCapability,

...

}

H261VideoCapability	::=SEQUENCE

{

qcifMPI	INTEGER (1..4) OPTIONAL,	-- units 1/29.97 Hz

cifMPI		INTEGER (1..4) OPTIONAL,	-- units 1/29.97 Hz

temporalSpatialTradeOffCapability	BOOLEAN,

maxBitRate	INTEGER (1..19200),	-- units of 100 bit/s

stillImageTransmission	BOOLEAN,	-- Annex D of H.261

...

}

H262VideoCapability	::=SEQUENCE

{

profileAndLevel-SPatML	BOOLEAN,

profileAndLevel-MPatLL	BOOLEAN,

profileAndLevel-MPatML	BOOLEAN,

profileAndLevel-MPatH-14	BOOLEAN,

profileAndLevel-MPatHL	BOOLEAN,

profileAndLevel-SNRatLL	BOOLEAN,

profileAndLevel-SNRatML	BOOLEAN,

profileAndLevel-SpatialatH-14	BOOLEAN,

profileAndLevel-HPatML	BOOLEAN,

profileAndLevel-HPatH-14	BOOLEAN,

profileAndLevel-HPatHL	BOOLEAN,

videoBitRate	INTEGER (0.. 1073741823) OPTIONAL,	-- units 400 bit/s

vbvBufferSize	INTEGER (0.. 262143) OPTIONAL,	-- units 16384 bits

samplesPerLine	INTEGER (0..16383) OPTIONAL,	-- units samples/line

linesPerFrame	INTEGER (0..16383) OPTIONAL,	-- units lines/frame

framesPerSecond	INTEGER (0..15) OPTIONAL,	-- frame_rate_code

luminanceSampleRate	INTEGER (0..4294967295) OPTIONAL,	-- units samples/sec

...

}

H263VideoCapability	::=SEQUENCE

{

sqcifMPI	INTEGER (1..32) OPTIONAL,	-- units 1/29.97 Hz

qcifMPI	INTEGER (1..32) OPTIONAL,	-- units 1/29.97 Hz

cifMPI		INTEGER (1..32) OPTIONAL,	-- units 1/29.97 Hz

cif4MPI	INTEGER (1..32) OPTIONAL,	-- units 1/29.97 Hz

cif16MPI	INTEGER (1..32) OPTIONAL,	-- units 1/29.97 Hz

maxBitRate	INTEGER (1..192400),	-- units 100 bit/s

unrestrictedVector	BOOLEAN,

arithmeticCoding	BOOLEAN,

advancedPrediction	BOOLEAN,

pbFrames	BOOLEAN,

temporalSpatialTradeOffCapability	BOOLEAN,

hrd-B		INTEGER (0..524287) OPTIONAL,	-- units 128 bits

bppMaxKb	INTEGER (0..65535) OPTIONAL,	-- units 1024 bits

...,

slowSqcifMPI	INTEGER (1..3600) OPTIONAL,	-- units seconds/frame

slowQcifMPI	INTEGER (1..3600) OPTIONAL, --units seconds/frame

slowCifMPI	INTEGER (1..3600) OPTIONAL, --units seconds/frame

slowCif4MPI	INTEGER (1..3600) OPTIONAL, --units seconds/frame

slowCif16MPI	INTEGER (1..3600) OPTIONAL, --units seconds/frame

errorCompensation	BOOLEAN,

enhancementLayerInfo	EnhancementLayerInfo OPTIONAL,

h263Options	H263Options OPTIONAL

}

EnhancementLayerInfo	::=SEQUENCE

{

baseBitRateConstrained	BOOLEAN,

snrEnhancement 	SET SIZE(1..14) OF EnhancementOptions OPTIONAL,

spatialEnhancement 	SET SIZE(1..14) OF EnhancementOptions OPTIONAL,

bPictureEnhancement	SET SIZE(1..14) OF BEnhancementParameters OPTIONAL,

...

}

BEnhancementParameters	::=SEQUENCE

{

enhancementOptions	EnhancementOptions,

numberOfBPictures	INTEGER (1..64),

...

}

EnhancementOptions	::=SEQUENCE

{

sqcifMPI	INTEGER (1..32) OPTIONAL,  -- units 1/29.97 Hz

qcifMPI	INTEGER (1..32) OPTIONAL,  -- units 1/29.97 Hz

cifMPI		INTEGER (1..32) OPTIONAL,  -- units 1/29.97 Hz

cif4MPI 	INTEGER (1..32) OPTIONAL,  -- units 1/29.97 Hz

cif16MPI	INTEGER (1..32) OPTIONAL,  -- units 1/29.97 Hz

maxBitRate	INTEGER (1..192400),             -- units 100 bit/s

unrestrictedVector	BOOLEAN,

arithmeticCoding	BOOLEAN,

temporalSpatialTradeOffCapability	BOOLEAN,

slowSqcifMPI	INTEGER (1..3600) OPTIONAL,  -- units econds/frame

slowQcifMPI	INTEGER (1..3600) OPTIONAL,   --units seconds/frame

slowCifMPI	INTEGER (1..3600) OPTIONAL,   --units seconds/frame

slowCif4MPI	INTEGER (1..3600) OPTIONAL,   --units seconds/frame

slowCif16MPI	INTEGER (1..3600) OPTIONAL,   --units seconds/frame

errorCompensation	BOOLEAN,

h263Options	H263Options OPTIONAL,

...

}

H263Options	::= SEQUENCE

{

advancedIntraCodingMode	BOOLEAN,

deblockingFilterMode	BOOLEAN,

improvedPBFramesMode	BOOLEAN,

unlimitedMotionVectors	BOOLEAN,

fullPictureFreeze	BOOLEAN,

partialPictureFreezeAndRelease	BOOLEAN,

resizingPartPicFreezeAndRelease	BOOLEAN,

fullPictureSnapshot	BOOLEAN,

partialPictureSnapshot	BOOLEAN,

videoSegmentTagging	BOOLEAN,

progressiveRefinement	BOOLEAN,

dynamicPictureResizingByFour	BOOLEAN,

dynamicPictureResizingSixteenthPel	BOOLEAN,

dynamicWarpingHalfPel	BOOLEAN,

dynamicWarpingSixteenthPel	BOOLEAN,

independentSegmentDecoding	BOOLEAN,

slicesInOrder-NonRect	BOOLEAN,

slicesInOrder-Rect	BOOLEAN,

slicesNoOrder-NonRect	BOOLEAN,

slicesNoOrder-Rect	BOOLEAN,

alternateInterVLCMode	BOOLEAN,

modifiedQuantizationMode	BOOLEAN,

reducedResolutionUpdate	BOOLEAN,

transparencyParameters	TransparencyParameters OPTIONAL,

separateVideoBackChannel	BOOLEAN,

refPictureSelection	RefPictureSelection OPTIONAL,

customPictureClockFrequency	SET SIZE (1..16) OF CustomPictureClockFrequency	 OPTIONAL,

customPictureFormat	SET SIZE (1..16) OF CustomPictureFormat OPTIONAL,

modeCombos	SET SIZE (1..16) OF H263VideoModeCombos OPTIONAL,

...

}

TransparencyParameters	::= SEQUENCE

{

presentationOrder	INTEGER(1..256),

offset-x	INTEGER(-262144..262143), -- 1/8 pixels

offset-y	INTEGER(-262144..262143), -- 1/8 pixels

scale-x		INTEGER(1..255),

scale-y		INTEGER(1..255),

...

}

RefPictureSelection	::=SEQUENCE

{

additionalPictureMemory	SEQUENCE

{

sqcifAdditionalPictureMemory	INTEGER (1..256) OPTIONAL,	-- units frame

qcifAdditionalPictureMemory	INTEGER (1..256) OPTIONAL,	-- units frame

cifAdditionalPictureMemory	INTEGER (1..256) OPTIONAL,	-- units frame

cif4AdditionalPictureMemory	INTEGER (1..256) OPTIONAL,	-- units frame

cif16AdditionalPictureMemory	INTEGER (1..256) OPTIONAL,	-- units frame

bigCpfAdditionalPictureMemory	INTEGER (1..256) OPTIONAL,	-- units frame

...

} OPTIONAL,

videoMux	BOOLEAN,

videoBackChannelSend	CHOICE

{

none	NULL,

ackMessageOnly	NULL,

nackMessageOnly	NULL,

ackOrNackMessageOnly	NULL,

ackAndNackMessage	NULL,

...

},

...

}

CustomPictureClockFrequency	::=SEQUENCE

{

clockConversionCode	INTEGER(1000..1001),

clockDivisor	INTEGER(1..127),

sqcifMPI	INTEGER (1..2048) OPTIONAL,

qcifMPI	INTEGER (1..2048) OPTIONAL,

cifMPI		INTEGER (1..2048) OPTIONAL,

cif4MPI	INTEGER (1..2048) OPTIONAL,

cif16MPI	INTEGER (1..2048) OPTIONAL,

...

}

CustomPictureFormat	::=SEQUENCE

{

maxCustomPictureWidth	INTEGER(1..2048),	-- units 4 pixels

maxCustomPictureHeight	INTEGER(1..2048),	-- units 4 pixels

minCustomPictureWidth	INTEGER(1..2048),	-- units 4 pixels

minCustomPictureHeight	INTEGER(1..2048),	-- units 4 pixels

mPI			SEQUENCE

{

standardMPI	INTEGER (1..31) OPTIONAL,

customPCF	SET SIZE (1..16) OF SEQUENCE

{

clockConversionCode	INTEGER (1000..1001),

clockDivisor	INTEGER (1..127),

customMPI	INTEGER (1..2048),

...

} OPTIONAL,

...

},

pixelAspectInformation	CHOICE

{

anyPixelAspectRatio	BOOLEAN,

pixelAspectCode	SET SIZE (1..14) OF INTEGER(1..14),

extendedPAR	SET SIZE (1..256) OF SEQUENCE

{

width	INTEGER(1..255),

height	INTEGER(1..255),

...

},

...

} ,

...

}

H263VideoModeCombos	::= SEQUENCE

{

h263VideoUncoupledModes	H263ModeComboFlags,

h263VideoCoupledModes	SET SIZE (1..16) OF H263ModeComboFlags,

...

}

H263ModeComboFlags	::= SEQUENCE

{

unrestrictedVector	BOOLEAN,

arithmeticCoding	BOOLEAN,

advancedPrediction	BOOLEAN,

pbFrames	BOOLEAN,

advancedIntraCodingMode	BOOLEAN,

deblockingFilterMode	BOOLEAN,

unlimitedMotionVectors	BOOLEAN,

slicesInOrder-NonRect	BOOLEAN,

slicesInOrder-Rect	BOOLEAN,

slicesNoOrder-NonRect	BOOLEAN,

slicesNoOrder-Rect	BOOLEAN,

improvedPBFramesMode	BOOLEAN,

referencePicSelect	BOOLEAN,

dynamicPictureResizingByFour	BOOLEAN,

dynamicPictureResizingSixteenthPel	BOOLEAN,

dynamicWarpingHalfPel	BOOLEAN,

dynamicWarpingSixteenthPel	BOOLEAN,

reducedResolutionUpdate	BOOLEAN,

independentSegmentDecoding	BOOLEAN,

alternateInterVLCMode	BOOLEAN,

modifiedQuantizationMode	BOOLEAN,

...

}

IS11172VideoCapability	::=SEQUENCE

{

constrainedBitstream	BOOLEAN,

videoBitRate	INTEGER (0.. 1073741823) OPTIONAL,	-- units 400 bit/s

vbvBufferSize	INTEGER (0.. 262143) OPTIONAL,	-- units 16384 bits

samplesPerLine	INTEGER (0..16383) OPTIONAL,	-- units samples/line

linesPerFrame	INTEGER (0..16383) OPTIONAL,	-- units lines/frame

pictureRate	INTEGER (0..15) OPTIONAL,

luminanceSampleRate	INTEGER (0..4294967295) OPTIONAL,	-- units samples/sec

...

}

-- ===================================================================================

-- Capability exchange definitions: Audio capabilities

-- ===================================================================================

-- For an H.222 multiplex, the integers indicate the size of the STD buffer in units of 256 octets

-- For an H.223 multiplex, the integers indicate the maximum number of audio frames per AL-SDU

-- For an H.225.0 multiplex, the integers indicate the maximum number of audio frames per packet

AudioCapability	::=CHOICE

{

nonStandard	NonStandardParameter,

g711Alaw64k	INTEGER (1..256),

g711Alaw56k	INTEGER (1..256),

g711Ulaw64k	INTEGER (1..256),

g711Ulaw56k	INTEGER (1..256),

g722-64k	INTEGER (1..256),

g722-56k	INTEGER (1..256),

g722-48k	INTEGER (1..256),

g7231		SEQUENCE

{

maxAl-sduAudioFrames	INTEGER (1..256),

silenceSuppression	BOOLEAN

},

g728		INTEGER (1..256),

g729		INTEGER (1..256),

g729AnnexA	INTEGER (1..256),

is11172AudioCapability	IS11172AudioCapability,

is13818AudioCapability	IS13818AudioCapability,

...,

g729wAnnexB	INTEGER(1..256),

g729AnnexAwAnnexB	INTEGER(1..256),

g7231AnnexCCapability	G7231AnnexCCapability,

gsmFullRate	GSMAudioCapability,

gsmHalfRate	GSMAudioCapability,

gsmEnhancedFullRate	GSMAudioCapability

}

G7231AnnexCCapability	::= SEQUENCE

{

maxAl-sduAudioFrames	INTEGER (1..256),

silenceSuppression	BOOLEAN,

g723AnnexCAudioMode	SEQUENCE

{

highRateMode0	INTEGER (27..78),	-- units octets

highRateMode1	INTEGER (27..78),	-- units octets

lowRateMode0	INTEGER (23..66),	-- units octets

lowRateMode1	INTEGER (23..66),	-- units octets

sidMode0	INTEGER (6..17),	-- units octets

sidMode1	INTEGER (6..17),	-- units octets

...

} OPTIONAL,

...

}

IS11172AudioCapability	::=SEQUENCE

{

audioLayer1	BOOLEAN,

audioLayer2	BOOLEAN,

audioLayer3	BOOLEAN,

audioSampling32k	BOOLEAN,

audioSampling44k1	BOOLEAN,

audioSampling48k	BOOLEAN,

singleChannel	BOOLEAN,

twoChannels	BOOLEAN,

bitRate	INTEGER (1..448),	-- units kbit/s

...

}

IS13818AudioCapability	::=SEQUENCE

{

audioLayer1	BOOLEAN,

audioLayer2	BOOLEAN,

audioLayer3	BOOLEAN,

audioSampling16k	BOOLEAN,

audioSampling22k05	BOOLEAN,

audioSampling24k	BOOLEAN,

audioSampling32k	BOOLEAN,

audioSampling44k1	BOOLEAN,

audioSampling48k	BOOLEAN,

singleChannel	BOOLEAN,

twoChannels	BOOLEAN,

threeChannels2-1	BOOLEAN,

threeChannels3-0	BOOLEAN,

fourChannels2-0-2-0	BOOLEAN,

fourChannels2-2	BOOLEAN,

fourChannels3-1	BOOLEAN,

fiveChannels3-0-2-0	BOOLEAN,

fiveChannels3-2	BOOLEAN,

lowFrequencyEnhancement	BOOLEAN,

multilingual	BOOLEAN,

bitRate	INTEGER (1..1130),	-- units kbit/s

...

}

GSMAudioCapability	::= SEQUENCE

{

audioUnitSize	INTEGER (1..256),

comfortNoise	BOOLEAN,

scrambled	BOOLEAN,

...

}

-- ===================================================================================

-- Capability exchange definitions: Data capabilities

-- ===================================================================================

DataApplicationCapability	::=SEQUENCE

{

application	CHOICE

{

nonStandard	NonStandardParameter,

t120	DataProtocolCapability,

dsm-cc	DataProtocolCapability,

userData	DataProtocolCapability,

t84	SEQUENCE

{

t84Protocol	DataProtocolCapability,

t84Profile	T84Profile

},

t434	DataProtocolCapability,

h224	DataProtocolCapability,

nlpid	SEQUENCE

{

nlpidProtocol	DataProtocolCapability,

nlpidData	OCTET STRING

},

dsvdControl	NULL,

h222DataPartitioning	DataProtocolCapability,

...,

t30fax	DataProtocolCapability,

t140	DataProtocolCapability,

t38fax	SEQUENCE

{

t38FaxProtocol	DataProtocolCapability,

t38FaxProfile	T38FaxProfile

}

},

maxBitRate	INTEGER (0..4294967295),	-- units 100 bit/s

...

}

DataProtocolCapability	::=CHOICE

{

nonStandard	NonStandardParameter,

v14buffered	NULL,

v42lapm	NULL,	-- may negotiate to V.42bis

hdlcFrameTunnelling	NULL,

h310SeparateVCStack	NULL,

h310SingleVCStack	NULL,

transparent	NULL,

...,

segmentationAndReassembly	NULL,

hdlcFrameTunnelingwSAR	NULL,

v120		NULL,	-- as in H.230

separateLANStack	NULL,

v76wCompression         	CHOICE

{

transmitCompression     	CompressionType,

receiveCompression  	CompressionType,

transmitAndReceiveCompression 	CompressionType,

...

},

tcp			NULL,

udp			NULL

}

CompressionType 	::=CHOICE

{

v42bis		V42bis,

...

}

V42bis			::=SEQUENCE

{

numberOfCodewords	INTEGER (1..65536),

maximumStringLength	INTEGER (1..256),

...

}

T84Profile		::=CHOICE

{

t84Unrestricted	NULL,

t84Restricted	SEQUENCE

{
		qcif	BOOLEAN,

cif		BOOLEAN,

ccir601Seq	BOOLEAN,

ccir601Prog	BOOLEAN,

hdtvSeq	BOOLEAN,

hdtvProg	BOOLEAN,

g3FacsMH200x100	BOOLEAN,

g3FacsMH200x200	BOOLEAN,

g4FacsMMR200x100	BOOLEAN,

g4FacsMMR200x200	BOOLEAN,

jbig200x200Seq	BOOLEAN,

jbig200x200Prog	BOOLEAN,

jbig300x300Seq	BOOLEAN,

jbig300x300Prog	BOOLEAN,

digPhotoLow	BOOLEAN,

digPhotoMedSeq	BOOLEAN,

digPhotoMedProg	BOOLEAN,

digPhotoHighSeq	BOOLEAN,

digPhotoHighProg	BOOLEAN,

...

}

}

T38FaxProfile	::=SEQUENCE

{

fillBitRemoval	BOOLEAN,

transcodingJBIG	BOOLEAN,

transcodingMMR	BOOLEAN,

...

}

-- ===================================================================================

-- Encryption Capability Definitions:

-- ===================================================================================

EncryptionAuthenticationAndIntegrity	::=SEQUENCE

{

encryptionCapability	EncryptionCapability OPTIONAL,

authenticationCapability	AuthenticationCapability OPTIONAL,

integrityCapability	IntegrityCapability OPTIONAL,

...

}

EncryptionCapability	::=SEQUENCE SIZE(1..256) OF MediaEncryptionAlgorithm

MediaEncryptionAlgorithm	 ::=CHOICE

{

nonStandard            	NonStandardParameter,

algorithm	OBJECT IDENTIFIER,   -- many defined in ISO/IEC 9979

...

}

AuthenticationCapability	::=SEQUENCE

{

nonStandard            	NonStandardParameter OPTIONAL,

...

}

IntegrityCapability	::=SEQUENCE

{

nonStandard            	NonStandardParameter OPTIONAL,

...

}

-- ================================================================================

-- Capability Exchange Definitions: UserInput

-- ================================================================================

UserInputCapability	::= CHOICE

{

nonStandard	SEQUENCE SIZE(1..16) OF NonStandardParameter,

basicString	NULL,  -- alphanumeric

iA5String	NULL,  -- alphanumeric

generalString	NULL,  -- alphanumeric

dtmf		NULL,  -- supports dtmf using signal and signalUpdate

hookflash	NULL,  -- supports hookflash using signal

...

}

-- ===================================================================================

-- Capability Exchange Definitions: Conference

-- ===================================================================================

ConferenceCapability	::=SEQUENCE

{

nonStandardData	SEQUENCE OF NonStandardParameter OPTIONAL,

chairControlCapability	BOOLEAN,

...

}

-- ===================================================================================

-- Logical channel signalling definitions

-- ===================================================================================

-- "Forward" is used to refer to transmission in the direction from the terminal making the

-- original request for a logical channel to the other terminal, and "reverse" is used to refer

-- to the opposite direction of transmission, in the case of a bi-directional channel request.

OpenLogicalChannel	::=SEQUENCE

{

forwardLogicalChannelNumber	LogicalChannelNumber,

forwardLogicalChannelParameters	SEQUENCE

{

portNumber	INTEGER (0..65535) OPTIONAL,

dataType	DataType,

multiplexParameters	CHOICE

{

h222LogicalChannelParameters	H222LogicalChannelParameters,

h223LogicalChannelParameters	H223LogicalChannelParameters,

v76LogicalChannelParameters	V76LogicalChannelParameters,

...,

h2250LogicalChannelParameters	H2250LogicalChannelParameters,

none	NULL  -- for use with Separate Stack when

-- multiplexParameters are not required

-- or appropriate

},

...,

forwardLogicalChannelDependency	LogicalChannelNumber OPTIONAL,

-- also used to refer to the primary logical channel when using video redundancy coding

replacementFor	LogicalChannelNumber OPTIONAL

},

-- Used to specify the reverse channel for bi-directional open request

reverseLogicalChannelParameters	SEQUENCE

{

dataType	DataType,

multiplexParameters	CHOICE

{

-- H.222 parameters are never present in reverse direction

h223LogicalChannelParameters	H223LogicalChannelParameters,

v76LogicalChannelParameters	V76LogicalChannelParameters,

...,

h2250LogicalChannelParameters H2250LogicalChannelParameters

} OPTIONAL,	-- Not present for H.222

...,

reverseLogicalChannelDependency	LogicalChannelNumber OPTIONAL,

-- also used to refer to the primary logical channel when using video redundancy coding

replacementFor	LogicalChannelNumber OPTIONAL

} OPTIONAL,	-- Not present for uni-directional channel request

...,

separateStack	NetworkAccessParameters OPTIONAL,

-- for Open responder to establish the stack

encryptionSync	EncryptionSync OPTIONAL	-- used only by Master

}

LogicalChannelNumber	::=INTEGER (1..65535)

NetworkAccessParameters	::=SEQUENCE

{

distribution	CHOICE

{

unicast	NULL,

multicast	NULL,	-- For Further Study in T.120

...

} OPTIONAL,

networkAddress	CHOICE

{

q2931Address	Q2931Address,

e164Address	IA5String(SIZE(1..128)) (FROM ("0123456789#*,")),

localAreaAddress	TransportAddress,

...

},

associateConference	BOOLEAN,

externalReference	OCTET STRING(SIZE(1..255)) OPTIONAL,

...,

t120SetupProcedure	CHOICE

{

originateCall	NULL,

waitForCall	NULL,

issueQuery	NULL,

...

} OPTIONAL

}

Q2931Address	::=SEQUENCE

{

address	CHOICE

{

internationalNumber	NumericString(SIZE(1..16)),

nsapAddress	OCTET STRING (SIZE(1..20)),

...

},

subaddress	OCTET STRING (SIZE(1..20)) OPTIONAL,

...

}

V75Parameters	::= SEQUENCE

{

audioHeaderPresent	BOOLEAN,

...

}

DataType		::=CHOICE

{

nonStandard	NonStandardParameter,

nullData	NULL,

videoData	VideoCapability,

audioData	AudioCapability,

data		DataApplicationCapability,

encryptionData	EncryptionMode,

...,

h235Control	NonStandardParameter,

h235Media	H235Media

}

H235Media		::=SEQUENCE

{

encryptionAuthenticationAndIntegrity	EncryptionAuthenticationAndIntegrity,

mediaType	CHOICE

{

nonStandard	NonStandardParameter,

videoData	VideoCapability,

audioData	AudioCapability,

data	DataApplicationCapability,

...

},

...

}

H222LogicalChannelParameters	::=SEQUENCE

{

resourceID	INTEGER (0..65535),

subChannelID	INTEGER (0..8191),

pcr-pid	INTEGER (0..8191) OPTIONAL,

programDescriptors	OCTET STRING OPTIONAL,

streamDescriptors	OCTET STRING OPTIONAL,

...

}

H223LogicalChannelParameters	::=SEQUENCE

{

adaptationLayerType	CHOICE

{

nonStandard	NonStandardParameter,

al1Framed	NULL,

al1NotFramed	NULL,

al2WithoutSequenceNumbers	NULL,

al2WithSequenceNumbers	NULL,

al3		SEQUENCE

{

controlFieldOctets	INTEGER (0..2),

sendBufferSize	INTEGER (0..16777215)	-- units octets

},

...,

al1M	H223AL1MParameters,

al2M	H223AL2MParameters,

al3M	H223AL3MParameters

},

segmentableFlag	BOOLEAN,

...

}

H223AL1MParameters	::=SEQUENCE

{

transferMode	CHOICE

{

framed	NULL,

unframed	NULL,

...

},

headerFEC	CHOICE

{

sebch16-7	NULL,

golay24-12	NULL,

...

},

crcLength	CHOICE

{

crc4bit	NULL,

crc12bit	NULL,

crc20bit	NULL,

crc28bit	NULL,

...

},

rcpcCodeRate	INTEGER (8..32),

arqType	CHOICE

{

noArq	NULL,

typeIArq	H223AnnexCArqParameters,

typeIIArq	H223AnnexCArqParameters,

...

},

alpduInterleaving	BOOLEAN,

alsduSplitting	BOOLEAN,

...

}

H223AL2MParameters	::=SEQUENCE

{

headerFEC	CHOICE

{

sebch16-5	NULL,

golay24-12	NULL,

...

},

alpduInterleaving	BOOLEAN,

...

}

H223AL3MParameters	::=SEQUENCE

{

headerFormat	CHOICE

{

sebch16-7	NULL,

golay24-12	NULL,

...

},

crcLength	CHOICE

{

crc4bit	NULL,

crc12bit	NULL,

crc20bit	NULL,

crc28bit	NULL,

...

},

rcpcCodeRate	INTEGER (8..32),

arqType	CHOICE

{

noArq	NULL,

typeIArq	H223AnnexCArqParameters,

typeIIArq	H223AnnexCArqParameters,

...

},

alpduInterleaving	BOOLEAN,

...

}

H223AnnexCArqParameters	::=SEQUENCE

{

numberOfRetransmissions	CHOICE

{

finite	INTEGER (0..16),

infinite	NULL,

...

},

sendBufferSize	INTEGER (0..16777215),	-- units octets

...

}

V76LogicalChannelParameters	::=SEQUENCE

{

hdlcParameters	V76HDLCParameters,

suspendResume	CHOICE

{

noSuspendResume	NULL,

suspendResumewAddress	NULL,

suspendResumewoAddress    	NULL,

...

},

uIH			BOOLEAN,

mode		CHOICE

{

eRM	SEQUENCE

{

windowSize	INTEGER (1..127) ,

recovery	CHOICE

{

rej	NULL,

sREJ	NULL,

mSREJ	NULL,

...

},

...

},

uNERM	NULL,

...

},

v75Parameters	V75Parameters,

...

}

V76HDLCParameters	::=SEQUENCE

{

crcLength	CRCLength,

n401		INTEGER (1..4095),

loopbackTestProcedure	BOOLEAN,

...

}

CRCLength	::=CHOICE

{

crc8bit		NULL,

crc16bit	NULL,

crc32bit	NULL,

...

}

H2250LogicalChannelParameters	::=SEQUENCE

{

nonStandard	SEQUENCE OF NonStandardParameter OPTIONAL,

sessionID	INTEGER(0..255),

associatedSessionID	INTEGER(1..255) OPTIONAL,

mediaChannel	TransportAddress OPTIONAL,

mediaGuaranteedDelivery	BOOLEAN OPTIONAL,

mediaControlChannel	TransportAddress OPTIONAL,  -- reverse RTCP channel

mediaControlGuaranteedDelivery	BOOLEAN OPTIONAL,

silenceSuppression	BOOLEAN OPTIONAL,

destination	TerminalLabel OPTIONAL,

dynamicRTPPayloadType	INTEGER(96..127) OPTIONAL,

mediaPacketization	CHOICE

{

h261aVideoPacketization	NULL,

...,

rtpPayloadType	RTPPayloadType

} OPTIONAL,

...,

transportCapability	TransportCapability OPTIONAL,

redundancyEncoding	RedundancyEncoding OPTIONAL,

source		TerminalLabel OPTIONAL

}

RTPPayloadType	::= SEQUENCE

{

payloadDescriptor	CHOICE

{

nonStandardIdentifier	NonStandardParameter,

rfc-number	INTEGER (1..32768, ...),

oid	OBJECT IDENTIFIER,

...

},

payloadType	INTEGER (0..127) OPTIONAL,

...

}

RedundancyEncoding	::=SEQUENCE

{

redundancyEncodingMethod	RedundancyEncodingMethod,

secondaryEncoding	DataType OPTIONAL,	-- depends on method

...

}

TransportAddress	::=CHOICE

{

unicastAddress	UnicastAddress,

multicastAddress	MulticastAddress,

...

}

UnicastAddress	::=CHOICE

{

iPAddress	SEQUENCE

{

network	OCTET STRING (SIZE(4)),

tsapIdentifier	INTEGER(0..65535),

...

},

iPXAddress	SEQUENCE

{

node	OCTET STRING (SIZE(6)),

netnum	OCTET STRING (SIZE(4)),

tsapIdentifier	OCTET STRING (SIZE(2)),

...

},

iP6Address	SEQUENCE

{

network	OCTET STRING (SIZE(16)),

tsapIdentifier	INTEGER(0..65535),

...

},

netBios	OCTET STRING (SIZE(16)),

iPSourceRouteAddress	SEQUENCE

{

routing	CHOICE

{

strict	NULL,

loose	NULL

},

network	OCTET STRING (SIZE(4)),

tsapIdentifier	INTEGER(0..65535),

route	SEQUENCE OF OCTET STRING (SIZE(4)),

...

},

...,

nsap		OCTET STRING (SIZE(1..20)),

nonStandardAddress	NonStandardParameter

}

MulticastAddress	::=CHOICE

{

iPAddress	SEQUENCE

{

network	OCTET STRING (SIZE(4)),

tsapIdentifier	INTEGER(0..65535),

...

},

iP6Address	SEQUENCE

{

network	OCTET STRING (SIZE(16)),

tsapIdentifier	INTEGER(0..65535),

...

},

...,

nsap		OCTET STRING (SIZE(1..20)),

nonStandardAddress	NonStandardParameter

}

EncryptionSync	::=SEQUENCE

-- used to supply new key and synchronization point

{

nonStandard            	NonStandardParameter OPTIONAL,

synchFlag	INTEGER(0..255) , 	-- may need to be larger for H.324, etc

-- shall be the Dynamic Payload# for H.323

h235Key	OCTET STRING (SIZE(1..65535)),  -- H.235 encoded value

escrowentry	SEQUENCE SIZE(1..256) OF EscrowData OPTIONAL,

...

}

EscrowData	::=SEQUENCE

{

escrowID	OBJECT IDENTIFIER,

escrowValue	BIT STRING (SIZE(1..65535)),

...

}

OpenLogicalChannelAck	::=SEQUENCE

{

forwardLogicalChannelNumber	LogicalChannelNumber,

reverseLogicalChannelParameters	SEQUENCE

{

reverseLogicalChannelNumber	LogicalChannelNumber,

portNumber	INTEGER (0..65535) OPTIONAL,

multiplexParameters	CHOICE

{

h222LogicalChannelParameters	H222LogicalChannelParameters,

-- H.223 parameters are never present in reverse direction

...,

h2250LogicalChannelParameters H2250LogicalChannelParameters

} OPTIONAL,	-- Not present for H.223

...,

replacementFor	LogicalChannelNumber OPTIONAL

} OPTIONAL,	-- Not present for uni-directional channel request

...,

separateStack	NetworkAccessParameters OPTIONAL,

-- for Open requester to establish the stack

forwardMultiplexAckParameters	CHOICE

{

-- H.222 parameters are never present in the Ack

-- H.223 parameters are never present in the Ack

--V.76 parameters are never present in the Ack

h2250LogicalChannelAckParameters H2250LogicalChannelAckParameters,

...

} OPTIONAL,

encryptionSync	EncryptionSync OPTIONAL	-- used only by Master

}

OpenLogicalChannelReject	::=SEQUENCE

{

forwardLogicalChannelNumber	LogicalChannelNumber,

cause		CHOICE

{

unspecified	NULL,

unsuitableReverseParameters	NULL,

dataTypeNotSupported	NULL,

dataTypeNotAvailable	NULL,

unknownDataType	NULL,

dataTypeALCombinationNotSupported	NULL,

...,

multicastChannelNotAllowed	NULL,

insufficientBandwidth	NULL,

separateStackEstablishmentFailed	NULL,

invalidSessionID	NULL,

masterSlaveConflict	NULL,

waitForCommunicationMode	NULL,

invalidDependentChannel	NULL,

replacementForRejected	NULL

},

...

}

OpenLogicalChannelConfirm	::=SEQUENCE

{

forwardLogicalChannelNumber	LogicalChannelNumber,

...

}

H2250LogicalChannelAckParameters	::=SEQUENCE

{

nonStandard	SEQUENCE OF NonStandardParameter OPTIONAL,

sessionID	INTEGER(1..255) OPTIONAL,

mediaChannel	TransportAddress OPTIONAL,

mediaControlChannel	TransportAddress OPTIONAL, -- forward RTCP channel

dynamicRTPPayloadType	INTEGER(96..127) OPTIONAL, -- used only by the master or MC

...,

flowControlToZero	BOOLEAN,

portNumber	INTEGER (0..65535) OPTIONAL

}

CloseLogicalChannel	::=SEQUENCE

{

forwardLogicalChannelNumber	LogicalChannelNumber,

source		CHOICE

{

user	NULL,

lcse	NULL

},

...,

reason		CHOICE

{

unknown	NULL,

reopen	NULL,

reservationFailure	NULL,

...

}

}

CloseLogicalChannelAck	::=SEQUENCE

{

forwardLogicalChannelNumber	LogicalChannelNumber,

...

}

RequestChannelClose	::=SEQUENCE

{

forwardLogicalChannelNumber	LogicalChannelNumber,

...,

qosCapability	QOSCapability OPTIONAL,

reason		CHOICE

{

unknown	NULL,

normal	NULL,

reopen	NULL,

reservationFailure	NULL,

...

}

}

RequestChannelCloseAck	::=SEQUENCE

{

forwardLogicalChannelNumber	LogicalChannelNumber,

...

}

RequestChannelCloseReject	::=SEQUENCE

{

forwardLogicalChannelNumber	LogicalChannelNumber,

cause		CHOICE

{

unspecified	NULL,

...

},

...

}

RequestChannelCloseRelease	::=SEQUENCE

{

forwardLogicalChannelNumber	LogicalChannelNumber,

...

}

-- ===================================================================================

-- H.223 multiplex table definitions

-- ===================================================================================

MultiplexEntrySend	::=SEQUENCE

{

sequenceNumber	SequenceNumber,

multiplexEntryDescriptors	SET SIZE (1..15) OF MultiplexEntryDescriptor,

...

}

MultiplexEntryDescriptor	::=SEQUENCE

{

multiplexTableEntryNumber	MultiplexTableEntryNumber,

elementList	SEQUENCE SIZE (1..256) OF MultiplexElement OPTIONAL

}

MultiplexElement	::=SEQUENCE

{

type		CHOICE

{

logicalChannelNumber	INTEGER(0..65535),

subElementList	SEQUENCE SIZE (2..255) OF MultiplexElement

},

repeatCount	CHOICE

{

finite	INTEGER (1..65535),	-- repeats of type

untilClosingFlag	NULL	-- used for last element

}

}

MultiplexTableEntryNumber	::=INTEGER (1..15)

MultiplexEntrySendAck	::=SEQUENCE

{

sequenceNumber	SequenceNumber,

multiplexTableEntryNumber	SET SIZE (1..15) OF MultiplexTableEntryNumber,

...

}

MultiplexEntrySendReject	::=SEQUENCE

{

sequenceNumber	SequenceNumber,

rejectionDescriptions	SET SIZE (1..15) OF MultiplexEntryRejectionDescriptions,

...

}

MultiplexEntryRejectionDescriptions	::=SEQUENCE

{

multiplexTableEntryNumber	MultiplexTableEntryNumber,

cause		CHOICE

{

unspecifiedCause	NULL,

descriptorTooComplex	NULL,

...

},

...

}

MultiplexEntrySendRelease	::=SEQUENCE

{

multiplexTableEntryNumber	SET SIZE (1..15) OF MultiplexTableEntryNumber,

...

}

RequestMultiplexEntry	::=SEQUENCE

{

entryNumbers	SET SIZE (1..15) OF MultiplexTableEntryNumber,

...

}

RequestMultiplexEntryAck	::=SEQUENCE

{

entryNumbers	SET SIZE (1..15) OF MultiplexTableEntryNumber,

...

}

RequestMultiplexEntryReject	::=SEQUENCE

{

entryNumbers	SET SIZE (1..15) OF MultiplexTableEntryNumber,

rejectionDescriptions	SET SIZE (1..15) OF RequestMultiplexEntryRejectionDescriptions,

...

}

RequestMultiplexEntryRejectionDescriptions	::=SEQUENCE

{

multiplexTableEntryNumber	MultiplexTableEntryNumber,

cause		CHOICE

{

unspecifiedCause	NULL,

...

},

...

}

RequestMultiplexEntryRelease	::=SEQUENCE

{

entryNumbers	SET SIZE (1..15) OF MultiplexTableEntryNumber,

...

}

-- ===================================================================================

-- Request mode definitions

-- ===================================================================================

-- RequestMode is a list, in order or preference, of modes that a terminal would like

-- to have transmitted to it.

RequestMode	::=SEQUENCE

{

sequenceNumber	SequenceNumber,

requestedModes	SEQUENCE SIZE (1..256) OF ModeDescription,

...

}

RequestModeAck	::=SEQUENCE

{

sequenceNumber	SequenceNumber,

response	CHOICE

{

willTransmitMostPreferredMode	NULL,

willTransmitLessPreferredMode	NULL,

...

},

...

}

RequestModeReject	::=SEQUENCE

{

sequenceNumber	SequenceNumber,

cause		CHOICE

{

modeUnavailable	NULL,

multipointConstraint	NULL,

requestDenied	NULL,

...

},

...

}

RequestModeRelease	::=SEQUENCE

{

...

}

-- ===================================================================================

-- Request mode definitions: Mode description

-- ===================================================================================

ModeDescription	::=SET SIZE (1..256) OF ModeElement

ModeElement	::= SEQUENCE

{

type		CHOICE

{

nonStandard	NonStandardParameter,

videoMode	VideoMode,

audioMode	AudioMode,

dataMode	DataMode,

encryptionMode	EncryptionMode,

...,

h235Mode	H235Mode

},

h223ModeParameters	H223ModeParameters OPTIONAL,

...,

v76ModeParameters	V76ModeParameters OPTIONAL,

h2250ModeParameters	H2250ModeParameters OPTIONAL

}

H235Mode		::=SEQUENCE

{

encryptionAuthenticationAndIntegrity	EncryptionAuthenticationAndIntegrity,

mediaMode	CHOICE

{

nonStandard	NonStandardParameter,

videoMode	VideoMode,

audioMode	AudioMode,

dataMode	DataMode,

...

},

...

}

H223ModeParameters	::=SEQUENCE

{

adaptationLayerType	CHOICE

{

nonStandard	NonStandardParameter,

al1Framed	NULL,

al1NotFramed	NULL,

al2WithoutSequenceNumbers	NULL,

al2WithSequenceNumbers	NULL,

al3		SEQUENCE

{

controlFieldOctets	INTEGER(0..2),

sendBufferSize	INTEGER(0..16777215)	-- units octets

},

...,

al1M	H223AL1MParameters,

al2M	H223AL2MParameters,

al3M	H223AL3MParameters

},

segmentableFlag	BOOLEAN,

...

}

V76ModeParameters	::=CHOICE

{

suspendResumewAddress	NULL,

suspendResumewoAddress	NULL,

...

}

H2250ModeParameters	::=SEQUENCE

{

redundancyEncodingMode	RedundancyEncodingMode OPTIONAL,

...

}

RedundancyEncodingMode	::=SEQUENCE

{

redundancyEncodingMethod	RedundancyEncodingMethod,

secondaryEncoding	CHOICE

{

nonStandard	NonStandardParameter,

audioData	AudioMode,

...

} OPTIONAL,

...

}

-- ===================================================================================

-- Request mode definitions: Video modes

-- ===================================================================================

VideoMode		::=CHOICE

{

nonStandard	NonStandardParameter,

h261VideoMode	H261VideoMode,

h262VideoMode	H262VideoMode,

h263VideoMode	H263VideoMode,

is11172VideoMode	IS11172VideoMode,

...

}

H261VideoMode	::=SEQUENCE

{

resolution	CHOICE

{

qcif	NULL,

cif		NULL

},

bitRate	INTEGER (1..19200),	-- units 100 bit/s

stillImageTransmission	BOOLEAN,

...

}

H262VideoMode	::=SEQUENCE

{

profileAndLevel	CHOICE

{

profileAndLevel-SPatML	NULL,

profileAndLevel-MPatLL	NULL,

profileAndLevel-MPatML	NULL,

profileAndLevel-MPatH-14	NULL,

profileAndLevel-MPatHL	NULL,

profileAndLevel-SNRatLL	NULL,

profileAndLevel-SNRatML	NULL,

profileAndLevel-SpatialatH-14	NULL,

profileAndLevel-HPatML	NULL,

profileAndLevel-HPatH-14	NULL,

profileAndLevel-HPatHL	NULL,

...

},

videoBitRate	INTEGER(0..1073741823) OPTIONAL,	-- units 400bit/s

vbvBufferSize	INTEGER(0..262143) OPTIONAL,	-- units 16384bits

samplesPerLine	INTEGER(0..16383) OPTIONAL,	-- units samples/line

linesPerFrame	INTEGER(0..16383) OPTIONAL,	-- units lines/frame

framesPerSecond	INTEGER(0..15) OPTIONAL,	-- frame_rate_code

luminanceSampleRate	INTEGER(0..4294967295) OPTIONAL,	-- units samples/sec

...

}

H263VideoMode	::=SEQUENCE

{

resolution	CHOICE

{

sqcif	NULL,

qcif	NULL,

cif		NULL,

cif4	NULL,

cif16	NULL,

...

},

bitRate	INTEGER (1..19200),	-- units 100 bit/s

unrestrictedVector	BOOLEAN,

arithmeticCoding	BOOLEAN,

advancedPrediction	BOOLEAN,

pbFrames	BOOLEAN,

...,

errorCompensation	BOOLEAN,

enhancementLayerInfo	EnhancementLayerInfo OPTIONAL,

h263Options	H263Options OPTIONAL

}

IS11172VideoMode	::=SEQUENCE

{

constrainedBitstream	BOOLEAN,

videoBitRate	INTEGER(0..1073741823) OPTIONAL,	-- units 400bit/s

vbvBufferSize	INTEGER(0..262143) OPTIONAL,	-- units 16384bits

samplesPerLine	INTEGER(0..16383) OPTIONAL,	-- units samples/line

linesPerFrame	INTEGER(0..16383) OPTIONAL,	-- units lines/frame

pictureRate	INTEGER(0..15) OPTIONAL,

luminanceSampleRate	INTEGER(0..4294967295) OPTIONAL,	-- units samples/sec

...

}

-- ===================================================================================

-- Request mode definitions: Audio modes

-- ===================================================================================

AudioMode	::=CHOICE

{

nonStandard	NonStandardParameter,

g711Alaw64k	NULL,

g711Alaw56k	NULL,

g711Ulaw64k	NULL,

g711Ulaw56k	NULL,

g722-64k	NULL,

g722-56k	NULL,

g722-48k	NULL,

g728		NULL,

g729		NULL,

g729AnnexA	NULL,

g7231		CHOICE

{

noSilenceSuppressionLowRate	NULL,

noSilenceSuppressionHighRate	NULL,

silenceSuppressionLowRate	NULL,

silenceSuppressionHighRate	NULL

},

is11172AudioMode	IS11172AudioMode,

is13818AudioMode	IS13818AudioMode,

...,

g729wAnnexB	INTEGER(1..256),

g729AnnexAwAnnexB	INTEGER(1..256),

g7231AnnexCMode	G7231AnnexCMode,

gsmFullRate	GSMAudioCapability,

gsmHalfRate	GSMAudioCapability,

gsmEnhancedFullRate	GSMAudioCapability

}

IS11172AudioMode	::=SEQUENCE

{

audioLayer	CHOICE

{

audioLayer1	NULL,

audioLayer2	NULL,

audioLayer3	NULL

},

audioSampling	CHOICE

{

audioSampling32k	NULL,

audioSampling44k1	NULL,

audioSampling48k	NULL

},

multichannelType	CHOICE

{

singleChannel	NULL,

twoChannelStereo	NULL,

twoChannelDual	NULL

},

bitRate	INTEGER (1..448),	--units kbit/s

...

}

IS13818AudioMode	::=SEQUENCE

{

audioLayer	CHOICE

{

audioLayer1	NULL,

audioLayer2	NULL,

audioLayer3	NULL

},

audioSampling	CHOICE

{

audioSampling16k	NULL,

audioSampling22k05	NULL,

audioSampling24k	NULL,

audioSampling32k	NULL,

audioSampling44k1	NULL,

audioSampling48k	NULL

},

multichannelType	CHOICE

{

singleChannel	NULL,

twoChannelStereo	NULL,

twoChannelDual	NULL,

threeChannels2-1	NULL,

threeChannels3-0	NULL,

fourChannels2-0-2-0	NULL,

fourChannels2-2	NULL,

fourChannels3-1	NULL,

fiveChannels3-0-2-0	NULL,

fiveChannels3-2	NULL

},

lowFrequencyEnhancement	BOOLEAN,

multilingual	BOOLEAN,

bitRate	INTEGER (1..1130),	--units kbit/s

...

}

G7231AnnexCMode	::= SEQUENCE

{

maxAl-sduAudioFrames	INTEGER (1..256),

silenceSuppression	BOOLEAN,

g723AnnexCAudioMode	SEQUENCE

{

highRateMode0	INTEGER (27..78),	-- units octets

highRateMode1	INTEGER (27..78),	-- units octets

lowRateMode0	INTEGER (23..66),	-- units octets

lowRateMode1	INTEGER (23..66),	-- units octets

sidMode0	INTEGER (6..17),	-- units octets

sidMode1	INTEGER (6..17),	-- units octets

...

},

...

}

-- ===================================================================================

-- Request mode definitions: Data modes

-- ===================================================================================

DataMode		::=SEQUENCE

{

application	CHOICE

{

nonStandard	NonStandardParameter,

t120	DataProtocolCapability,

dsm-cc	DataProtocolCapability,

userData	DataProtocolCapability,

t84	DataProtocolCapability,

t434	DataProtocolCapability,

h224	DataProtocolCapability,

nlpid	SEQUENCE

{

nlpidProtocol	DataProtocolCapability,

nlpidData	OCTET STRING

},

dsvdControl	NULL,

h222DataPartitioning	DataProtocolCapability,

...,

t30fax	DataProtocolCapability,

t140	DataProtocolCapability,

t38fax	SEQUENCE

{

t38FaxProtocol	DataProtocolCapability,

t38FaxProfile	T38FaxProfile

}

},

bitRate	INTEGER (0..4294967295),	-- units 100 bit/s

...

}

-- ===================================================================================

-- Request mode definitions: Encryption modes

-- ===================================================================================

EncryptionMode	::=CHOICE

{

nonStandard	NonStandardParameter,

h233Encryption	NULL,

...

}

-- ===================================================================================

-- Round Trip Delay definitions

-- ===================================================================================

RoundTripDelayRequest	::=SEQUENCE

{

sequenceNumber	SequenceNumber,

...

}

RoundTripDelayResponse	::=SEQUENCE

{

sequenceNumber	SequenceNumber,

...

}

-- ===================================================================================

-- Maintenance Loop definitions

-- ===================================================================================

MaintenanceLoopRequest	::=SEQUENCE

{

type		CHOICE

{

systemLoop	NULL,

mediaLoop	LogicalChannelNumber,

logicalChannelLoop	LogicalChannelNumber,

...

},

...

}

MaintenanceLoopAck	::=SEQUENCE

{

type		CHOICE

{

systemLoop	NULL,

mediaLoop	LogicalChannelNumber,

logicalChannelLoop	LogicalChannelNumber,

...

},

...

}

MaintenanceLoopReject	::=SEQUENCE

{

type		CHOICE

{

systemLoop	NULL,

mediaLoop	LogicalChannelNumber,

logicalChannelLoop	LogicalChannelNumber,

...

},

cause		CHOICE

{

canNotPerformLoop	NULL,

...

},

...

}

MaintenanceLoopOffCommand	::=SEQUENCE

{

...

}

-- ===================================================================================

-- Communication Mode definitions

-- ===================================================================================

CommunicationModeCommand	::=SEQUENCE

{

communicationModeTable	SET SIZE(1..256) OF CommunicationModeTableEntry,

...

}

CommunicationModeRequest	::=SEQUENCE

{

...

}

CommunicationModeResponse	::=CHOICE

{

communicationModeTable	SET SIZE(1..256) OF CommunicationModeTableEntry,

...

}

CommunicationModeTableEntry	::=SEQUENCE

{

nonStandard	SEQUENCE OF NonStandardParameter OPTIONAL,

sessionID	INTEGER(1..255),

associatedSessionID	INTEGER(1..255) OPTIONAL,

terminalLabel	TerminalLabel OPTIONAL, -- if not present,

-- it refers to all participants

-- in the conference

sessionDescription	BMPString (SIZE(1..128)) ,

-- Basic ISO/IEC 10646-1 (Unicode)

dataType	CHOICE

{

videoData	VideoCapability,

audioData	AudioCapability,

data	DataApplicationCapability,

...

},

mediaChannel	TransportAddress OPTIONAL,

mediaGuaranteedDelivery	BOOLEAN OPTIONAL,

mediaControlChannel	TransportAddress OPTIONAL,  -- reverse RTCP channel

mediaControlGuaranteedDelivery	BOOLEAN OPTIONAL,

...,

redundancyEncoding	RedundancyEncoding OPTIONAL,

sessionDependency	INTEGER (1..255) OPTIONAL,

destination	TerminalLabel OPTIONAL

}

-- ===================================================================================

-- Conference Request definitions

-- ===================================================================================

ConferenceRequest	::=CHOICE

{

terminalListRequest	NULL,                    -- same as H.230 TCU (term->MC)

makeMeChair	NULL,                    -- same as H.230 CCA (term->MC)

cancelMakeMeChair	NULL,                    -- same as H.230 CIS (term->MC)

dropTerminal	TerminalLabel,       -- same as H.230 CCD(term->MC)

requestTerminalID	TerminalLabel,       -- sames as TCP (term->MC)

enterH243Password	NULL,                    -- same as H.230 TCS1(MC->term)

enterH243TerminalID	NULL,                    -- same as H.230 TCS2/TCI

--  (MC->term)

enterH243ConferenceID	NULL,                    -- same as H.230 TCS3 (MC->term)

...,

enterExtensionAddress	NULL,                     -- same as H.230 TCS4 (GW->term)

requestChairTokenOwner	NULL,                    -- same as H.230 TCA (term->MC)

requestTerminalCertificate 	SEQUENCE

{

terminalLabel	TerminalLabel OPTIONAL,

certSelectionCriteria	CertSelectionCriteria OPTIONAL,

sRandom	INTEGER (1..4294967295) OPTIONAL,

-- this is the requester's challenge

...

},

broadcastMyLogicalChannel	LogicalChannelNumber,	-- similar to H.230 MCV

makeTerminalBroadcaster	TerminalLabel,	-- similar to H.230 VCB

sendThisSource	TerminalLabel,	-- similar to H.230 VCS

requestAllTerminalIDs	NULL,

remoteMCRequest	RemoteMCRequest

}

CertSelectionCriteria	::=SEQUENCE SIZE (1..16) OF Criteria

Criteria			::=SEQUENCE

{

field		OBJECT IDENTIFIER, -- may include certificate type

value		OCTET STRING (SIZE(1..65535)),

...

}

TerminalLabel	::=SEQUENCE

{

mcuNumber	McuNumber,

terminalNumber	TerminalNumber,

...

}

McuNumber	::=INTEGER(0..192)

TerminalNumber	::=INTEGER(0..192)

-- ===================================================================================

-- Conference Response definitions

-- ===================================================================================

ConferenceResponse	::=CHOICE

{

mCTerminalIDResponse	SEQUENCE             -- response to TCP(same as TIP)

{				-- sent by MC only

terminalLabel	TerminalLabel,

terminalID	TerminalID,

...

},

terminalIDResponse	SEQUENCE	-- response to TCS2 or TCI

{				-- same as IIS

terminalLabel	TerminalLabel,	-- (term->MC)

terminalID	TerminalID,

...

},

conferenceIDResponse	SEQUENCE	-- response to TCS3

{				-- same as IIS

terminalLabel	TerminalLabel,	-- (term->MC)

conferenceID	ConferenceID,

...

},

passwordResponse	SEQUENCE	-- response to TCS1

{				-- same as IIS

terminalLabel	TerminalLabel,	-- (term->MC)

password	Password,

...

},

terminalListResponse	SET SIZE (1..256) OF TerminalLabel,

videoCommandReject	NULL,	-- same as H.230 VCR

terminalDropReject	NULL,	-- same as H.230 CIR

makeMeChairResponse	CHOICE	-- same as H.230 CCR

{

grantedChairToken	NULL,	-- same as H.230 CIT

deniedChairToken	NULL,	-- same as H.230 CCR

...

},

...,

extensionAddressResponse	SEQUENCE	-- response to TCS4

{

extensionAddress	TerminalID,	-- same as IIS (term->GW)

...

},

chairTokenOwnerResponse	SEQUENCE    -- response to TCA(same as TIR) sent by MC only

{

terminalLabel	TerminalLabel,

terminalID	TerminalID,

...

},

terminalCertificateResponse  	SEQUENCE

{

terminalLabel           	TerminalLabel OPTIONAL,

certificateResponse           	OCTET STRING (SIZE(1..65535)) OPTIONAL,

...

},

broadcastMyLogicalChannelResponse	CHOICE

{

grantedBroadcastMyLogicalChannel	NULL,

deniedBroadcastMyLogicalChannel	NULL,

...

},

makeTerminalBroadcasterResponse	CHOICE

{

grantedMakeTerminalBroadcaster	NULL,

deniedMakeTerminalBroadcaster	NULL,

...

},

sendThisSourceResponse	CHOICE

{

grantedSendThisSource	NULL,

deniedSendThisSource	NULL,

...

},

requestAllTerminalIDsResponse	RequestAllTerminalIDsResponse,

remoteMCResponse	RemoteMCResponse

}

TerminalID 	::=OCTET STRING (SIZE(1..128))	-- as per H.230

ConferenceID	::=OCTET STRING (SIZE(1..32))

Password		::=OCTET STRING (SIZE(1..32))

RequestAllTerminalIDsResponse 	::=SEQUENCE

{

terminalInformation	SEQUENCE OF TerminalInformation,

...

}

TerminalInformation	::=SEQUENCE

{

terminalLabel	TerminalLabel,

terminalID	TerminalID,

...

}

-- ===================================================================================

-- Remote MC Request definitions

-- ===================================================================================

RemoteMCRequest	::=CHOICE

{

masterActivate	NULL,

slaveActivate	NULL,

deActivate	NULL,

...

}

RemoteMCResponse	::=CHOICE

{

accept	NULL,

reject	CHOICE

{

unspecified	NULL,

functionNotSupported	NULL,

...

},

...

}

-- ===================================================================================

-- Multilink definitions

-- ===================================================================================

MultilinkRequest	::=CHOICE

{

nonStandard	NonStandardMessage,

callInformation	SEQUENCE

{

maxNumberOfAdditionalConnections	INTEGER (1..65535),

...

},

addConnection 	SEQUENCE

{

sequenceNumber	SequenceNumber,  -- Unique ID of request

dialingInformation	DialingInformation,

...

},

removeConnection	SEQUENCE

{

connectionIdentifier	ConnectionIdentifier,

...

},

maximumHeaderInterval	SEQUENCE

{

requestType	CHOICE

{

currentIntervalInformation	NULL,

requestedInterval	INTEGER (0..65535), -- Max Header Interval, milliseconds

...

},

...

},

...

}

MultilinkResponse	::=CHOICE

{

nonStandard	NonStandardMessage,

callInformation	SEQUENCE

{

dialingInformation	DialingInformation,

callAssociationNumber	INTEGER (0..4294967295),

...

},

addConnection	SEQUENCE

{

sequenceNumber	SequenceNumber, -- Equal to value in request

responseCode 	CHOICE

{

accepted	NULL,

rejected	CHOICE

{

connectionsNotAvailable	NULL, -- due to any technical reason

userRejected	NULL,

...

},

...

},

...

},

removeConnection	SEQUENCE

{

connectionIdentifier	ConnectionIdentifier,

...

},

maximumHeaderInterval	SEQUENCE

{

currentInterval	INTEGER (0..65535), -- Max Header Interval, milliseconds

...

},

...

}

MultilinkIndication	::=CHOICE

{

nonStandard	NonStandardMessage,

crcDesired	SEQUENCE

{

...

},

excessiveError	SEQUENCE

{

connectionIdentifier	ConnectionIdentifier,

...

},

...

}

DialingInformation 	::= CHOICE

{

nonStandard	NonStandardMessage,

differential	SET SIZE (1..65535) OF DialingInformationNumber,

-- list of numbers for all additional channels; only least

-- significant digits different from initial channel’s number

infoNotAvailable	INTEGER (1..65535), -- maximum no. of additional channels

...

}

DialingInformationNumber 	::= SEQUENCE
{

networkAddress	NumericString (SIZE (0..40)),

subAddress	IA5String (SIZE (1..40)) OPTIONAL,

networkType	SET SIZE (1..255) OF DialingInformationNetworkType,

...

}

DialingInformationNetworkType	::= CHOICE

{

nonStandard	NonStandardMessage,

n-isdn		NULL,

gstn		NULL,

...

}

ConnectionIdentifier 	::= SEQUENCE
{

channelTag	INTEGER (0..4294967295),	-- from H.MULTILINK

sequenceNumber	INTEGER (0..4294967295),	-- from H.MULTILINK

...

}

-- ===================================================================================

-- Command Message definitions

-- ===================================================================================

-- ===================================================================================

-- Command Message : Send Terminal Capability Set

-- ===================================================================================

SendTerminalCapabilitySet	::=CHOICE

{

specificRequest	SEQUENCE

{

multiplexCapability	BOOLEAN,

capabilityTableEntryNumbers	SET SIZE (1..65535) OF CapabilityTableEntryNumber OPTIONAL,

capabilityDescriptorNumbers	SET SIZE (1..256) OF CapabilityDescriptorNumber OPTIONAL,

...

},

genericRequest	NULL,

...

}

-- ===================================================================================

-- Command Message : Encryption

-- ===================================================================================

EncryptionCommand	::=CHOICE

{

encryptionSE	OCTET STRING,	-- per H.233, but no error protection

encryptionIVRequest	NULL,	-- requests new IV

encryptionAlgorithmID	SEQUENCE

{

h233AlgorithmIdentifier	SequenceNumber,

associatedAlgorithm	NonStandardParameter

},

...

}

-- ===================================================================================

-- Command Message : Flow Control

-- ===================================================================================

FlowControlCommand	::=SEQUENCE

{

scope		CHOICE

{

logicalChannelNumber	LogicalChannelNumber,

resourceID	INTEGER (0..65535),

wholeMultiplex	NULL

},

restriction	CHOICE

{

maximumBitRate	INTEGER (0..16777215),	-- units 100 bit/s

noRestriction	NULL

},

...

}

-- ===================================================================================

-- Command Message : Change or End Session

-- ===================================================================================

EndSessionCommand	::=CHOICE

{

nonStandard	NonStandardParameter,

disconnect	NULL,

gstnOptions	CHOICE

{

telephonyMode	NULL,

v8bis	NULL,

v34DSVD	NULL,

v34DuplexFAX	NULL,

v34H324	NULL,

...

},

...,

isdnOptions	CHOICE

{

telephonyMode	NULL,

v140	NULL,

terminalOnHold	NULL,

...

}

}

-- ===================================================================================

-- Command Message : Conference Commands

-- ===================================================================================

ConferenceCommand	::=CHOICE

{

broadcastMyLogicalChannel	LogicalChannelNumber,	-- similar to H.230 MCV

cancelBroadcastMyLogicalChannel	LogicalChannelNumber, -- similar to H.230 Cancel-MCV

makeTerminalBroadcaster	TerminalLabel,	-- same as H.230 VCB

cancelMakeTerminalBroadcaster	NULL,	-- same as H.230 Cancel-VCB

sendThisSource	TerminalLabel,	-- same as H.230 VCS

cancelSendThisSource	NULL,	-- same as H.230 cancel VCS

dropConference	NULL,	-- same as H.230 CCK

...,

substituteConferenceIDCommand	SubstituteConferenceIDCommand

}

SubstituteConferenceIDCommand	::=SEQUENCE

{

conferenceIdentifier	OCTET STRING (SIZE(16)),

...

}

-- ===================================================================================

-- Command Message : Miscellaneous H.230-like commands

-- ===================================================================================

MiscellaneousCommand	::=SEQUENCE

{

logicalChannelNumber	LogicalChannelNumber,

type		CHOICE

{

equaliseDelay	NULL,	-- same as H.230 ACE

zeroDelay	NULL,	-- same as H.230 ACZ

multipointModeCommand	NULL,

cancelMultipointModeCommand	NULL,

videoFreezePicture	NULL,

videoFastUpdatePicture	NULL,

videoFastUpdateGOB	SEQUENCE

{

firstGOB	INTEGER (0..17),

numberOfGOBs	INTEGER (1..18)

},

videoTemporalSpatialTradeOff	INTEGER (0..31),	-- commands a trade-off value

videoSendSyncEveryGOB	NULL,

videoSendSyncEveryGOBCancel	NULL,

...,

videoFastUpdateMB	SEQUENCE

{

firstGOB	INTEGER (0..255) OPTIONAL,

firstMB	INTEGER (1..8192) OPTIONAL,

numberOfMBs	INTEGER (1..8192),

...

},

maxH223MUXPDUsize	INTEGER(1..65535),	-- units octets

encryptionUpdate	EncryptionSync,

encryptionUpdateRequest	EncryptionUpdateRequest,

switchReceiveMediaOff	NULL,

switchReceiveMediaOn	NULL,

progressiveRefinementStart	SEQUENCE

{

repeatCount	CHOICE

{

doOneProgression		NULL,

doContinuousProgressions		NULL,

doOneIndependentProgression	NULL,

doContinuousIndependentProgressions	NULL,

...

},

...

},

progressiveRefinementAbortOne		NULL,

progressiveRefinementAbortContinuous		NULL

},

...

}

KeyProtectionMethod	::=SEQUENCE -- specify how the new key is to be protected

{

secureChannel	BOOLEAN,

sharedSecret	BOOLEAN,

certProtectedKey	BOOLEAN,

...

}

EncryptionUpdateRequest	::=SEQUENCE

{

keyProtectionMethod	KeyProtectionMethod OPTIONAL,

...

}

-- ===================================================================================

-- Command Message : H.223 Multiplex Reconfiguration

-- ===================================================================================

H223MultiplexReconfiguration	::=CHOICE

{

h223ModeChange	CHOICE

{

toLevel0	NULL,

toLevel1	NULL,

toLevel2	NULL,

toLevel2withOptionalHeader	NULL,

...

},

h223AnnexADoubleFlag	CHOICE

{

start	NULL,

stop	NULL,

...

},

...

}

-- ===================================================================================

-- Command Message : New ATM virtual channel command

-- ===================================================================================

NewATMVCCommand	::=SEQUENCE

{

resourceID	INTEGER(0..65535),

bitRate		INTEGER(1..65535),	-- units 64 kbit/s

bitRateLockedToPCRClock	BOOLEAN,

bitRateLockedToNetworkClock	BOOLEAN,

aal			CHOICE

{

aal1	SEQUENCE

{

clockRecovery	CHOICE

{

nullClockRecovery	NULL,

srtsClockRecovery	NULL,

adaptiveClockRecovery	NULL,

...

},

errorCorrection	CHOICE

{

nullErrorCorrection	NULL,

longInterleaver	NULL,

shortInterleaver	NULL,

errorCorrectionOnly	NULL,

...

},

structuredDataTransfer	BOOLEAN,

partiallyFilledCells	BOOLEAN,

...

},

aal5	SEQUENCE

{

forwardMaximumSDUSize	INTEGER (0..65535),	-- units octets

backwardMaximumSDUSize	INTEGER (0..65535),	-- units octets

...

},

...

},

multiplex	CHOICE

{

noMultiplex	NULL,

transportStream	NULL,

programStream	NULL,

...

},

reverseParameters	SEQUENCE

{

bitRate	INTEGER(1..65535),	-- units 64 kbit/s

bitRateLockedToPCRClock	BOOLEAN,

bitRateLockedToNetworkClock	BOOLEAN,

multiplex	CHOICE

{

noMultiplex	NULL,

transportStream	NULL,

programStream	NULL,

...

},

...

},

...

}

-- ===================================================================================

-- Indication Message definitions

-- ===================================================================================

-- ===================================================================================

-- Indication Message : Function not understood

-- ===================================================================================

-- This is used to return a request, response or command that is not understood

FunctionNotUnderstood	::=CHOICE

{

request	RequestMessage,

response	ResponseMessage,

command	CommandMessage

}

-- ===================================================================================

-- Indication Message : Function not Supported

-- ===================================================================================

-- This is used to return a complete request, response or command that is not recognized

FunctionNotSupported	::=SEQUENCE

{

cause		CHOICE

{

syntaxError	NULL,

semanticError	NULL,

unknownFunction	NULL,

...

},

returnedFunction	OCTET STRING OPTIONAL,

...

}

-- ===================================================================================

-- Indication Message : Conference

-- ===================================================================================

ConferenceIndication	::=CHOICE

{

sbeNumber		INTEGER (0..9),	-- same as H.230 SBE Number

terminalNumberAssign		TerminalLabel,	-- same as H.230 TIA

terminalJoinedConference		TerminalLabel,	-- same as H.230 TIN

terminalLeftConference		TerminalLabel,	-- same as H.230 TID

seenByAtLeastOneOther		NULL,	-- same as H.230 MIV

cancelSeenByAtLeastOneOther		NULL,	-- same as H.230 cancel MIV

seenByAll		NULL,	-- like H.230 MIV

cancelSeenByAll		NULL,	-- like H.230 MIV

terminalYouAreSeeing		TerminalLabel,	-- same as H.230 VIN

requestForFloor	NULL,	-- same as H.230 TIF

...,

withdrawChairToken	NULL,	-- same as H.230 CCR

-- MC-> chair

floorRequested	TerminalLabel	-- same as H.230 TIF

-- MC-> chair

}

-- ===================================================================================

-- Indication Message : Miscellaneous H.230-like indication

-- ===================================================================================

MiscellaneousIndication	::=SEQUENCE

{

logicalChannelNumber	LogicalChannelNumber,

type		CHOICE

{

logicalChannelActive	NULL,	-- same as H.230 AIA and VIA

logicalChannelInactive	NULL,	-- same as H.230 AIM and VIS

multipointConference	NULL,

cancelMultipointConference	NULL,

multipointZeroComm	NULL,	-- same as H.230 MIZ

cancelMultipointZeroComm	NULL,	-- same as H.230 cancel MIZ

multipointSecondaryStatus	NULL,	-- same as H.230 MIS

cancelMultipointSecondaryStatus	NULL,	-- same as H.230 cancel MIS

videoIndicateReadyToActivate	NULL,	-- same as H.230 VIR

videoTemporalSpatialTradeOff	INTEGER (0..31),	-- indicates current trade-off

...,

videoNotDecodedMBs	SEQUENCE

{

firstMB	INTEGER (1..8192),

numberOfMBs	INTEGER (1..8192),

temporalReference	INTEGER (0..255),

...

},

transportCapability	TransportCapability

},

...

}

-- ===================================================================================

-- Indication Message : Jitter Indication

-- ===================================================================================

JitterIndication	::=SEQUENCE

{

scope		CHOICE

{

logicalChannelNumber	LogicalChannelNumber,

resourceID	INTEGER (0..65535),

wholeMultiplex	NULL

},

estimatedReceivedJitterMantissa	INTEGER (0..3),

estimatedReceivedJitterExponent	INTEGER (0..7),

skippedFrameCount	INTEGER (0..15) OPTIONAL,

additionalDecoderBuffer	INTEGER (0..262143) OPTIONAL,	-- 262143 is 2^18 - 1

...

}

-- ===================================================================================

-- Indication Message : H.223 logical channel skew

-- ===================================================================================

H223SkewIndication	::=SEQUENCE

{

logicalChannelNumber1	LogicalChannelNumber,

logicalChannelNumber2	LogicalChannelNumber,

skew		INTEGER (0..4095),	-- units milliseconds

...

}

-- ===================================================================================

-- Indication Message : H.225.0 maximum logical channel skew

-- ===================================================================================

H2250MaximumSkewIndication	::=SEQUENCE

{

logicalChannelNumber1	LogicalChannelNumber,

logicalChannelNumber2	LogicalChannelNumber,

maximumSkew	INTEGER (0..4095),	-- units milliseconds

...

}

-- ===================================================================================

-- Indication Message : MC Location Indication

-- ===================================================================================

MCLocationIndication	::=SEQUENCE

{

signalAddress	TransportAddress, -- this is the H.323 Call Signalling

-- address of the entity which

-- contains the MC

...

}

-- ===================================================================================

-- Indication Message : Vendor Identification

-- ===================================================================================

VendorIdentification		::=SEQUENCE

{

vendor				NonStandardIdentifier,

productNumber		OCTET STRING (SIZE(1..256)) OPTIONAL,  -- per  vendor

versionNumber		OCTET STRING (SIZE(1..256)) OPTIONAL,  -- per productNumber

...

}

-- ===================================================================================

-- Indication Message : New ATM virtual channel indication

-- ===================================================================================

NewATMVCIndication	::=SEQUENCE

{

resourceID	INTEGER(0..65535),

bitRate	INTEGER(1..65535),	-- units 64 kbit/s

bitRateLockedToPCRClock	BOOLEAN,

bitRateLockedToNetworkClock	BOOLEAN,

aal			CHOICE

{

aal1	SEQUENCE

{

clockRecovery	CHOICE

{

nullClockRecovery	NULL,

srtsClockRecovery	NULL,

adaptiveClockRecovery	NULL,

...

},

errorCorrection	CHOICE

{

nullErrorCorrection	NULL,

longInterleaver	NULL,

shortInterleaver	NULL,

errorCorrectionOnly	NULL,

...

},

structuredDataTransfer	BOOLEAN,

partiallyFilledCells	BOOLEAN,

...

},

aal5	SEQUENCE

{

forwardMaximumSDUSize	INTEGER (0..65535),	-- units octets

backwardMaximumSDUSize	INTEGER (0..65535),	-- units octets

...

},

...

},

multiplex	CHOICE

{

noMultiplex	NULL,

transportStream	NULL,

programStream	NULL,

...

},

...,

reverseParameters	SEQUENCE

{

bitRate	INTEGER(1..65535),	-- units 64 kbit/s

bitRateLockedToPCRClock	BOOLEAN,

bitRateLockedToNetworkClock	BOOLEAN,

multiplex	CHOICE

{

noMultiplex	NULL,

transportStream	NULL,

programStream	NULL,

...

},

...

}

}

-- ===================================================================================

-- Indication Message : user input

-- ===================================================================================

UserInputIndication	::=CHOICE

{

nonStandard	NonStandardParameter,

alphanumeric	GeneralString,

...,

userInputSupportIndication	CHOICE

{

nonStandard 	NonStandardParameter,

basicString	NULL,

iA5String	NULL,

generalString	NULL,

...

},

signal			SEQUENCE

{

signalType	IA5String (SIZE (1) ^ FROM ("0123456789#*ABCD!")),

duration	INTEGER (1..65535) OPTIONAL,  -- milliseconds

rtp			SEQUENCE

{

timestamp	INTEGER (0..4294967295) OPTIONAL,

expirationTime	INTEGER (0..4294967295) OPTIONAL,

logicalChannelNumber	LogicalChannelNumber,

...

} OPTIONAL,

...

},

signalUpdate	SEQUENCE

{

duration	INTEGER (1..65535),  -- milliseconds

rtp			SEQUENCE

{

logicalChannelNumber	LogicalChannelNumber,

...

} OPTIONAL,

...

}

}

END