| INTERNATIONAL TELECOMMUNICATION UNION |  |
| --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 1997 - 2000 | TD- -E(PLEN) October 1999 Original: English |

Question 3, 11, 12, 13, 14, 15/16

STUDY GROUP 16 - CONTRIBUTION

SOURCE* :		RAPPORTEUR for Question 14 (G.Freundlich)

TITLE:			RECOMMENDATION H.245 - VERSION 6

__________

This contribution, known as H245NCML, provides text for version 6 of Recommendation H.245 "Control protocol for multimedia communication". No changes are shown. It has been created from H245NCMI, COM 16-102 (which includes the changes determined in the May 1999 meeting of SG16 in TDs 14, 22, 24, and 69 (PLEN), and TD xxx (PLEN). The version number in the syntax has been changed and Annex D has been updated to reference the new version.

The changes from version 5 are summarised below.

1) TD14 : Generic capability support of MPEG-4 Systems.

2) TD22 : Generic capability support of MPEG-4 Audio.

3) TD24 : Generic capability support of GSM Adaptive Multi Rate Speech Codec.

4) TD69 : Add atm-AAL5-compressed to the ASN.1 syntax for MediaTransportType.

5) APC-1644 : APC-1568 : Generic Capability support for ACELP and US1.

6) APC-1644 : APC-1575 : Addition of more ASN.1 syntax and semantics for G.729 .

7) APC-1644 : APC-1596 : Add Generic Capability OID definition to Appendix VIII.

8) TD-57: APC-1650 : New Annex for EVRC (CDMA) wireless vocoder

9) TD-57: APC-1662 : Support operation behind basic NAT devices

10) TD-57: APC-1700 : Modified syntax for G.729

11) TD-57: APC-1702 : Text from Implementors Guide: H.320/H.323 Continuous Presence; Clarification of OLCs within the Context of Fast Start

12) TD-57: APC-1746 : T.38 Annex B

13) TD-57: Annexes D, E, G and H : ISO/IEC 14496 References

14) TD-57: Tables I.8 and I.10 have invisible text

15) TD-57: Incorrect text in Table I.2 (J.2, K.2 and L.2)

16) TD-57: Q11-K-17 : Missing Parameter Table in Annex H

17) D-327 : Addition of Multipoint Visualization Capability as an equivalent to the H.243 MVC.

18) D-403 : Update the references to TDMA Wireless Coders

19) TD15/WP2 : LIAISON TO ITU-T SG16/Q.11, 14 ON MPEG-4/AUDIO

Summary

This Recommendation specifies syntax and semantics of terminal information messages as well as procedures to use them for in-band negotiation at the start of or during communication. The messages cover receiving and transmitting capabilities as well as mode preference from the receiving end, logical channel signalling, and Control & Indication. Acknowledged signalling procedures are specified to ensure reliable audiovisual and data communication.

|  | INTERNATIONAL TELECOMMUNICATION UNION |
| --- | --- |

ITU-T	H.245

TELECOMMUNICATION	(February 17, 2000)
STANDARDIZATION  SECTOR
OF  ITU

LINE  TRANSMISSION  OF  NON-TELEPHONE
SIGNALS

ITU-T  Recommendation  H.245

FOREWORD

The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of the International Telecommunication Union. The ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Conference (WTSC), which meets every four years, established the topics for study by the ITU-T Study Groups which, in their turn, produce Recommendations on these topics.

ITU-T Recommendation H.245 was prepared by the ITU-T Study Group 16 (199x-199x) and was approved by the WTSC (Place, Month xx-xx, 199x).

___________________

NOTES

1	"Shall" is used in this Recommendation to specify a mandatory requirement. "Should" is used in this Recommendation to specify a suggested, but not required, course of action. "May" is used to specify an optional course of action, without expressing a preference.

ã  ITU  199x

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

CONTENTS

Page

µ1	Scope	1

2	References	1

3	Definitions	5

4	Abbreviations	7

5	General	9

5.1	Master-slave determination	9

5.2	Capability exchange	9

5.3	Logical channel signalling procedures	10

5.4	Receive terminal close logical channel request	10

5.5	H.223 multiplex table entry modification	10

5.6	Audiovisual and data mode request	10

5.7	Round trip delay determination	10

5.8	Maintenance loops	10

5.9	Commands and indications	10

Annex A	Messages: syntax	12

Annex B	Messages: semantic definitions	66

B.1	Master Slave Determination messages	66

B.1.1	Master Slave Determination	66

B.1.2	Master Slave Determination Acknowledge	66

B.1.3	Master Slave Determination Reject	66

B.1.4	Master Slave Determination Release	66

B.2	Terminal capability messages	67

B.2.1	Overview	67

B.2.2	Terminal Capability Set	67

B.2.3	Terminal Capability Set Acknowledge	85

B.2.4	Terminal Capability Set Reject	85

B.2.5	Terminal Capability Set Release	86

B.3	Logical channel signalling messages	87

B.3.1	Open Logical Channel	87

B.3.2	Open Logical Channel Acknowledge	90

B.3.3	Open Logical Channel Reject	91

B.3.4	Open Logical Channel Confirm	92

B.3.5	Close Logical Channel	92

B.3.6	Close Logical Channel Acknowledge	93

B.3.7	Request Channel Close	93

B.3.8	Request Channel Close Acknowledge	93

B.3.9	Request Channel Close Reject	93

B.3.10	Request Channel Close Release	93

B.4	Multiplex Table signalling messages	94

B.4.1	Multiplex Entry Send	94

B.4.2	Multiplex Entry Send Acknowledge	94

B.4.3	Multiplex Entry Send Reject	94

B.4.4	Multiplex Entry Send Release	94

B.5	Request Multiplex Table signalling messages	95

B.5.1	Request Multiplex Entry	95

B.5.2	Request Multiplex Entry Acknowledge	95

B.5.3	Request Multiplex Entry Reject	95

B.5.4	Request Multiplex Entry Release	95

B.6	Request Mode messages	96

B.6.1	Request Mode	96

B.6.2	Request Mode Acknowledge	98

B.6.3	Request Mode Reject	98

B.6.4	Request Mode Release	99

B.7	Round Trip Delay messages	100

B.7.1	Round Trip Delay Request	100

B.7.2	Round Trip Delay Response	100

B.8	Maintenance Loop messages	100

B.8.1	Maintenance Loop Request	100

B.8.2	Maintenance Loop Acknowledge	100

B.8.3	Maintenance Loop Reject	100

B.8.4	Maintenance Loop Command Off	100

B.9	Communication Mode Messages	100

B.9.1	Communication Mode Command	100

B.9.2	Communication Mode Request	101

B.9.3	Communication Mode Response	101

B.10	Conference Request and Response Messages	101

B.10.1	Terminal List Request	101

B.10.2	Terminal List Response	101

B.10.3	Make Me Chair	101

B.10.4	Cancel Make Me Chair	101

B.10.5	Make Me Chair Response	101

B.10.6	Drop Terminal	101

B.10.7	Terminal Drop Reject	101

B.10.8	RequestTerminal ID	101

B.10.9	MC Terminal ID Response	102

B.10.10	Enter H.243 Password Request	102

B.10.11	Password Response	102

B.10.12	Enter H.243 Terminal ID Request	102

B.10.13	Terminal ID Response	102

B.10.14	Enter H.243 Conference ID Request	102

B.10.15	Conference ID Response	102

B.10.16	Video Command Reject	102

B.10.17	Enter Extension Address Request	102

B.10.18	Extension Address Response	102

B.10.19	Request Chair Control Token Owner	102

B.10.20	Chair Token Owner Response	102

B.10.21	Request Terminal Certificate	102

B.10.22	Terminal Certificate Response	103

B.10.23	Broadcast My Logical Channel	103

B.10.24	Broadcast My Logical Channel Response	103

B.10.25	Make Terminal Broadcaster	103

B.10.26	Make Terminal Broadcaster Response	103

B.10.27	Send This Source	103

B.10.28	SendThis Source Response	103

B.10.29	Request All Terminals Ids	103

B.10.30	Request All Terminal Ids Response	103

B.10.31	RemoteMC Request	103

B.10.32	RemoteMC Response	103

B.11	Multilink Messages	104

B.11.1	callInformation Request and Response	104

B.11.2	addConnection Request and Response	104

B.11.3	removeConnection Request and Response	104

B.11.4	maximumHeaderInterval Request and Response	104

B.11.5	Multilink Indications	104

B.11.6	DialingInformation	105

B.11.7	DialingInformationNumber	105

B.11.8	DialingInformationNetworkType	105

B.11.9	ConnectionIdentifier	105

B.12	Logical Channel Bit rate Change Messages	105

B.12.1	Logical Channel Rate Request	105

B.12.2	Logical Channel Rate Acknowledge	105

B.12.3	Logical Channel Rate Reject	106

B.12.4	Logical Channel Rate Release	106

B.13	Commands	107

B.13.1	Send Terminal Capability Set	107

B.13.2	Encryption	107

B.13.3	Flow Control	107

B.13.4	End session	107

B.13.5	Miscellaneous Command	108

B.13.6	Conference Command	110

B.13.7	H.223 Multiplex Reconfiguration	110

B.13.8	New ATM Virtual Channel Command	110

B.14	Indications	111

B.14.1	Function Not Understood	111

B.14.2	Miscellaneous Indication	111

B.14.3	Jitter Indication	111

B.14.4	H.223 Skew Indication	112

B.14.5	New ATM Virtual Channel Indication	113

B.14.6	User Input	113

B.14.7	Conference Indications	114

B.14.8	H2250 Maximum Logical Channel Skew	115

B.14.9	MC Location Indication	115

B.14.10	Vendor Identification Indication	115

B.14.11	Function Not Supported	115

B.14.12	Flow Control Indication	115

Annex C	Procedures	117

C.1	Introduction	117

C.1.1	Method of specification	117

C.1.2	Communication between protocol entity and protocol user	117

C.1.3	Peer-to-peer communication	117

C.1.4	SDL Diagrams	118

C.1.5	SDL Key	118

C.2	Master slave determination procedures	120

C.2.1	Introduction	120

C.2.2	Communication between the MSDSE and the MSDSE user	121

C.2.3	Peer to peer MSDSE communication	123

C.2.4	MSDSE procedures	124

C.3	Capability exchange procedures	131

C.3.1	Introduction	131

C.3.2	Communication between CESE and CESE user	131

C.3.3	Peer to peer CESE communication	133

C.3.4	CESE procedures	134

C.4	Uni-directional Logical Channel signalling procedures	141

C.4.1	Introduction	141

C.4.2	Communication between the LCSE and the LCSE user	142

C.4.3	Peer to peer LCSE communication	144

C.4.4	LCSE procedures	145

C.5	Bi-directional Logical Channel signalling procedures	153

C.5.1	Introduction	153

C.5.2	Communication between the B-LCSE and the B-LCSE user	154

C.5.3	Peer to peer B-LCSE communication	157

C.5.4	B-LCSE procedures	158

C.6	Close Logical Channel procedures	167

C.6.1	Introduction	167

C.6.2	Communication between CLCSE and CLCSE user	167

C.6.3	Peer to peer CLCSE communication	169

C.6.4	CLCSE procedures	170

C.7	H.223 Multiplex Table Procedures	175

C.7.1	Introduction	175

C.7.2	Communication between the MTSE and MTSE user	176

C.7.3	Peer to peer MTSE communication	178

C.7.4	MTSE procedures	179

C.8	Request Multiplex Entry procedures	186

C.8.1	Introduction	186

C.8.2	Communication between RMESE and RMESE user	186

C.8.3	Peer to peer RMESE communication	188

C.8.4	RMESE procedures	189

C.9	Mode Request procedures	193

C.9.1	Introduction	193

C.9.2	Communication between MRSE and MRSE user	194

C.9.3	Peer to peer MRSE communication	195

C.9.4	MRSE procedures	196

C.10	Round trip delay procedures	203

C.10.1	Introduction	203

C.10.2	Communication between the RTDSE and the RTDSE user	203

C.10.3	Peer to peer RTDSE communication	204

C.10.4	RTDSE procedures	205

C.11	Maintenance Loop procedures	208

C.11.1	Introduction	208

C.11.2	Communication between the MLSE and the MLSE user	208

C.11.3	Peer to peer MLSE communication	211

C.11.4	MLSE procedures	212

Annex D	Object identifier assignments	218

Annex E	ISO/IEC 14496-2 Capability Definitions	220

Annex F	Logical Channel Bit Rate Management Capability Definitions	223

Annex G	ISO/IEC 14496-1 Capability Definitions	225

G.1	Capability Identifier	225

G.2	Capability parameters used for capability negotiations and logical channel signalling	225

G.3	Capability parameters used for logical channel signalling only	227

Annex H	ISO/IEC 14496-3 Capability Definitions	228

Annex I	GSM Adaptive Multi Rate Capability Definitions	233

I.1	Definition of mode signaling and bit stuffing to achieve octet alignment	235

Annex J	TDMA ACELP Voice Codec Definitions	245

Annex K	TDMA US1 Voice Codec Definitions	247

Annex L	CDMA  EVRC Voice Codec Definitions	249

Appendix I	Overview of ASN.1 syntax	251

I.1	Introduction to ASN.1	251

I.2	Basic ASN.1 data types	251

I.3	Aggregate data types	253

I.4	Object identifier type	253

Appendix II	Examples of H.245 procedures	255

II.1	Introduction	255

II.2	Master Slave Determination Signalling Entity	255

II.3	Capability Exchange Signalling Entity	259

II.4	Logical Channel Signalling Entity	260

II.5	Close Logical Channel Signalling Entity	262

II.6	Multiplex Table Signalling Entity	263

II.7	Mode Request Signalling Entity	265

II.8	Round Trip Delay Signalling Entity	267

II.9.	Bi-directional Logical Channel Signalling Entity	268

Appendix III	Summary of procedure timers and counters	271

III.1	Timers	271

III.2	Counters	272

Appendix IV	H.245 Extension Procedure	273

Appendix V	The ReplacementFor Procedure	275

Appendix VI	Examples of H.263 Capability Structure Settings	276

VI.1	Examples of Enhancement Layer H.245 parameter setting	276

VI.2	Examples of Video Back Channel H.245 parameter setting	278

Appendix VIIProcedure and template for defining new capabilities with H.245 generic capabilities	282

VII.1	Procedure	282

VII.1.1	Definition of generic capabilities in H.245	282

VII.1.2	Definition of generic capabilities in other ITU Recommendations	282

VII.1.3	Definition of generic capabilities in non-ITU Standards	282

VII.2	Template	282

VII.2.1	Capability Identifier	282

VII.2.2	Capability parameters	283

VII.3	Example Template - H.261	283

VII.3.1	H.261 Capability Identifier	283

VII.3.2	H.261 Capability parameters	284

Appendix VIII	List of generic capabilities defined in other Recommendations/Standards than this Recommendation	286

SUMMARY

This Recommendation specifies syntax and semantics of terminal information messages as well as procedures to use them for in-band negotiation at the start of or during communication. The messages cover receiving and transmitting capabilities as well as mode preference from the receiving end, logical channel signalling, and Control & Indication. Acknowledged signalling procedures are specified to ensure reliable audiovisual and data communication.

Recommendation H.245

Recommendation H.245     (Error: Reference source not found)

(Place, 199x)

# 1	Scope

This Recommendation specifies syntax and semantics of terminal information messages as well as procedures to use them for in-band negotiation at the start of or during communication. The messages cover receiving and transmitting capabilities as well as mode preference from the receiving end, logical channel signalling, and Control & Indication. Acknowledged signalling procedures are specified to ensure reliable audiovisual and data communication.

This Recommendation covers a wide range of applications, including storage/retrieval, messaging and distribution services as well as conversational. It applies to, but is not limited to, multimedia systems that use the multiplexes defined in H.222.0, H.223, and H.225.0. These different systems share the same syntax and semantics, and are therefore bit-wise compatible. Some of the procedures are applicable to all systems, while the others are more specific to particular systems.

The different systems that make use of this Recommendation may specify the use of different transport protocols. However, it is intended to be used with a reliable transport layer, that is, one that provides guaranteed delivery of correct data.

NOTE - There should be no confusion with the T.120 management system, which is carried within the data stream, and covers different functionalities from those described here - the H.245 stream and the T.120-data stream are complementary.

# 2	References

The following ITU-T Recommendations, and other references, contain provisions which, through reference in this text, constitute the provisions of the Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; all users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

[1]	ITU-T Recommendation E.164 (1997) - Numbering Plan for the ISDN ERA.

[2]	ITU-T Recommendation G.711 (1988) - Pulse code modulation (PCM) of voice frequencies.

[3]	ITU-T Recommendation G.722 (1988) - 7 kHz audio-coding within 64 kbit/s.

[4]	ITU-T Recommendation G.723.1 (1996) - Dual rate speech coder for multimedia communication transmitting at 5.3 and 6.3 kbit/s.

[5]	ITU-T Recommendation G.728 (1992) - Coding of speech at 16 kbit/s using low-delay code excited linear prediction.

[6]	ITU-T Recommendation G.729 (1996) - Coding of speech at 8 kbit/s using conjugate structure algebraic-code-excited linear-prediction (CS-ACELP).

[7]	ITU-T Recommendation H.221 (1995) - Frame structure for a 64 to 1920 kbit/s channel in audiovisual teleservices.

[8]	ITU-T Recommendation H.222.0 (1996) - Coding of Moving Pictures and Associated Audio: Systems - ISO/IEC 13818-1.

[9]	ITU-T Recommendation H.222.1 (1996) - Multimedia multiplex and synchronization for audiovisual communication in ATM environments.

[10]	ITU-T Recommendation H.223 (1996) - Multiplexing protocol for low bitrate multimedia communication.

[11]	ITU-T Recommendation H.224 (1994) - A real time control protocol for simplex applications using the H.221 LSD/HSD/MLP channels.

[12]	ITU-T Recommendation H.225.0 (1998) - Call Signalling protocols and media stream packetization for packet based multimedia communications systems.

[13]	ITU-T Recommendation H.230 (1995) - Frame-synchronous control and indication signals for audiovisual systems.

[14]	ITU-T Recommendation H.233 (1995) - Confidentiality system for audiovisual services.

[15]	ITU-T Recommendation H.234 (1994) - Authentication and key management.

[16]	ITU-T Recommendation H.235 (1998) - Security and Encryption for H Series (H.323 and other H.245 based) multimedia terminals.

[17] ITU-T Recommendation H.243 (1996) - .Procedures for Establishing Communication Between Three or More Audio Visual Terminals using Digital Channels up to 2 Mbit/s.

[18]	ITU-T Recommendation H.261 (1993) - Video Codec for audiovisual services at px64 kbit/s.

[19]	ITU-T Recommendation H.262 (1996) - Generic Coding of Moving Pictures and Associated Audio: Video - ISO/IEC 13818-2.

[20]	ITU-T Recommendation H.263 (1998) - Video coding for low bit rate communication.

[21]	ITU-T Recommendation H.310 (1996) - Broadband audiovisual communication systems and terminals.

[22]	ITU-T Recommendation H.320 (1996) - Narrow-band visual telephone systems and terminal equipment.

[23]	ITU-T Recommendation H.323 (1998) - Packet-based multimedia communication systems.

[24]	ITU-T Recommendation H.324 (1998) - Terminal for low bitrate multimedia communication.

[25]	ITU-T Recommendation I.363 (1996) - B-ISDN ATM adaptation layer (AAL) specification.

[26]	ITU-T Recommendation Q.2931 (1996) - Broadband integrated services digital network (B-ISDN) - Digital subscriber signalling No. 2 (DSS 2) - User network interface layer 3 specification for basic call/connection control.

[27]	ITU-T Recommendation T.30 (1996) - Procedures for document facsimile transmission in the general switched telephone network.

[28]	ITU-T Recommendation T.35 (1991) - Procedure for the allocation of CCITT defined codes for non-standard facilities.

[29]	ITU-T Recommendation T.38 (1998) - Procedures for Real Time Group 3 Facsimile Communication Between Terminals Using IP Networks.

[30]	ITU-T Recommendation T.51 (1993) - Latin based coded character sets for telematic services.

[31]	ITU-T Recommendation T.84 | ISO/IEC 10918-3 (1996): "Digital Compression and Coding of Continuous Tone Still Images - Extensions".

[32]	ITU-T Recommendation T.120 (1996) - Data protocols for multimedia conferencing - under development.

[33] ITU-T Recommendation T.123 (1996) - Protocol Stacks for Audiographic and Audiovisual Teleconference Applications.

[34]	ITU-T Recommendation T.140 (1998) - Text conversation protocol for multimedia application

[35]	ITU-T Recommendation T.434 (1996) - Binary File Transfer Format for the Telematic Services.

[36]	ITU-T Recommendation V.14 (1993) - Transmission of start-stop characters over synchronous bearer channels.

[37]	ITU-T Recommendation V.34 (1996) - A modem operating at data signalling rates of up to 33 600 bit/s for use on the general switched telephone network and on leased point-to-point 2-wire telephone-type circuits.

[38]	ITU-T Recommendation V.42 (1996) - Error-correcting procedures for DCEs using asynchronous-to-synchronous conversion.

[39]	ITU-T Recommendation V.140 (1998) - Procedures for establishing communication between two multiprotocol audiovisual terminals using digital channels at a multiple of 64 or 56 kbit/s.

[40]	ITU-T Recommendation X.680 (1994): Information Technology - Abstract Syntax Notation One (ASN.1) - Specification of basic notation.

[41]	ITU-T Recommendation X.680 Amendment 1 (1995): Information Technology - Abstract Syntax Notation One (ASN.1) - Specification of Basic Notation; Amendment 1: Rules of extensibility.

[42]	ITU-T Recommendation X.681 Amendment 1 (1995): Information Technology - Abstract Syntax Notation One (ASN.1) - Information Object Specification; Amendment 1: Rules of extensibility.

[43]	ITU-T Recommendation X.691 (1996): Information Technology - ASN.1 Encoding Rules - Specification of Packed Encoding Rules (PER).

[44]	ISO/IEC 3309 (1991): Information Technology - Telecommunications and information exchange between systems - High-level data link control (HDLC) procedures - Frame Structure.

[45]	ISO/IEC 11172-2 (1993): Information Technology - Coding of Moving Pictures and Associated Audio for digital storage media at up to about 1.5 Mbit/s - Part 2: Video.

[46]	ISO/IEC 11172-3 (1993): Information Technology - Coding of Moving Pictures and Associated Audio for digital storage media at up to about 1.5 Mbit/s - Part 3: Audio.

[47]	ISO/IEC 13818-3 (1996): Information Technology - Generic Coding of Moving Pictures and Associated Audio - Part 3: Audio.

[48]	ISO/IEC 13818-6 (1996): Information Technology - Generic Coding of Moving Pictures and Associated Audio - Part 6: Digital Storage Media Command and Control.

[49]	ISO/IEC 14496-1 (1999): Information Technology - Generic Coding of Audio-Visual Objects - Part 1: Systems.

[50]	ISO/IEC 14496-2 (1999): Information Technology - Generic Coding of Audio-Visual Objects - Part 2: Visual.

[51]	ISO/IEC 14496-3 (1999): Information Technology - Generic Coding of Audio-Visual Objects - Part 3: Audio.

[52]	ISO/IEC 14496-3/AMD1 (2000), Information technology - Generic coding of audio-visual objects - Part 3: Audio.

[53]	ISO/IEC TR9577 (1990): Information Technology - Telecommunications information exchange between systems - protocol identification in the network layer.

[54] I-ETS Recommendation 300 036 (GSM 06.10) - Full-rate speech transcoding.

[55] I-ETS Recommendation 300 969 (GSM 06.20) - Half-rate speech transcoding.

[56] I-ETS Recommendation 300 726 (GSM 06.60) - Enhanced Full-rate speech transcoding.

[57] I-ETS Recommendation 300 038 (GSM 06.12) - Comfort noise aspects for full rate speech traffic channels.

[58] I-ETS Recommendation 300 971 (GSM 06.22) - Comfort noise aspects for half rate speech traffic channels.

[59] I-ETS Recommendation 300 728 (GSM 06.62) - Comfort noise aspects for enhanced full rate speech traffic channels.

[60] I-ETS Recommendation 300 039 (GSM 06.31) - Discontinuous transmission for full rate speech traffic channels.

[61] I-ETS Recommendation 300 972 (GSM 06.41) - Discontinuous transmission for half rate speech traffic channels.

[62] I-ETS Recommendation 300 729 (GSM 06.81) - Discontinuous transmission for enhanced full rate speech traffic channels.

[63]	I-ETS Recommendation 300 037 (GSM 06.11) - Substitution and muting of lost frames for full rate speech traffic channels.

[64]	I-ETS Recommendation 300 970 (GSM 06.21) - Substitution and muting of lost frames for half rate speech traffic channels.

[65]	I-ETS Recommendation 300 727 (GSM 06.61) - Substitution and muting of lost frames for enhanced full rate speech traffic channels.

[66]	I-ETS Recommendation 300 040 (GSM 06.32) - Voice activity detection for full rate speech traffic channels.

[67]	I-ETS Recommendation 300 973 (GSM 06.42) - Voice activity detection for half rate speech traffic channels.

[68]	I-ETS Recommendation 300 730 (GSM 06.82) - Voice activity detection for enhanced full rate speech traffic channels.

[69]	 I-ETS Recommendation 300 724 (GSM 06.53) - ANSI-C code for the GSM Enhanced Full Rate speech.

[70]	 GSM 06.73 - ANSI-C code for the GSM Adaptive Multi Rate speech codec.

[71]	 GSM 06.90 - Adaptive Multi Rate speech transcoding.

[72]	 GSM 06.91 - Substitution and muting of lost frames for Adaptive Multi Rate speech traffic channels.

[73]	 GSM 06.92 - Comfort noise aspects for Adaptive Multi Rate speech traffic channels

[74]	 GSM 06.94 - Voice Activity Detector  for Adaptive Multi Rate speech traffic channels.

[75]	 RCR STD-27H, Personal Digital Cellular Telecommunication System RCR Standard.

[76] TIA/EIA -136-Rev.A, part 410  – TDMA Cellular/PCS – Radio Interface, Enhanced Full Rate Voice Codec (ACELP). Formerly IS-641. TIA published standard,  1998.

[77]	TIA/EIA -136-Rev.A, part 430– TDMA Cellular/PCS – Radio Interface, US1 Full Rate Voice Codec. TIA published standard, 1998.

# 3	Definitions

For the purpose of this Recommendation, the following definitions apply:

Bi-directional Logical Channel: A bi-directional logical channel consists of a pair of associated transmission paths between two terminals, one in each direction of transmission.

Capability: A terminal has a particular capability if it is able to encode and transmit or receive and decode that particular signal.

Channel: A channel is a uni-directional link between two end points.

Command: A command is a message that requires action but no explicit response.

Elementary Stream: Elementary Stream is a generic term for a coded video, coded audio or other coded bitstream.

Entry: The word entry is used to refer to elements in sets or tables, such as capability sets and multiplex tables.

Forward: Forward is used to refer to transmission directed from the terminal making the request for a bi-directional logical channel to the other terminal.

In-band: In-band messages are those that are transported within the channel or logical channel to which they refer.

Incoming: An Incoming Signalling Entity cannot initiate a procedure, but responds to messages from the remote Signalling Entity and its own user's primitives.

Indication: An indication is a message that contains information but does not require action or response.

Logical Channel: A logical channel is a uni-directional path or bi-directional path for the transmission of information.

Logical Channel Number: A logical channel number is a number that identifies a single logical channel.

Logical Channel Signalling: Logical channel signalling is a set of procedures that are used to open and close logical channels.

Master Terminal: A master terminal is the terminal that is determined as being the master terminal by the master-slave determination procedure defined in this Recommendation, or by some other procedure.

Medium Type: A medium type is a single form of information that is presented to a user or the data representing that information: video, audio and text are example Medium Types.

Mode: A mode is a set of elementary streams that a terminal is transmitting, intends to transmit, or would like to receive.

Multimedia communication: Multimedia communication refers to the transmission and/or reception of signals of two or more Medium Types simultaneously.

Non-standard: Not conforming to a national or international standard referenced in this Recommendation.

Outgoing: An Outgoing Signalling Entity is one which initiates a procedure.

Multipoint: Multipoint refers to the simultaneous interconnection of three or more terminals to allow communication among several sites through the use of multipoint control units (bridges) that centrally direct the flow of information.

Request: A request is a message that results in action by the remote terminal and requires an immediate response from it.

Response: A response is a message that is the response to a request.

Reverse: Reverse is used to refer to transmission directed from the terminal receiving a request for a bi-directional logical channel to the terminal making the request.

Session: A session is a period of communication between two terminals which may be conversational or non-conversational (for example retrieval from a database).

Slave Terminal: A slave terminal is the terminal that is determined as being the slave terminal by the master-slave determination procedure defined in this Recommendation, or by some other procedure.

Support: The ability to operate in a given mode, however a requirement to support a mode does not mean that the mode must actually be used at all times: unless prohibited, other modes may be used by mutual negotiation.

Terminal: A terminal is any endpoint and may be a user's terminal or some other communication system such as an MCU or an information server.

TSAP Identifier: The piece of information used to multiplex several transport connections of the same type on a single H.323 entity with all transport connections sharing the same LAN address, (e. g. the port number in a TCP/UDP/IP environment). TSAP identifiers may be (pre)assigned by some international authority or may be allocated dynamically during setup of a call. Dynamically assigned TSAP identifiers are of transient nature, i. e. their values are only valid for the duration of a single call.

Uni-directional Logical Channel: A uni-directional logical channel is a path for the transmission of a single elementary stream from one terminal to another.

# 4	Abbreviations

For the purpose of this Recommendation, the following abbreviations are used:

AAL	ATM Adaptation layer

AL1,2,3	H.223 Adaptation layers 1, 2 and 3

ASN.1	Abstract syntax notation 1

ATM	Asynchronous transfer mode

B-LCSE	Bi-directional Logical Channel Signalling Entity

CESE	Capability Exchange Signalling Entity

CLCSE	Close Logical Channel Signalling Entity

CIF	Common Intermediate Format (of a video picture: refer to H.261 and H.263)

CPCS	Common Part Convergence Sublayer (of ATM Adaptation Layer 5)

DTMF	Dual tone multi-frequency

DSM-CC	Digital storage media - command and control

GOB	Group of blocks (of a video picture: refer to H.261 and H.263)

GSTN	General switched telephone network

HDLC	High-level data link control

HRD	Hypothetical Reference Decoder (refer to H.261 and H.263)

IV	Initialization Vector (used for encryption: refer to H.233 and H.234)

LAPM	Link access protocol for modems

LCSE	Logical Channel Signalling Entity

MC	 H.323 Multipoint Control Entity

MCU	Multipoint control unit

MLSE	Maintenance Loop Signalling Entity

MPI	Minimum picture interval

MSDSE	Master Slave Determination Signalling Entity

MTSE	Multiplex Table Signalling Entity

MRSE	Mode Request Signalling Entity

PCR	Program Clock Reference (refer to ISO/IEC 13818-1 | H.222.0)

PID	Packet Identifier (refer to ISO/IEC 13818-1 | H.222.0)

QCIF	Quarter CIF

RMESE	Request Multiplex Entry Signalling Entity

RTDSE	Round Trip Delay Signalling Entity

RTP	Real-time Transport Protocol

RTCP	Real-time Transport Control Protocol

SDL	Specification and description Language

SDU	Service data unit

SE	Session Exchange message (used for encryption: refer to H.233 and H.234)

SQCIF	Sub QCIF

STD	System Target Decoder (refer to ISO/IEC 13818-1 | H.222.0)

VC	ATM Virtual channel

# 5	General

This Recommendation provides a number of different services, some of which are expected to be applicable to all terminals that use it and some that are more specific to particular ones. Procedures are defined to allow the exchange of audiovisual and data capabilities; to request the transmission of a particular audiovisual and data mode; to manage the logical channels used to transport the audiovisual and data information; to establish which terminal is the master terminal and which is the slave terminal for the purposes of managing logical channels; to carry various control and indication signals; to control the bit rate of individual logical channels and the whole multiplex; and to measure the round trip delay, from one terminal to the other and back. These procedures are explained in more detail below.

Following this general introduction, there are sections detailing the message syntax and semantics and the procedures. The syntax has been defined using ASN.1 notation [78] and the semantics define the meaning of syntax elements as well as providing syntactic constraints that are not specified in the ASN.1 syntax. The procedures section defines the protocols that use the messages defined in the other sections.

Although not all of the messages and procedures defined in this Recommendation will be applicable to all terminals, no indication of such restrictions is given here. These restrictions are the responsibility of the recommendations that use this Recommendation.

This Recommendation has been defined to be independent of the underlying transport mechanism, but is intended to be used with a reliable transport layer, that is, one that provides guaranteed delivery of correct data.

## 5.1	Master-slave determination

Conflicts may arise when two terminals involved in a call initiate similar events simultaneously and only one such event is possible or desired, for example, when resources are available for only one occurrence of the event. To resolve such conflicts, one terminal shall act as a master and the other terminal shall act as a slave terminal. Rules specify how the master and slave terminal shall respond at times of conflict.

The master-slave determination procedure allow terminals in a call to determine which terminal is the master and which terminal is the slave. The terminal status may be re-determined at any time during a call; however, a terminal may only initiate the master slave determination process if no procedure which depends upon its result is locally active.

## 5.2	Capability exchange

The capability exchange procedures are intended to ensure that the only multimedia signals to be transmitted are those that can be received and treated appropriately by the receive terminal. This requires that the capabilities of each terminal to receive and decode be known to the other terminal. It is not necessary that a terminal understand or store all incoming capabilities; those that are not understood, or can not be used shall be ignored, and no fault shall be considered to have occurred. When a capability is received which contains extensions not understood by the terminal, the capability shall be accepted as if it did not contain the extensions.

The total capability of a terminal to receive and decode various signals is made known to the other terminal by transmission of its capability set.

Receive capabilities describe the terminal's ability to receive and process incoming information streams. Transmitters shall limit the content of their transmitted information to that which the receiver has indicated it is capable of receiving. The absence of a receive capability indicates that the terminal cannot receive (is a transmitter only).

Transmit capabilities describe the terminal's ability to transmit information streams. Transmit capabilities serve to offer receivers a choice of possible modes of operation, so that the receiver may request the mode which it prefers to receive. The absence of a transmit capability indicates that the terminal is not offering a choice of preferred modes to the receiver (but it may still transmit anything within the capability of the receiver).

These capability sets provide for more than one stream of a given medium type to be sent simultaneously. For example, a terminal may declare its ability to receive (or send) two independent H.262 video streams and two independent G.722 audio streams at the same time. Capability messages have been defined to allow a terminal to indicate that it does not have fixed capabilities, but that they depend on which other modes are being used simultaneously. For example, it is possible to indicate that higher resolution video can be decoded when a simpler audio algorithm is used; or that either two low resolution video sequences can be decoded or a single high resolution one. It is also possible to indicate trade-offs between the capability to transmit and the capability to receive.

Non-standard capabilities and control messages may be issued using the NonStandardParameter structure. Note that while the meaning of non-standard messages is defined by individual organizations, equipment built by any manufacturer may signal any non-standard message, if the meaning is known.

Terminals may reissue capability sets at any time.

## 5.3	Logical channel signalling procedures

An acknowledged protocol is defined for the opening and closing of logical channels which carry the audiovisual and data information. The aim of these procedures is to ensure that a terminal is capable of receiving and decoding the data that will be transmitted on a logical channel at the time the logical channel is opened rather than at the time the first data is transmitted on it; and to ensure that the receive terminal is ready to receive and decode the data that will be transmitted on the logical channel before that transmission starts. The OpenLogicalChannel message includes a description of the data to be transported, for example, H.262 MP@ML at 6 Mbit/s. Logical channels should only be opened when there is sufficient capability to receive data on all open logical channels simultaneously.

A part of this protocol is concerned with the opening of bi-directional channels. To avoid conflicts which may arise when two terminals initiate similar events simultaneously, one terminal is defined as the master terminal, and the other as the slave terminal. A protocol is defined to establish which terminal is the master and which is the slave. However, systems that use this Recommendation may specify the procedure specified in this Recommendation or another means of determining which terminal is the master and which is the slave.

## 5.4	Receive terminal close logical channel request

A logical channel is opened and closed from the transmitter side. A mechanism is defined which allows a receive terminal to request the closure of an incoming logical channel. The transmit terminal may accept or reject the logical channel closure request. A terminal may, for example, use these procedures to request the closure of an incoming logical channel which, for whatever reason, cannot be decoded. These procedures may also be used to request the closure of a bi-directional logical channel by the terminal that did not open the channel.

## 5.5	H.223 multiplex table entry modification

The H.223 multiplex table associates each octet within an H.223 MUX message with a particular logical channel number. The H.223 multiplex table may have up to 15 entries. A mechanism is provided that allows the transmit terminal to specify and inform the receiver of new H.223 multiplex table entries. A receive terminal may also request the retransmission of a multiplex table entry.

## 5.6	Audiovisual and data mode request

When the capability exchange protocol has been completed, both terminals will be aware of each other's capability to transmit and receive as specified in the capability descriptors that have been exchanged. It is not mandatory for a terminal to declare all its capabilities; it need only declare those that it wishes to be used.

A terminal may indicate its capabilities to transmit. A terminal that receives transmission capabilities from the remote terminal may request a particular mode to be transmitted to it. A terminal indicates that it does not want its transmission mode to be controlled by the remote terminal by sending no transmission capabilities.

## 5.7	Round trip delay determination

It may be useful in some applications to have knowledge of the round trip delay between a transmit terminal and a receive terminal. A mechanism is provided to measure this round trip delay. This mechanism may also be useful as a means to detect whether the remote terminal is still functioning.

## 5.8	Maintenance loops

Procedures are specified to establish maintenance loops. It is possible to specify the loop of a single logical channel either as a digital loop or decoded loop, and the loop of the whole multiplex.

## 5.9	Commands and indications

Commands and indications are provided for various purposes: video/audio active/inactive signals to inform the user; fast update request for source switching in multipoint applications are some examples. Neither commands nor indications elicit response messages from the remote terminal. Commands force an action at the remote terminal whilst indications merely provide information and do not force any action.

A command is defined to allow the bit rate of logical channels and the whole multiplex to be controlled from the remote terminal. This has a number of purposes: interworking with terminals using multiplexes in which only a finite number of bit rates are available; multi-point applications where the rates from different sources should be matched; and flow control in congested networks.

ANNEX A

# Annex A	Messages: syntax

A==

(This annex forms an integral part of this Recommendation)

This annex specifies the syntax of messages using the notation defined in ASN.1 [79]. Messages shall be encoded for transmission by applying the packed encoding rules specified in [80] using the basic aligned variant. The first bit in each octet which is transmitted is the most significant bit of the octet as is specified in X.691.

MULTIMEDIA-SYSTEM-CONTROL DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- Export all symbols

-- ===================================================================================

-- Top level Messages

-- ===================================================================================

MultimediaSystemControlMessage	::=CHOICE

{

request	RequestMessage,

response	ResponseMessage,

command	CommandMessage,

indication	IndicationMessage,

...

}

-- A RequestMessage results in action and requires an immediate response

RequestMessage	::=CHOICE

{

nonStandard	NonStandardMessage,

masterSlaveDetermination	MasterSlaveDetermination,

terminalCapabilitySet	TerminalCapabilitySet,

openLogicalChannel	OpenLogicalChannel,

closeLogicalChannel	CloseLogicalChannel,

requestChannelClose	RequestChannelClose,

multiplexEntrySend	MultiplexEntrySend,

requestMultiplexEntry	RequestMultiplexEntry,

requestMode	RequestMode,

roundTripDelayRequest	RoundTripDelayRequest,

maintenanceLoopRequest	MaintenanceLoopRequest,

...,

communicationModeRequest	CommunicationModeRequest,

conferenceRequest	ConferenceRequest,

multilinkRequest	MultilinkRequest,

logicalChannelRateRequest	LogicalChannelRateRequest

}

-- A ResponseMessage is the response to a request Message

ResponseMessage	::=CHOICE

{

nonStandard	NonStandardMessage,

masterSlaveDeterminationAck	MasterSlaveDeterminationAck,

masterSlaveDeterminationReject	MasterSlaveDeterminationReject,

terminalCapabilitySetAck	TerminalCapabilitySetAck,

terminalCapabilitySetReject	TerminalCapabilitySetReject,

openLogicalChannelAck	OpenLogicalChannelAck,

openLogicalChannelReject	OpenLogicalChannelReject,

closeLogicalChannelAck	CloseLogicalChannelAck,

requestChannelCloseAck	RequestChannelCloseAck,

requestChannelCloseReject	RequestChannelCloseReject,

multiplexEntrySendAck	MultiplexEntrySendAck,

multiplexEntrySendReject	MultiplexEntrySendReject,

requestMultiplexEntryAck	RequestMultiplexEntryAck,

requestMultiplexEntryReject	RequestMultiplexEntryReject,

requestModeAck	RequestModeAck,

requestModeReject	RequestModeReject,

roundTripDelayResponse	RoundTripDelayResponse,

maintenanceLoopAck	MaintenanceLoopAck,

maintenanceLoopReject	MaintenanceLoopReject,

...,

communicationModeResponse	CommunicationModeResponse,

conferenceResponse	ConferenceResponse,

multilinkResponse	MultilinkResponse,

logicalChannelRateAcknowledge	LogicalChannelRateAcknowledge,

logicalChannelRateReject	LogicalChannelRateReject

}

-- A CommandMessage requires action, but no explicit response

CommandMessage	::=CHOICE

{

nonStandard	NonStandardMessage,

maintenanceLoopOffCommand	MaintenanceLoopOffCommand,

sendTerminalCapabilitySet	SendTerminalCapabilitySet,

encryptionCommand	EncryptionCommand,

flowControlCommand	FlowControlCommand,

endSessionCommand	EndSessionCommand,

miscellaneousCommand	MiscellaneousCommand,

...,

communicationModeCommand	CommunicationModeCommand,

conferenceCommand	ConferenceCommand,

h223MultiplexReconfiguration	H223MultiplexReconfiguration,

newATMVCCommand	NewATMVCCommand

}

-- An IndicationMessage is information that does not require action or response

IndicationMessage	::=CHOICE

{

nonStandard	NonStandardMessage,

functionNotUnderstood	FunctionNotUnderstood,

masterSlaveDeterminationRelease	MasterSlaveDeterminationRelease,

terminalCapabilitySetRelease	TerminalCapabilitySetRelease,

openLogicalChannelConfirm	OpenLogicalChannelConfirm,

requestChannelCloseRelease	RequestChannelCloseRelease,

multiplexEntrySendRelease	MultiplexEntrySendRelease,

requestMultiplexEntryRelease	RequestMultiplexEntryRelease,

requestModeRelease	RequestModeRelease,

miscellaneousIndication	MiscellaneousIndication,

jitterIndication	JitterIndication,

h223SkewIndication	H223SkewIndication,

newATMVCIndication	NewATMVCIndication,

userInput	UserInputIndication,

...,

h2250MaximumSkewIndication	H2250MaximumSkewIndication,

mcLocationIndication	MCLocationIndication,

conferenceIndication	ConferenceIndication,

vendorIdentification	VendorIdentification,

functionNotSupported	FunctionNotSupported,

multilinkIndication	MultilinkIndication,

logicalChannelRateRelease	LogicalChannelRateRelease,

flowControlIndication	FlowControlIndication

}

-- SequenceNumber is defined here as it is used in a number of Messages

SequenceNumber	::=INTEGER (0..255)

-- ===================================================================================

-- Non standard Message definitions

-- ===================================================================================

NonStandardMessage	::=SEQUENCE

{

nonStandardData	NonStandardParameter,

...

}

NonStandardParameter	::=SEQUENCE

{

nonStandardIdentifier	NonStandardIdentifier,

data		OCTET STRING

}

NonStandardIdentifier	::=CHOICE

{

object		OBJECT IDENTIFIER,

h221NonStandard	SEQUENCE

{

t35CountryCode	INTEGER (0..255),	-- country, per T.35 Annex A

t35Extension	INTEGER (0..255),

-- assigned nationally unless t35CountryCode is 1111 1111,

-- in which case it shall contain the country code according

-- to T.35 Annex B

manufacturerCode	INTEGER (0..65535)	-- assigned nationally

}

}

-- ===================================================================================

-- Master-slave determination definitions

-- ===================================================================================

MasterSlaveDetermination	::=SEQUENCE

{

terminalType	INTEGER (0..255),

statusDeterminationNumber	INTEGER (0..16777215),

...

}

MasterSlaveDeterminationAck	::=SEQUENCE

{

decision	CHOICE

{

master	NULL,

slave	NULL

},

...

}

MasterSlaveDeterminationReject	::=SEQUENCE

{

cause		CHOICE

{

identicalNumbers	NULL,

...

},

...

}

MasterSlaveDeterminationRelease	::=SEQUENCE

{

...

}

-- ===================================================================================

-- Capability exchange definitions

-- ===================================================================================

TerminalCapabilitySet	::=SEQUENCE

{

sequenceNumber	SequenceNumber,

protocolIdentifier	OBJECT IDENTIFIER,

-- shall be set to the value

-- {itu-t (0) recommendation (0) h (8) 245 version (0) 6}

multiplexCapability	MultiplexCapability OPTIONAL,

capabilityTable	SET SIZE (1..256) OF CapabilityTableEntry OPTIONAL,

capabilityDescriptors	SET SIZE (1..256) OF CapabilityDescriptor OPTIONAL,

...

}

CapabilityTableEntry	::=SEQUENCE

{

capabilityTableEntryNumber	CapabilityTableEntryNumber,

capability	Capability OPTIONAL

}

CapabilityDescriptor	::=SEQUENCE

{

capabilityDescriptorNumber	CapabilityDescriptorNumber,

simultaneousCapabilities	SET SIZE (1..256) OF AlternativeCapabilitySet OPTIONAL

}

AlternativeCapabilitySet	::=SEQUENCE SIZE (1..256) OF CapabilityTableEntryNumber

CapabilityTableEntryNumber	::=INTEGER (1..65535)

CapabilityDescriptorNumber	::=INTEGER (0..255)

TerminalCapabilitySetAck	::=SEQUENCE

{

sequenceNumber	SequenceNumber,

...

}

TerminalCapabilitySetReject	::=SEQUENCE

{

sequenceNumber	SequenceNumber,

cause		CHOICE

{

unspecified	NULL,

undefinedTableEntryUsed	NULL,

descriptorCapacityExceeded	NULL,

tableEntryCapacityExceeded	CHOICE

{

highestEntryNumberProcessed	CapabilityTableEntryNumber,

noneProcessed	NULL

},

...

},

...

}

TerminalCapabilitySetRelease	::=SEQUENCE

{

...

}

-- ===================================================================================

-- Capability exchange definitions: top level capability description

-- ===================================================================================

Capability		::=CHOICE

{

nonStandard	NonStandardParameter,

receiveVideoCapability	VideoCapability,

transmitVideoCapability	VideoCapability,

receiveAndTransmitVideoCapability	VideoCapability,

receiveAudioCapability	AudioCapability,

transmitAudioCapability	AudioCapability,

receiveAndTransmitAudioCapability	AudioCapability,

receiveDataApplicationCapability	DataApplicationCapability,

transmitDataApplicationCapability	DataApplicationCapability,

receiveAndTransmitDataApplicationCapability	DataApplicationCapability,

h233EncryptionTransmitCapability	BOOLEAN,

h233EncryptionReceiveCapability	SEQUENCE

{

h233IVResponseTime	INTEGER (0..255),	-- units milliseconds

...

},

...,

conferenceCapability	ConferenceCapability,

h235SecurityCapability	H235SecurityCapability,

maxPendingReplacementFor	INTEGER (0..255),

receiveUserInputCapability	UserInputCapability,

transmitUserInputCapability	UserInputCapability,

receiveAndTransmitUserInputCapability	UserInputCapability,

genericControlCapability	GenericCapability

}

H235SecurityCapability	::=SEQUENCE

{

encryptionAuthenticationAndIntegrity	EncryptionAuthenticationAndIntegrity,

mediaCapability	CapabilityTableEntryNumber,

-- Note: the mediaCapability shall refer to Capability Table Entries that do contain a transmit,

-- receive, or receiveAndTransmit AudioCapability, VideoCapability,

-- DataApplicationCapability, or similar capability indicated by a NonStandardParameter only

...

}

-- ===================================================================================

-- Capability exchange definitions: Multiplex capabilities

-- ===================================================================================

MultiplexCapability	::=CHOICE

{

nonStandard	NonStandardParameter,

h222Capability	H222Capability,

h223Capability	H223Capability,

v76Capability	V76Capability,

...,

h2250Capability	H2250Capability,

genericMultiplexCapability	GenericCapability

}

H222Capability	::= SEQUENCE

{

numberOfVCs	INTEGER (1..256),

vcCapability	SET OF VCCapability,

...

}

VCCapability	::=SEQUENCE

{

aal1		SEQUENCE

{

nullClockRecovery	BOOLEAN,

srtsClockRecovery	BOOLEAN,

adaptiveClockRecovery	BOOLEAN,

nullErrorCorrection	BOOLEAN,

longInterleaver	BOOLEAN,

shortInterleaver	BOOLEAN,

errorCorrectionOnly	BOOLEAN,

structuredDataTransfer	BOOLEAN,

partiallyFilledCells	BOOLEAN,

...

} OPTIONAL,

aal5		SEQUENCE

{

forwardMaximumSDUSize	INTEGER (0..65535),	-- units octets

backwardMaximumSDUSize	INTEGER (0..65535),	-- units octets

...

} OPTIONAL,

transportStream	BOOLEAN,

programStream	BOOLEAN,

availableBitRates	SEQUENCE

{

type	CHOICE

{

singleBitRate	INTEGER (1..65535),	-- units 64 kbit/s

rangeOfBitRates	SEQUENCE

{

lowerBitRate	INTEGER (1..65535),	-- units 64 kbit/s

higherBitRate	INTEGER (1..65535)	-- units 64 kbit/s

}

},

...

},

...,

aal1ViaGateway	SEQUENCE

{

gatewayAddress	SET SIZE(1..256) OF Q2931Address,

nullClockRecovery	BOOLEAN,

srtsClockRecovery	BOOLEAN,

adaptiveClockRecovery	BOOLEAN,

nullErrorCorrection	BOOLEAN,

longInterleaver	BOOLEAN,

shortInterleaver	BOOLEAN,

errorCorrectionOnly	BOOLEAN,

structuredDataTransfer	BOOLEAN,

partiallyFilledCells	BOOLEAN,

...

} OPTIONAL

}

H223Capability	::=SEQUENCE

{

transportWithI-frames	BOOLEAN,	-- I-frame transport of H.245

videoWithAL1	BOOLEAN,

videoWithAL2	BOOLEAN,

videoWithAL3	BOOLEAN,

audioWithAL1	BOOLEAN,

audioWithAL2	BOOLEAN,

audioWithAL3	BOOLEAN,

dataWithAL1	BOOLEAN,

dataWithAL2	BOOLEAN,

dataWithAL3	BOOLEAN,

maximumAl2SDUSize	INTEGER (0..65535),	-- units octets

maximumAl3SDUSize	INTEGER (0..65535),	-- units octets

maximumDelayJitter	INTEGER (0..1023),	-- units milliseconds

h223MultiplexTableCapability	CHOICE

{

basic	NULL,

enhanced	SEQUENCE

{

maximumNestingDepth	INTEGER (1..15),

maximumElementListSize	INTEGER (2..255),

maximumSubElementListSize	INTEGER (2..255),

...

}

},

...,

maxMUXPDUSizeCapability	BOOLEAN,

nsrpSupport	BOOLEAN,

mobileOperationTransmitCapability	SEQUENCE

{

modeChangeCapability	BOOLEAN,

h223AnnexA	BOOLEAN,

h223AnnexADoubleFlag	BOOLEAN,

h223AnnexB	BOOLEAN,

h223AnnexBwithHeader	BOOLEAN,

...

} OPTIONAL,

h223AnnexCCapability	H223AnnexCCapability OPTIONAL

}

H223AnnexCCapability	::= SEQUENCE

{

videoWithAL1M	BOOLEAN,

videoWithAL2M	BOOLEAN,

videoWithAL3M	BOOLEAN,

audioWithAL1M	BOOLEAN,

audioWithAL2M	BOOLEAN,

audioWithAL3M	BOOLEAN,

dataWithAL1M	BOOLEAN,

dataWithAL2M	BOOLEAN,

dataWithAL3M	BOOLEAN,

alpduInterleaving	BOOLEAN,

maximumAL1MPDUSize	INTEGER (0..65535),	-- units octets

maximumAL2MSDUSize	INTEGER (0..65535),	-- units octets

maximumAL3MSDUSize	INTEGER (0..65535),	-- units octets

...,

rsCodeCapability	BOOLEAN OPTIONAL

}

V76Capability	::=SEQUENCE

{

suspendResumeCapabilitywAddress	BOOLEAN,

suspendResumeCapabilitywoAddress	BOOLEAN,

rejCapability	BOOLEAN,

sREJCapability	BOOLEAN,

mREJCapability	BOOLEAN,

crc8bitCapability	BOOLEAN,

crc16bitCapability	BOOLEAN,

crc32bitCapability	BOOLEAN,

uihCapability	BOOLEAN,

numOfDLCS	INTEGER (2..8191),

twoOctetAddressFieldCapability	BOOLEAN,

loopBackTestCapability	BOOLEAN,

n401Capability	INTEGER (1..4095),

maxWindowSizeCapability	INTEGER (1..127),

v75Capability	V75Capability,

...

}

V75Capability	::=SEQUENCE

{

audioHeader	BOOLEAN,

...

}

H2250Capability	::=SEQUENCE

{

maximumAudioDelayJitter	INTEGER(0..1023),	-- units in milliseconds

receiveMultipointCapability	MultipointCapability,

transmitMultipointCapability	MultipointCapability,

receiveAndTransmitMultipointCapability	MultipointCapability,

mcCapability	SEQUENCE

{

centralizedConferenceMC	BOOLEAN,

decentralizedConferenceMC	BOOLEAN,

...

},

rtcpVideoControlCapability	BOOLEAN,	-- FIR and NACK

mediaPacketizationCapability	MediaPacketizationCapability,

...,

transportCapability	TransportCapability OPTIONAL,

redundancyEncodingCapability	SEQUENCE SIZE(1..256) OF RedundancyEncodingCapability OPTIONAL,

logicalChannelSwitchingCapability	BOOLEAN,

t120DynamicPortCapability	BOOLEAN

}

MediaPacketizationCapability	::=SEQUENCE

{

h261aVideoPacketization	BOOLEAN,

...,

rtpPayloadType	SEQUENCE SIZE(1..256) OF RTPPayloadType OPTIONAL

}

RSVPParameters	::=SEQUENCE

{

qosMode	QOSMode OPTIONAL,

tokenRate	INTEGER (1..4294967295) OPTIONAL,

-- rate in bytes/sec

bucketSize	INTEGER (1..4294967295) OPTIONAL,        -- size in bytes

peakRate	INTEGER (1..4294967295) OPTIONAL,

-- peak bandwidth bytes/sec

minPoliced	INTEGER (1..4294967295) OPTIONAL,        --

maxPktSize	INTEGER (1..4294967295) OPTIONAL,        -- size in bytes

...

}

QOSMode		::=CHOICE

{

guaranteedQOS	NULL,

controlledLoad	NULL,

...

}

-- Ed. Replace with ITU-T ATM Transfer Capability Types

ATMParameters	::=SEQUENCE

{

maxNTUSize	INTEGER(0..65535),        --units in octets

atmUBR	BOOLEAN,                       --unspecified bit rate

atmrtVBR	BOOLEAN,                       --real time variable bit rate

atmnrtVBR	BOOLEAN,                       --non real time variable bit rate

atmABR	BOOLEAN,                       --available bit rate

atmCBR	BOOLEAN,                       --constant bit rate

...

}

QOSCapability	::=SEQUENCE

{

nonStandardData	NonStandardParameter OPTIONAL,

rsvpParameters	RSVPParameters OPTIONAL,

atmParameters	ATMParameters OPTIONAL,

...

}

MediaTransportType	::=CHOICE

{

ip-UDP		NULL,

ip-TCP		NULL,

atm-AAL5-UNIDIR	NULL, -- virtual circuits used as unidirectional

atm-AAL5-BIDIR	NULL, -- virtual circuits used as bidirectional

...,

atm-AAL5-compressed	SEQUENCE

{

variable-delta	BOOLEAN,

...

}

}

MediaChannelCapability	::=SEQUENCE

{

mediaTransport	MediaTransportType  OPTIONAL,

...

}

TransportCapability	::=SEQUENCE

{

nonStandard            	NonStandardParameter OPTIONAL,

qOSCapabilities	SEQUENCE SIZE(1..256) OF QOSCapability OPTIONAL,

mediaChannelCapabilities	SEQUENCE SIZE(1..256) OF MediaChannelCapability OPTIONAL,

...

}

RedundancyEncodingCapability	::=SEQUENCE

{

redundancyEncodingMethod	RedundancyEncodingMethod,

primaryEncoding	CapabilityTableEntryNumber,

secondaryEncoding	SEQUENCE SIZE(1..256) OF CapabilityTableEntryNumber OPTIONAL,

...

}

RedundancyEncodingMethod	::=CHOICE

{

nonStandard	NonStandardParameter,

rtpAudioRedundancyEncoding	NULL,

...,

rtpH263VideoRedundancyEncoding	RTPH263VideoRedundancyEncoding

}

RTPH263VideoRedundancyEncoding	::= SEQUENCE

{

numberOfThreads	INTEGER (1..16),

framesBetweenSyncPoints	INTEGER (1..256),

frameToThreadMapping	CHOICE

{

roundrobin	NULL,

custom	SEQUENCE SIZE(1..256) OF

RTPH263VideoRedundancyFrameMapping,

-- empty SEQUENCE for capability negotiation

-- meaningful contents only OpenLogicalChannel

...

},

containedThreads	SEQUENCE SIZE(1..256) OF INTEGER (0..15) OPTIONAL,

-- only used for opening of logical channels

...

}

RTPH263VideoRedundancyFrameMapping	::= SEQUENCE

{

threadNumber	INTEGER (0..15),

frameSequence	SEQUENCE SIZE(1..256) OF INTEGER (0..255),

...

}

MultipointCapability	::=SEQUENCE

{

multicastCapability	BOOLEAN,

multiUniCastConference	BOOLEAN,

mediaDistributionCapability	SEQUENCE OF MediaDistributionCapability,

...

}

MediaDistributionCapability	::=SEQUENCE

{

centralizedControl	BOOLEAN,

distributedControl	BOOLEAN,	-- for further study in H.323

centralizedAudio	BOOLEAN,

distributedAudio	BOOLEAN,

centralizedVideo	BOOLEAN,

distributedVideo	BOOLEAN,

centralizedData	SEQUENCE OF DataApplicationCapability OPTIONAL,

distributedData	SEQUENCE OF DataApplicationCapability OPTIONAL,

-- for further study in H.323

...

}

-- ===================================================================================

-- Capability exchange definitions: Video capabilities

-- ===================================================================================

VideoCapability	::=CHOICE

{

nonStandard	NonStandardParameter ,

h261VideoCapability	H261VideoCapability,

h262VideoCapability	H262VideoCapability,

h263VideoCapability	H263VideoCapability,

is11172VideoCapability	IS11172VideoCapability,

...,

genericVideoCapability	GenericCapability

}

H261VideoCapability	::=SEQUENCE

{

qcifMPI	INTEGER (1..4) OPTIONAL,	-- units 1/29.97 Hz

cifMPI		INTEGER (1..4) OPTIONAL,	-- units 1/29.97 Hz

temporalSpatialTradeOffCapability	BOOLEAN,

maxBitRate	INTEGER (1..19200),	-- units of 100 bit/s

stillImageTransmission	BOOLEAN,	-- Annex D of H.261

...

}

H262VideoCapability	::=SEQUENCE

{

profileAndLevel-SPatML	BOOLEAN,

profileAndLevel-MPatLL	BOOLEAN,

profileAndLevel-MPatML	BOOLEAN,

profileAndLevel-MPatH-14	BOOLEAN,

profileAndLevel-MPatHL	BOOLEAN,

profileAndLevel-SNRatLL	BOOLEAN,

profileAndLevel-SNRatML	BOOLEAN,

profileAndLevel-SpatialatH-14	BOOLEAN,

profileAndLevel-HPatML	BOOLEAN,

profileAndLevel-HPatH-14	BOOLEAN,

profileAndLevel-HPatHL	BOOLEAN,

videoBitRate	INTEGER (0.. 1073741823) OPTIONAL,	-- units 400 bit/s

vbvBufferSize	INTEGER (0.. 262143) OPTIONAL,	-- units 16384 bits

samplesPerLine	INTEGER (0..16383) OPTIONAL,	-- units samples/line

linesPerFrame	INTEGER (0..16383) OPTIONAL,	-- units lines/frame

framesPerSecond	INTEGER (0..15) OPTIONAL,	-- frame_rate_code

luminanceSampleRate	INTEGER (0..4294967295) OPTIONAL,	-- units samples/sec

...

}

H263VideoCapability	::=SEQUENCE

{

sqcifMPI	INTEGER (1..32) OPTIONAL,	-- units 1/29.97 Hz

qcifMPI	INTEGER (1..32) OPTIONAL,	-- units 1/29.97 Hz

cifMPI		INTEGER (1..32) OPTIONAL,	-- units 1/29.97 Hz

cif4MPI	INTEGER (1..32) OPTIONAL,	-- units 1/29.97 Hz

cif16MPI	INTEGER (1..32) OPTIONAL,	-- units 1/29.97 Hz

maxBitRate	INTEGER (1..192400),	-- units 100 bit/s

unrestrictedVector	BOOLEAN,

arithmeticCoding	BOOLEAN,

advancedPrediction	BOOLEAN,

pbFrames	BOOLEAN,

temporalSpatialTradeOffCapability	BOOLEAN,

hrd-B		INTEGER (0..524287) OPTIONAL,	-- units 128 bits

bppMaxKb	INTEGER (0..65535) OPTIONAL,	-- units 1024 bits

...,

slowSqcifMPI	INTEGER (1..3600) OPTIONAL,	-- units seconds/frame

slowQcifMPI	INTEGER (1..3600) OPTIONAL, --units seconds/frame

slowCifMPI	INTEGER (1..3600) OPTIONAL, --units seconds/frame

slowCif4MPI	INTEGER (1..3600) OPTIONAL, --units seconds/frame

slowCif16MPI	INTEGER (1..3600) OPTIONAL, --units seconds/frame

errorCompensation	BOOLEAN,

enhancementLayerInfo	EnhancementLayerInfo OPTIONAL,

h263Options	H263Options OPTIONAL

}

EnhancementLayerInfo	::=SEQUENCE

{

baseBitRateConstrained	BOOLEAN,

snrEnhancement 	SET SIZE(1..14) OF EnhancementOptions OPTIONAL,

spatialEnhancement 	SET SIZE(1..14) OF EnhancementOptions OPTIONAL,

bPictureEnhancement	SET SIZE(1..14) OF BEnhancementParameters OPTIONAL,

...

}

BEnhancementParameters	::=SEQUENCE

{

enhancementOptions	EnhancementOptions,

numberOfBPictures	INTEGER (1..64),

...

}

EnhancementOptions	::=SEQUENCE

{

sqcifMPI	INTEGER (1..32) OPTIONAL,  -- units 1/29.97 Hz

qcifMPI	INTEGER (1..32) OPTIONAL,  -- units 1/29.97 Hz

cifMPI		INTEGER (1..32) OPTIONAL,  -- units 1/29.97 Hz

cif4MPI 	INTEGER (1..32) OPTIONAL,  -- units 1/29.97 Hz

cif16MPI	INTEGER (1..32) OPTIONAL,  -- units 1/29.97 Hz

maxBitRate	INTEGER (1..192400),             -- units 100 bit/s

unrestrictedVector	BOOLEAN,

arithmeticCoding	BOOLEAN,

temporalSpatialTradeOffCapability	BOOLEAN,

slowSqcifMPI	INTEGER (1..3600) OPTIONAL,  -- units seconds/frame

slowQcifMPI	INTEGER (1..3600) OPTIONAL,   --units seconds/frame

slowCifMPI	INTEGER (1..3600) OPTIONAL,   --units seconds/frame

slowCif4MPI	INTEGER (1..3600) OPTIONAL,   --units seconds/frame

slowCif16MPI	INTEGER (1..3600) OPTIONAL,   --units seconds/frame

errorCompensation	BOOLEAN,

h263Options	H263Options OPTIONAL,

...

}

H263Options	::= SEQUENCE

{

advancedIntraCodingMode	BOOLEAN,

deblockingFilterMode	BOOLEAN,

improvedPBFramesMode	BOOLEAN,

unlimitedMotionVectors	BOOLEAN,

fullPictureFreeze	BOOLEAN,

partialPictureFreezeAndRelease	BOOLEAN,

resizingPartPicFreezeAndRelease	BOOLEAN,

fullPictureSnapshot	BOOLEAN,

partialPictureSnapshot	BOOLEAN,

videoSegmentTagging	BOOLEAN,

progressiveRefinement	BOOLEAN,

dynamicPictureResizingByFour	BOOLEAN,

dynamicPictureResizingSixteenthPel	BOOLEAN,

dynamicWarpingHalfPel	BOOLEAN,

dynamicWarpingSixteenthPel	BOOLEAN,

independentSegmentDecoding	BOOLEAN,

slicesInOrder-NonRect	BOOLEAN,

slicesInOrder-Rect	BOOLEAN,

slicesNoOrder-NonRect	BOOLEAN,

slicesNoOrder-Rect	BOOLEAN,

alternateInterVLCMode	BOOLEAN,

modifiedQuantizationMode	BOOLEAN,

reducedResolutionUpdate	BOOLEAN,

transparencyParameters	TransparencyParameters OPTIONAL,

separateVideoBackChannel	BOOLEAN,

refPictureSelection	RefPictureSelection OPTIONAL,

customPictureClockFrequency	SET SIZE (1..16) OF CustomPictureClockFrequency	 OPTIONAL,

customPictureFormat	SET SIZE (1..16) OF CustomPictureFormat OPTIONAL,

modeCombos	SET SIZE (1..16) OF H263VideoModeCombos OPTIONAL,

...

}

TransparencyParameters	::= SEQUENCE

{

presentationOrder	INTEGER(1..256),

offset-x	INTEGER(-262144..262143), -- 1/8 pixels

offset-y	INTEGER(-262144..262143), -- 1/8 pixels

scale-x		INTEGER(1..255),

scale-y		INTEGER(1..255),

...

}

RefPictureSelection	::=SEQUENCE

{

additionalPictureMemory	SEQUENCE

{

sqcifAdditionalPictureMemory	INTEGER (1..256) OPTIONAL,	-- units frame

qcifAdditionalPictureMemory	INTEGER (1..256) OPTIONAL,	-- units frame

cifAdditionalPictureMemory	INTEGER (1..256) OPTIONAL,	-- units frame

cif4AdditionalPictureMemory	INTEGER (1..256) OPTIONAL,	-- units frame

cif16AdditionalPictureMemory	INTEGER (1..256) OPTIONAL,	-- units frame

bigCpfAdditionalPictureMemory	INTEGER (1..256) OPTIONAL,	-- units frame

...

} OPTIONAL,

videoMux	BOOLEAN,

videoBackChannelSend	CHOICE

{

none	NULL,

ackMessageOnly	NULL,

nackMessageOnly	NULL,

ackOrNackMessageOnly	NULL,

ackAndNackMessage	NULL,

...

},

...

}

CustomPictureClockFrequency	::=SEQUENCE

{

clockConversionCode	INTEGER(1000..1001),

clockDivisor	INTEGER(1..127),

sqcifMPI	INTEGER (1..2048) OPTIONAL,

qcifMPI	INTEGER (1..2048) OPTIONAL,

cifMPI		INTEGER (1..2048) OPTIONAL,

cif4MPI	INTEGER (1..2048) OPTIONAL,

cif16MPI	INTEGER (1..2048) OPTIONAL,

...

}

CustomPictureFormat	::=SEQUENCE

{

maxCustomPictureWidth	INTEGER(1..2048),	-- units 4 pixels

maxCustomPictureHeight	INTEGER(1..2048),	-- units 4 pixels

minCustomPictureWidth	INTEGER(1..2048),	-- units 4 pixels

minCustomPictureHeight	INTEGER(1..2048),	-- units 4 pixels

mPI			SEQUENCE

{

standardMPI	INTEGER (1..31) OPTIONAL,

customPCF	SET SIZE (1..16) OF SEQUENCE

{

clockConversionCode	INTEGER (1000..1001),

clockDivisor	INTEGER (1..127),

customMPI	INTEGER (1..2048),

...

} OPTIONAL,

...

},

pixelAspectInformation	CHOICE

{

anyPixelAspectRatio	BOOLEAN,

pixelAspectCode	SET SIZE (1..14) OF INTEGER(1..14),

extendedPAR	SET SIZE (1..256) OF SEQUENCE

{

width	INTEGER(1..255),

height	INTEGER(1..255),

...

},

...

} ,

...

}

H263VideoModeCombos	::= SEQUENCE

{

h263VideoUncoupledModes	H263ModeComboFlags,

h263VideoCoupledModes	SET SIZE (1..16) OF H263ModeComboFlags,

...

}

H263ModeComboFlags	::= SEQUENCE

{

unrestrictedVector	BOOLEAN,

arithmeticCoding	BOOLEAN,

advancedPrediction	BOOLEAN,

pbFrames	BOOLEAN,

advancedIntraCodingMode	BOOLEAN,

deblockingFilterMode	BOOLEAN,

unlimitedMotionVectors	BOOLEAN,

slicesInOrder-NonRect	BOOLEAN,

slicesInOrder-Rect	BOOLEAN,

slicesNoOrder-NonRect	BOOLEAN,

slicesNoOrder-Rect	BOOLEAN,

improvedPBFramesMode	BOOLEAN,

referencePicSelect	BOOLEAN,

dynamicPictureResizingByFour	BOOLEAN,

dynamicPictureResizingSixteenthPel	BOOLEAN,

dynamicWarpingHalfPel	BOOLEAN,

dynamicWarpingSixteenthPel	BOOLEAN,

reducedResolutionUpdate	BOOLEAN,

independentSegmentDecoding	BOOLEAN,

alternateInterVLCMode	BOOLEAN,

modifiedQuantizationMode	BOOLEAN,

...

}

IS11172VideoCapability	::=SEQUENCE

{

constrainedBitstream	BOOLEAN,

videoBitRate	INTEGER (0.. 1073741823) OPTIONAL,	-- units 400 bit/s

vbvBufferSize	INTEGER (0.. 262143) OPTIONAL,	-- units 16384 bits

samplesPerLine	INTEGER (0..16383) OPTIONAL,	-- units samples/line

linesPerFrame	INTEGER (0..16383) OPTIONAL,	-- units lines/frame

pictureRate	INTEGER (0..15) OPTIONAL,

luminanceSampleRate	INTEGER (0..4294967295) OPTIONAL,	-- units samples/sec

...

}

-- ===================================================================================

-- Capability exchange definitions: Audio capabilities

-- ===================================================================================

-- For an H.222 multiplex, the integers indicate the size of the STD buffer in units of 256 octets

-- For an H.223 multiplex, the integers indicate the maximum number of audio frames per AL-SDU

-- For an H.225.0 multiplex, the integers indicate the maximum number of audio frames per packet

AudioCapability	::=CHOICE

{

nonStandard	NonStandardParameter,

g711Alaw64k	INTEGER (1..256),

g711Alaw56k	INTEGER (1..256),

g711Ulaw64k	INTEGER (1..256),

g711Ulaw56k	INTEGER (1..256),

g722-64k	INTEGER (1..256),

g722-56k	INTEGER (1..256),

g722-48k	INTEGER (1..256),

g7231		SEQUENCE

{

maxAl-sduAudioFrames	INTEGER (1..256),

silenceSuppression	BOOLEAN

},

g728		INTEGER (1..256),

g729		INTEGER (1..256),

g729AnnexA	INTEGER (1..256),

is11172AudioCapability	IS11172AudioCapability,

is13818AudioCapability	IS13818AudioCapability,

...,

g729wAnnexB	INTEGER(1..256),

g729AnnexAwAnnexB	INTEGER(1..256),

g7231AnnexCCapability	G7231AnnexCCapability,

gsmFullRate	GSMAudioCapability,

gsmHalfRate	GSMAudioCapability,

gsmEnhancedFullRate	GSMAudioCapability,

genericAudioCapability	GenericCapability,

g729Extensions	G729Extensions

}

G729Extensions	::= SEQUENCE

{

audioUnit	INTEGER (1..256) OPTIONAL,

annexA	BOOLEAN,

annexB	BOOLEAN,

annexD	BOOLEAN,

annexE	BOOLEAN,

annexF	BOOLEAN,

annexG	BOOLEAN,

annexH	BOOLEAN,

...

}

G7231AnnexCCapability	::= SEQUENCE

{

maxAl-sduAudioFrames	INTEGER (1..256),

silenceSuppression	BOOLEAN,

g723AnnexCAudioMode	SEQUENCE

{

highRateMode0	INTEGER (27..78),	-- units octets

highRateMode1	INTEGER (27..78),	-- units octets

lowRateMode0	INTEGER (23..66),	-- units octets

lowRateMode1	INTEGER (23..66),	-- units octets

sidMode0	INTEGER (6..17),	-- units octets

sidMode1	INTEGER (6..17),	-- units octets

...

} OPTIONAL,

...

}

IS11172AudioCapability	::=SEQUENCE

{

audioLayer1	BOOLEAN,

audioLayer2	BOOLEAN,

audioLayer3	BOOLEAN,

audioSampling32k	BOOLEAN,

audioSampling44k1	BOOLEAN,

audioSampling48k	BOOLEAN,

singleChannel	BOOLEAN,

twoChannels	BOOLEAN,

bitRate	INTEGER (1..448),	-- units kbit/s

...

}

IS13818AudioCapability	::=SEQUENCE

{

audioLayer1	BOOLEAN,

audioLayer2	BOOLEAN,

audioLayer3	BOOLEAN,

audioSampling16k	BOOLEAN,

audioSampling22k05	BOOLEAN,

audioSampling24k	BOOLEAN,

audioSampling32k	BOOLEAN,

audioSampling44k1	BOOLEAN,

audioSampling48k	BOOLEAN,

singleChannel	BOOLEAN,

twoChannels	BOOLEAN,

threeChannels2-1	BOOLEAN,

threeChannels3-0	BOOLEAN,

fourChannels2-0-2-0	BOOLEAN,

fourChannels2-2	BOOLEAN,

fourChannels3-1	BOOLEAN,

fiveChannels3-0-2-0	BOOLEAN,

fiveChannels3-2	BOOLEAN,

lowFrequencyEnhancement	BOOLEAN,

multilingual	BOOLEAN,

bitRate	INTEGER (1..1130),	-- units kbit/s

...

}

GSMAudioCapability	::= SEQUENCE

{

audioUnitSize	INTEGER (1..256),

comfortNoise	BOOLEAN,

scrambled	BOOLEAN,

...

}

-- ===================================================================================

-- Capability exchange definitions: Data capabilities

-- ===================================================================================

DataApplicationCapability	::=SEQUENCE

{

application	CHOICE

{

nonStandard	NonStandardParameter,

t120	DataProtocolCapability,

dsm-cc	DataProtocolCapability,

userData	DataProtocolCapability,

t84	SEQUENCE

{

t84Protocol	DataProtocolCapability,

t84Profile	T84Profile

},

t434	DataProtocolCapability,

h224	DataProtocolCapability,

nlpid	SEQUENCE

{

nlpidProtocol	DataProtocolCapability,

nlpidData	OCTET STRING

},

dsvdControl	NULL,

h222DataPartitioning	DataProtocolCapability,

...,

t30fax	DataProtocolCapability,

t140	DataProtocolCapability,

t38fax	SEQUENCE

{

t38FaxProtocol	DataProtocolCapability,

t38FaxProfile	T38FaxProfile

},

genericDataCapability	GenericCapability

},

maxBitRate	INTEGER (0..4294967295),	-- units 100 bit/s

...

}

DataProtocolCapability	::=CHOICE

{

nonStandard	NonStandardParameter,

v14buffered	NULL,

v42lapm	NULL,	-- may negotiate to V.42bis

hdlcFrameTunnelling	NULL,

h310SeparateVCStack	NULL,

h310SingleVCStack	NULL,

transparent	NULL,

...,

segmentationAndReassembly	NULL,

hdlcFrameTunnelingwSAR	NULL,

v120		NULL,	-- as in H.230

separateLANStack	NULL,

v76wCompression         	CHOICE

{

transmitCompression     	CompressionType,

receiveCompression  	CompressionType,

transmitAndReceiveCompression 	CompressionType,

...

},

tcp			NULL,

udp			NULL

}

CompressionType 	::=CHOICE

{

v42bis		V42bis,

...

}

V42bis			::=SEQUENCE

{

numberOfCodewords	INTEGER (1..65536),

maximumStringLength	INTEGER (1..256),

...

}

T84Profile		::=CHOICE

{

t84Unrestricted	NULL,

t84Restricted	SEQUENCE

{
		qcif	BOOLEAN,

cif		BOOLEAN,

ccir601Seq	BOOLEAN,

ccir601Prog	BOOLEAN,

hdtvSeq	BOOLEAN,

hdtvProg	BOOLEAN,

g3FacsMH200x100	BOOLEAN,

g3FacsMH200x200	BOOLEAN,

g4FacsMMR200x100	BOOLEAN,

g4FacsMMR200x200	BOOLEAN,

jbig200x200Seq	BOOLEAN,

jbig200x200Prog	BOOLEAN,

jbig300x300Seq	BOOLEAN,

jbig300x300Prog	BOOLEAN,

digPhotoLow	BOOLEAN,

digPhotoMedSeq	BOOLEAN,

digPhotoMedProg	BOOLEAN,

digPhotoHighSeq	BOOLEAN,

digPhotoHighProg	BOOLEAN,

...

}

}

T38FaxProfile	::=SEQUENCE

{

fillBitRemoval	BOOLEAN,

transcodingJBIG	BOOLEAN,

transcodingMMR	BOOLEAN,

...,

version	INTEGER (0..255),

-- Version 0, the default, refers to T.38 (1998)

t38FaxRateManagement	T38FaxRateManagement,

-- The default Data Rate Management is

-- determined by the choice of

-- DataProtocolCapability

t38FaxUdpOptions	T38FaxUdpOptions OPTIONAL

-- For UDP,  t38UDPRedundancy is the default

}

T38FaxRateManagement	::= CHOICE

{

localTCF	NULL,

transferredTCF	NULL,

...

}

T38FaxUdpOptions  	::= SEQUENCE

{

t38FaxMaxBuffer	INTEGER OPTIONAL,

t38FaxMaxDatagram	INTEGER OPTIONAL,

t38FaxUdpEC	CHOICE

{

t38UDPFEC	NULL,

t38UDPRedundancy	NULL,

...

}

}

-- ===================================================================================

-- Encryption Capability Definitions:

-- ===================================================================================

EncryptionAuthenticationAndIntegrity	::=SEQUENCE

{

encryptionCapability	EncryptionCapability OPTIONAL,

authenticationCapability	AuthenticationCapability OPTIONAL,

integrityCapability	IntegrityCapability OPTIONAL,

...

}

EncryptionCapability	::=SEQUENCE SIZE(1..256) OF MediaEncryptionAlgorithm

MediaEncryptionAlgorithm	 ::=CHOICE

{

nonStandard            	NonStandardParameter,

algorithm	OBJECT IDENTIFIER,   -- many defined in ISO/IEC 9979

...

}

AuthenticationCapability	::=SEQUENCE

{

nonStandard            	NonStandardParameter OPTIONAL,

...

}

IntegrityCapability	::=SEQUENCE

{

nonStandard            	NonStandardParameter OPTIONAL,

...

}

-- ================================================================================

-- Capability Exchange Definitions: UserInput

-- ================================================================================

UserInputCapability	::= CHOICE

{

nonStandard	SEQUENCE SIZE(1..16) OF NonStandardParameter,

basicString	NULL,  -- alphanumeric

iA5String	NULL,  -- alphanumeric

generalString	NULL,  -- alphanumeric

dtmf		NULL,  -- supports dtmf using signal and signalUpdate

hookflash	NULL,  -- supports hookflash using signal

...

}

-- ===================================================================================

-- Capability Exchange Definitions: Conference

-- ===================================================================================

ConferenceCapability	::=SEQUENCE

{

nonStandardData	SEQUENCE OF NonStandardParameter OPTIONAL,

chairControlCapability	BOOLEAN,

...,

VideoIndicateMixingCapability	BOOLEAN,

multipointVisualizationCapability	BOOLEAN OPTIONAL	-- same as H.230 MVC

}

-- ===================================================================================

-- Capability Exchange Definitions: Generic Capability

-- ===================================================================================

GenericCapability	::=SEQUENCE

{

capabilityIdentifier	CapabilityIdentifier,

maxBitRate	INTEGER (0..4294967295) OPTIONAL,	-- Units 100 bit/s

collapsing	SEQUENCE OF GenericParameter  OPTIONAL,

nonCollapsing	SEQUENCE OF GenericParameter  OPTIONAL,

nonCollapsingRaw	OCTET STRING  OPTIONAL,	-- Typically contains ASN.1

-- PER encoded data describing capability

transport	DataProtocolCapability  OPTIONAL,

...

}

CapabilityIdentifier	::=CHOICE

{

standard	OBJECT IDENTIFIER,

-- e.g. { itu-t (0) recommendation (0) h (8) 267

-- version (0) 2 subIdentifier (0)}

h221NonStandard	NonStandardParameter,

uuid		OCTET STRING ( SIZE (16) ),

domainBased	IA5String ( SIZE (1..64) ),

...

}

-- Note. The ranges of parameter values have been selected to ensure that the GenericParameter

-- preamble, standard part of ParameterIdentifier and the encoding of that choice, and the preamble

-- of ParameterValue to fit into 2 octets.

GenericParameter	::=SEQUENCE

{

parameterIdentifier	ParameterIdentifier,

parameterValue	ParameterValue,

supersedes	SEQUENCE OF ParameterIdentifier OPTIONAL,

...

}

ParameterIdentifier	::=CHOICE

{

standard	INTEGER (0..127),  -- Assigned by Capability specifications

h221NonStandard	NonStandardParameter,  -- N.B. NonStandardIdentifier is

-- not sufficient in this case

uuid		OCTET STRING ( SIZE (16) ),	-- For non-standard

domainBased	IA5String ( SIZE (1..64) ),

...

}

ParameterValue	::=CHOICE

{

logical		NULL,	-- Only acceptable if all

-- entities include this option

booleanArray	INTEGER (0..255),	-- array of 8 logical types

unsignedMin	INTEGER (0..65535),	-- Look for min common value

unsignedMax	INTEGER (0..65535),	-- Look for max common value

unsigned32Min	INTEGER (0..4294967295),	-- Look for min common value

unsigned32Max	INTEGER (0..4294967295),	-- Look for max common value

octetString	OCTET STRING,	-- non-collapsing octet string

genericParameter	SEQUENCE OF GenericParameter,

...

}

-- ===================================================================================

-- Logical channel signalling definitions

-- ===================================================================================

-- "Forward" is used to refer to transmission in the direction from the terminal making the

-- original request for a logical channel to the other terminal, and "reverse" is used to refer

-- to the opposite direction of transmission, in the case of a bi-directional channel request.

OpenLogicalChannel	::=SEQUENCE

{

forwardLogicalChannelNumber	LogicalChannelNumber,

forwardLogicalChannelParameters	SEQUENCE

{

portNumber	INTEGER (0..65535) OPTIONAL,

dataType	DataType,

multiplexParameters	CHOICE

{

h222LogicalChannelParameters	H222LogicalChannelParameters,

h223LogicalChannelParameters	H223LogicalChannelParameters,

v76LogicalChannelParameters	V76LogicalChannelParameters,

...,

h2250LogicalChannelParameters	H2250LogicalChannelParameters,

none	NULL  -- for use with Separate Stack when

-- multiplexParameters are not required

-- or appropriate

},

...,

forwardLogicalChannelDependency	LogicalChannelNumber OPTIONAL,

-- also used to refer to the primary logical channel when using video redundancy coding

replacementFor	LogicalChannelNumber OPTIONAL

},

-- Used to specify the reverse channel for bi-directional open request

reverseLogicalChannelParameters	SEQUENCE

{

dataType	DataType,

multiplexParameters	CHOICE

{

-- H.222 parameters are never present in reverse direction

h223LogicalChannelParameters	H223LogicalChannelParameters,

v76LogicalChannelParameters	V76LogicalChannelParameters,

...,

h2250LogicalChannelParameters H2250LogicalChannelParameters

} OPTIONAL,	-- Not present for H.222

...,

reverseLogicalChannelDependency	LogicalChannelNumber OPTIONAL,

-- also used to refer to the primary logical channel when using video redundancy coding

replacementFor	LogicalChannelNumber OPTIONAL

} OPTIONAL,	-- Not present for uni-directional channel request

...,

separateStack	NetworkAccessParameters OPTIONAL,

-- for Open responder to establish the stack

encryptionSync	EncryptionSync OPTIONAL	-- used only by Master

}

LogicalChannelNumber	::=INTEGER (1..65535)

NetworkAccessParameters	::=SEQUENCE

{

distribution	CHOICE

{

unicast	NULL,

multicast	NULL,	-- For Further Study in T.120

...

} OPTIONAL,

networkAddress	CHOICE

{

q2931Address	Q2931Address,

e164Address	IA5String(SIZE(1..128)) (FROM ("0123456789#*,")),

localAreaAddress	TransportAddress,

...

},

associateConference	BOOLEAN,

externalReference	OCTET STRING(SIZE(1..255)) OPTIONAL,

...,

t120SetupProcedure	CHOICE

{

originateCall	NULL,

waitForCall	NULL,

issueQuery	NULL,

...

} OPTIONAL

}

Q2931Address	::=SEQUENCE

{

address	CHOICE

{

internationalNumber	NumericString(SIZE(1..16)),

nsapAddress	OCTET STRING (SIZE(1..20)),

...

},

subaddress	OCTET STRING (SIZE(1..20)) OPTIONAL,

...

}

V75Parameters	::= SEQUENCE

{

audioHeaderPresent	BOOLEAN,

...

}

DataType		::=CHOICE

{

nonStandard	NonStandardParameter,

nullData	NULL,

videoData	VideoCapability,

audioData	AudioCapability,

data		DataApplicationCapability,

encryptionData	EncryptionMode,

...,

h235Control	NonStandardParameter,

h235Media	H235Media

}

H235Media		::=SEQUENCE

{

encryptionAuthenticationAndIntegrity	EncryptionAuthenticationAndIntegrity,

mediaType	CHOICE

{

nonStandard	NonStandardParameter,

videoData	VideoCapability,

audioData	AudioCapability,

data	DataApplicationCapability,

...

},

...

}

H222LogicalChannelParameters	::=SEQUENCE

{

resourceID	INTEGER (0..65535),

subChannelID	INTEGER (0..8191),

pcr-pid	INTEGER (0..8191) OPTIONAL,

programDescriptors	OCTET STRING OPTIONAL,

streamDescriptors	OCTET STRING OPTIONAL,

...

}

H223LogicalChannelParameters	::=SEQUENCE

{

adaptationLayerType	CHOICE

{

nonStandard	NonStandardParameter,

al1Framed	NULL,

al1NotFramed	NULL,

al2WithoutSequenceNumbers	NULL,

al2WithSequenceNumbers	NULL,

al3		SEQUENCE

{

controlFieldOctets	INTEGER (0..2),

sendBufferSize	INTEGER (0..16777215)	-- units octets

},

...,

al1M	H223AL1MParameters,

al2M	H223AL2MParameters,

al3M	H223AL3MParameters

},

segmentableFlag	BOOLEAN,

...

}

H223AL1MParameters	::=SEQUENCE

{

transferMode	CHOICE

{

framed	NULL,

unframed	NULL,

...

},

headerFEC	CHOICE

{

sebch16-7	NULL,

golay24-12	NULL,

...

},

crcLength	CHOICE

{

crc4bit	NULL,

crc12bit	NULL,

crc20bit	NULL,

crc28bit	NULL,

...,

crc8bit	NULL,

crc16bit	NULL,

crc32bit	NULL,

crcNotUsed	NULL

},

rcpcCodeRate	INTEGER (8..32),

arqType	CHOICE

{

noArq	NULL,

typeIArq	H223AnnexCArqParameters,

typeIIArq	H223AnnexCArqParameters,

...

},

alpduInterleaving	BOOLEAN,

alsduSplitting	BOOLEAN,

...,

rsCodeCorrection	INTEGER (0..127) OPTIONAL

}

H223AL2MParameters	::=SEQUENCE

{

headerFEC	CHOICE

{

sebch16-5	NULL,

golay24-12	NULL,

...

},

alpduInterleaving	BOOLEAN,

...

}

H223AL3MParameters	::=SEQUENCE

{

headerFormat	CHOICE

{

sebch16-7	NULL,

golay24-12	NULL,

...

},

crcLength	CHOICE

{

crc4bit	NULL,

crc12bit	NULL,

crc20bit	NULL,

crc28bit	NULL,

...,

crc8bit	NULL,

crc16bit	NULL,

crc32bit	NULL,

crcNotUsed	NULL

},

rcpcCodeRate	INTEGER (8..32),

arqType	CHOICE

{

noArq	NULL,

typeIArq	H223AnnexCArqParameters,

typeIIArq	H223AnnexCArqParameters,

...

},

alpduInterleaving	BOOLEAN,

...,

rsCodeCorrection	INTEGER (0..127) OPTIONAL

}

H223AnnexCArqParameters	::=SEQUENCE

{

numberOfRetransmissions	CHOICE

{

finite	INTEGER (0..16),

infinite	NULL,

...

},

sendBufferSize	INTEGER (0..16777215),	-- units octets

...

}

V76LogicalChannelParameters	::=SEQUENCE

{

hdlcParameters	V76HDLCParameters,

suspendResume	CHOICE

{

noSuspendResume	NULL,

suspendResumewAddress	NULL,

suspendResumewoAddress    	NULL,

...

},

uIH			BOOLEAN,

mode		CHOICE

{

eRM	SEQUENCE

{

windowSize	INTEGER (1..127) ,

recovery	CHOICE

{

rej	NULL,

sREJ	NULL,

mSREJ	NULL,

...

},

...

},

uNERM	NULL,

...

},

v75Parameters	V75Parameters,

...

}

V76HDLCParameters	::=SEQUENCE

{

crcLength	CRCLength,

n401		INTEGER (1..4095),

loopbackTestProcedure	BOOLEAN,

...

}

CRCLength	::=CHOICE

{

crc8bit		NULL,

crc16bit	NULL,

crc32bit	NULL,

...

}

H2250LogicalChannelParameters	::=SEQUENCE

{

nonStandard	SEQUENCE OF NonStandardParameter OPTIONAL,

sessionID	INTEGER(0..255),

associatedSessionID	INTEGER(1..255) OPTIONAL,

mediaChannel	TransportAddress OPTIONAL,

mediaGuaranteedDelivery	BOOLEAN OPTIONAL,

mediaControlChannel	TransportAddress OPTIONAL,  -- reverse RTCP channel

mediaControlGuaranteedDelivery	BOOLEAN OPTIONAL,

silenceSuppression	BOOLEAN OPTIONAL,

destination	TerminalLabel OPTIONAL,

dynamicRTPPayloadType	INTEGER(96..127) OPTIONAL,

mediaPacketization	CHOICE

{

h261aVideoPacketization	NULL,

...,

rtpPayloadType	RTPPayloadType

} OPTIONAL,

...,

transportCapability	TransportCapability OPTIONAL,

redundancyEncoding	RedundancyEncoding OPTIONAL,

source		TerminalLabel OPTIONAL,

symmetricMediaAddress	BOOLEAN

}

RTPPayloadType	::= SEQUENCE

{

payloadDescriptor	CHOICE

{

nonStandardIdentifier	NonStandardParameter,

rfc-number	INTEGER (1..32768, ...),

oid	OBJECT IDENTIFIER,

...

},

payloadType	INTEGER (0..127) OPTIONAL,

...

}

RedundancyEncoding	::=SEQUENCE

{

redundancyEncodingMethod	RedundancyEncodingMethod,

secondaryEncoding	DataType OPTIONAL,	-- depends on method

...

}

TransportAddress	::=CHOICE

{

unicastAddress	UnicastAddress,

multicastAddress	MulticastAddress,

...

}

UnicastAddress	::=CHOICE

{

iPAddress	SEQUENCE

{

network	OCTET STRING (SIZE(4)),

tsapIdentifier	INTEGER(0..65535),

...

},

iPXAddress	SEQUENCE

{

node	OCTET STRING (SIZE(6)),

netnum	OCTET STRING (SIZE(4)),

tsapIdentifier	OCTET STRING (SIZE(2)),

...

},

iP6Address	SEQUENCE

{

network	OCTET STRING (SIZE(16)),

tsapIdentifier	INTEGER(0..65535),

...

},

netBios	OCTET STRING (SIZE(16)),

iPSourceRouteAddress	SEQUENCE

{

routing	CHOICE

{

strict	NULL,

loose	NULL

},

network	OCTET STRING (SIZE(4)),

tsapIdentifier	INTEGER(0..65535),

route	SEQUENCE OF OCTET STRING (SIZE(4)),

...

},

...,

nsap		OCTET STRING (SIZE(1..20)),

nonStandardAddress	NonStandardParameter

}

MulticastAddress	::=CHOICE

{

iPAddress	SEQUENCE

{

network	OCTET STRING (SIZE(4)),

tsapIdentifier	INTEGER(0..65535),

...

},

iP6Address	SEQUENCE

{

network	OCTET STRING (SIZE(16)),

tsapIdentifier	INTEGER(0..65535),

...

},

...,

nsap		OCTET STRING (SIZE(1..20)),

nonStandardAddress	NonStandardParameter

}

EncryptionSync	::=SEQUENCE

-- used to supply new key and synchronization point

{

nonStandard            	NonStandardParameter OPTIONAL,

synchFlag	INTEGER(0..255) , 	-- may need to be larger for H.324, etc

-- shall be the Dynamic Payload# for H.323

h235Key	OCTET STRING (SIZE(1..65535)),  -- H.235 encoded value

escrowentry	SEQUENCE SIZE(1..256) OF EscrowData OPTIONAL,

...

}

EscrowData	::=SEQUENCE

{

escrowID	OBJECT IDENTIFIER,

escrowValue	BIT STRING (SIZE(1..65535)),

...

}

OpenLogicalChannelAck	::=SEQUENCE

{

forwardLogicalChannelNumber	LogicalChannelNumber,

reverseLogicalChannelParameters	SEQUENCE

{

reverseLogicalChannelNumber	LogicalChannelNumber,

portNumber	INTEGER (0..65535) OPTIONAL,

multiplexParameters	CHOICE

{

h222LogicalChannelParameters	H222LogicalChannelParameters,

-- H.223 parameters are never present in reverse direction

...,

h2250LogicalChannelParameters H2250LogicalChannelParameters

} OPTIONAL,	-- Not present for H.223

...,

replacementFor	LogicalChannelNumber OPTIONAL

} OPTIONAL,	-- Not present for uni-directional channel request

...,

separateStack	NetworkAccessParameters OPTIONAL,

-- for Open requester to establish the stack

forwardMultiplexAckParameters	CHOICE

{

-- H.222 parameters are never present in the Ack

-- H.223 parameters are never present in the Ack

--V.76 parameters are never present in the Ack

h2250LogicalChannelAckParameters H2250LogicalChannelAckParameters,

...

} OPTIONAL,

encryptionSync	EncryptionSync OPTIONAL	-- used only by Master

}

OpenLogicalChannelReject	::=SEQUENCE

{

forwardLogicalChannelNumber	LogicalChannelNumber,

cause		CHOICE

{

unspecified	NULL,

unsuitableReverseParameters	NULL,

dataTypeNotSupported	NULL,

dataTypeNotAvailable	NULL,

unknownDataType	NULL,

dataTypeALCombinationNotSupported	NULL,

...,

multicastChannelNotAllowed	NULL,

insufficientBandwidth	NULL,

separateStackEstablishmentFailed	NULL,

invalidSessionID	NULL,

masterSlaveConflict	NULL,

waitForCommunicationMode	NULL,

invalidDependentChannel	NULL,

replacementForRejected	NULL

},

...

}

OpenLogicalChannelConfirm	::=SEQUENCE

{

forwardLogicalChannelNumber	LogicalChannelNumber,

...

}

H2250LogicalChannelAckParameters	::=SEQUENCE

{

nonStandard	SEQUENCE OF NonStandardParameter OPTIONAL,

sessionID	INTEGER(1..255) OPTIONAL,

mediaChannel	TransportAddress OPTIONAL,

mediaControlChannel	TransportAddress OPTIONAL, -- forward RTCP channel

dynamicRTPPayloadType	INTEGER(96..127) OPTIONAL, -- used only by the master or MC

...,

flowControlToZero	BOOLEAN,

portNumber	INTEGER (0..65535) OPTIONAL

}

CloseLogicalChannel	::=SEQUENCE

{

forwardLogicalChannelNumber	LogicalChannelNumber,

source		CHOICE

{

user	NULL,

lcse	NULL

},

...,

reason		CHOICE

{

unknown	NULL,

reopen	NULL,

reservationFailure	NULL,

...

}

}

CloseLogicalChannelAck	::=SEQUENCE

{

forwardLogicalChannelNumber	LogicalChannelNumber,

...

}

RequestChannelClose	::=SEQUENCE

{

forwardLogicalChannelNumber	LogicalChannelNumber,

...,

qosCapability	QOSCapability OPTIONAL,

reason		CHOICE

{

unknown	NULL,

normal	NULL,

reopen	NULL,

reservationFailure	NULL,

...

}

}

RequestChannelCloseAck	::=SEQUENCE

{

forwardLogicalChannelNumber	LogicalChannelNumber,

...

}

RequestChannelCloseReject	::=SEQUENCE

{

forwardLogicalChannelNumber	LogicalChannelNumber,

cause		CHOICE

{

unspecified	NULL,

...

},

...

}

RequestChannelCloseRelease	::=SEQUENCE

{

forwardLogicalChannelNumber	LogicalChannelNumber,

...

}

-- ===================================================================================

-- H.223 multiplex table definitions

-- ===================================================================================

MultiplexEntrySend	::=SEQUENCE

{

sequenceNumber	SequenceNumber,

multiplexEntryDescriptors	SET SIZE (1..15) OF MultiplexEntryDescriptor,

...

}

MultiplexEntryDescriptor	::=SEQUENCE

{

multiplexTableEntryNumber	MultiplexTableEntryNumber,

elementList	SEQUENCE SIZE (1..256) OF MultiplexElement OPTIONAL

}

MultiplexElement	::=SEQUENCE

{

type		CHOICE

{

logicalChannelNumber	INTEGER(0..65535),

subElementList	SEQUENCE SIZE (2..255) OF MultiplexElement

},

repeatCount	CHOICE

{

finite	INTEGER (1..65535),	-- repeats of type

untilClosingFlag	NULL	-- used for last element

}

}

MultiplexTableEntryNumber	::=INTEGER (1..15)

MultiplexEntrySendAck	::=SEQUENCE

{

sequenceNumber	SequenceNumber,

multiplexTableEntryNumber	SET SIZE (1..15) OF MultiplexTableEntryNumber,

...

}

MultiplexEntrySendReject	::=SEQUENCE

{

sequenceNumber	SequenceNumber,

rejectionDescriptions	SET SIZE (1..15) OF MultiplexEntryRejectionDescriptions,

...

}

MultiplexEntryRejectionDescriptions	::=SEQUENCE

{

multiplexTableEntryNumber	MultiplexTableEntryNumber,

cause		CHOICE

{

unspecifiedCause	NULL,

descriptorTooComplex	NULL,

...

},

...

}

MultiplexEntrySendRelease	::=SEQUENCE

{

multiplexTableEntryNumber	SET SIZE (1..15) OF MultiplexTableEntryNumber,

...

}

RequestMultiplexEntry	::=SEQUENCE

{

entryNumbers	SET SIZE (1..15) OF MultiplexTableEntryNumber,

...

}

RequestMultiplexEntryAck	::=SEQUENCE

{

entryNumbers	SET SIZE (1..15) OF MultiplexTableEntryNumber,

...

}

RequestMultiplexEntryReject	::=SEQUENCE

{

entryNumbers	SET SIZE (1..15) OF MultiplexTableEntryNumber,

rejectionDescriptions	SET SIZE (1..15) OF RequestMultiplexEntryRejectionDescriptions,

...

}

RequestMultiplexEntryRejectionDescriptions	::=SEQUENCE

{

multiplexTableEntryNumber	MultiplexTableEntryNumber,

cause		CHOICE

{

unspecifiedCause	NULL,

...

},

...

}

RequestMultiplexEntryRelease	::=SEQUENCE

{

entryNumbers	SET SIZE (1..15) OF MultiplexTableEntryNumber,

...

}

-- ===================================================================================

-- Request mode definitions

-- ===================================================================================

-- RequestMode is a list, in order or preference, of modes that a terminal would like

-- to have transmitted to it.

RequestMode	::=SEQUENCE

{

sequenceNumber	SequenceNumber,

requestedModes	SEQUENCE SIZE (1..256) OF ModeDescription,

...

}

RequestModeAck	::=SEQUENCE

{

sequenceNumber	SequenceNumber,

response	CHOICE

{

willTransmitMostPreferredMode	NULL,

willTransmitLessPreferredMode	NULL,

...

},

...

}

RequestModeReject	::=SEQUENCE

{

sequenceNumber	SequenceNumber,

cause		CHOICE

{

modeUnavailable	NULL,

multipointConstraint	NULL,

requestDenied	NULL,

...

},

...

}

RequestModeRelease	::=SEQUENCE

{

...

}

-- ===================================================================================

-- Request mode definitions: Mode description

-- ===================================================================================

ModeDescription	::=SET SIZE (1..256) OF ModeElement

ModeElement	::= SEQUENCE

{

type		CHOICE

{

nonStandard	NonStandardParameter,

videoMode	VideoMode,

audioMode	AudioMode,

dataMode	DataMode,

encryptionMode	EncryptionMode,

...,

h235Mode	H235Mode

},

h223ModeParameters	H223ModeParameters OPTIONAL,

...,

v76ModeParameters	V76ModeParameters OPTIONAL,

h2250ModeParameters	H2250ModeParameters OPTIONAL,

genericModeParameters	GenericCapability OPTIONAL

}

H235Mode		::=SEQUENCE

{

encryptionAuthenticationAndIntegrity	EncryptionAuthenticationAndIntegrity,

mediaMode	CHOICE

{

nonStandard	NonStandardParameter,

videoMode	VideoMode,

audioMode	AudioMode,

dataMode	DataMode,

...

},

...

}

H223ModeParameters	::=SEQUENCE

{

adaptationLayerType	CHOICE

{

nonStandard	NonStandardParameter,

al1Framed	NULL,

al1NotFramed	NULL,

al2WithoutSequenceNumbers	NULL,

al2WithSequenceNumbers	NULL,

al3		SEQUENCE

{

controlFieldOctets	INTEGER(0..2),

sendBufferSize	INTEGER(0..16777215)	-- units octets

},

...,

al1M	H223AL1MParameters,

al2M	H223AL2MParameters,

al3M	H223AL3MParameters

},

segmentableFlag	BOOLEAN,

...

}

V76ModeParameters	::=CHOICE

{

suspendResumewAddress	NULL,

suspendResumewoAddress	NULL,

...

}

H2250ModeParameters	::=SEQUENCE

{

redundancyEncodingMode	RedundancyEncodingMode OPTIONAL,

...

}

RedundancyEncodingMode	::=SEQUENCE

{

redundancyEncodingMethod	RedundancyEncodingMethod,

secondaryEncoding	CHOICE

{

nonStandard	NonStandardParameter,

audioData	AudioMode,

...

} OPTIONAL,

...

}

-- ===================================================================================

-- Request mode definitions: Video modes

-- ===================================================================================

VideoMode		::=CHOICE

{

nonStandard	NonStandardParameter,

h261VideoMode	H261VideoMode,

h262VideoMode	H262VideoMode,

h263VideoMode	H263VideoMode,

is11172VideoMode	IS11172VideoMode,

...,

genericVideoMode	GenericCapability

}

H261VideoMode	::=SEQUENCE

{

resolution	CHOICE

{

qcif	NULL,

cif		NULL

},

bitRate	INTEGER (1..19200),	-- units 100 bit/s

stillImageTransmission	BOOLEAN,

...

}

H262VideoMode	::=SEQUENCE

{

profileAndLevel	CHOICE

{

profileAndLevel-SPatML	NULL,

profileAndLevel-MPatLL	NULL,

profileAndLevel-MPatML	NULL,

profileAndLevel-MPatH-14	NULL,

profileAndLevel-MPatHL	NULL,

profileAndLevel-SNRatLL	NULL,

profileAndLevel-SNRatML	NULL,

profileAndLevel-SpatialatH-14	NULL,

profileAndLevel-HPatML	NULL,

profileAndLevel-HPatH-14	NULL,

profileAndLevel-HPatHL	NULL,

...

},

videoBitRate	INTEGER(0..1073741823) OPTIONAL,	-- units 400bit/s

vbvBufferSize	INTEGER(0..262143) OPTIONAL,	-- units 16384bits

samplesPerLine	INTEGER(0..16383) OPTIONAL,	-- units samples/line

linesPerFrame	INTEGER(0..16383) OPTIONAL,	-- units lines/frame

framesPerSecond	INTEGER(0..15) OPTIONAL,	-- frame_rate_code

luminanceSampleRate	INTEGER(0..4294967295) OPTIONAL,	-- units samples/sec

...

}

H263VideoMode	::=SEQUENCE

{

resolution	CHOICE

{

sqcif	NULL,

qcif	NULL,

cif		NULL,

cif4	NULL,

cif16	NULL,

...

},

bitRate	INTEGER (1..19200),	-- units 100 bit/s

unrestrictedVector	BOOLEAN,

arithmeticCoding	BOOLEAN,

advancedPrediction	BOOLEAN,

pbFrames	BOOLEAN,

...,

errorCompensation	BOOLEAN,

enhancementLayerInfo	EnhancementLayerInfo OPTIONAL,

h263Options	H263Options OPTIONAL

}

IS11172VideoMode	::=SEQUENCE

{

constrainedBitstream	BOOLEAN,

videoBitRate	INTEGER(0..1073741823) OPTIONAL,	-- units 400bit/s

vbvBufferSize	INTEGER(0..262143) OPTIONAL,	-- units 16384bits

samplesPerLine	INTEGER(0..16383) OPTIONAL,	-- units samples/line

linesPerFrame	INTEGER(0..16383) OPTIONAL,	-- units lines/frame

pictureRate	INTEGER(0..15) OPTIONAL,

luminanceSampleRate	INTEGER(0..4294967295) OPTIONAL,	-- units samples/sec

...

}

-- ===================================================================================

-- Request mode definitions: Audio modes

-- ===================================================================================

AudioMode	::=CHOICE

{

nonStandard	NonStandardParameter,

g711Alaw64k	NULL,

g711Alaw56k	NULL,

g711Ulaw64k	NULL,

g711Ulaw56k	NULL,

g722-64k	NULL,

g722-56k	NULL,

g722-48k	NULL,

g728		NULL,

g729		NULL,

g729AnnexA	NULL,

g7231		CHOICE

{

noSilenceSuppressionLowRate	NULL,

noSilenceSuppressionHighRate	NULL,

silenceSuppressionLowRate	NULL,

silenceSuppressionHighRate	NULL

},

is11172AudioMode	IS11172AudioMode,

is13818AudioMode	IS13818AudioMode,

...,

g729wAnnexB	INTEGER(1..256),

g729AnnexAwAnnexB	INTEGER(1..256),

g7231AnnexCMode	G7231AnnexCMode,

gsmFullRate	GSMAudioCapability,

gsmHalfRate	GSMAudioCapability,

gsmEnhancedFullRate	GSMAudioCapability,

genericAudioMode	GenericCapability,

g729Extensions	G729Extensions

}

IS11172AudioMode	::=SEQUENCE

{

audioLayer	CHOICE

{

audioLayer1	NULL,

audioLayer2	NULL,

audioLayer3	NULL

},

audioSampling	CHOICE

{

audioSampling32k	NULL,

audioSampling44k1	NULL,

audioSampling48k	NULL

},

multichannelType	CHOICE

{

singleChannel	NULL,

twoChannelStereo	NULL,

twoChannelDual	NULL

},

bitRate	INTEGER (1..448),	--units kbit/s

...

}

IS13818AudioMode	::=SEQUENCE

{

audioLayer	CHOICE

{

audioLayer1	NULL,

audioLayer2	NULL,

audioLayer3	NULL

},

audioSampling	CHOICE

{

audioSampling16k	NULL,

audioSampling22k05	NULL,

audioSampling24k	NULL,

audioSampling32k	NULL,

audioSampling44k1	NULL,

audioSampling48k	NULL

},

multichannelType	CHOICE

{

singleChannel	NULL,

twoChannelStereo	NULL,

twoChannelDual	NULL,

threeChannels2-1	NULL,

threeChannels3-0	NULL,

fourChannels2-0-2-0	NULL,

fourChannels2-2	NULL,

fourChannels3-1	NULL,

fiveChannels3-0-2-0	NULL,

fiveChannels3-2	NULL

},

lowFrequencyEnhancement	BOOLEAN,

multilingual	BOOLEAN,

bitRate	INTEGER (1..1130),	--units kbit/s

...

}

G7231AnnexCMode	::= SEQUENCE

{

maxAl-sduAudioFrames	INTEGER (1..256),

silenceSuppression	BOOLEAN,

g723AnnexCAudioMode	SEQUENCE

{

highRateMode0	INTEGER (27..78),	-- units octets

highRateMode1	INTEGER (27..78),	-- units octets

lowRateMode0	INTEGER (23..66),	-- units octets

lowRateMode1	INTEGER (23..66),	-- units octets

sidMode0	INTEGER (6..17),	-- units octets

sidMode1	INTEGER (6..17),	-- units octets

...

},

...

}

-- ===================================================================================

-- Request mode definitions: Data modes

-- ===================================================================================

DataMode		::=SEQUENCE

{

application	CHOICE

{

nonStandard	NonStandardParameter,

t120	DataProtocolCapability,

dsm-cc	DataProtocolCapability,

userData	DataProtocolCapability,

t84	DataProtocolCapability,

t434	DataProtocolCapability,

h224	DataProtocolCapability,

nlpid	SEQUENCE

{

nlpidProtocol	DataProtocolCapability,

nlpidData	OCTET STRING

},

dsvdControl	NULL,

h222DataPartitioning	DataProtocolCapability,

...,

t30fax	DataProtocolCapability,

t140	DataProtocolCapability,

t38fax	SEQUENCE

{

t38FaxProtocol	DataProtocolCapability,

t38FaxProfile	T38FaxProfile

},

genericDataMode	GenericCapability

},

bitRate	INTEGER (0..4294967295),	-- units 100 bit/s

...

}

-- ===================================================================================

-- Request mode definitions: Encryption modes

-- ===================================================================================

EncryptionMode	::=CHOICE

{

nonStandard	NonStandardParameter,

h233Encryption	NULL,

...

}

-- ===================================================================================

-- Round Trip Delay definitions

-- ===================================================================================

RoundTripDelayRequest	::=SEQUENCE

{

sequenceNumber	SequenceNumber,

...

}

RoundTripDelayResponse	::=SEQUENCE

{

sequenceNumber	SequenceNumber,

...

}

-- ===================================================================================

-- Maintenance Loop definitions

-- ===================================================================================

MaintenanceLoopRequest	::=SEQUENCE

{

type		CHOICE

{

systemLoop	NULL,

mediaLoop	LogicalChannelNumber,

logicalChannelLoop	LogicalChannelNumber,

...

},

...

}

MaintenanceLoopAck	::=SEQUENCE

{

type		CHOICE

{

systemLoop	NULL,

mediaLoop	LogicalChannelNumber,

logicalChannelLoop	LogicalChannelNumber,

...

},

...

}

MaintenanceLoopReject	::=SEQUENCE

{

type		CHOICE

{

systemLoop	NULL,

mediaLoop	LogicalChannelNumber,

logicalChannelLoop	LogicalChannelNumber,

...

},

cause		CHOICE

{

canNotPerformLoop	NULL,

...

},

...

}

MaintenanceLoopOffCommand	::=SEQUENCE

{

...

}

-- ===================================================================================

-- Communication Mode definitions

-- ===================================================================================

CommunicationModeCommand	::=SEQUENCE

{

communicationModeTable	SET SIZE(1..256) OF CommunicationModeTableEntry,

...

}

CommunicationModeRequest	::=SEQUENCE

{

...

}

CommunicationModeResponse	::=CHOICE

{

communicationModeTable	SET SIZE(1..256) OF CommunicationModeTableEntry,

...

}

CommunicationModeTableEntry	::=SEQUENCE

{

nonStandard	SEQUENCE OF NonStandardParameter OPTIONAL,

sessionID	INTEGER(1..255),

associatedSessionID	INTEGER(1..255) OPTIONAL,

terminalLabel	TerminalLabel OPTIONAL, -- if not present,

-- it refers to all participants

-- in the conference

sessionDescription	BMPString (SIZE(1..128)) ,

-- Basic ISO/IEC 10646-1 (Unicode)

dataType	CHOICE

{

videoData	VideoCapability,

audioData	AudioCapability,

data	DataApplicationCapability,

...

},

mediaChannel	TransportAddress OPTIONAL,

mediaGuaranteedDelivery	BOOLEAN OPTIONAL,

mediaControlChannel	TransportAddress OPTIONAL,  -- reverse RTCP channel

mediaControlGuaranteedDelivery	BOOLEAN OPTIONAL,

...,

redundancyEncoding	RedundancyEncoding OPTIONAL,

sessionDependency	INTEGER (1..255) OPTIONAL,

destination	TerminalLabel OPTIONAL

}

-- ===================================================================================

-- Conference Request definitions

-- ===================================================================================

ConferenceRequest	::=CHOICE

{

terminalListRequest	NULL,                    -- same as H.230 TCU (term->MC)

makeMeChair	NULL,                    -- same as H.230 CCA (term->MC)

cancelMakeMeChair	NULL,                    -- same as H.230 CIS (term->MC)

dropTerminal	TerminalLabel,       -- same as H.230 CCD(term->MC)

requestTerminalID	TerminalLabel,       -- same as TCP (term->MC)

enterH243Password	NULL,                    -- same as H.230 TCS1(MC->term)

enterH243TerminalID	NULL,                    -- same as H.230 TCS2/TCI

--  (MC->term)

enterH243ConferenceID	NULL,                    -- same as H.230 TCS3 (MC->term)

...,

enterExtensionAddress	NULL,                     -- same as H.230 TCS4 (GW->term)

requestChairTokenOwner	NULL,                    -- same as H.230 TCA (term->MC)

requestTerminalCertificate 	SEQUENCE

{

terminalLabel	TerminalLabel OPTIONAL,

certSelectionCriteria	CertSelectionCriteria OPTIONAL,

sRandom	INTEGER (1..4294967295) OPTIONAL,

-- this is the requester's challenge

...

},

broadcastMyLogicalChannel	LogicalChannelNumber,	-- similar to H.230 MCV

makeTerminalBroadcaster	TerminalLabel,	-- similar to H.230 VCB

sendThisSource	TerminalLabel,	-- similar to H.230 VCS

requestAllTerminalIDs	NULL,

remoteMCRequest	RemoteMCRequest

}

CertSelectionCriteria	::=SEQUENCE SIZE (1..16) OF Criteria

Criteria			::=SEQUENCE

{

field		OBJECT IDENTIFIER, -- may include certificate type

value		OCTET STRING (SIZE(1..65535)),

...

}

TerminalLabel	::=SEQUENCE

{

mcuNumber	McuNumber,

terminalNumber	TerminalNumber,

...

}

McuNumber	::=INTEGER(0..192)

TerminalNumber	::=INTEGER(0..192)

-- ===================================================================================

-- Conference Response definitions

-- ===================================================================================

ConferenceResponse	::=CHOICE

{

mCTerminalIDResponse	SEQUENCE             -- response to TCP(same as TIP)

{				-- sent by MC only

terminalLabel	TerminalLabel,

terminalID	TerminalID,

...

},

terminalIDResponse	SEQUENCE	-- response to TCS2 or TCI

{				-- same as IIS

terminalLabel	TerminalLabel,	-- (term->MC)

terminalID	TerminalID,

...

},

conferenceIDResponse	SEQUENCE	-- response to TCS3

{				-- same as IIS

terminalLabel	TerminalLabel,	-- (term->MC)

conferenceID	ConferenceID,

...

},

passwordResponse	SEQUENCE	-- response to TCS1

{				-- same as IIS

terminalLabel	TerminalLabel,	-- (term->MC)

password	Password,

...

},

terminalListResponse	SET SIZE (1..256) OF TerminalLabel,

videoCommandReject	NULL,	-- same as H.230 VCR

terminalDropReject	NULL,	-- same as H.230 CIR

makeMeChairResponse	CHOICE	-- same as H.230 CCR

{

grantedChairToken	NULL,	-- same as H.230 CIT

deniedChairToken	NULL,	-- same as H.230 CCR

...

},

...,

extensionAddressResponse	SEQUENCE	-- response to TCS4

{

extensionAddress	TerminalID,	-- same as IIS (term->GW)

...

},

chairTokenOwnerResponse	SEQUENCE    -- response to TCA(same as TIR) sent by MC only

{

terminalLabel	TerminalLabel,

terminalID	TerminalID,

...

},

terminalCertificateResponse  	SEQUENCE

{

terminalLabel           	TerminalLabel OPTIONAL,

certificateResponse           	OCTET STRING (SIZE(1..65535)) OPTIONAL,

...

},

broadcastMyLogicalChannelResponse	CHOICE

{

grantedBroadcastMyLogicalChannel	NULL,	-- similar to H.230 MVA

deniedBroadcastMyLogicalChannel	NULL,	-- similar to H.230 MVR

...

},

makeTerminalBroadcasterResponse	CHOICE

{

grantedMakeTerminalBroadcaster	NULL,

deniedMakeTerminalBroadcaster	NULL,

...

},

sendThisSourceResponse	CHOICE

{

grantedSendThisSource	NULL,

deniedSendThisSource	NULL,

...

},

requestAllTerminalIDsResponse	RequestAllTerminalIDsResponse,

remoteMCResponse	RemoteMCResponse

}

TerminalID 	::=OCTET STRING (SIZE(1..128))	-- as per H.230

ConferenceID	::=OCTET STRING (SIZE(1..32))

Password		::=OCTET STRING (SIZE(1..32))

RequestAllTerminalIDsResponse 	::=SEQUENCE

{

terminalInformation	SEQUENCE OF TerminalInformation,

...

}

TerminalInformation	::=SEQUENCE

{

terminalLabel	TerminalLabel,

terminalID	TerminalID,

...

}

-- ===================================================================================

-- Remote MC Request definitions

-- ===================================================================================

RemoteMCRequest	::=CHOICE

{

masterActivate	NULL,

slaveActivate	NULL,

deActivate	NULL,

...

}

RemoteMCResponse	::=CHOICE

{

accept	NULL,

reject	CHOICE

{

unspecified	NULL,

functionNotSupported	NULL,

...

},

...

}

-- ===================================================================================

-- Multilink definitions

-- ===================================================================================

MultilinkRequest	::=CHOICE

{

nonStandard	NonStandardMessage,

callInformation	SEQUENCE

{

maxNumberOfAdditionalConnections	INTEGER (1..65535),

...

},

addConnection 	SEQUENCE

{

sequenceNumber	SequenceNumber,  -- Unique ID of request

dialingInformation	DialingInformation,

...

},

removeConnection	SEQUENCE

{

connectionIdentifier	ConnectionIdentifier,

...

},

maximumHeaderInterval	SEQUENCE

{

requestType	CHOICE

{

currentIntervalInformation	NULL,

requestedInterval	INTEGER (0..65535), -- Max Header Interval, milliseconds

...

},

...

},

...

}

MultilinkResponse	::=CHOICE

{

nonStandard	NonStandardMessage,

callInformation	SEQUENCE

{

dialingInformation	DialingInformation,

callAssociationNumber	INTEGER (0..4294967295),

...

},

addConnection	SEQUENCE

{

sequenceNumber	SequenceNumber, -- Equal to value in request

responseCode 	CHOICE

{

accepted	NULL,

rejected	CHOICE

{

connectionsNotAvailable	NULL, -- due to any technical reason

userRejected	NULL,

...

},

...

},

...

},

removeConnection	SEQUENCE

{

connectionIdentifier	ConnectionIdentifier,

...

},

maximumHeaderInterval	SEQUENCE

{

currentInterval	INTEGER (0..65535), -- Max Header Interval, milliseconds

...

},

...

}

MultilinkIndication	::=CHOICE

{

nonStandard	NonStandardMessage,

crcDesired	SEQUENCE

{

...

},

excessiveError	SEQUENCE

{

connectionIdentifier	ConnectionIdentifier,

...

},

...

}

DialingInformation 	::= CHOICE

{

nonStandard	NonStandardMessage,

differential	SET SIZE (1..65535) OF DialingInformationNumber,

-- list of numbers for all additional channels; only least

-- significant digits different from initial channel’s number

infoNotAvailable	INTEGER (1..65535), -- maximum no. of additional channels

...

}

DialingInformationNumber 	::= SEQUENCE
{

networkAddress	NumericString (SIZE (0..40)),

subAddress	IA5String (SIZE (1..40)) OPTIONAL,

networkType	SET SIZE (1..255) OF DialingInformationNetworkType,

...

}

DialingInformationNetworkType	::= CHOICE

{

nonStandard	NonStandardMessage,

n-isdn		NULL,

gstn		NULL,

...

}

ConnectionIdentifier 	::= SEQUENCE
{

channelTag	INTEGER (0..4294967295),	-- from H.MULTILINK

sequenceNumber	INTEGER (0..4294967295),	-- from H.MULTILINK

...

}

-- ===================================================================================

-- Logical channel bit-rate change definitions

-- ===================================================================================

MaximumBitRate	::=INTEGER (0.. 4294967295) 	-- units of 100 bit/s

LogicalChannelRateRequest	::=SEQUENCE

{

sequenceNumber	SequenceNumber,

logicalChannelNumber	LogicalChannelNumber,

maximumBitRate	MaximumBitRate,

...

}

LogicalChannelRateAcknowledge	::=SEQUENCE

{

sequenceNumber	SequenceNumber,

logicalChannelNumber	LogicalChannelNumber,

maximumBitRate	MaximumBitRate,

...

}

LogicalChannelRateReject	::=SEQUENCE

{

sequenceNumber	SequenceNumber,

logicalChannelNumber	LogicalChannelNumber,

rejectReason	LogicalChannelRateRejectReason,

currentMaximumBitRate        	MaximumBitRate OPTIONAL,

...

}

LogicalChannelRateRejectReason	::=CHOICE

{

undefinedReason	NULL,

insufficientResources	NULL,

...

}

LogicalChannelRateRelease	::=SEQUENCE

{

...

}

-- ===================================================================================

-- Command Message definitions

-- ===================================================================================

-- ===================================================================================

-- Command Message : Send Terminal Capability Set

-- ===================================================================================

SendTerminalCapabilitySet	::=CHOICE

{

specificRequest	SEQUENCE

{

multiplexCapability	BOOLEAN,

capabilityTableEntryNumbers	SET SIZE (1..65535) OF CapabilityTableEntryNumber OPTIONAL,

capabilityDescriptorNumbers	SET SIZE (1..256) OF CapabilityDescriptorNumber OPTIONAL,

...

},

genericRequest	NULL,

...

}

-- ===================================================================================

-- Command Message : Encryption

-- ===================================================================================

EncryptionCommand	::=CHOICE

{

encryptionSE	OCTET STRING,	-- per H.233, but no error protection

encryptionIVRequest	NULL,	-- requests new IV

encryptionAlgorithmID	SEQUENCE

{

h233AlgorithmIdentifier	SequenceNumber,

associatedAlgorithm	NonStandardParameter

},

...

}

-- ===================================================================================

-- Command Message : Flow Control

-- ===================================================================================

FlowControlCommand	::=SEQUENCE

{

scope		CHOICE

{

logicalChannelNumber	LogicalChannelNumber,

resourceID	INTEGER (0..65535),

wholeMultiplex	NULL

},

restriction	CHOICE

{

maximumBitRate	INTEGER (0..16777215),	-- units 100 bit/s

noRestriction	NULL

},

...

}

-- ===================================================================================

-- Command Message : Change or End Session

-- ===================================================================================

EndSessionCommand	::=CHOICE

{

nonStandard	NonStandardParameter,

disconnect	NULL,

gstnOptions	CHOICE

{

telephonyMode	NULL,

v8bis	NULL,

v34DSVD	NULL,

v34DuplexFAX	NULL,

v34H324	NULL,

...

},

...,

isdnOptions	CHOICE

{

telephonyMode	NULL,

v140	NULL,

terminalOnHold	NULL,

...

}

}

-- ===================================================================================

-- Command Message : Conference Commands

-- ===================================================================================

ConferenceCommand	::=CHOICE

{

broadcastMyLogicalChannel	LogicalChannelNumber,	-- similar to H.230 MCV

cancelBroadcastMyLogicalChannel	LogicalChannelNumber, -- similar to H.230 Cancel-MCV

makeTerminalBroadcaster	TerminalLabel,	-- same as H.230 VCB

cancelMakeTerminalBroadcaster	NULL,	-- same as H.230 Cancel-VCB

sendThisSource	TerminalLabel,	-- same as H.230 VCS

cancelSendThisSource	NULL,	-- same as H.230 cancel VCS

dropConference	NULL,	-- same as H.230 CCK

...,

substituteConferenceIDCommand	SubstituteConferenceIDCommand

}

SubstituteConferenceIDCommand	::=SEQUENCE

{

conferenceIdentifier	OCTET STRING (SIZE(16)),

...

}

-- ===================================================================================

-- Command Message : Miscellaneous H.230-like commands

-- ===================================================================================

MiscellaneousCommand	::=SEQUENCE

{

logicalChannelNumber	LogicalChannelNumber,

type		CHOICE

{

equaliseDelay	NULL,	-- same as H.230 ACE

zeroDelay	NULL,	-- same as H.230 ACZ

multipointModeCommand	NULL,

cancelMultipointModeCommand	NULL,

videoFreezePicture	NULL,

videoFastUpdatePicture	NULL,

videoFastUpdateGOB	SEQUENCE

{

firstGOB	INTEGER (0..17),

numberOfGOBs	INTEGER (1..18)

},

videoTemporalSpatialTradeOff	INTEGER (0..31),	-- commands a trade-off value

videoSendSyncEveryGOB	NULL,

videoSendSyncEveryGOBCancel	NULL,

...,

videoFastUpdateMB	SEQUENCE

{

firstGOB	INTEGER (0..255) OPTIONAL,

firstMB	INTEGER (1..8192) OPTIONAL,

numberOfMBs	INTEGER (1..8192),

...

},

maxH223MUXPDUsize	INTEGER(1..65535),	-- units octets

encryptionUpdate	EncryptionSync,

encryptionUpdateRequest	EncryptionUpdateRequest,

switchReceiveMediaOff	NULL,

switchReceiveMediaOn	NULL,

progressiveRefinementStart	SEQUENCE

{

repeatCount	CHOICE

{

doOneProgression		NULL,

doContinuousProgressions		NULL,

doOneIndependentProgression	NULL,

doContinuousIndependentProgressions	NULL,

...

},

...

},

progressiveRefinementAbortOne		NULL,

progressiveRefinementAbortContinuous		NULL

},

...

}

KeyProtectionMethod	::=SEQUENCE -- specify how the new key is to be protected

{

secureChannel	BOOLEAN,

sharedSecret	BOOLEAN,

certProtectedKey	BOOLEAN,

...

}

EncryptionUpdateRequest	::=SEQUENCE

{

keyProtectionMethod	KeyProtectionMethod OPTIONAL,

...

}

-- ===================================================================================

-- Command Message : H.223 Multiplex Reconfiguration

-- ===================================================================================

H223MultiplexReconfiguration	::=CHOICE

{

h223ModeChange	CHOICE

{

toLevel0	NULL,

toLevel1	NULL,

toLevel2	NULL,

toLevel2withOptionalHeader	NULL,

...

},

h223AnnexADoubleFlag	CHOICE

{

start	NULL,

stop	NULL,

...

},

...

}

-- ===================================================================================

-- Command Message : New ATM virtual channel command

-- ===================================================================================

NewATMVCCommand	::=SEQUENCE

{

resourceID	INTEGER(0..65535),

bitRate		INTEGER(1..65535),	-- units 64 kbit/s

bitRateLockedToPCRClock	BOOLEAN,

bitRateLockedToNetworkClock	BOOLEAN,

aal			CHOICE

{

aal1	SEQUENCE

{

clockRecovery	CHOICE

{

nullClockRecovery	NULL,

srtsClockRecovery	NULL,

adaptiveClockRecovery	NULL,

...

},

errorCorrection	CHOICE

{

nullErrorCorrection	NULL,

longInterleaver	NULL,

shortInterleaver	NULL,

errorCorrectionOnly	NULL,

...

},

structuredDataTransfer	BOOLEAN,

partiallyFilledCells	BOOLEAN,

...

},

aal5	SEQUENCE

{

forwardMaximumSDUSize	INTEGER (0..65535),	-- units octets

backwardMaximumSDUSize	INTEGER (0..65535),	-- units octets

...

},

...

},

multiplex	CHOICE

{

noMultiplex	NULL,

transportStream	NULL,

programStream	NULL,

...

},

reverseParameters	SEQUENCE

{

bitRate	INTEGER(1..65535),	-- units 64 kbit/s

bitRateLockedToPCRClock	BOOLEAN,

bitRateLockedToNetworkClock	BOOLEAN,

multiplex	CHOICE

{

noMultiplex	NULL,

transportStream	NULL,

programStream	NULL,

...

},

...

},

...

}

-- ===================================================================================

-- Indication Message definitions

-- ===================================================================================

-- ===================================================================================

-- Indication Message : Function not understood

-- ===================================================================================

-- This is used to return a request, response or command that is not understood

FunctionNotUnderstood	::=CHOICE

{

request	RequestMessage,

response	ResponseMessage,

command	CommandMessage

}

-- ===================================================================================

-- Indication Message : Function not Supported

-- ===================================================================================

-- This is used to return a complete request, response or command that is not recognized

FunctionNotSupported	::=SEQUENCE

{

cause		CHOICE

{

syntaxError	NULL,

semanticError	NULL,

unknownFunction	NULL,

...

},

returnedFunction	OCTET STRING OPTIONAL,

...

}

-- ===================================================================================

-- Indication Message : Conference

-- ===================================================================================

ConferenceIndication	::=CHOICE

{

sbeNumber		INTEGER (0..9),	-- same as H.230 SBE Number

terminalNumberAssign		TerminalLabel,	-- same as H.230 TIA

terminalJoinedConference		TerminalLabel,	-- same as H.230 TIN

terminalLeftConference		TerminalLabel,	-- same as H.230 TID

seenByAtLeastOneOther		NULL,	-- same as H.230 MIV

cancelSeenByAtLeastOneOther		NULL,	-- same as H.230 cancel MIV

seenByAll		NULL,	-- like H.230 MIV

cancelSeenByAll		NULL,	-- like H.230 MIV

terminalYouAreSeeing		TerminalLabel,	-- same as H.230 VIN

requestForFloor	NULL,	-- same as H.230 TIF

...,

withdrawChairToken	NULL,	-- same as H.230 CCR

-- MC-> chair

floorRequested	TerminalLabel,	-- same as H.230 TIF

-- MC-> chair

terminalYouAreSeeingInSubPictureNumber TerminalYouAreSeeingInSubPictureNumber,

videoIndicateCompose 	VideoIndicateCompose,

}

TerminalYouAreSeeingInSubPictureNumber ::= SEQUENCE

{

terminalNumber	TerminalNumber,

subPictureNumber	INTEGER (0..255),

...

}

VideoIndicateCompose ::= SEQUENCE

{

compositionNumber	INTEGER (0..255),

...

}

-- ===================================================================================

-- Indication Message : Miscellaneous H.230-like indication

-- ===================================================================================

MiscellaneousIndication	::=SEQUENCE

{

logicalChannelNumber	LogicalChannelNumber,

type		CHOICE

{

logicalChannelActive	NULL,	-- same as H.230 AIA and VIA

logicalChannelInactive	NULL,	-- same as H.230 AIM and VIS

multipointConference	NULL,

cancelMultipointConference	NULL,

multipointZeroComm	NULL,	-- same as H.230 MIZ

cancelMultipointZeroComm	NULL,	-- same as H.230 cancel MIZ

multipointSecondaryStatus	NULL,	-- same as H.230 MIS

cancelMultipointSecondaryStatus	NULL,	-- same as H.230 cancel MIS

videoIndicateReadyToActivate	NULL,	-- same as H.230 VIR

videoTemporalSpatialTradeOff	INTEGER (0..31),	-- indicates current trade-off

...,

videoNotDecodedMBs	SEQUENCE

{

firstMB	INTEGER (1..8192),

numberOfMBs	INTEGER (1..8192),

temporalReference	INTEGER (0..255),

...

},

transportCapability	TransportCapability

},

...

}

-- ===================================================================================

-- Indication Message : Jitter Indication

-- ===================================================================================

JitterIndication	::=SEQUENCE

{

scope		CHOICE

{

logicalChannelNumber	LogicalChannelNumber,

resourceID	INTEGER (0..65535),

wholeMultiplex	NULL

},

estimatedReceivedJitterMantissa	INTEGER (0..3),

estimatedReceivedJitterExponent	INTEGER (0..7),

skippedFrameCount	INTEGER (0..15) OPTIONAL,

additionalDecoderBuffer	INTEGER (0..262143) OPTIONAL,	-- 262143 is 2^18 - 1

...

}

-- ===================================================================================

-- Indication Message : H.223 logical channel skew

-- ===================================================================================

H223SkewIndication	::=SEQUENCE

{

logicalChannelNumber1	LogicalChannelNumber,

logicalChannelNumber2	LogicalChannelNumber,

skew		INTEGER (0..4095),	-- units milliseconds

...

}

-- ===================================================================================

-- Indication Message : H.225.0 maximum logical channel skew

-- ===================================================================================

H2250MaximumSkewIndication	::=SEQUENCE

{

logicalChannelNumber1	LogicalChannelNumber,

logicalChannelNumber2	LogicalChannelNumber,

maximumSkew	INTEGER (0..4095),	-- units milliseconds

...

}

-- ===================================================================================

-- Indication Message : MC Location Indication

-- ===================================================================================

MCLocationIndication	::=SEQUENCE

{

signalAddress	TransportAddress, -- this is the H.323 Call Signalling

-- address of the entity which

-- contains the MC

...

}

-- ===================================================================================

-- Indication Message : Vendor Identification

-- ===================================================================================

VendorIdentification		::=SEQUENCE

{

vendor				NonStandardIdentifier,

productNumber		OCTET STRING (SIZE(1..256)) OPTIONAL,  -- per  vendor

versionNumber		OCTET STRING (SIZE(1..256)) OPTIONAL,  -- per productNumber

...

}

-- ===================================================================================

-- Indication Message : New ATM virtual channel indication

-- ===================================================================================

NewATMVCIndication	::=SEQUENCE

{

resourceID	INTEGER(0..65535),

bitRate	INTEGER(1..65535),	-- units 64 kbit/s

bitRateLockedToPCRClock	BOOLEAN,

bitRateLockedToNetworkClock	BOOLEAN,

aal			CHOICE

{

aal1	SEQUENCE

{

clockRecovery	CHOICE

{

nullClockRecovery	NULL,

srtsClockRecovery	NULL,

adaptiveClockRecovery	NULL,

...

},

errorCorrection	CHOICE

{

nullErrorCorrection	NULL,

longInterleaver	NULL,

shortInterleaver	NULL,

errorCorrectionOnly	NULL,

...

},

structuredDataTransfer	BOOLEAN,

partiallyFilledCells	BOOLEAN,

...

},

aal5	SEQUENCE

{

forwardMaximumSDUSize	INTEGER (0..65535),	-- units octets

backwardMaximumSDUSize	INTEGER (0..65535),	-- units octets

...

},

...

},

multiplex	CHOICE

{

noMultiplex	NULL,

transportStream	NULL,

programStream	NULL,

...

},

...,

reverseParameters	SEQUENCE

{

bitRate	INTEGER(1..65535),	-- units 64 kbit/s

bitRateLockedToPCRClock	BOOLEAN,

bitRateLockedToNetworkClock	BOOLEAN,

multiplex	CHOICE

{

noMultiplex	NULL,

transportStream	NULL,

programStream	NULL,

...

},

...

}

}

-- ===================================================================================

-- Indication Message : user input

-- ===================================================================================

UserInputIndication	::=CHOICE

{

nonStandard	NonStandardParameter,

alphanumeric	GeneralString,

...,

userInputSupportIndication	CHOICE

{

nonStandard 	NonStandardParameter,

basicString	NULL,

iA5String	NULL,

generalString	NULL,

...

},

signal			SEQUENCE

{

signalType	IA5String (SIZE (1) ^ FROM ("0123456789#*ABCD!")),

duration	INTEGER (1..65535) OPTIONAL,  -- milliseconds

rtp			SEQUENCE

{

timestamp	INTEGER (0..4294967295) OPTIONAL,

expirationTime	INTEGER (0..4294967295) OPTIONAL,

logicalChannelNumber	LogicalChannelNumber,

...

} OPTIONAL,

...

},

signalUpdate	SEQUENCE

{

duration	INTEGER (1..65535),  -- milliseconds

rtp			SEQUENCE

{

logicalChannelNumber	LogicalChannelNumber,

...

} OPTIONAL,

...

}

}

-- ===================================================================================

-- Indication Message : Flow Control

-- ===================================================================================

FlowControlIndication	::=SEQUENCE

{

scope		CHOICE

{

logicalChannelNumber	LogicalChannelNumber,

resourceID	INTEGER (0..65535),

wholeMultiplex	NULL

},

restriction	CHOICE

{

maximumBitRate	INTEGER (0..16777215),	-- units 100 bit/s

noRestriction	NULL

},

...

}

END

ANNEX B

# Annex B	Messages: semantic definitions

A==

(This annex forms an integral part of this Recommendation)

This annex provides semantic definitions and constraints on the syntax elements defined in the previous section.

MultimediaSystemControlMessage: is a choice of message types. Messages defined in this Recommendation are classified as request, response, command and indication messages.

RequestMessage: a request message results in an action by the remote terminal and requires an immediate response from it. The nonStandard message may be used to send non-standard requests.

ResponseMessage: a response message is the response to a request message. The nonStandard message may be used to send non-standard responses.

CommandMessage: a command message requires action but no explicit response. The nonStandard message may be used to send non-standard commands.

IndicationMessage: an indication contains information that does not require action or response. The nonStandard message may be used to send non-standard indications.

NonStandardParameter: this may be used to indicate a non standard parameter. It consists of an identity and the actual parameters, which are coded as an octet string.

NonStandardIdentifier: is used to identify the type of non-standard parameter. It is either an object identifier, or an H.221 type of identifier that is an octet string consisting of exactly four octets as follows. Country code consists of two octets, the first being according to Recommendation T.35 Annex A. The second octet is assigned nationally, unless the first octet is 1111 1111, in which case the second octet shall contain the country code according to T.35 Annex B. The terminal manufacturer code consists of two octets assigned nationally.. The manufacturer codes are the same as those assigned for use in H.320 [81]. H.245 non-standard identifiers may be either “object” type or “h221NonStandard” type at the discretion of the manufacturer defining the non-standard message, as OBJECT IDENTIFIERs and h221NonStandard messages come from non-overlapping spaces and cannot be confused. However, since h221NonStandard messages are also used by H.320, such messages come from the same space as H.320 messages, and shall have the same meaning.

## B.1	Master Slave Determination messages

This set of messages is used by a protocol to determine which terminal is the master terminal and which is the slave terminal.

### B.1.1	Master Slave Determination

This is sent from a MSDSE to a peer MSDSE.

terminalType is a number that identifies different types of terminal, such as, terminals, MCUs and gateways. The allocation of values to terminal types is outside the scope of this Recommendation.

statusDeterminationNumber is a random number in the range 0 ... 224-1.

### B.1.2	Master Slave Determination Acknowledge

This is used to confirm whether the terminal is the master terminal or the slave terminal, as indicated by decision. When decision is of type master, the terminal receiving this message is the master terminal and when decision is of type slave, it is the slave terminal.

### B.1.3	Master Slave Determination Reject

This is used to reject the MasterSlaveDetermination message. When the cause is of type identicalNumbers, the rejection was due to the random numbers being equivalent and the terminal types being the same.

### B.1.4	Master Slave Determination Release

This is sent in the case of a timeout.

## B.2	Terminal capability messages

This set of messages is for the secure exchange of capabilities between the two terminals.

### B.2.1	Overview

The transmitting terminal assigns each individual mode the terminal is capable of operating in a number in a capabilityTable. For example, G.723.1 audio, G.728 audio, and CIF H.263 video would each be assigned separate numbers.

These capability numbers are grouped into AlternativeCapabilitySet structures. Each AlternativeCapabilitySet indicates that the terminal is capable of operating in exactly one mode listed in the set. For example, an AlternativeCapabilitySet listing {G.711, G.723.1, G.728} means that the terminal can operate in any one of those audio modes, but not more than one.

These AlternativeCapabilitySet structures are grouped into simultaneousCapabilities structures. Each simultaneousCapabilities structure indicates a set of modes the terminal is capable of using simultaneously. For example, a simultaneousCapabilities structure containing the two AlternativeCapabilitySet structures {H.261, H.263} and {G.711, G.723.1, G.728} means that the terminal can operate either of the video codecs simultaneously with any one of the audio codecs. The simultaneousCapabilities set {{H.261}, {H.261, H.263}, {G.711, G.723.1, G.728} } means the terminal can operate two video channels and one audio channel simultaneously: one video channel per H.261, another video channel per either H.261 or H.263, and one audio channel per either G.711, G.723.1, or G.728.

NOTE - The actual capabilities stored in the capabilityTable are often more complex than presented here. For example, each H.263 capability indicates details including ability to support various picture formats at given minimum picture intervals, and ability to use optional coding modes.

The terminal's total capabilities are described by a set of CapabilityDescriptor structures, each of which is a single simultaneousCapabilities structure and a capabilityDescriptorNumber. By sending more than one CapabilityDescriptor, the terminal may signal dependencies between operating modes by describing different sets of modes which it can simultaneously use. For example, a terminal issuing two CapabilityDescriptor structures, one {{H.261, H.263}, {G.711, G.723.1, G.728}} as in the previous example, and the other {{H.262}, {G.711}}, means the terminal can also operate the H.262 video codec, but only with the low-complexity G.711 audio codec.

Terminals may dynamically add capabilities during a communication session by issuing additional CapabilityDescriptor structures, or remove capabilities by sending revised CapabilityDescriptor structures. All terminals shall transmit at least one CapabilityDescriptor structure.

### B.2.2	Terminal Capability Set

This message contains information about the terminal's capability to transmit and receive. It also indicates the version of this Recommendation that is in use. It is sent from an outgoing CESE to a peer incoming CESE.

sequenceNumber is used to label instances of TerminalCapabilitySet so that the corresponding response can be identified.

protocolIdentifier is used to indicate the version of this Recommendation that is in use. Annex A lists the object identifiers defined for use by this Recommendation.

multiplexCapability indicates capabilities relating to multiplexing and network adaptation. A terminal shall include multiplexCapability in the first TerminalCapabilitySet sent.

V75Capability indicates the capabilities of the V.75 control entity. The audioHeader indicates the capability of the V.75 audio header.

#### B.2.2.1	Capability Table

A capability table is a numbered list of capabilities. A terminal shall be capable of everything that it lists in its capability table, but shall not necessarily be capable of simultaneously performing more than one of them.

A TerminalCapabilitySet may contain zero or more CapabilityTableEntrys. At the start, no table entries are defined. When a CapabilityTableEntry is received, it replaces the previously received CapabilityTableEntry with the same CapabilityTableEntryNumber. A CapabilityTableEntry without a Capability may be used to remove the previously received CapabilityTableEntry with the same CapabilityTableEntryNumber.

#### B.2.2.2	Capability Descriptors

CapabilityDescriptors are used to indicate a terminal's capability to transmit and receive. Each CapabilityDescriptor provides an independent statement about the terminal's capabilities.

capabilityDescriptorNumber is used to number CapabilityDescriptors. If a terminal has a preference for the mode it would like to transmit or receive, and wishes to express this when transmitting its capabilities, it may do so by giving CapabilityDescriptors that relate to its preferred mode or modes small values of capabilityDescriptorNumber.

simultaneousCapabilities is a set of AlternativeCapabilitySet. It is used to list the simultaneous capabilities of the terminal.

An AlternativeCapabilitySet is a sequence of CapabilityTableEntryNumbers. Only those CapabilityTableEntrys that have been defined shall be present in an AlternativeCapabilitySet, although it is possible to define CapabilityTableEntrys and refer to them in the same TerminalCapabilitySet. If a terminal has a preference for the mode it would like to transmit or receive, and wishes to express this when transmitting its capabilities, it may do so by listing elements in AlternativeCapabilitySets in order of decreasing preference.

A terminal shall be capable of simultaneously performing any one capability from each AlternativeCapabilitySet listed in simultaneousCapabilities.

At least one capability descriptor shall have the following structure: there shall be at least one AlternativeCapabilitySet containing only capabilities of a single medium type for each medium type that the terminal can support. This is to ensure that the remote terminal can select a mode of transmission that includes at least one instance of each medium type that the receiver can support.

NOTE - A repetition of a capability in an AlternativeCapabilitySet is redundant and conveys no further information, while the repetition of a capability in different AlternativeCapabilitySets in the same CapabilityDescriptor indicates the possibility of an additional, simultaneous, instance of the particular capability.

NOTE - Terminals that can not vary the allocation of resources can indicate their capability completely by use of a single CapabilityDescriptor.

#### B.2.2.3	Capability

The choices receiveVideoCapability, receiveAudioCapability, receiveDataApplicationCapability and receiveUserInputCapability indicate the capability to receive according to the respective VideoCapability, AudioCapability, DataApplicationCapability and UserInputCapability.

The choices transmitVideoCapability, transmitAudioCapability, transmitDataApplicationCapability and transmitUserInputCapability indicate the capability to transmit according to the respective VideoCapability, AudioCapability, DataApplicationCapability and UserInputCapability.

The choices receiveAndTransmitVideoCapability, receiveAndTransmitAudioCapability, receiveAndTransmitDataApplicationCapability and receiveAndTransmitUserInputCapability indicate the capability to receive and transmit according to the respective VideoCapability, AudioCapability, DataApplicationCapability and UserInputCapability. These code points may be useful for indicating that the receive and transmit capabilities are not independent.

The boolean h233EncryptionTransmitCapability, when true, indicates that the terminal supports encryption according to H.233 and H.234 [82][83].

h233IVResponseTime is measured in units of milliseconds, and indicates the minimum time the receiver requires the transmitter to wait after the completion of transmission of an IV message before starting to use the new IV. The means of transmitting the IV is not defined in this Recommendation.

ConferenceCapability indicates various conference capabilities.

multipointVisualizationCapability (similar to H.230 MVC) is included in the cap-set of an MCU or terminal to indicate that it shall properly generate or process conferenceResponse.broadcastMyLogicalChannel.grantedBroadcastMyLogicalChannel (similar to H.230 MVA) and conferenceResponse.broadcastMyLogicalChannel.deniedBroadcastMyLogicalChannel (similar to H.230 MVR) in response to conferenceRequest. BroadcastMyLogicalChannel (similar to H.230 MCV).

h235SecurityCapability indicates the capabilities that the terminal supports according to H.235[84]. The mediaCapability field shall refer to Capability Table Entries that do contain a transmit, receive, or receiveAndTransmit AudioCapability, VideoCapability, DataApplicationCapability, or similar capability indicated by a NonStandardParameter only.

EncryptionAuthenticationAndIntegrity indicates which encryption, authentication, and integrity capabilities are supported for the signalled mediaCapability. mediaCapability defines the supported audio, video, or data algorithms as well as the supported distribution methods (e.g. receive, transmit, or receive and transmit). The maxPendingReplacementFor parameter indicates the maximum number of open logical channel operations which are allowed to be in the REPLACEMENT PENDING state simultaneously. The REPLACEMENT PENDING state occurs when a logical channel has been established using the replacementFor parameter, but the replaced logical channel has not yet been closed.

genericControlCapability indicates generic control capabilities.

#### B.2.2.4	Multiplex Capabilities

MultiplexCapability indicates capabilities relating to multiplexing and network adaptation. A terminal shall send MultiplexCapability in the first TerminalCapabilitySet sent. Unless stated otherwise, these are capabilities to receive.

H222Capability: indicates multiplexing and network adaptation capabilities that are specific to the multiplex defined in H.222.1 [85].

numberOfVCs indicates how many simultaneous ATM Virtual Channels (VCs) can be supported by the terminal. This includes any VCs that transport H.245, T.120, DSM-CC or any other data, and all VCs that carry audiovisual information. It does not include the VC used for Q.2931 signalling [86].

vcCapability is a set, of size equal to the value of numberOfVCs, that indicates the capabilities present for each available VC.

The sequence aal1, when present, indicates the capability for ATM adaptation layer 1, and which of its options, as specified in I.363 [87], are supported. The codepoints are defined in Table B.1.

TABLE B.2/H.245

ATM Adaptation Layer 1 codepoints

| ASN.1 Codepoint | Semantic meaning of codepoint |
| --- | --- |
| nullClockRecovery | Null source clock frequency recovery method: synchronous circuit transport. |
| srtsClockRecovery | Synchronous residual timestamp source clock frequency recovery method. |
| adaptiveClockRecovery | Adaptive clock source clock frequency recovery method. |
| nullErrorCorrection | No error correction is supported. |
| longInterleaver | The forward error correction method for loss sensitive signal transport is supported. |
| shortInterleaver | The forward error correction method for delay sensitive signal transport is supported. |
| errorCorrectionOnly | The forward error correction method without cell interleaving is supported. |
| structuredDataTransfer | Structured data transfer is supported. |
| partiallyFilledCells | Partially filled cells is supported. |

The sequence aal5, when present, indicates the capability for ATM adaptation layer 5, and which of its options, as specified in I.363 [88], are supported. forwardMaximumSDUSize and backwardMaximumSDUSize indicate the maximum CPCS-SDU size in the forward and reverse directions, measured in octets. Either aal1 or aal5 or both shall be present.

The booleans transportStream and programStream, when equal to true, indicate the capability to support the Transport Stream and Program Stream multiplexes respectively [89].

availableBitRates indicates the bit rate capabilities for the VC. It is a sequence of different bit rates that can be supported, measured in units of 64 kbit/s. Bit rates are listed in decreasing order, that is, the highest bit rate supported is listed first. Supported bit rates can be listed as individual values using the field singleBitRate, or as a rangeOfBitRates between lowerBitRate and higherBitRate, indicating that all values between this lower limit and higher limit, including these limits, are supported. The bit rates indicated are measured at the AAL-SAP.

The sequence aal1ViaGateway, when present, indicates the capability of ATM adaptation layer 1 supported by AAL1/5 conversion gateways. The codepoints are the same as those of sequence aal1. The sequence Q2931Address indicates one or more sets of Q.2931 party number and party subaddress.

H223Capability: indicates capabilities specific to the H.223 multiplex [90].

The boolean transportWithI-frames, when true, indicates that the terminal is capable of sending and receiving control channel messages using LAPM I-frames as defined in V.42 [91].

The booleans videoWithAL1, videoWithAL2, videoWithAL3, audioWithAL1, audioWithAL2, audioWithAL3, dataWithAL1, dataWithAL2 and dataWithAL3, when true, indicate the capability to receive the stated medium type (video, audio, or data) using the stated adaptation layer (AL1, AL2, or AL3).

The integers maximumAl2SDUSize and maximumAl3SDUSize indicate the maximum number of octets in each SDU that the terminal can receive when using adaptation layer types 2 and 3 respectively.

maximumDelayJitter indicates the maximum peak-to-peak multiplexing jitter that the transmitter shall cause. It is measured in milliseconds. Multiplexing jitter is defined as the difference in time of delivery of the first octet of an audio frame when delivered in the multiplexed stream and when it would be delivered at constant bit rate without a multiplex.

h223MultiplexTableCapability: indicates the terminals ability to receive and process multiplex table entries.

basic indicates that the multiplex can only receive basic MultiplexEntryDescriptors as defined in H.223 [8].

enhanced indicates that the multiplex can receive enhanced MultiplexEntryDescriptors with the additional parameters defined below.

maximumNestingDepth depth indicates the maximum nesting depth of recursively invoked subElementList fields. MultiplexEntryDescriptors which do not use the subElementList field shall be considered to have a nesting depth of zero.

maximumElementListSize indicates the maximum number of fields in the ASN.1 SEQUENCE.

maximumSubElementListSize indicates the maximum number of subelements in the subElementList.

The boolean maxMUXPDUSizeCapability, when true, indicates that the transmitter is able to restrict the size of the H.223 MUX-PDUs that it transmits. It has no meaning when part of a receive capability.

The boolean nsrpSupport, when true, indicates support of the H.324 Annex A NSRP mode.

MobileOperationTransmitCapability: indicates the capability transmit the multiplex layers described in Annex A/H.223 and Annex B/H.223.

The boolean h223AnnexA, if true, indicates the terminal can transmit the MUX-PDUs as defined in Annex A/H.223.

The boolean h223AnnexADoubleFlag, if true, indicates the terminal can transmit the MUX-PDUs as defined in Annex A/H.223 with its optional double-flag mode.

The boolean h223AnnexB, if true, indicates the terminal can transmit the MUX-PDUs as defined in Annex B/H.223.

The boolean h223AnnexBwithOptionalHeaderField, if true, indicates the terminal can transmit the MUX-PDU as defined in Annex B/H.223 with its optional header field.

h223AnnexCCapability: indicates the capability to receive and process AL-PDUs as described in Annex C/H.223, with following condition.

The booleans videoWithAL1M, videoWithAL2M, videoWithAL3M, audioWithAL1M, audioWithAL2M, audioWithAL3M, dataWithAL1M, dataWithAL2M and dataWithAL3M, when true, indicate the capability to receive the stated medium type (video, audio, or data) using the stated adaptation layer (AL1M, AL2M, or AL3M).

alpduInterleaving, if true, indicates the capability to receive and process AL-PDU's for which interleaving is applied.

The integer maximumAl1MPDUSize indicates the maximum number of octets in each PDU that the terminal can receive when using adaptation layer AL1M.

The integers maximumAl2MSDUSize and maximumAL3MSDUSize indicate the maximum number of octets in each SDU that the terminal can receive when using adaptation layer AL2M and AL3M, respectively.

rsCodeCapability, if true, indicates the capability to receive the AL-PDU's for which Reed-Solomon coding is indicated.

V76Capability: indicates capabilities specific to the V.76 multiplex.

The suspendResumeCapabilitywAddress indicates the capability of supporting V.76 suspend/resume with an address field. The suspendResumeCapabilitywoAddress indicates the capability of supporting V.76 suspend/resume without an address field.

rejCapability indicates the capability of the V.76 multiplex error control function to perform reject.

sREJCapability indicates the capability of the multiplex error control function to perform selective reject.

mREJCapability indicates the capability of the multiplex error control function to perform multiple selective reject.

crc8bitCapability is the capability of the multiplex to use 8 bit CRC.

crc16bitCapability is the capability of multiplex to use 16 bit CRC.

crc32bitCapability is the capability of the multiplex to use 32 bit CRC.

uihCapability indicates support of V.76 UIH frames.

numOfDLCS indicates the number of DLCs which the V.76 multiplex can support.

twoOctetAddressFieldCapability indicates the ability of the V.76 multiplex to support an address field of two octets.

loopBackTestCapability indicates the support of loop back per Recommendation V.76. n401Capability indicates the maximum value of N401 described in Recommendation V.76. maxWindowSizeCapability indicates the maximum window size the V.76 multiplex can support.

H2250Capability: indicates capabilities specific to the H.225.0 media packetization layer.

maximumAudioDelayJitter indicates the maximum peak-to-peak delivery of audio packets to the transport layer that the transmitter shall cause. It is measured in milliseconds.

receiveMultipointCapability indicates the receive capabilities of a terminal in a multipoint conference.

transmitMultipointCapability indicates the transmit capabilities of a terminal in a multipoint conference.

receiveAndTransmitMultipointCapability indicates the receive and transmit capabilities of a terminal in a multipoint conference.

mcCapability indicates the ability of a terminal to act as an MC in a centralized or distributed conference.

rtcpVideoControlCapability indicates a terminal's ability to process both RTCP Full Intra Request (FIR) and Negative Acknowledgement (NACK) messages.

MediaPacketizationCapability indicates which optional media packetization scheme are supported by the endpoint.

h261aVideoPacketization indicates that the H261 alternative RTP payload format described in H.225.0 is in use.

rtpPayloadType indicates the RTP payload packetization schemes supported by the endpoint as follows.

payloadDescriptor identifies the semantics associated with the payloadType: if the rfc-number is chosen, it indicates the official document of the IETF in which the payload format is defined; obsoleted RFCs should not be referenced here.  If the oid component is chosen, this identifies a payload format specified as part of a Recommendation defined by the ITU or an International Standard defined by the ISO and registered in the respective document under this Object Identifier.  This applies equally well to both capability exchange and opening logical channels.

payloadType may be included to indicate which payload type is associated with this format.  If used in capability exchange, the payloadType shall be set to a statically assigned payload type iff one exists for this payload format.  Otherwise, the payloadType shall be omitted.  If used in conjunction with OpenLogicalChannel, the payloadType shall indicate the payload type value to be used.  If a dynamic payload type is used, the values of the payloadType field and the dynamicPayloadType field shall match.

TransportCapability indicates optional transport capabilities such as quality of service and median channel type capabilities.

redundancyEncodingCapability indicates which redundancy encoding modes are supported (if any). For each capability entry, the redundancyEncodingMethod specifies the type of encoding to be used; the primary encoding; and which secondary encodings are supported for this primary encoding. The choice of encoding schemes depends on the mode selected. rtpAudioRedundancyEncoding refers to the audio redundancy encoding; if this mode is the selected redundancyEncodingMethod, only CapabilityEntryNumbers referring to audio encodings are valid. rtpH263VideoRedundancyEncoding indicates that video redundancy coding according to H.263+ Annex N is possible or that a logical channel shall be opened using video redundancy coding. Additional parameters are provided as follows:

numberOfThreads indicates the maximum number of the threads the sender/receiver is able to support when used during capability exchange; contains the actual number of threads for a specific stream when opening a logical channel

framesBetweenSyncPoints defines the maximum number of video frames that may be transmitted (summed across all threads) between two synchronization points of all threads during capability exchange; defines the actual number of frames for a specific stream for OpenLogicalChannel

frameToThreadMapping defines which modes are supported by a sender / receiver during capability exchange and which mode is to be used when opening a logical channel: ”round-robin” indicates that frames are assigned in a round robin fashion to the threads, with the first frame after a synchronization point being assigned to thread 0, the second to thread 1, an so forth. The ”custom” format allows to specify arbitrary mappings of frames to threads; during caps exchange, support for custom format is indicated by choosing this component and encoding an arbitrary (possibly empty) SEQUENCE. Support for custom formats implies support for round-robin mappings.

containedThreads applies only to commands that open logical channels: this parameter then indicates which of the threads are transmitted in logical channel to be opened. A logical channel may contain any number of 1 through 15 threads, however, two logical channels shall not specify to contain the same thread.

In case of rtpH263VideoRedundancyEncoding, the secondaryEncoding parameter shall not be present; this also applies to the H2250ModeParameters and the RedundancyEncoding ASN.1 structures of H.245.

When a logical channel for video redundancy coding is opened, the logical channel containing thread 0 shall be opened first, and this logical channel shall be referenced by all other logical channels by means of the forwardLogicalChannelDependency parameter in the OpenLogicalChannel command.

LogicalChannelSwitchingCapability indicates the ability of a receiver to switch which stream (e.g. which logical channel) is being rendered based on the switchReceiveMedia on and off commands.

t120DynamicPortCapability indicates that the endpoint can place a T.120 [92] call to a dynamic transport address instead of the standard well known port address as defined in T.123 [93].

MultipointCapability: indicates a terminal's capabilities specific to multipoint.

multicastCapability indicates the ability of a terminal to multicast audio or video traffic.

multiUniCastConference indicates the ability of a terminal to participate in a multiUniCast conference.

MediaDistributionCapability : indicates a terminal's capabilities for transmission and reception of media in a multipoint conference. Centralized Control and Audio shall be TRUE for H.323 terminals. If Video is supported, the Centralized Video shall be set TRUE. If T.120 is supported, the Centralized Data T.120 Data Application Capability shall be present.

Centralized and distributed control, audio, and video, indicate the ability of a terminal to participate in a conference with those media distribution types. Centralized and distributed data indicate the ability of a terminal to participate in conference with those media distribution types for the specific Data Application Protocol. MediaDistributionCapability is a sequence to allow for the definition of simultaneous capabilities (e.g. centralized audio with distributed video or centralized video with distributed audio, or specific data capabilities per a Data Application Protocol).

QOSCapabilities indicates quality of service capabilities such as RSVPParameters and ATMParameters parameters.

mediaChannelCapabilities indicate what transports the media channel may be carried on. IP-UDP indicates that the endpoint supports transporting the media channel over an the IP network layer and a UDP transport layer. IP-TCP indicates that the endpoint supports transporting the media channel over an IP network layer and a TCP transport layer. atm-AAL5-UNIDIR indicates that the endpoint support transporting the media channel over an ATM AAL5 uni-directional virtual circuiit. atm-AAL5-BIDIR indicates that the endpoint support transporting the media channel over an ATM AAL5 bi-directional virtual circuiit.

RSVPParameters indicate specific parameter information about the RSVP protocol.

ATMParameters indicate specific parameter information about an ATM virtual circuit.

QosMode indicates whether the mode is a guaranteed quality of service or controlled load mode where no upper bound on end-to-end delay is enforced.

genericMultiplexCapability: indicates generic multiplex capabilities.

#### B.2.2.5	Video Capabilities

This indicates video capabilities. The indication of more than a single capability within a single VideoCapability does not indicate simultaneous processing capability. Simultaneous processing capability can be indicated by instances of VideoCapability in different AlternativeCapabilitySets in a single CapabilityDescriptor.

H261VideoCapability: indicates H.261 [94] capabilities.

If present, qcifMPI indicates the minimum picture interval in units of 1/29.97 for the encoding and/or decoding of QCIF pictures, and if not present, no capability for QCIF pictures is indicated.

If present, cifMPI indicates the minimum picture interval in units of 1/29.97 for the encoding and/or decoding of CIF pictures, and if not present, no capability for CIF pictures is indicated.

The boolean temporalSpatialTradeOffCapability, when true, indicates that the encoder is able to vary its trade-off between temporal and spatial resolution as commanded by the remote terminal. It has no meaning when part of a receive capability.

maxBitRate indicates the maximum bit rate in units of 100 bit/s at which a transmitter can transmit video or a receiver can receive video.

stillImageTransmission indicates the capability for still images as specified in Annex D of H.261.

H262VideoCapability: indicates H.262 [95] capabilities.

The list of booleans indicate the capability of processing the particular profiles and levels: a value of true indicates that such operation is possible, while a value of false indicates that such operation is not possible. An encoder shall produce bitstreams compliant to the specifications of a profile and level for which it has indicated capability, but also within the limitations imposed by the optional fields (see below). A decoder shall be able to accept all bit streams conforming to a profile and level for which it has indicated capability, provided it is within the limitations indicated by the optional fields. The optional fields are integers with units defined in Table B.3.

TABLE B.4/H.245

Units for H.262 codepoints

| ASN.1 Codepoint | Units for referenced parameter |
| --- | --- |
| videoBitRate | 400 bit/s |
| vbvBufferSize | 16384 bits |
| samplesPerLine | samples per line |
| linesPerFrame | lines per frame |
| framesPerSecond | The index, frame_rate_code, into table 6-4/H.262 |
| luminanceSampleRate | samples per second |

H263VideoCapability: indicates H.263 [96] capabilities.

If present, sqcifMPI indicates the minimum picture interval in units of 1/29.97 for the encoding and/or decoding of SQCIF pictures, and if not present, no capability for SQCIF pictures is indicated.

If present, qcifMPI indicates the minimum picture interval in units of 1/29.97 for the encoding and/or decoding of QCIF pictures, and if not present, no capability for QCIF pictures is indicated.

If present, cifMPI indicates the minimum picture interval in units of 1/29.97 for the encoding and/or decoding of CIF pictures, and if not present, no capability for CIF pictures is indicated.

If present, cif4MPI indicates the minimum picture interval in units of 1/29.97 for the encoding and/or decoding of 4CIF pictures, and if not present, no capability for 4CIF pictures is indicated.

If present, cif16MPI indicates the minimum picture interval in units of 1/29.97 for the encoding and/or decoding of 16CIF pictures, and if not present, no capability for 16CIF pictures is indicated.

maxBitRate indicates the maximum bit rate in units of 100 bit/s at which a transmitter can transmit video or a receiver can receive video.

The booleans unrestrictedVector (H.263, Annex D), arithmeticCoding (H.263, Annex E), advancedPrediction (H.263, Annex F), and pbFrames (H.263, Annex G), when true, indicate the capability to transmit and/or receive these optional modes defined in the Annexes of H.263.

The boolean temporalSpatialTradeOffCapability, when true, indicates that the encoder is able to vary its trade-off between temporal and spatial resolution as commanded by the remote terminal. It has no meaning when part of a receive capability.

The integer hrd-B, when present, indicates the HRD parameter B in H.263 Annex B, and is measured in units of 128 bits. When not present, the default value defined in H.263 Annex B applies. It is a receiver capability and has no meaning in transmission capability sets.

The integer bppMaxKb, when present, indicates the maximum number of bits for one coded picture that the receiver can receive and decode correctly, and is measured in units of 1 024 bits. When not present, the default value defined in H.263 applies. It is a receiver capability and has no meaning in transmission capability sets.

The following capabilities are intended for use in certain very low frame rate applications such as surveillance applications:

If present, slowSqcifMPI indicates the minimum picture interval in units of seconds per frame for the encoding and/or decoding of SQCIF pictures. If not present and sqcifMPI is not present, no capability for SQCIF pictures is indicated. If sqcifMPI is present, slowSqcifMPI shall not be present.

If present, slowQcifMPI indicates the minimum picture interval in units of seconds per frame for the encoding and/or decoding of QCIF pictures. If not present and qcifMPI is not present, no capability for QCIF pictures is indicated. If qcifMPI is present, slowQcifMPI shall not be present.

If present, slowCifMPI indicates the minimum picture interval in units of seconds per frame for the encoding and/or decoding of CIF pictures. If not present and cifMPI is not present, no capability for CIF pictures is indicated. If cifMPI is present, slowCifMPI shall not be present.

If present, slowCif4MPI indicates the minimum picture interval in units of seconds per frame for the encoding and/or decoding of 4CIF pictures. If not present and cif4MPI is not present, no capability for 4CIF pictures is indicated. If cif4MPI is present, slowCif4MPI shall not be present.

If present, slowCif16MPI indicates the minimum picture interval in units of seconds per frame for the encoding and/or decoding of 16CIF pictures. If not present and cif16MPI is not present, no capability for 16CIF pictures is indicated. If cif16MPI is present, slowCif16MPI shall not be present.

The values of MPI are applicable when all of the optional modes, for which capability is indicated, are being used, as well as when any combination of them is used. A terminal may signal the capability for a smaller MPI when some options are not used by transmitting another VideoCapability including this smaller MPI and indicating the reduced set of options.

The boolean errorCompensation, when true, indicates the capability to transmit and/or receive feedback information for error compensation as illustrated in Appendix I of H.263. When part of a transmit capability, it indicates the ability of the encoder to process videoNotDecodedMBs indications and compensate errors. When part of a receive capability, it indicates the ability of the decoder to identify erroneous MBs, treat them as not coded, and send appropriate videoNotDecodedMBs indications.

If present, enhancementLayerInfo indicates the capability of the encoder to transmit, or the decoder to receive, bitstreams with the optional scalability mode (Annex O) of H.263. enhancementLayerInfo is a sequence which indicates the configuration parameters of the scalability mode.

If present, H263Options indicates the capability for optional modes of H.263.

EnhancementLayerInfo: indicates capability for the Scalability Mode of H.263.

baseBitRateConstrained indicates whether the base layer is constrained not to exceed the maximum bit rate in the video capability minus the sum of the maximum bit rate in each of the enhancement options.

When present, snrEnhancement indicates the presence of an snr enhancement layer capability. The set size indicates the number of snrEnhancement layers the terminal is capable of supporting within a single logical channel.

When present, spatialEnhancement indicates the presence of a spatial enhancement layer capability. An enhancement layer bit stream contains a picture size which is either twice the width, or twice the height, or both, of the picture size in the layer which it references. For a terminal to be capable of spatial enhancement in one-dimension (width or height), a terminal must also indicate the capability to support the associated custom picture format required in the enhancement layer. The set size indicates the number of spatialEnhancement layers the terminal is capable of supporting within a single logical channel.

When present, bPictureEnhancement indicates the presence of a B pictures enhancement layer capability. The set size indicates the number of bPictureEnhancement layers the terminal is capable of supporting within a single logical channel.

EnhancementOptions inside the bPictureEnhancement sequence indicates which additional options an encoder may transmit or a decoder can receive in the B pictures.

numberOfBPictures indicates the maximum number of B pictures the terminal is capable of supporting between successive pairs of anchor reference pictures used in the prediction of the B pictures. For example, if equal to 2, then two B pictures can be sent between each pair of P pictures or other anchor pictures.

EnhancementOptions: indicates scalability enhancement layer capabilities.

The parameters in EnhancementOptions have the same semantic definitions as the parameters of the same name in H263VideoCapability.

H263Options: indicates capability of additional optional modes of H.263

advancedIntraCodingMode, when true, indicates the capability to transmit or receive the Advanced INTRA Coding Mode (Annex I) of H.263.

deblockingFilterMode, when true, indicates the capability to transmit or receive the Deblocking Filter Mode (Annex J) of H.263.

improvedPBFramesMode, when true, indicates the capability to transmit or receive the Improved PB frames Mode (Annex M) of H.263.

unlimitedMotionVectors, when true, indicates the capability of the encoder or decoder to support unlimited motion vector range when Unrestricted Motion Vector Mode (H.263 Annex D) is also indicated. unlimitedMotionVectors shall be FALSE if unrestrictedVector is FALSE in the same H263VideoCapability or H263VideoMode.

fullPictureFreeze, when true, indicates the capability of the encoder to send or the decoder to receive Full Picture Freeze commands as described in Annex L of H.263.

partialPictureFreezeAndRelease, when true, indicates the capability of the encoder to send or the decoder to receive Full Picture Freeze and Release commands as described in Annex L of H.263

resizingPartPicFreezeAndRelease, when true, indicates the capability of the encoder to send or the decoder to receive the Resizing Partial Picture Freeze and Release commands as described in Annex L of H.263.

fullPictureSnapshot, when true, indicates the capability of the encoder to send or the decoder to receive Full Picture snapshots of the video content as described in Annex L of H.263.

partialPictureSnapshot, when true, indicates the capability of the encoder to send or the decoder to receive Partial Picture Snapshots of the video content as described in Annex L of H.263.

videoSegmentTagging, when true, indicates the capability of the encoder to send or the decoder to receive Video Segment tagging for the video content as described in Annex L of H.263.

progressiveRefinement, when true, indicates the capability of the encoder to send or the decoder to receive Progressive Refinement tagging as described in Annex L of H.263. In addition, when true, the encoder shall respond to the progressive refinement miscellaneous commands doOneProgression, doContinuousProgressions, doOneIndependentProgression, doContinuousIndependentProgressions, progressiveRefinementAbortOne, and progressiveRefinementAbortContinuous. In addition, the encoder shall insert the Progressive Refinement Segment Start Tags and the Progressive Refinement Segment End Tags as defined in the Supplemental Enhancement Information Specification (Annex L) of Recommendation H.263.

NOTE - Progressive Refinement tagging can be sent by an encoder and received by a decoder even when not commanded in a miscellaneous command.

dynamicPictureResizingByFour, when true, indicates the capability of an encoder or decoder to support the picture resizing-by-four (with clipping) submode of the implicit Reference Picture Resampling Mode (Annex P) of H.263.

dynamicPictureResizingSixteenthPel, when true, indicates the capability of an encoder or decoder to support resizing a reference picture to any width and height using the implicit Reference Picture Resampling mode (Annex P) of H.263 (with clipping).

dynamicWarpingHalfPel, when true, indicates the capability of an encoder or decoder to support the arbitrary picture warping operation within the Reference Picture Resampling mode (Annex P) of H.263 (with any fill mode) using half-pixel accuracy warping.

dynamicWarpingSixteenthPel, when true, indicates the capability of an encoder or decoder to support the arbitrary picture warping operation within the Reference Picture Resampling mode (Annex P) of H.263 (with any fill mode) using either half-pixel or sixteenth pixel accuracy warping.

If DynamicPictureResizingSixteenthPel is true then DynamicPictureResizingByFour shall be true. If DynamicWarpingSixteenthPel is true, then DynamicWarpingHalfPel, DynamicPictureResizingByFour, and DynamicPictureResizingSixteenthPel shall be true.

independentSegmentDecoding, when true, indicates the capability of an encoder or decoder to support the Independent Segment Decoding Mode (H.263 Annex R) of H.263.

slicesInOrder-NonRect, when true, indicates the capability of an encoder or decoder to support the submode of Slice Structured Mode (H.263 Annex K) where slices are transmitted in order and contain macroblocks in scanning order of the picture.

slicesInOrder-Rect, when true, indicates the capability of an encoder or decoder to support the submode of Slice Structured Mode (H.263 Annex K) where slices are transmitted in order and the slice occupies a rectangular region of the picture.

slicesNoOrder-NonRect, when true, indicates the capability of an encoder or decoder to support the submode of Slice Structured Mode (H.263 Annex K) where slices contain macroblocks in scanning order of the picture and need not be transmitted in order.

slicesNoOrder-Rect, when true, indicates the capability of an encoder or decoder to support the submode of Slice Structured Mode (H.263 Annex K) where slices occupy a rectangular region of the picture and need not be transmitted in order.

alternateInterVLCMode, when true, indicates the capability of an encoder or decoder to support Alternate Inter VLC Mode (Annex S) of H.263.

modifiedQuantizationMode, when true, indicates the capability of an encoder or decoder to support the Modified Quantization Mode (Annex T) of H.263.

reducedResolutionUpdate, when true, indicates the capability of an encoder or decoder to support the Reduced Resolution Update mode defined in Annex Q of Recommendation H.263.

TransparencyParameters: indicate parameters specifying a transparent video layer.

presentationOrder indicates the layering of transparent video layers. During capability exchange, the value of presentationOrder shall take one of the values 0, 1 and 2: if 0, it indicates that the Reference Picture Background (RPB) type of transparency support as defined in Annex L of H.263 is supported; if 1, it indicates that an externally controlled background picture can be used; and if 2, it indicates that the bitstream can specify use of either the Reference Picture Background transparency or an externally-controlled background picture type of transparency. During logical open channel, the INTEGER value specifies the presentation order. A layer with a higher presentation order shall be layered on top of a layer with a lower presentation order. The presentationOrder can be viewed as an axis perpendicular to the screen with direction of increasing parameter towards the viewer.

offset-x and offset-y indicate the pixel offset, in 1/8 pixels, of the signalled transparent layer to base layer, in units relative to the base layer. When used in a capability, these denote the capability to offset the location of the transparent video layer, and shall have values restricted to 1, 2, 4, or 8, in 1/8 pel units: for example, if the value is 4, the capability to offset the transparent layer in 1/2 pixel increments is indicated.

scale-x and scale-y indicate a scaling factor to be applied in the corresponding x and y coordinates to the signalled transparent layer before video layering, in units relative to the base layer. In a capability message, they indicate the maximum scale factor that can be applied: 1 indicates rescaling is not supported, 2 indicates it can double the size of the layer or keep it unscaled, 3 indicates it can double it, triple it or keep it unscaled, etc.).

The boolean separateVideoBackChannel indicates, when true, that the terminal can support the Separate Logical Channel mode: no other video capability shall be indicated in the same H263VideoCapability: no MPI values shall be present, and all other mode flags and contents have no meaning and shall be false or absent. When sent in a mode request message, separateVideoBackChannel = true, shall be sent as the only video capability in that H263VideoMode, and indicates that the receiver wants to receive a channel containing only H.263 back-channel data. If present in the OpenLogicalChannel message, it indicates that the logical channel is for video back channel messages only and no other H.263 video bitstream shall be delivered by that logical channel

refPictureSelection: indicates the capability of Reference Picture Selection mode (H.263 Annex N).

If present, additionalPictureMemory indicates the presence of the extra amount of memory, in addition to the amount which can be used by the normal decoder which does not support reference picture selection mode. If not present, it indicates that no information regarding the additional amount of memory which the decoder can use is available for an encoder at the other terminal. If it is indicated in H263VideoMode, it indicates the presence of the additional amount of picture memory used for decoding.

sqcifAdditionalPictureMemory indicates that the encoder can send or the decoder can receive an H.263 bitstream which requires the decoder to have the additional memory to store the indicated number of pictures of size SQCIF, or of a smaller size in both horizontal and vertical dimension if custom picture format support for such pictures is indicated in customPictureFormat.

qcifAdditionalPictureMemory indicates that the encoder can send or the decoder can receive an H.263 bitstream which requires the decoder to have the additional memory to store the indicated number of pictures of size QCIF, or of a smaller size in both horizontal and vertical dimension if custom picture format support for such pictures is indicated in customPictureFormat. The number of picture memories indicated in qcifAdditionalPictureMemory shall not be larger than the number of pictures indicated in sqcifAdditionalPictureMemory (if present).

cifAdditionalPictureMemory indicates that the encoder can send or the decoder can receive an H.263 bitstream which requires the decoder to have the additional memory to store the indicated number of pictures of size CIF, or of a smaller size in both horizontal and vertical dimension if custom picture format support for such pictures is indicated in customPictureFormat. The number of picture memories indicated in cifAdditionalPictureMemory shall not be larger than the number of pictures indicated in sqcifAdditionalPictureMemory, or qcifAdditionalPictureMemory (if present).

cif4AdditionalPictureMemory indicates that the encoder can send or the decoder can receive an H.263 bitstream which requires the decoder to have the additional memory to store the indicated number of pictures of size 4CIF, or of a smaller size in both horizontal and vertical dimension if custom picture format support for such pictures is indicated in customPictureFormat. The number of picture memories indicated in cif4AdditionalPictureMemory shall not be larger than the number of pictures indicated in sqcifAdditionalPictureMemory, qcifAdditionalPictureMemory, or cifAdditionalPictureMemory (if present).

cif16AdditionalPictureMemory indicates that the encoder can send or the decoder can receive an H.263 bitstream which requires the decoder to have the additional memory to store the indicated number of pictures of size 16CIF, or of a smaller size in both horizontal and vertical dimension if custom picture format support for such pictures is indicated in customPictureFormat. The number of picture memories indicated in cif16AdditionalPictureMemory shall not be larger than the number of pictures indicated in sqcifAdditionalPictureMemory, qcifAdditionalPictureMemory, cifAdditionalPictureMemory, or cif4AdditionalPictureMemory (if present).

bigCpfAdditionalPictureMemory indicates that the encoder can send or the decoder can receive an H.263 bitstream which requires the decoder to have the additional memory to store the indicated number of pictures having a custom picture format of a size indicated in customPictureFormat which are larger than 16CIF in the horizontal or vertical dimension. The number of picture memories indicated in bigCPFAdditionalPictureMemory shall not be larger than the number of pictures indicated in sqcifAdditionalPictureMemory, qcifAdditionalPictureMemory, cifAdditionalPictureMemory, cif4AdditionalPictureMemory, or cif16AdditionalPictureMemory (if present).

videoMux indicates, during the capability exchange procedure, that the terminal can support the VideoMux mode illustrated in H.263 Annex N. When true, the encoder or decoder can use a video bitstream containing video back channel message. If it is indicated in H263VideoMode, it indicates that receiving video back channel messages in VideoMux mode is preferable. When used in H263VideoMode, videoMux and separateVideoBackChannel shall not both be true.

videoBackChannelSend indicates which type of video back channel message is supported by the terminal. If it is indicated in H263VideoMode, it indicates the preferred type of back channel message.

none indicates that the encoder is not capable of sending or the decoder is not capable of receiving an H.263 bitstream which contains requests for any back-channel messages to be returned.

ackMessageOnly indicates that the encoder is capable of sending or the decoder is capable of receiving an H.263 bitstream which contains requests for only acknowledgement back channel messages to be returned.

nackMessageOnly indicates that the encoder is capable of sending or the decoder is capable of receiving an H.263 bitstream which contains requests for only non-acknowledgement back channel messages to be returned.

ackOrNackMessageOnly indicates that the encoder is capable of sending or the decoder is capable of receiving an H.263 bitstream which contains requests for either acknowledgement or non-acknowledgement back channel messages to be returned, but only one for a particular video bitstream.

ackAndNackMessage indicates that the encoder is capable of sending or the decoder is capable of receiving an H.263 bitstream which contains requests for acknowledgement and non-acknowledgement back channel messages to be returned.

CustomPictureClockFrequency: indicates the capability to support custom picture clock frequency when present as a capability, and parameters for custom picture clock frequency when present in OpenLogicalChannel and RequestMode.

When used in OpenLogicalChannel, if customPictureClockFrequency has more than one member in its set, then the video bitstream is allowed to switch between the various Picture Clock Frequencies (PCFs) within that set within the same video bitstream. Even if there is only one PCF in the set, if any MPI values are sent for the standard PCF at higher levels in the same message (e.g., in the same H263VideoCapability), then within the same bitstream there can be switching between the standard PCF and the custom one. If one wishes to indicate that the PCF should not change within the bitstream, then data relevant to only one PCF should be sent (either only MPI values for the standard PCF or just the customPictureClockFrequency).

clockConversionCode indicates the clock conversion code when custom picture clock frequency is used in H.263.

clockDivisor indicates the natural binary representation of the value of the clock divisor. The custom picture clock frequency is given by 1 800 000/(clock divisor * clock conversion factor) Hz.

If present, sqcifMPI indicates the minimum picture interval in units of 1/(custom picture clock frequency) for the encoding and/or decoding of SQCIF pictures, and if not present, no capability for SQCIF pictures is indicated.

If present, qcifMPI indicates the minimum picture interval in units of 1/(custom picture clock frequency) for the encoding and/or decoding of QCIF pictures, and if not present, no capability for QCIF pictures is indicated.

If present, cifMPI indicates the minimum picture interval in units of 1/(custom picture clock frequency) for the encoding and/or decoding of CIF pictures, and if not present, no capability for CIF pictures is indicated.

If present, cif4MPI indicates the minimum picture interval in units of 1/(custom picture clock frequency) for the encoding and/or decoding of 4CIF pictures, and if not present, no capability for 4CIF pictures is indicated.

If present, cif16MPI indicates the minimum picture interval in units of 1/(custom picture clock frequency) for the encoding and/or decoding of 16CIF pictures, and if not present, no capability for 16CIF pictures is indicated.

CustomPictureFormat: indicates the capability to support a custom picture format when present as a capability, and parameters for custom picture format when present in OpenLogicalChannel and RequestMode.

The parameters maxCustomPictureWidth, maxCustomPictureHeight, minCustomPictureWidth, minCustomPictureHeight indicate the range of picture sizes that an encoder or decoder can support; and the requested picture size in the case of use with RequestMode.

standardMPI indicates the minimum picture interval in units of 1/29.97 when no custom picture clock frequency is used.

customPCF indicates the parameters for custom picture clock frequency when used in conjunction with custom picture format.

clockConversionCode indicates a clock conversion code when custom picture clock frequency is used in H.263.

clockDivisor indicates the natural binary representation of the value of the clock divisor. The custom picture clock frequency is given by 1 800 000/(clock divisor * clock conversion factor) Hz.

customMPI indicates the minimum picture interval in units of 1/(custom picture clock frequency) for the encoding and/or decoding of pictures in the custom picture format size.

pixelAspectInformation indicates the capability of an encoder or decoder to support various pixel aspect ratios; and the requested pixel aspect ratio in the case of use with RequestMode.

pixelAspectCode indicates the capability to support the pixel aspect ratio as indicated by the PAR code of H.263.

extendedPAR: width, height indicate the capability to support the pixel aspect ratio as indicated by the extended pixel aspect ratio code (EPAR) of H.263.

H263VideoModeCombos

When present, h263VideoModeCombos is used to indicate dependencies among optional modes of H.263. The mode combinations for which capabilities are indicated in h263VideoModeCombos are not implied to be allowed for use with other optional modes signalled at higher levels within the same H263Options or H263VideoCapability or H263VideoMode message except as noted in the fourth paragraph of this section and in the third paragraph of the following section. In other words, if support for some mode booleans for the same modes that contain booleans in H263VideoModeCombos are indicated at higher levels of the syntax in h263Mode or H263Capability, these modes are not assumed to also apply in uncoupled combinations with the modes declared in H263VideoModeCombos.

h263VideoUncoupledModes indicates which optional modes of H.263 operation can be switched on or off independently to each other in any syntactically correct way for a picture and which can be switched on or off independently to the modes indicated within the h263VideoCoupledModes sent in the same H263VideoModeCombos sequence.

h263VideoCoupledModes indicates one or more sets of the optional modes of H.263 operation which can be switched on or off together for a picture within an H.263 bitstream, but for which the ability to independently switch on or off any subset of these modes is not implied. Any set of modes which are indicated as coupled in an h263VideoCoupledModes message can be used along with the full set or any subset of the modes that are indicated as uncoupled in the accompanying h263VideoUncoupledModes message within the same H263VideoModeCombos message. Within the contents of each H263ModeComboFlags message of an h263VideoCoupledModes message there shall be at least two boolean flags set to true, and there shall not be a set of mode flags set to true which indicates a coupled combination of modes which is not syntactically allowed within the same picture of an H.263 bitstream.

Some optional features of Recommendation H.263 are not included in H263ModeComboFlags since they are thought unlikely to require coupling in implementation. Specifically, these include the features specified in Annex L of H.263 (for example, fullPictureFreeze, partialPictureFreezeAndRelease, and resizingPartPicFreezeAndRelease) and the optional picture formats and optional picture clock frequencies. If support of any of these such features is signalled at a higher level within the same H263Options or H263VideoCapability or H263VideoMode message, these features shall operate in an uncoupled manner with the mode combinations signalled in H263VideoModeCombos. A fairly complex example of the use of H263VideoModeCombos follows.

The example consists of a case in which H263VideoCapability indicates that advancedPrediction and unrestrictedVector are supported, and (in the same H263VideoCapability message) inside an H263Options message it is indicated that dynamicPictureResizingByFour is supported and (in the same H263VideoCapability message) inside a H263VideoModeCombos message is an h263VideoUncoupledModes message which indicates that advancedIntraCodingMode is supported in an uncoupled manner along with an h263VideoCoupledModes message which indicates that modifiedQuantizationMode and slicesInOrder-NonRect are supported in a coupled manner.  This then means that the video bitstream can contain (only) pictures with the following mode combinations: None, advancedPrediction, unrestrictedVector, dynamicPictureResizingByFour, advancedPrediction with unrestrictedVector, advancedPrediction with dynamicPictureResizingByFour, unrestrictedVector with dynamicPictureResizingByFour, advancedPrediction with unrestrictedVector with dynamicPictureResizingByFour, advancedIntraCodingMode, modifiedQuantizationMode with slicesInOrder-NonRect, and advancedIntraCodingMode with modifiedQuantizationMode with slicesInOrder-NonRect.

H263ModeComboFlags

The individual parameters of H263ModeComboFlags have the same meaning as the parameters with the same name in H263VideoCapability and H263Options.

unlimitedMotionVectors shall be FALSE if unrestrictedVector is FALSE in the same H263VideoUncoupledModes message. unlimitedMotionVectors shall be FALSE if unrestrictedVector is FALSE in the same H263VideoCoupledModes message and in the H263VideoUncoupledModes message in the same H263VideoModeCombos message.

referencePicSelect, when true, indicates the ability of the encoder or decoder to use the Reference Picture Selection mode of H.263. When true, the specific parameters specifying how the Reference Picture Selection mode can be used shall be as sent in the refPictureSelection field of the same H263Options message. referencePicSelect shall not be true unless refPicturesSelection is present in the same H263Options message.

IS11172 VideoCapability: indicates IS11172 [97] capabilities.

constrainedBitstream indicates the capability for bitstreams in which constrained_parameters flag is set to "1": a value of true indicates that such operation is possible, while a value of false indicates that such operation is not possible. An encoder shall produce bitstreams within the limitations imposed by the optional fields (see below). A decoder shall be able to accept all bit streams within the limitations indicated by the optional fields. The optional fields are integers with units defined in Table B.5.

TABLE B.6/H.245

Units for IS11172-2 codepoints

| ASN.1 Codepoint | Units for referenced parameter |
| --- | --- |
| videoBitRate | 400 bit/s |
| vbvBufferSize | 16384 bits |
| samplesPerLine | samples per line |
| linesPerFrame | lines per frame |
| pictureRate | refer to Section 2.4.3.2 of IS11172-2 |
| luminanceSampleRate | samples per second |

genericVideoCapability: indicates generic video capabilities.

#### B.2.2.6	Audio Capabilities

This indicates audio capabilities. The indication of more than a single capability within a single AudioCapability does not indicate simultaneous processing capability. Simultaneous processing capability can be indicated by instances of AudioCapability in different AlternativeCapabilitySets in a single CapabilityDescriptor.

The capability to transmit and/or receive G-series audio is indicated by a choice of integers. When an H.222.1 multiplex is used, these numbers refer to the available STD buffer size in units of 256 octets. When an H.223 multiplex is used, these numbers refer to the maximum number of audio frames per AL-SDU. When an H.225.0 multiplex is used, these numbers indicate the maximum number of audio frames per packet. The exact meaning of the codepoints is given in Table B.7.

TABLE B.8/H.245

G-series audio codepoints

| ASN.1 Codepoint | Semantic meaning of codepoint |
| --- | --- |
| g711Alaw64k | G.711 audio at 64 kbit/s, A-law |
| g711Alaw56k | G.711 audio at 56 kbit/s, A-law, truncated to 7 bits |
| g711Ulaw64k | G.711 audio at 64 kbit/s, m-law |
| g711Ulaw56k | G.711 audio at 56 kbit/s, m-law, truncated to 7 bits |
| g722-64k | G.722 7 kHz audio at 64 kbit/s |
| g722-56k | G.722 7 kHz audio at 56 kbit/s |
| g722-48k | G.722 7 kHz audio at 48 kbit/s |
| g7231 | G.723.1 at either 5.3 or 6.3 kbit/s |
| g728 | G.728 audio at 16 kbit/s |
| g729 | G.729 audio at 8 kbit/s |
| g729AnnexA | G.729AnnexA audio at 8 kbit/s |
| g729wAnnexB | G.729 audio at 8 kbit/s with silence suppression as in Annex B |
| g729AnnexAwAnnexB | G.729AnnexA audio at 8 kbit/s with silence suppression as in Annex B |
| g7231AnnexCCapability | G.723.1 with Annex C |
| gsmFullRate | Full-rate speech transcoding (GSM 06.10) |
| gsmHalfRate | Half-rate speech transcoding (GSM 06.20) |
| gsmEnhancedFullRate | Enhanced Full Rate (EFR) speech transoding (GSM 06.60) |
| g729Extensions | G.729 Extensions |

G7231: indicates the ability to process audio codec G723.1. maxAl-sduAudioFrames indicates the maximum number of audio frames per AL-SDU. The boolean silenceSupression, when true, indicates the capability to use silence compression defined in G.723.1 Annex A.

G7231AnnexCCapability: indicates the ability to process audio codec G723.1 with its Annex C. maxAl-sduAudioFrames indicates the maximum number of audio frames per AL-SDU. The boolean silenceSupression, when true, indicates the capability to use silence compression defined in G.723.1 Annex A. g723AnnexCAudioMode shall not be present when G7231AnnexCCapability is included in a TerminalCapabilitySet message, but shall be present when G7231AnnexCCapability is included in an OpenLogicalChannel message. The fields highRateMode0, highRateMode1, lowRateMode0, lowRateMode1, sidMode0, and sidMode1 indicate the number of octets per frame for each of the audio and error protection modes of G.723.1 and G.723.1 Annex C that will be used on the logical channel.

IS11172AudioCapability: indicates the ability to process audio coded according to ISO/IEC 11172-3 [98].

Booleans that have the value of true indicate that the particular mode of operation is possible, while a value of false indicates that it is not. The booleans audioLayer1, audioLayer2 and audioLayer3 indicate which audio coding layers can be processed. The booleans audioSampling32k, audioSampling44k1 and audioSampling48k indicate which of the audio sample rates, 32 kHz, 44.1 kHz and 48 kHz respectively, can be processed. The booleans singleChannel and twoChannels indicate capability for single channel and stereo/dual channel operation respectively. The integer bitRate indicates the maximum audio bit rate capability, and is measured in units of kbit/s.

IS13818AudioCapability: indicates the ability to process audio coded according to ISO/IEC 13818-3 [99].

Booleans that have the value of true indicate that the particular mode of operation is possible, while a value of false indicates that it is not. The booleans audioLayer1, audioLayer2 and audioLayer3 indicate which audio coding layers can be processed. The booleans audioSampling16k, audioSampling22k05, audioSampling24k, audioSampling32k, audioSampling44k1 and audioSampling48k indicate which of the audio sample rates, 16 kHz, 22.05 kHz, 24 kHz, 32 kHz, 44.1 kHz and 48 kHz respectively, can be processed.

The booleans concerned with multi-channel operation indicate capability to operate in the particular modes, as specified in Table B.9.

TABLE B.10/H.245

ISO/IEC 13818-3 multi-channel codepoints

| ASN.1 Codepoint | Semantic meaning of codepoint |
| --- | --- |
| singleChannel | One channel, using the 1/0 configuration. Single channel mode (as in ISO/IEC 11172-3) |
| twoChannels | Two channels, using the 2/0 configuration. Stereo or dual channel mode (as in ISO/IEC 11172-3) |
| threeChannels2-1 | Three channels, using the 2/1 configuration. Left, Right and single surround channel |
| threeChannels3-0 | Three channels, using the 3/0 configuration. Left, Centre and Right, without surround channel |
| fourChannels2-0-2-0 | Four channels, using the 2/0 + 2/0 configuration. Left and Right of the first programme and Left and Right of the second programme |
| fourChannels2-2 | Four channels, using the 2/2 configuration. Left, Right, Left surround and Right surround |
| fourChannels3-1 | Four channels, using the 3/1 configuration. Left, Centre, Right, and a single surround channel |
| fiveChannels3-0-2-0 | Five channels, using the 3/0 + 2/0 configuration. Left, Centre and Right of the first programme and Left and Right of the second programme |
| fiveChannels3-2 | Five channels, using the 3/2 configuration. Left, Centre, Right, Left surround and Right surround |

The boolean lowFrequencyEnhancement indicates the capability for a low frequency enhancement channel.

The boolean multilingual, when true, indicates the capability to support up to seven multilingual channels, and when false that no multilingual channel is supported.

The integer bitRate indicates the maximum audio bit rate capability, and is measured in units of kbit/s.

GSMAudioCapability: indicates capabilities for the GSM full rate, half rate and enhanced full rate speech trancoding audio codecs. audioUnitSize indicates the maximum number of bytes to be sent in each packet, comfortNoise, when true, indicates the capability to support the comfort noise processing for the full, half or enhanced full rate speech traffic channel (GSM 06.12, GSM 06.22, GSM 06.62), and scrambled, when true, indicates the capability to support bit scrambling for the full, half, or enhanced full rate speech traffic channels (GSM 06.10, GSM 06.20, GSM 06.60).

genericAudioCapability: indicates generic audio capabilities.

g729Extensions: indicates capability of additional optional modes of G.729. This codepoint shall not be used to indicate g729AnnexA, g729wAnnexB and g729AnnexAwAnnexB, for which g729AnnexA, g729AnnexB and g729AnnexAwAnnexB shall be used.

audioUnit:

for an H.222 multiplex, indicates the size of the STD buffer in units of 256 octets,

for an H.223 multiplex, indicates the maximum number of audio frames per AL-SDU, and

for an H.225.0 multiplex, indicates the maximum number of audio frames per packet.

audioUnit shall be present for Capability Exchange. It may be present for Mode Request.

annexA, when true, indicates the capability to transmit or receive the G.729AnnexA audio at 8kbit/s, instead of G.729 main body.

annexB,  when true, indicates the capability of silence suppression as in Annex B.

annexD, when true, indicates the capability to transmit or receive the G.729 Annex D audio at 6.4kbit/s.

annexE, when true, indicates the capability to transmit or receive the G.729 Annex E audio at 11.8kbit/s.

annexF, when true, indicates the capability of silence supression as in Annex F.

annexG, when true, indicates the capability of silence supression as in Annex G.

annexH, when true, indicates the capability of switching operation between 6.4 kbit/s (Annex D) and 11.8 kbit/s (Annex E).

#### B.2.2.7	Data Application Capabilities

This indicates data capabilities. The indication of more than a single capability within a single DataApplicationCapability does not indicate simultaneous processing capability. Simultaneous processing capability can be indicated by instances of DataApplicationCapability in different AlternativeCapabilitySets in a single CapabilityDescriptor.

Recommendations that use this Recommendation may place restrictions on which of these modes may be signalled.

Some of the data capabilities require bi-directional logical channels, for example, to run a retransmission protocol. This requirement is implicitly included in the appropriate capability codepoints.

DataApplicationCapability: is a list of data applications and bit rates. Each data application indicated shall be supported by one or more DataProtocolCapabilitys.

maxBitRate indicates the maximum bit rate in units of 100 bit/s at which a transmitter can transmit video or a receiver can receive the given data application.

t120 indicates the capability to support the T.120 [100] protocol.

dsm-cc indicates the capability to support the DSM-CC [101] protocol.

userData indicates the capability to support unspecified user data from external data ports.

t84 indicates the capability to support the transfer of T.84 [102] type images (JPEG, JBIG, Facsimile Gr.3/4).

t434 indicates the capability to support the transfer of T.434 [103] telematic binary files.

h224 indicates the capability to support the real-time simplex device control protocol H.224 [104].

nlpid indicates the capability to support the network layer protocol as specified by nlpidData as defined in ISO/IEC TR9577 [105]. These protocols include Internet protocol (IP) and IETF Point-to-Point protocol (PPP), among others.

NOTE - The use of the NLPID is extensively described in IETC RFC1490, "Multiprotocol Interconnect over Frame Relay".

dsvdControl indicates the capability of the DSVD terminal to support an out-of-band control channel.

h222DataPartitioning indicates the capability to support the modified and restricted usage of data partitioning of H.262, as specified in H.222.1, in which the enhancement data is transmitted as a data channel supported by the listed DataProtocolCapability.

t30fax: This codepoint indicates the capability to use T.30 Annex C analog mode (G3V), as specified in Rec. T.39 for the DSVF/MSVF modes.

t140: This codepoint indicates the capability to use the T.140 text conversation protocol, as specified in Recommendation T.140.

t38fax: This codepoint indicates a data protocol conforming to ITU-T T.38 [106].

The fields version, t38FaxRateManagement and t38FaxUdpOptions are defined in T.38.

fillBitRemoval, when true, indicates that the gateway/terminal has the ability to remove and insert fill bits.

transcodingJBIG, when true, indicates that the gateway has the ability to transcode in real time between the line compression and JBIG for transfer over the IP network.

transcodingMMG, when true, indicates that the gateway has the ability to transcode in real time between the line compression and MMG for transfer over the IP network.

genericDataCapability indicates generic data capabilities. When maxBitRate is included in the genericDataCapability, its value shall be the same as the value of maxBitRate in the DataApplicationCapability.

DataProtocolCapability: contains a list of data protocols.

v14buffered indicates the capability to support a specified data application using buffered V.14 [107].

v42lapm indicates the capability to support a specified data application using the LAPM protocol defined in V.42 [108].

hdlcFrameTunnelling indicates the capability to support a specified data application using HDLC Frame Tunnelling. Refer to clause 4.5.2 of ISO/IEC 3309 [109].

h310SeparateVCStack indicates the capability to support a specified data application using the protocol stack defined in H.310 for the transport of H.245 messages over a separate ATM VC to that used for audiovisual communication.

h310SingleVCStack indicates the capability to support a specified data application using the protocol stack defined in H.310 for the transport of H.245 messages in the same ATM VC as that used for audiovisual communication.

transparent indicates the capability to support a specified data application using transparent data transfer.

v120: use of v120 is for further study in H.323.

separateLANStack indicates that a separate transport stack will be used to transport the data. The intent of a separate network connection for data is indicated by dataType in OpenLogicalChannel resolving to values h310SeparateVCStack or separateLANStack of DataProtocolCapability. When the selected DataApplicationCapability is t120, these choices imply use of the T.123 basic profile for B-ISDN and LAN, respectively. Alternative LAN profiles may be selected by a nonStandard DataProtocolCapability.

If separateLANStack is selected and separateStack is present in the OpenLogicalChannel request, the receiver should attempt to establish the stack indicated. It will respond OpenLogicalChannelAck if successful, otherwise OpenLogicalChannelReject with a suitable cause.

If separateLANStack is selected and separateStack is absent in the OpenLogicalChannel request, the receiver should supply an appropriate separateStack in its OpenLogicalChannelAck response. The receiver of this (the original requester) should then attempt to establish the stack indicated. It will issue CloseLogicalChannel if unsuccessful.

If separateLANStack is selected and separateStack is present in the OpenLogicalChannel request, it can be overridden by separateStack in the OpenLogicalChannelAck response. If the original requester does not tolerate an override, it will issue CloseLogicalChannel.

If separateLANStack is selected and separateStack is absent in the OpenLogicalChannel request and also absent in the OpenLogicalChannelAck response, the original requester can infer that the responder does not understand these ASN.1 extensions and should issue CloseLogicalChannel to clean up.

v76wCompression indicates the capability to support data compression on a V.76 data channel.

tcp incidates the capability to support TCP/IP for this application.

udp incidates the capability to support UDP for this application.T84Profile: indicates the types of still image profile that the terminal is able to support.

t84Unrestricted provides no indication of the type of T.84 still image that the terminal is able to support: information in the T.84 layer should be used to determine whether a particular image can be received.

t84Restricted indicates the type of T.84 still image that the terminal is able to support.

qcif indicates the support of a sequential colour YCrCb type image with QCIF resolution.

cif indicates the support of a sequential colour YCrCb type image with CIF resolution.

ccir601Seq indicates the support of a sequential colour YCrCb type image with CCIR601 resolution.

ccir601Prog indicates the support of a progressive colour YCrCb type image with CCIR601 resolution.

hdtvSeq indicates the support of a sequential colour YCrCb type image with HDTV resolution.

hdtvProg indicates the support of a progressive colour YCrCb type image with HDTV resolution.

g3FacsMH200x100 indicates the support of a sequential Facsimile Gr. 3 MH (Modified Huffman) coded bi-level image at the normal (200x100ppi) resolution.

g3FacsMH200x200 indicates the support of a sequential Facsimile Gr. 3 MH (Modified Huffman) coded bi-level image at the high (200x200ppi) resolution.

g4FacsMMR200x100 indicates the support of a sequential Facsimile Gr. 4 MMR (Modified Modified Reed) coded bi-level image at the normal (200x100ppi) resolution.

g4FacsMMR200x200 indicates the support of a sequential Facsimile Gr. 4 MMR (Modified Modified Reed) coded bi-level image at the high (200x200ppi) resolution.

jbig200x200Seq indicates the support of a sequential bi-level JBIG coded bi-level image at the 200x200ppi resolution.

jbig200x200Prog indicates the support of a progressive bi-level JBIG coded bi-level image at the 200x200ppi resolution.

jbig300x300Seq indicates the support of a sequential bi-level JBIG coded bi-level image at the 300x300ppi resolution.

jbig300x300Prog	indicates the support of a progressive bi-level JBIG coded bi-level image at the 300x300ppi resolution.

digPhotoLow	indicates the support of a sequential JPEG coded colour image of up to 720x576 image size.

digPhotoMedSeq indicates the support of a sequential JPEG coded colour image of up to 1440x1152 image size.

digPhotoMedProg indicates the support of a progressive JPEG coded colour image of up to 1440x1152 image size.

digPhotoHighSeq indicates the support of a sequential JPEG coded colour image of up to 2880x2304 image size.

digPhotoHighProg indicates the support of a progressive JPEG coded colour image of up to 2880x2304 image size.

#### B.2.2.8	Encryption, Authentication and Integrity Capabilities

EncryptionCapabilities, if present, indicate the encryption capabilities of a terminal for each media type where the capabilities are present. The scope of encryption indicates whether the encryption is applied to the entire bit stream, a portion of the bit stream in a standard way, or a portion of the stream in a non standard way. The algorithm selects the encryption algorithm.

AuthenticationCapabilities if present, indicate that the authentication components of H.235 [110] are supported by the terminal.

IntegrityCapabilities if present, indicate that the integrity components of H.235 [111] are supported by the terminal.

#### B.2.2.9	Conference Capabilities

ConferenceCapability indicates conference capabilities such as the ability to support Chair Control as described in H.243.

videoIndicateMixingCapability shall be defined as H.230 VIM.

#### B.2.2.10	User Input Capabilities

UserInputCapabilities indicates which parameters in the UserInputIndication message are supported by the terminal. BasicString indicates that the terminal supports the basicString option of userInputSupportIndication, iA5String indicates that the terminal supports the iA5String option of userInputSupportIndication, and generalString indicates that the terminal supports the generalString option of userInputSupportIndication. Dtmf indicates that the terminal supports dtmf using the signal and signal update components of the userInputIndication message. Hookflash indicates that the terminal supports hookflash using the signal and signal update components of the userInputIndication message.

#### B.2.2.11	Generic Capabilities

The GenericCapability type allows new capabilities to be specified in such a way that a new version of H.245 syntax does not need to be issued. This generic means of specifying capabilities allows network based devices, such as MCs, to determine a highest common operating mode without having a detailed knowledge of the capability being used. It allows for both ITU-T and other standards based capability descriptions (including proprietary capability descriptions) to be defined. ITU-T standards based capability descriptions should be included as Annexes to H.245. Non ITU-T standards capability descriptions may be published in any suitable form.

The field capabilityIdentifier indicates which type of capability is being defined. ITU-T based capability descriptions shall use the standard OBJECT IDENTIFIER, while other standards based and proprietary capability descriptions shall use one of standard, h221NonStandard, uuid and domainBased as appropriate.

The field subIdentifier indicates a type or set of parameters associated with the capabilityIdentifier.

maxBitRate indicates the maximum rate at which the capability can operate when capabilities are exchanged, and the actual bit rate to be used when open logical channel signalling takes place. It shall be present whenever a meaningful value can be indicated, and when mandated by the specification of the particular capability description. It is defined separately so that intermediaries on the signalling path can have visibility of the bandwidth being used without having detailed knowledge of each capability.

The parameters of the capability can be described as any combination of collapsing, nonCollapsing, and nonCollapsingRaw, together with transport, as specified in the capability description.

The collapsing field indicates capabilities that are described in such a way that an MC can combine the capabilities from a number of endpoints and build a common capability set using a simple set of rules without having detailed knowledge of the individual codec.

The nonCollapsing field indicates capabilities using the same syntax as collapsing but which can not be processed by an MC. In this case, the semantics of ParameterValue change to indictate only values and not collapsing rules. For example, unsignedMin and unsignedMax have the same semantics, and simply indicate a 16 bit integer parameter.

The nonCollapsingRaw field indicates capabilities using an OCTET STRING. Typically this may consist of a ASN.1 PER encoded data structure. Note that an MC must have specific knowledge of capabilities described in this way to make use of them.

The transport field indicates transport parameters specific to the capability being described.

It is recommended when specifying capability descriptions to define as many parameters as possible as collapsing, as it is only parameters defined in this was that are ensured to be processed rather than simply forwarded by network elements.

GenericParameter indicates one capability parameter or one group of capability parameters.

The parameterIdentifier allows the values of standard (i.e. defined in the capability description) and proprietry parameters to be indicated. The parameters defined in the capability description use the standard form which identifies the parameter with an integer. Parameters that are propriety extensions use the h221NonStandard, uuid, or domainBased forms.

The parameterValue field indicates the parameter’s value. The presence of a logical parameter indicates that the endpoint supports the option that the parameter represents. The booleanArray field contains up to eight independent boolean variables. The unsignedMin and unsignedMax fields indicate a parameter using an unsigned 16 bit integer. The unsigned32Min and unsigned32Max fields indicate a parameter using an unsigned 32 bit integer. The octetString field indicates a parameter as an OCTET STRING. The genericParameter field indicates a sequence of parameters that have been grouped together at this layer of the capability heirarchy.

To combine capability descriptions from multiple endpoints into a common capability description for a capability which an MC has no in built knowledge, an MC should first ignore any parameters which are not supported by all endpoints that the MC has decided are candidates for using a particular capability. Then, for each of the candidate endpoints’ parameters with the same parameterIdentifier, the MC should:

- perform a logical AND in the case of booleanArray;

- choose the minimum value in the case of unsignedMin or unsigned32Min; and

- choose the maximum value in the case of unsignedMax or unsigned32Max.

The supersedes field allows a capability description to contain a group of parameters from which only one should be selected when a common capability description is determined. This might be the case for a video codec that supports SQCIF, QCIF and CIF resolutions with different minimum picture intervals. The value in the parameterIdentifier refers to a parameter at the same level of nesting. Multiple supersedes fields are included with a parameter so that a tree of parameter dependencies can be expressed as is found in the H.262 capability description. Each of the parameters identified in the supersedes field should be discarded from the common capability description.  The parameters that the discarded parameters supersede shall in turn also be discarded and this process shall be repeated until all superseded parameters are discarded.

The result of this operation is the common capability description.

NOTE - An MC that has in-built knowledge of a particular capability description may use its own set of rules to produce a common capability description.

### B.2.3	Terminal Capability Set Acknowledge

This is used to confirm receipt of a TerminalCapabilitySet from the peer CESE.

The sequenceNumber shall be the same as the sequenceNumber in the TerminalCapabilitySet for which this is the confirmation.

### B.2.4	Terminal Capability Set Reject

This is used to reject a TerminalCapabilitySet from the peer CESE.

The sequenceNumber shall be the same as the sequenceNumber in the TerminalCapabilitySet for which this is the negative acknowledgement.

The reasons for sending this message are given in Table B.11.

TABLE B.12/H.245

Reasons for rejecting a TerminalCapabilitySet

| ASN.1 codepoint | Cause |
| --- | --- |
| unspecified | No cause for rejection specified. |
| undefinedTableEntryUsed | A capability descriptor made reference to a capabilityTable entry that is not defined. |
| descriptorCapacityExceeded | The terminal was incapable of storing all of the information in the TerminalCapabilitySet. |
| tableEntryCapacityExceeded | The terminal was incapable of storing more entries than that indicated in highestEntryNumberProcessed or else could not store any. |

### B.2.5	Terminal Capability Set Release

This is sent in the case of a timeout.

## B.3	Logical channel signalling messages

This set of messages is for logical channel signalling. The same set of messages is used for uni-directional and bi-directional logical channel signalling; however, some parameters are only present in the case of bi-directional logical channel signalling.

"Forward" is used to refer to transmission in the direction from the terminal making the original request for a logical channel to the other terminal, and "reverse" is used to refer to the opposite direction of transmission, in the case of a bi-directional channel request.

### B.3.1	Open Logical Channel

This is used to attempt to open a uni-directional logical channel connection between an outgoing LCSE and a peer incoming LCSE and to open a bi-directional logical channel connection between an outgoing B-LCSE and a peer incoming B-LCSE.

forwardLogicalChannelNumber: indicates the logical channel number of the forward logical channel that is to be opened.

forwardLogicalChannelParameters: include parameters associated with the logical channel in the case of attempting to open a uni-directional channel and parameters associated with the forward logical channel in the case of attempting to open a bi-directional channel.

reverseLogicalChannelParameters: include parameters associated with the reverse logical channel in the case of attempting to open a bi-directional channel. Its presence indicates that the request is for a bi-directional logical channel with the stated parameters, and its absence indicates that the request is for a uni-directional logical channel.

NOTE - H.222 parameters are not included in reverseLogicalChannelParameters as their values are not known to the terminal initiating the request.

portNumber is a user to user parameter that may be used by a user for such purposes as associating an input or output port, or higher layer channel number, with the logical channel.

dataType indicates the data that is to be carried on the logical channel.

If it is nullData, the logical channel will not be used for the transport of elementary stream data, but only for adaptation layer information - if video is to be transmitted in one direction only, but a retransmission protocol is to be used, such as AL3 defined in H.223, a return channel is needed to transport the retransmission requests - it may also be used to describe a logical channel that only contains PCR values in the case of H.222.1 Transport Streams [112].

Terminals capable only of uni-directional (transmit or receive) operation on media types which make use of bi-directional channels shall send capabilities only for the supported direction of operation. The reverse direction shall use the nullData type, for which no capability is necessary. Transmit-only terminals should send transmit capabilities, but terminals should not assume that the absence of transmit capabilities implies that transmit-only operation is not possible.

separateStack indicates that a separate transport stack will be used to transport the data and provides an address to use to establish the stack which is either a Q.2931, E.164, or local area network transport address.

networkAccessParameters define the distribution, network address, and creation and association information to be used for the separateStack.

distribution shall be present when networkAddress is set to localAreaNetwork and shall indicate whether the networkAddress is a uni or multicast transport address.

networkAddress indicates the address of the actual stack in use: Q.2931, E.164, or local area network transport address.

associateConference indicates whether or not the data conference is new (associateConference=FALSE) or is an existing data conference which should be associated with the audio/video call (associateConference=TRUE).

externalReference indicates information which may be used to further provide association or information concerning the separateStack.

If it is of type VideoCapability, AudioCapability, the logical channel may be used for any of the variations indicated by each individual capability; and it shall be possible to switch between these variations using only signalling that is in-band to the logical channel - for example, in the case of H.261 video, if both QCIF and CIF are indicated, it shall be possible to switch between these on a picture by picture basis. In the case of DataApplicationCapability, only one instance of a capability can be indicated since there is no in-band signalling allowing a switch between variations.

If it is encryptionData, the logical channel will be used for the transport of encryption information as specified.

forwardLogicalChannelDependency indicates which logical channel number the forward channel to be opened is dependent on.

reverseLogicalChannelDependency indicates which logical channel number the reverse channel to be opened is dependent on.

The replacementFor parameter indicates that the logical channel to be opened will be a replacement for the specified existing, already-open logical channel. This parameter shall be used only to refer to logical channels already in the ESTABLISHED state. Logical channels opened using this parameter shall not carry any data traffic until after all traffic on the referenced established logical channel ceases. Media decoders will in this case never be required to decode data traffic from both logical channels simultaneously. Once traffic on the newly established logical channel has begun, the old logical channel shall immediately be closed. Receivers may acknowledge logical channels opened using the replacementFor mechanism with the understanding that the old and new logical channels shall not be used simultaneously, and therefore will not exceed the receiver's capability to decode.

The encryptionSync field shall be used by the master when acknowledging the opening of a channel by a slave. It is used to provide the encryption key value and the synchronization point at which the key should be used. For H.323, the syncFlag shall be set to the RTP dynamic payload number which matches the key.

H222LogicalChannelParameters: is used to indicate parameters specific to using H.222.1 [113]. It shall be present in forwardLogicalChannelParameters and shall not be present in reverseLogicalChannelParameters.

resourceID indicates in which ATM Virtual Channel the logical channel is to be transported. The means by which this parameter is associated with an ATM Virtual Channel is not specified in this Recommendation.

subChannelID indicates which H.222.1 sub-channel is used for the logical channel. It shall be equal to the PID in a Transport Stream and the stream_id in a Program Stream.

pcr-pid indicates the PID used for the transport of Program Clock References when the Transport Stream is used. It shall be present when the ATM virtual channel carries a Transport Stream and shall not be present when the ATM virtual channel carries a Program Stream.

programDescriptors is an optional octet string, which, if present, contains one or more descriptors, as specified in H.222.0 and H.222.1, that describe the program that the information to be carried in the logical channel is a part of.

streamDescriptors is an optional octet string, which, if present, contains one or more descriptors, as specified in H.222.0 and H.222.1, that describe the information that is to be carried in the logical channel.

H223LogicalChannelParameters: is used to indicate parameters specific to using H.223 [114]. It shall be present in forwardLogicalChannelParameters and reverseLogicalChannelParameters.

adaptationLayerType indicates which adaptation layer and options will be used on the logical channel. The codepoints are as follows: nonStandard, al1Framed (AL1 framed mode), al1NotFramed (AL1 unframed mode), al2WithoutSequenceNumbers (AL2 with no sequence numbers present), al2WithSequenceNumbers (AL2 with sequence numbers present), and al3 (AL3, indicating the number of control field octets that will be present and the size of the send buffer, Bs, that will be used, the size being measured in octets), al1M (AL1M defined in Annex C with the specified parameters), al2M (AL3M defined in Annex C with the specified parameters) or al3M (AL3M defined in Annex C with the specified parameters). .

segmentableFlag, when equal to true indicates that the channel is designated to be segmentable, and when equal to false indicates that the channel is designated to be non-segmentable.

H223AL1MParameters: is used to indicate parameters specific to using adaptation Layer AL1M.

transferMode indicates whether framed mode or unframed mode is used.

headerFEC indicates whether FEC is SEBCH(16,7) or Golay(24,12).

The length of CRC bits for the payload is indicted by crcLength as 4, 8, 12, 16, 20, 28 or 32 bits or by crcNotUsed.

rcpcCodeRate indicates the RCPC code rate as 8/8, 8/9, ..., 8/32.

arqType indicates the ARQ mode of operation: noARQ indicates no retransmission, typeIArq indicates ARQ type I, and typeIIArq indicates ARQ type II.

alpduInterleaving, if true, indicates the use of AL-PDU interleaving.

alsduSplitting, if true, indicates the use of AL-SDU splitting mode.

rsCodeCorrection indicates the RS code correction ability as 0, 1, …, 127 octets. A fixed number of the RS code parity symbols (octets) corresponding to rsCodeCorrection is added to each variable length AL-SDU and CRC field. When the RS coding is used, typeIIArq and alpduInterleaving are not supported.

H223AL2MParameters: is used to indicate parameters specific to using adaptation Layer AL2M.

headerFEC indicates whether FEC is SEBCH(16,5) or Golay(24,12).

alpduInterleaving, if true, indicates the use of AL-PDU interleaving.

H223AL3MParameters: is used to indicate parameters specific to using adaptation Layer AL3M.

This has the same parameters as AL1MParameters, except transferMode and alsduSplitting are not present.

H223AnnexCArqParameters

numberOfRetransmissions indicates the maximum number of retransmissions that may be used: finite indicates a finite limit on the number of retransmissions that may be used in the range 0 to 16; and infinite indicates that there is no limit on the number of retransmissions that may be used. numberOfRetransmissions equal to the finite value of 0 indicates that the control field is used for the splitting mode but retransmissions are not used.

sendBufferSize indicates the size of the send buffer that will be used, the size being measured in octets.

V76LogicalChannelParameters: is used to indicate parameters specific to using V.76.

audioHeader is used to indicate the use of an audio header on the logical channel. This is a valid parameter for channels of the DataType audio.

suspendResume is used to indicate that the channel may use the suspend/resume procedures to suspend other logical channels. Three channel options may be selected; no suspend resume on the channel, suspend resume using an address or suspend resume without an address as defined in V.76. suspendResumewAddress indicates that the suspend/resume channel shall use the address field as defined in V.76. suspendResumewoAddress indicates that the suspend/resume channel shall not use the address field.

eRM indicates that the logical channel shall perform error recovery procedures as defined in V.76.

uNERM indicates that the logical channel shall operate in non error recovery mode as defined in V.76.

For description of n401, windowSize and loopbackTestProcedure see Recommendation V.42, 12.2.1 and its subsections. For the purposes of V.70, n401 shall be encoded in octets.

crcLength is an optional parameter that indicates the CRC length used in error recovery mode. If this parameter is not present, the default CRC length shall be used. crc8bit indicates to use an 8 bit CRC, crc16bit indicates use of the 16 bit CRC and crc32bit indicates to use a 32 bit CRC as defined in V.76.

recovery is an optional parameter that indicates the error recover procedures defined in V.76. If this parameter is not present, the default error recovery procedure shall be used. sREJ indicates to use the selective frame reject procedure and mSREJ indicates to use the multiple selective reject procedure as defined in V.76.

uIH indicates the use of V.76 UIH frames.

rej indicates the use of the reject procedure in V.76.

V75Parameters is used to indicate parameter specific to using V.75. audioHeaderPresent indicates the presence of the V.75 audio header.

H2250LogicalChannelParameters: is used to indicate parameters specific to using H.225.0. It shall be present in forwardLogicalChannelParameters and reverseLogicalChannelParameters.

The sessionID is a unique RTP or T.120 Session Identifier in the conference. It is used by the transmitter to refer to the session to which the logical channel applies. Only the master can create the session identification. By convention, there are three primary sessions. The first primary session with a session identification of 1 is the audio session, the second primary session with a session identification of 2 is the video session, and the third primary session with a session identification of 3 is the data session. A slave entity can open an additional session by providing a session identification of 0 in the openLogicalChannel message. The master will create a unique session identification and provide it in the openLogicalChannelAck message.

The associatedSessionID is used to associate one session with another. Typical use will be to associate an audio session with a video session to indicate which sessions to process for lip synchronization.

The mediaChannel indicates a transportAddress to be used for the logical channel. When the transport is unicast, mediaChannel is not present in the OpenLogicalChannel forwardLogicalChannelParameters, but may be present in the reverseLogicChannelParameters.. If the transportAddress is multicast, the master is responsible for creating the multicast transport address and shall include the address in the OpenLogicalChannel message. A slave entity that wishes to open a new multicast channel will provide zeroes in the multicast transportAddress field. The master will create and provide the multicast transportAddress in the OpenLogicalChannelAck message for the slave entity. Note that the MC will use the communicationModeCommand to specify the details about all the RTP Sessions in the conference.

The mediaChannel is used to describe the transport address for the logical channel. IPv4 and IPv6 addresses shall be encoded with the most significant octet of the address being the first octet in the respective OCTET STRING, e.g. the class B IPv4 address 130.1.2.97 shall have the "130" being encoded in the first octet of the OCTET STRING, followed by the "1" and so forth. The IPv6 address a148:2:3:4:a:b:c:d shall have the "a1" encoded in the first octet, "48" in the second, "00" in the third, "02" in the fourth and so forth. IPX addresses, node, netnum, and port shall be encoded with the most significant octet of each field being the first octet in the respective OCTET STRING.

mediaGuaranteedDelivery indicates whether or not the underlying media transport should be selected to provide or not provide guaranteed delivery of data.

mediaControlChannel indicates the media control channel in which the sender of the open logical channel will be listening for media control messages for this session. This field is present only when a media control channel is required.

mediaControlGuaranteedDelivery indicates whether or not the underlying media control transport should be selected to provide or not provide guaranteed delivery of data. This field is present only when a media control channel is required.

The silenceSuppression is used to indicate whether the transmitter stops sending packets during times of silence. It shall be included in the openLogicalChannel message for an audio channel and omitted for any other type of channel.

destination indicates the terminalLabel of the destination if one has been assigned.

dynamicRTPPayloadType indicates a dynamic payload value which is used in H.323 for the H.225.0 alternative H.261 video packetization scheme. This field is present only when a dynamic RTP payload is in use.

mediaPacketization indicates which optional media packetization scheme is in use.

redundancyEncoding indicates that the redundant encoding method indicated in this parameter is to be used for the logical channel to be opened. The primary encoding is defined by the dataType of the forwardLogicalChannelParameters or the reverseLogicalChannelParameters, respectively. The type of redundancy encoding to be applied for this logical channel is identified by the redundancyEncodingMethod parameter, the secondary encoding is specified in the secondaryEncoding parameter. The DataType (audio, video, etc.) selected for both primary and secondary encoding shall match and shall be in accordance with the redundancyEncodingMethod selected. The source parameter is used to identify the terminal number of the sender of the OpenLogicalChannel message.

h235 Key: is used to include, and specify the method by which media specific session keys are protected as they are passed between two endpoints. The encoding of this field is a nested ASN.1 value as described in H.235

EscrowData is used to specify the type and contents of any key escrow mechanism in use. Specific types and contents may be required by implementations when media encryption is enabled.

T120SetupProcedure indicates how the T.120 conference is to be set up. For originateCall and waitForCall, the caller should derive the T.120 numeric conference name from the H.323 CID (as described in H.323), and issue the appropriate PDU (if the endpoint is master, it should issue an invite request, while a slave should issue a join request). For issueQuery, the caller should first issue a query request, and then set up the T.120 conference in accordance with the contents of the query response (as described in T.124).

symmetricMediaAddress, if TRUE, indicates that the destination transport addresses of the media channel and media control channel are the same as the source transport addresses of the reverse channels and may be derived from messages received on those channels. This element must not be set to TRUE by a sender if either the mediaChannel or the mediaControlChannel element is included. A receiver of this element, if it is set to TRUE, must ignore the mediaChannel and mediaControlChannel elements. This option is intended to facilitate transport of media through a network address translation device.

### B.3.2	Open Logical Channel Acknowledge

This is used to confirm acceptance of the logical channel connection request from the peer LCSE or B-LCSE. In the case of a request for a uni-directional logical channel, it indicates acceptance of that uni-directional logical channel. In the case of a request for a bi-directional logical channel, it indicates acceptance of that bi-directional logical channel, and indicates the appropriate parameters of the reverse channel.

forwardLogicalChannelNumber indicates the logical channel number of the forward channel that is being opened.

reverseLogicalChannelParameters is present if and only if responding to a bi-directional channel request.

reverseLogicalChannelNumber indicates the logical channel number of the reverse channel.

portNumber is a user to user parameter that may be used by a user for such purposes as associating an input or output port, or higher layer channel number, with the reverse logical channel.

multiplexParameters indicate parameters specific to the multiplex, H.222, H.223, or H.225.0, that is used to transport the reverse logical channel.

FlowControlToZero indicates whether the transmitter is allowed to start transmitting on the logical channel. If set to true, it indicates that the transmitter should not transmit on the logical channel until receiving a subsequent FlowControl message, applying to the logical channel, allowing it to do so. If set to false, or absent, the transmitter is allowed to begin transmitting immediately the channel is established.

The replacementFor parameter indicates that the logical channel to be opened will be a replacement for the specified existing, already-open logical channel. This parameter shall be used only to refer to logical channels already in the ESTABLISHED state. logical channels opened using this parameter shall not carry any data traffic until after all traffic on the referenced established logical channel ceases. Media decoders will in this case never be required to decode data traffic from both logical channels simultaneously. Once traffic on the newly established logical channel has begun, the old logical channel shall immediately be closed. Receivers may acknowledge logical channels opened using the replacementFor mechanism with the understanding that the old and new logical channels shall not be used simultaneously, and therefore will not exceed the receiver's capability to decode.

separateStack indicates that a separate transport stack will be used to transport the data and provides an address, to use to establish the stack, which is either a Q.2931, E.164, or local area network transport address.

forwardMultiplexAckParameters indicate parameters specific to the multiplex, H.222, H.223, or H.225.0 that is used to transport the forward logical channel.

The encryptionSync field shall be used by the master when acknowledging the opening of a channel by a slave. It is used to provide the encryption key value and the synchronization point at which the key should be used. For H.323, the syncFlag shall be set to the RTP dynamic payload number which matches the key.

H2250LogicalChannelAckParameters are used to indicate parameters specific to using H.225.0.

sessionID is a unique RTP Session Identifier in the conference that can only be created by the master. It is created and provided by the master if the slave wishes to create a new session by specifying an invalid session identification of 0 in the openLogicalChannelAck message.

The mediaChannel indicates a transportAddress to be used for the logical channel. It shall be present in the OpenLogicalChannelAck message when the transport is unicast except where the OpenLogicalChannel request specified a reverse unicast mediaChannel. If the transportAddress is multicast, the master is responsible for creating the multicast transport address and shall include the address in the OpenLogicalChannel message. A slave entity that wishes to open a new multicast channel will provide zeroes in the multicast transportAddress field. The master will create and provide the multicast transportAddress in the OpenLogicalChannelAck message for the slave entity. Note that the MC will use the communicationModeCommand to specify the details about all the RTP Sessions in the conference.

The mediaChannel is used to describe the transport address for the logical channel. IPv4 and IPv6 addresses shall be encoded with the most significant octet of the address being the first octet in the respective OCTET STRING, e.g. the class B IPv4 address 130.1.2.97 shall have the "130" being encoded in the first octet of the OCTET STRING, followed by the "1" and so forth. The IPv6 address a148:2:3:4:a:b:c:d shall have the "a1" encoded in the first octet, "48" in the second, "00" in the third, "02" in the fourth and so forth. IPX addresses, node, netnum, and port shall be encoded with the most significant octet of each field being the first octet in the respective OCTET STRING.

mediaControlChannel indicates the media control channel in which the sender of the openLogicalChannelAck will be listening for media control messages for this session. This field is present only when a media control channel is required.

dynamicRTPPayloadType indicates a dynamic payload value which is used in H.323 for the H.225.0 alternative H.261 video packetization scheme. This field is present only when a dynamic RTP payload is in use.

The portNumber field is used in Annex C of H.323 when the receiving endpoint finds that the B-HLI given by the portNumber field in the OpenLogicalChannel message is inappropriate, and indicates the alternative value that is to be used.NOTE - H.223 parameters are not included in reverseLogicalChannelParameters as their values were specified in the OpenLogicalChannel request message.

### B.3.3	Open Logical Channel Reject

This is used to reject the logical channel connection request from the peer LCSE or B-LCSE.

NOTE - In the case of a bi-directional channel request, rejection applies to both forward and reverse channels. It is not possible to accept one and reject the other.

forwardLogicalChannelNumber indicates the logical channel number of the forward channel specified in the request that is being rejected.

The cause field indicates the reason for rejection of the logical channel establishment. The cause values are given in Table B.13.

TABLE B.14/H.245

Reasons for rejecting a OpenLogicalChannel

| ASN.1 codepoint | Cause |
| --- | --- |
| unspecified | No cause for rejection specified. |
| unsuitableReverseParameters | This shall only be used to reject a bi-directional logical channel request when the only reason for rejection is that the requested reverseLogicalChannelParameters are inappropriate. Such a rejection shall immediately be followed by initiating procedures to open a similar but acceptable bi-directional logical channel. |
| dataTypeNotSupported | The terminal was not capable of supporting the dataType indicated in OpenLogicalChannel. |
| dataTypeNotAvailable | The terminal was not capable of supporting the dataType indicated in OpenLogicalChannel simultaneously with the dataTypes of logical channels that are already open. |
| unknownDataType | The terminal did not understand the dataType indicated in OpenLogicalChannel. |
| dataTypeALCombinationNotSupported | The terminal was not capable of supporting the dataType indicated in OpenLogicalChannel simultaneously with the Adaptation Layer type indicated in H223LogicalChannelParameters |
| multicastChannelNotAllowed | Multicast Channel could not be opened |
| insuffientBandwdith | The channel could not be opened because permission to use the requested bandwidth for the logical channel was denied. |
| separateStackEstablishmentFailed | A request to run the data portion of a call on a separate stack failed. |
| invalidSessionID | Attempt by slave to set SessionID when opening a logical channel to the master |
| masterSlaveConflict | Attempt by slave to open logical channel in which the master has determined a conflict may occur. (See C.4.1.3 and C.5.1.3) |
| waitForCommunicationMode | Attempt to open logical channel before MC has transmitted the CommunicationModeCommand |
| invalidDependentChannel | Attempt to open logical channel with a dependent channel specified which is not present |
| replacementForRejected | A logical channel of the type attempted cannot be opened using the replacementFor parameter. The transmitter may wish to re-try by firstly closing the logical channel which was to be replaced, and then opening the replacement. |

### B.3.4	Open Logical Channel Confirm

This is used in bi-directional signalling to indicate to the incoming B-LCSE that the reverse channel is open and can be used for transmission.

forwardLogicalChannelNumber indicates the logical channel number of the forward channel which was opened.

### B.3.5	Close Logical Channel

This is used by the outgoing LCSE or B-LCSE to close a logical channel connection between two peer LCSEs or B-LCSEs.

NOTE - In the case of a bi-directional logical channel, this closes both forward and reverse channels. It is not possible to close one and not the other.

forwardLogicalChannelNumber indicates the logical channel number of the forward channel of the logical channel that is to be closed.

The source of the logical channel release is given in Table B.15.

TABLE B.16/H.245

Sources of logical channel release

| ASN.1 codepoint | Cause |
| --- | --- |
| user | The LCSE or B-LCSE user is the source of the release. |
| lcse | The LCSE or B-LCSE is the source of the release. This may occur as a result of a protocol error. |

reason indicates why the channel is being closed. reservationFailure indicates that a QOS reservation was not able to be placed for the channel and it is therefore being closed. reopen indicates that the endpoint should close the channel and then re-open a channel using the OpenLogicalChannel procedures. As an example, this may occur if a multipoint call is reduced to a point to point call due to endpoints dropping from a conference.

### B.3.6	Close Logical Channel Acknowledge

This is used to confirm the closing of a logical channel connection.

forwardLogicalChannelNumber indicates the logical channel number of the forward channel of the logical channel that is being closed.

### B.3.7	Request Channel Close

This is used to by the outgoing CLCSE to request the closing of a logical channel connection between two peer LCSEs.

forwardLogicalChannelNumber indicates the logical channel number of the forward channel of the logical channel that is requested to close.

qosCapability is used to indicate the QOS parameters which were in use on the channel.

reason indicates why the request to close the channel is occuring. reservationFailure indicates that a QOS reservation was not able to be placed for the channel and it is therefore being closed. reopen indicates that the endpoint should close the channel and then re-open a channel using the OpenLogicalChannel procedures. As an example, this may occur if a multipoint call is reduced to a point to point call due to endpoints droping from a conference.

### B.3.8	Request Channel Close Acknowledge

This is used by the incoming CLCSE to indicate that the logical channel connection will be closed.

forwardLogicalChannelNumber indicates the logical channel number of the forward channel of the logical channel that it has been requested to close.

### B.3.9	Request Channel Close Reject

This is used by the incoming CLCSE to indicate that the logical channel connection will not be closed.

forwardLogicalChannelNumber indicates the logical channel number of the forward channel of the logical channel that it has been requested to close.

The cause field indicates the reason for rejection of the request to close the logical channel. The only valid cause value is unspecified.

### B.3.10	Request Channel Close Release

This is sent by the outgoing CLCSE in the case of a timeout.

forwardLogicalChannelNumber indicates the logical channel number of the forward channel of the logical channel that it has requested to close.

## B.4	Multiplex Table signalling messages

This set of messages is for the secure transmission of H.223 multiplex table entries from the transmitter to the receiver.

### B.4.1	Multiplex Entry Send

This is used to send H.223 multiplex table entries from the transmitter to the receiver. It is sent from an outgoing MTSE and a peer incoming MTSE.

sequenceNumber is used to label instances of MultiplexEntrySend so that the corresponding response can be identified.

MultiplexEntryDescriptors is a set of 1 to 15 MultiplexEntryDescriptors.

MultiplexEntryDescriptor: describes a single multiplex table entry. It includes the MultiplexTableEntryNumber and a list of MultiplexElements. A missing element list indicates that the entry is deactivated.

MultiplexElement: is a recursive structure that describes a single element and a repeat count. If of type logicalChannelNumber, the element indicates a single slot from the given logical channel, and the repeat count indicates the length of the slot in octets. If of type subElementList, the element indicates a sequence of nested MultiplexElements, and the repeat count indicates the number of times to repeat the sequence. In either case, if the repeatCount field is untilClosingFlag, this means to repeat the element indefinitely until the closing flag of the MUX-PDU.

In each MultiplexEntryDescriptor, the repeatCount of the final MultiplexElement in the elementList shall be set to “untilClosingFlag”, and the repeatCount of all other MultiplexElements in the elementList shall be set to “finite". This ensures that all multiplex table entries define a multiplex sequence pattern of indefinite length, repeating until the closing flag of the MUX-PDU. A MultiplexEntryDescriptor with a missing elementList field shall indicate a deactivated entry.

Each MultiplexEntrySend request may contain up to 15 MultiplexEntryDescriptors, each describing a single multiplex table entry. Multiplex entries may be sent in any order.

### B.4.2	Multiplex Entry Send Acknowledge

This is used to confirm receipt of one or more multiplexEntryDescriptors from a MultiplexEntrySend from the peer MTSE.

The sequenceNumber shall be the same as the sequenceNumber in the MultiplexEntrySend for which this is the confirmation.

multiplexTableEntryNumber indicates which multiplex table entries are being confirmed.

### B.4.3	Multiplex Entry Send Reject

This is used to reject one or more multiplexEntryDescriptors from a MultiplexEntrySend from the peer MTSE.

The sequenceNumber shall be the same as the sequenceNumber in the MultiplexEntrySend for which this is the rejection.

MultiplexEntryRejectionDescriptions specifies which table entries are being rejected, and why. The causes of rejection are given in Table B.17.

TABLE B.18/H.245

Reasons for rejecting a MultiplexEntrySend

| ASN.1 codepoint | Cause |
| --- | --- |
| unspecified | No cause for rejection specified. |
| descriptorTooComplex | The MultiplexEntryDescriptor exceeded the capability of the receive terminal. |

### B.4.4	Multiplex Entry Send Release

This is sent by the outgoing MTSE in the case of a timeout.

multiplexTableEntryNumber indicates which multiplex table entries have timedout.

## B.5	Request Multiplex Table signalling messages

This set of messages is for the secure request of retransmission of one or more MultiplexEntryDescriptors from the transmitter to the receiver.

### B.5.1	Request Multiplex Entry

This is used to request the retransmission of one or more MultiplexEntryDescriptors.

entryNumbers is a list of the MultiplexTableEntryNumbers of the MultiplexEntryDescriptors for which retransmission is requested.

### B.5.2	Request Multiplex Entry Acknowledge

This is used by the incoming RMESE to indicate that the multiplex entry will be transmitted.

entryNumbers is a list of the MultiplexTableEntryNumbers of the MultiplexEntryDescriptors will be transmitted.

### B.5.3	Request Multiplex Entry Reject

This is used by the incoming RMESE to indicate that the multiplex entry will not be transmitted.

entryNumbers is a list of the MultiplexTableEntryNumbers of the MultiplexEntryDescriptors will not be transmitted. The values of MultiplexTableEntryNumber in entryNumbers should match the values of MultiplexTableEntryNumber in rejectionDescriptions otherwise errors may occur during operation.

RequestMultiplexEntryRejectionDescriptions specifies which table entries are being rejected, and why. The causes of rejection are given in Table B.19.

TABLE B.20/H.245

Reasons for rejecting a MultiplexEntrySend

| ASN.1 codepoint | Cause |
| --- | --- |
| unspecified | No cause for rejection specified. |

### B.5.4	Request Multiplex Entry Release

This is sent by the outgoing RMESE in the case of a timeout.

entryNumbers is a list of the MultiplexTableEntryNumbers of the MultiplexEntryDescriptors for which time-out has occurred.

## B.6	Request Mode messages

This set of messages is used by a receive terminal to request particular modes of transmission from the transmit terminal.

### B.6.1	Request Mode

This is used to request particular modes of transmission from the transmit terminal. It is a list, in order or preference (most preferable first), of modes that the terminal would like to receive. Each mode is described using a ModeDescription.

sequenceNumber is used to label instances of RequestMode so that the corresponding response can be identified.

ModeDescription: is a set of one or more ModeElements.

ModeElement: is used to describe a mode element, that is, one of the constituent parts of a complete mode description. It indicates the type of elementary stream that is requested and optionally how it is requested to be multiplexed.

type is used to indicate the type of elementary stream that is requested. It is a choice of VideoMode, AudioMode, DataMode, EncryptionMode, and H235Mode. H235Mode indicates that encrypted media is requested.

h223ModeParameters: is used to indicate parameters specific to using H.223 [115].

adaptationLayerType indicates which adaptation layer and options are requested for the requested type. The codepoints are as follows: nonStandard, al1Framed (AL1 framed mode), al1NotFramed (AL1 unframed mode), al2WithoutSequenceNumbers (AL2 with no sequence numbers present), al2WithSequenceNumbers (AL2 with sequence numbers present), and al3 (AL3, indicating the number of control field octets that will be present and the size of the send buffer, Bs, that will be used, the size being measured in octets), al1M (Al1M defined in Annex C with specified parameters), al2M (Al2M defined in Annex C with specified parameters), and al3M (Al3M defined in Annex C with specified parameters).

segmentableFlag, when equal to true indicates that segmentable multiplexing is requested, and when equal to false indicates that non-segmentable multiplexing is requested.

h2250ModeParameters contains information specific for use along with H.225.0 and H.323.

redundancyEncodingMode (if present) specifies which redundancyEncodingMethod shall be used and which secondaryEncoding shall be used as redundancy encoding. The primary encoding is specified by the type element contained in the ModeElement.

genericModeParameters indicates generic mode parameters.

#### B.6.1.1	Video Mode

This is a choice of VideoModes.

H261VideoMode: Indicates the requested picture resolution (either QCIF or CIF), bit rate, in units of 100 bit/s, and still picture transmission

H262VideoMode: Indicates the requested profile and level, and the optional fields, if present, indicate the requested values of the parameters given. The optional fields are integers with units defined in Table B.21.

H263VideoMode: Indicates the requested picture resolution (SQCIF, QCIF, CIF, 4CIF and 16CIF or some custom picture format) and bit rate, in units of 100 bit/s.

The booleans unrestrictedVector, arithmeticCoding, advancedPrediction, and pbFrames, when true, indicate that is it requested to use these optional modes that are defined in the Annexes of H.263.

The boolean errorCompensation, when true, indicates that the encoder is capable of processing videoNotDecodedMBs indications and compensating errors as illustrated in Appendix I of H.263. The encoder is not required to respond to videoNotDecoded indications. In a multipoint control unit (MCU), it may not be practical for the MCU to respond to all indications.

EnhancementOptions: indicates the requested scalability enhancement layer parameters.

H263Options: indicates the requested optional modes of H.263.

IS11172VideoMode: Indicates request for constrainedBitstream and the optional fields, if present, indicate the requested values of the parameters given. The optional fields are integers with units defined in Table B.22.

genericVideoMode indicates generic video mode parameters.

#### B.6.1.2	Audio Mode

This is a choice of AudioModes.

The exact meaning of the G-series audio codepoints is given in Table B.23. There are four options for G.723.1 audio, to allow either of the bit rates (the low bit rate of 5.3 kbit/s or the high bit rate of 6.3 kbit/s) to be requested with or without the use of silence suppression.

G7231AnnexCMode: is used to request audio coded according to Annex C of G.723.1. maxAl-sduAudioFrames indicates the requested maximum number of audio frames per AL-SDU. The boolean silenceSupression, when true, requests the use of silence compression defined in G.723.1 Annex A. The fields of g723AnnexCAudioMode, highRateMode0, highRateMode1, lowRateMode0, lowRateMode1, sidMode0, and sidMode1 indicates the requested number of octets per frame for each of the audio and error protection modes of G.723.1 and G.723.1 Annex C.

IS11172AudioMode: is used to request audio coded according to ISO/IEC 11172-3 [116].

audioLayer indicates which coding layer is requested: either audioLayer1, audioLayer2 or audioLayer3.

audioSampling indicates which sample rate is requested: audioSampling32k, audioSampling44k1 and audioSampling48k indicate the audio sample rates 32 kHz, 44.1 kHz and 48 kHz respectively.

multichannelType indicates which multi-channel mode is requested: singleChannel, twoChannelStereo and twoChannelDual request single channel, stereo and dual channel operation respectively.

bitRate indicates the requested audio bit rate, and is measured in units of kbit/s.

IS13818AudioMode: is used to request audio coded according to ISO/IEC 13818-3 [117].

audioLayer indicates which coding layer is requested: either audioLayer1, audioLayer2 or audioLayer3.

audioSampling indicates which sample rate is requested: audioSampling16k, audioSampling22k05, audioSampling24k, audioSampling32k, audioSampling44k1 and audioSampling48k indicate the audio sample rates 16 kHz, 22.05 kHz, 24 kHz, 32 kHz, 44.1 kHz and 48 kHz respectively.

multichannelType indicates which multi-channel mode is requested as specified in Table B.24.

TABLE B.25/H.245

ISO/IEC 13818-3 multi-channel codepoints

| ASN.1 Codepoint | Semantic meaning of codepoint |
| --- | --- |
| singleChannel | One channel, using the 1/0 configuration. Single channel mode (as in ISO/IEC 11172-3) |
| twoChannelStereo | Two channels, using the 2/0 configuration, stereo channel mode (as in ISO/IEC 11172-3) |
| twoChannelDual | Two channels, using the 2/0 configuration, dual channel mode (as in ISO/IEC 11172-3) |
| threeChannels2-1 | Three channels, using the 2/1 configuration. Left, Right and single surround channel |
| threeChannels3-0 | Three channels, using the 3/0 configuration. Left, Centre and Right, without surround channel |
| fourChannels2-0-2-0 | Four channels, using the 2/0 + 2/0 configuration. Left and Right of the first programme and Left and Right of the second programme |
| fourChannels2-2 | Four channels, using the 2/2 configuration. Left, Right, Left surround and Right surround |
| fourChannels3-1 | Four channels, using the 3/1 configuration. Left, Centre, Right, and a single surround channel |
| fiveChannels3-0-2-0 | Five channels, using the 3/0 + 2/0 configuration. Left, Centre and Right of the first programme and Left and Right of the second programme |
| fiveChannels3-2 | Five channels, using the 3/2 configuration. Left, Centre, Right, Left surround and Right surround |

The boolean lowFrequencyEnhancement, when true, requests a low frequency enhancement channel.

The boolean multilingual, when true, requests up to seven multilingual channels.

bitRate indicates the requested audio bit rate, and is measured in units of kbit/s.

genericAudioMode indicates generic audio mode parameters.

#### B.6.1.3	Data Mode

This is a choice of data applications and bit rates.

bitRate indicates the requested bit rate in units of 100 bit/s.

t120 requests the use of the T.120 [118] protocol.

dsm-cc requests the use of the DSM-CC [119] protocol.

userData requests the use of unspecified user data from external data ports.

t84 requests the use of T.84 [120] for the transfer of such images (JPEG, JBIG, Facsimile Gr.3/4).

t434 requests the use of T.434 [121] for the transfer of telematic binary files.

h224 requests the use of the real-time simplex device control protocol H.224 [122].

nlpid requests the use of the use of the specified network link layer data application.

dsvdControl requests the use of the DSVD terminal to support an out-of-band control channel.

h222DataPartitioning requests the use of the modified and restricted usage of data partitioning of H.262, as specified in H.222.1, in which the enhancement data is transmitted as a data channel supported by the listed DataProtocolCapability.

t30fax requests the use of T.30 Annex C analog mode (G3V), as specified in Recommendation T.39 for the DSVF/MSVF modes.

t140 requests the use of the T.140 text conversation protocol, as specified in Recommendation T.140.

t38fax requests the use of the Recommendation T.38 [123].

genericDataMode indicates generic data mode parameters. When maxBitRate is included in the genericDataMode, its value shall be the same as the value of maxBitRate in the DataMode.

#### B.6.1.4	Encryption Mode

This is a choice of encryption modes.

h233Encryption requests the use of encryption according to H.233 and H.234 [124][125].

### B.6.2	Request Mode Acknowledge

This is sent to confirm that the transmit terminal intends to transmit in one of the modes requested by the receive terminal.

The sequenceNumber shall be the same as the sequenceNumber in the RequestMode for which this is the confirmation.

The response field indicates the action from the remote terminal. The possible values of response are given in Table B.26.

TABLE B.27/H.245

Confirmation responses to Request Mode

| ASN.1 codepoint | Response |
| --- | --- |
| willTransmitMostPreferredMode | The transmit terminal will change to the receiver's most preferred mode. |
| willTransmitLessPreferredMode | The transmit terminal will change to one of the receiver's preferred mode, but not the most preferred mode. |

### B.6.3	Request Mode Reject

This is sent to reject the request by the receive terminal.

The sequenceNumber shall be the same as the sequenceNumber in the RequestMode for which this is the response.

The cause field indicates the reason for rejection of the requested mode. The cause values are given in Table B.28.

TABLE B.29/H.245

Rejection responses to Request Mode

| ASN.1 codepoint | Response |
| --- | --- |
| modeUnavailable | The transmit terminal will not change its mode of transmission as the requested modes are not available. |
| multipointConstraint | The transmit terminal will not change its mode of transmission due to a multipoint constraint. |
| requestDenied | The transmit terminal will not change its mode of transmission. |

### B.6.4	Request Mode Release

This is used by the outgoing MRSE in the case of a timeout.

## B.7	Round Trip Delay messages

This set of messages is used by a terminal to determine the round trip delay between two communicating terminals. It also enables a H.245 user to determine whether the peer H.245 protocol entity is alive.

### B.7.1	Round Trip Delay Request

This is sent from the outgoing RTDSE to the incoming RTDSE.

sequenceNumber is used to label instances of RoundTripDelayRequest so that the corresponding response can be identified.

### B.7.2	Round Trip Delay Response

This is sent from the incoming RTDSE to the outgoing RTDSE.

The sequenceNumber shall be the same as the sequenceNumber in the RoundTripDelayRequest for which this is the response.

## B.8	Maintenance Loop messages

This set of messages is used by a terminal to perform maintenance loop functions.

### B.8.1	Maintenance Loop Request

This is sent to request a particular type of loop back. The types mediaLoop and logicalChannelLoop request the loopback of only one logical channel as indicated by LogicalChannelNumber, while the type systemLoop refers to all logical channels. The exact definition of these types is system specific and outside the scope of this Recommendation.

### B.8.2	Maintenance Loop Acknowledge

This is used confirm that the terminal will perform the loop as requested.

### B.8.3	Maintenance Loop Reject

This is used indicate that the terminal will not perform the loop as requested.

A terminal may use the cause canNotPerformLoop to indicate that it does not have the capability to perform the requested loop.

### B.8.4	Maintenance Loop Command Off

On receipt of this command, the terminal shall disconnect all loops and restore audio, video and data paths to their normal condition.

## B.9	Communication Mode Messages

This set of messages are used by an H.323 MC to convey the communication mode of an H.323 conference.

### B.9.1	Communication Mode Command

The CommunicationModeCommand is sent by a H.323 MC to specify the communication mode for each media type: unicast or multicast. This command may cause a switch between a centralized and decentralized conference and therefore may involve closing all existing logical channels and opening new ones.

The CommunicationModeCommand specifies all the sessions in the conference. For each session, the following data is specified: the RTP session identifier, the associated RTP session ID if applicable, a terminal label if applicable, a description of the session, the datatype of the sessions (e.g. G.711), and a unicast or multicast address for the media and media control channels as appropriate for the conference configuration and type. In case a redundancy encoding shall be used, the communicationModeTableEntry also specifies the redundancyEncodingMethod as well as the secondary encoding format.

The CommunicationModeCommand conveys the transmit modes which conference endpoints are to use in a conference. The command does not convey receive modes, as they are specified by OpenLogicalChannel commands which are sent from the MC to the endpoints.

It is presumed that the CommunicationModeCommand is defining the modes of a conference and is therefore sent after the multipointConference indication which notifies an endpoint that it must comply with the commands of the MC. Endpoints should wait for a CommunicationModeCommand before opening logical channels when they have received a multipointConference indication.

Endpoints receiving a CommunicationModeCommand use the terminalLabel field of each table entry to determine if the entry is applicable for its own processing. Entries which do not contain a terminalLabel apply to all endpoints in the conference. Entries which contain terminalLables are commands to specific endpoints and which match the terminalLabel in the entry. For example, when audio streams from all endpoints are placed on one multicast address (one session), the table entry for the audio mode, media address, and media control address will not contain a terminalLabel. When the table entry commands an endpoint to send its video to a multicast address, the MC will include that endpoint's terminalLabel.

SessionDependency is set by the MC to indicate when a session is dependent on another session for meaningful decoding of the data.

The destination field in the CommunicationModeTableEntry indicates the endpoint to which the transmitting endpoint should open a logical channel. If the destination field exists in the CommunicationModeTableEntry, the endpoint shall use the destination as the destination field in H2250LogicalChannelParameters of the OpenLogicalChannel message.

The CommunicationModeCommand can be used to instruct endpoints in a conference (or a point-to-point call) to change modes (by indicating a new mode with the mediaChannel already used) or to transmit to a new address (by indicating the mode currently in use, but with new mediaChannel). Similarly, an endpoint that receives a CommunicationModeCommand indicating the mode currently in use and no mediaChannel should close the appropriate channel and the attempt to reopen using the OpenLogicalChannel procedures, where the OpenLogicalChannelAck contains the address to which the endpoint will send the medium.

### B.9.2	Communication Mode Request

This is sent to the MC to request the communication mode of the current conference.

### B.9.3	Communication Mode Response

This is sent by the MC, in response to a CommunicationModeRequest to specify the communication mode of a conference.

## B.10	Conference Request and Response Messages

TerminalID, which is used in the Conference Request and Response Messages, has a length of 128 octets. When communicating between an H.323 terminal and an H.320 terminal via a H.323 Gateway, this field will be truncated to 32 octets.

### B.10.1	Terminal List Request

This request equates to H.230 TCU as described in H.243.

### B.10.2	Terminal List Response

This request equates to a sequence of terminalNumbers as described in H.230.

### B.10.3	Make Me Chair

This request equates to CCA as described in H.230.

### B.10.4	Cancel Make Me Chair

This request equates to CIS as described in H.230.

### B.10.5	Make Me Chair Response

This request equates to either H.230 CIT if the chair control token is granted or H.230 CCR if the chair control token is denied

### B.10.6	Drop Terminal

This request equates to CCD as described in H.230.

### B.10.7	Terminal Drop Reject

This response equates to CIR as described in H.230

### B.10.8	RequestTerminal ID

This request equates to TCP as described in H.230.

### B.10.9	MC Terminal ID Response

This response equates to TIP as described in H.230.

### B.10.10	Enter H.243 Password Request

This request equates to TCS1 as described in H.230.

### B.10.11	Password Response

This response equates to IIS as described in H.230.

### B.10.12	Enter H.243 Terminal ID Request

This request equates to TCS2/TCI as described in H.230.

### B.10.13	Terminal ID Response

This response equates to IIS as described in H.230.

### B.10.14	Enter H.243 Conference ID Request

This request equates to TCS3 as described in H.230.

### B.10.15	Conference ID Response

This response equates to IIS as described in H.230.

### B.10.16	Video Command Reject

This request equates to H.230 VCR.

### B.10.17	Enter Extension Address Request

This request equates to TCS4 as described in H.230.

### B.10.18	Extension Address Response

This response equates to IIS as described in H.230.

### B.10.19	Request Chair Control Token Owner

This request equates to TCA as described in H.230 for the Chair Control Token

### B.10.20	Chair Token Owner Response

This response equates to TIR as described in H.230 for the Chair Control Token

### B.10.21	Request Terminal Certificate

This request is issued by any endpoint in a conference to its MC. It allows an endpoint to obtain the digital certificate for the user at a particular terminal. The requesting terminal may optionally include its own terminalCertificate and a challangeString which is encrypted with the private key.

The CertSelectionCriteria defines a set certificates that are acceptable to the requester. The responder (the MC) should attempt to satisfy these criteria. CertSelectionCriteria may be present along with terminalLabel. In this case, the MC may use the criteria either to select an appropriate certificate from those presented by the specified terminal, or may request from the specified terminal for a certificate matching the criteria, which it then returns to the original, requesting terminal.

This response may return the digital certificate and optionally a signature associated with the certificate as per the following:

·	If the source of the terminalCertificateResponse does not have a suitable certificate, this message may be returned with no certificate (and therefore, no certificateResponse structure).

·	If an endpoint is requesting the certificate of another endpoint in a multipoint conference (indicated by the terminalLabel), the responding MC shall return a certificate associated with the requested endpoint (contained within the certificateResponse structure)

·	The certificateResponse structure should be present. In the event that the MC is presenting the certificate to the requester on behalf of another endpoint, there shall be a cryptographic link between the signatures and that of the MC. This shall be provided in one of two ways.

·	The private key used to protect session key material distributed in the most recent exchange shall be used.

·	If there has been no key material exchanged, or that key is not suitable for signing, the certificate that was used during the most recent endpoint-MC authentication shall be the source of the private key.

### B.10.22	Terminal Certificate Response

This response returns the digital certificate and a responseString which is encrypted with the private key for a specific terminal.

### B.10.23	Broadcast My Logical Channel

This request is similar to H.230 MCV but only refers to a single logical channel and has a response message broadcastMyLogicalChannelResponse which acknowledges the request.

### B.10.24	Broadcast My Logical Channel Response

Provides a granted or denied response to the BroadcastMyLogicalChannel Request.

### B.10.25	Make Terminal Broadcaster

This request is similar to H.230 VCB and has a response message makeTerminalBroadcasterResponse which acknowledges the request.

### B.10.26	Make Terminal Broadcaster Response

Provides a granted or denied response to the MakeTerminalBroadcaster Request.

### B.10.27	Send This Source

This request is similar to H.230 VCS and has a response message SendThisSourceResponse which acknowledges the request.

### B.10.28	SendThis Source Response

Provides a granted or denied response to SendThisSource Request.

### B.10.29	Request All Terminals Ids

Sent by an endpoint to the MC of a conference to obtain all of the terminal labels and terminal IDs of the conference participants.

### B.10.30	Request All Terminal Ids Response

Response to RequestAllTerminalIDs containing a list of all endpoints in conference by terminalLabel and terminalID.

### B.10.31	RemoteMC Request

This request is sent by an active MC to another MC to activate/de-activate it. A RemoteMC Request with a choice of masterActivate or slaveActivate may be sent by an active MC to an inactive MC to activate it as Master or Slave, respectively, of a cascaded connection. A RemoteMC Request with a choice of deActivate may be sent by a Master MC to an already active Slave MC to de-activate it.

### B.10.32	RemoteMC Response

The RemoteMC Response is sent to indicate acceptance or rejection of the RemoteMC Request. Acceptance of this request is determined by the following criteria:

Choice = activateSlave

The receiver is inactive and the sender of the request initiated this call with a conferenceGoal of INVITE in the H.225 Setup message or the receiver of the request initiated this call with a conferenceGoal of JOIN in the H.225 Setup message.

Choice = activateMaster

The receiver is inactive and the sender of the request initiated this call with a conferenceGoal of CREATE in the H.225 Setup message.

Choice = deActivate

The receiver is an active MC.

If the conditions stated above are not met, the request should be rejected with a choice of invalidConfiguration.

A reject choice of functionNotSupported is used by endpoints not supporting cascading.

## B.11	Multilink Messages

multilinkRequest, multilinkResponse, and multilinkIndication messages are used to support the use of channel aggregation according to Reccommendation H.MULTILINK, as specified in Annex F/H.324. These messages provide for the addition and removal of physical connections, automatic exchange of network addresses (telephone numbers), and control of H.MULTILINK operation.

### B.11.1	callInformation Request and Response

MultilinkRequest.callInformation is used by the initiator, as defined in Annex F/H.324, to request the information needed to establish and associate additional physical connections. The maximum number of additional connections that the sender is capable of establishing is sent in the maxNumberOfAdditionalConnections parameter.

The MultilinkResponse.callInformation message includes the DialingInformation parameter, with contents as described below, as well as a callAssociationNumber. The callAssociationNumber shall contain a 32-bit uniformly distributed random number. Any subsequent callInformation exchanges within the same session shall use the identical callAssociationNumber.

### B.11.2	addConnection Request and Response

The MultilinkRequest.addConnection message may be used by the responder, as defined in Annex F/H.324, to ask the initiator to add physical connections. The DialingInformation structure shall indicate the connections to be added. The sequenceNumber parameter shall be incremented by 1, modulo 256, for each new MultilinkRequest.addConnection message sent.

On receiving this message, the initiator shall respond with an MultilinkResponse.addConnection message indicating that it either intends to add the connections as requested, or that it does not intend to do so, along with the appropriate reason code. The sequenceNumber parameter shall be equal to the sequenceNumber parameter of the corresponding MultilinkRequest.addConnection message.

### B.11.3	removeConnection Request and Response

The MultilinkRequest.removeConnection message may be used by either the initiator or responder, as defined in Annex F/H.324, to request that the far-end remove a channel from the H.MULTILINK channel set. This is used as part of the procedure in Annex F/H.324 for removing physical connections. The connectionIdentifier parameter shall identify the channel to be removed, using the channel numbering received via H.MULTILINK from the terminal receiving the MultilinkRequest.removeConnection message.

The MultilinkRepsonse.removeConnection message shall be sent in response, after the channel has been removed from the H.MULTILINK channel set, indicating that this channel is no longer (or was never) in use. The connectionIdentifier parameter shall be identical to the value in the corresponding MultilinkRequest.removeConnection message.

### B.11.4	maximumHeaderInterval Request and Response

The MultilinkRequest.maximumHeaderInterval message may be used to request the actual H.MULTILINK Maximum Header Interval being used by the remote transmitter without altering it (the currentIntervalInformation choice), or to request a particular value to be used instead (the requestedInterval choice, with units in milliseconds).

The MultilinkResponse.maximumHeaderInterval message shall be sent in response. If the corresponding request was a request for information about the current minimum rate, the terminal shall provide the value that its transmitter is currently using as the Maximum Header Interval in the response. If the corresponding request specified a particular minimum rate to use, the terminal should attempt to comply with this request by modifying the Maximum Header Interval used by its transmitter. Whether or not it makes a change to the Maximum Header Interval, the response shall indicate the new value that is in use (which may be different from the requested value).

### B.11.5	Multilink Indications

The MultilinkIndication.crcDesired message may be sent by a terminal to indicate its desire that the remote terminal send the optional H.MULTILINK data CRC in all subsequent data sets. The receiving terminal may optionally comply: no explicit acknowledgment or response is required.

The MultilinkIndication.excessiveError message may be sent to indicate to the remote terminal that excessive errors are being received on a particular connection. The means for the terminal to determine the error rate or the criterion for determining what is excessive is defined locally at that terminal. The connection is indicated using the connectionIdentifier parameter. On receipt of this message, a terminal may choose to take corrective action. The particular corrective action that it should take is not specified.

### B.11.6	DialingInformation

The DialingInformation type is used to provide explicit dialing information (telephone numbers) to allow the automatic establishment of physical connections. The differential choice provides a list of DialingInformationNumber parameters, one for each potential additional connection. The length of this list indicates the maximum number of additional connections available. If such information is not available, the infoNotAvailable choice is used, indicating only the number of additional connections that are available.

### B.11.7	DialingInformationNumber

The DialingInformationNumber type includes up to three sub-parameters that indicate the dialing information for a physical connection differentially relative to the corresponding information for an already established initial connection.

The networkAddress parameter shall include the least significant (right-most) portion of the telephone number for the connection, up to and including the most significant digit that is different from the number for an initially established connection, and shall include no digits that are more significant than this. If the number for the connection is identical to that of the initial connection, the networkAddress parameter shall consist of a zero-length string (since there are no differing digits in the telephone number).

Note - The differential digit method is used instead of the full E.164 digit string because the first few digits of the number to be dialed can vary based on the geographic location of the two terminals; for example whether or not they are located in the same city.

If there is a sub-address used for dialing, and the sub-address of a given connection is different from that of the initial connection, the responder shall include the sub-address, in full, in the optional subAddress parameter.

The network types supported for the connection (GSTN, ISDN, or both) shall be indicated using the networkType parameter.

### B.11.8	DialingInformationNetworkType

The DialingInformationNetworkType type indicates a circuit-switched network type, n-isdn (N-ISDN) or gstn (GSTN).

### B.11.9	ConnectionIdentifier

The ConnectionIdentifier type is used to identify uniquely a single physical connection in H.MULTILINK, using a combination of channelTag and sequenceNumber from an H.MULTILINK Header. If a Channel Tag was not specified at all in the Header, a value of zero shall be used for the channelTag parameter.

## B.12	Logical Channel Bit rate Change Messages

LogicalChannelRateRequest, LogicalChannelRateAcknowledge, LogicalChannelRateReject and LogicalChannelRateRelease are used to change the bit-rate of a logical channel. The procedure for using these messages is that a terminal can request a target bit-rate for a specific logical channel, and the remote terminal can acknowledge or reject the request.

These messages provide an enhanced level of interaction, by allowing for a target bit-rate request as opposed to maximum bit-rate enforcement dictated by FlowControlCommand, and by providing feedback on whether the request is fulfilled or denied.

### B.12.1	Logical Channel Rate Request

This is used by a terminal to request a change in the bit rate of the given logical channel being transmitted to it.

sequenceNumber is used to label instances of LogicalChannelRateRequest so that the corresponding response can be identified.

logicalChannelNumber indicates the logical channel that the bit rate change request applies to.

maximumBitRate indicates, in units of 100 bit/s, the requested maximum bit rate for the logical channel.

### B.12.2	Logical Channel Rate Acknowledge

This message is sent to acknowledge a request for a change in bit rate of a logical channel.

The sequenceNumber shall be the same as the sequenceNumber in the LogicalChannelRateRequest for which this is the response.

logicalChannelNumber indicates the logical channel that the bit rate change request applies to.

maximumBitRate indicates, in units of 100 bit/s, the maximum bit rate for the logical channel that the terminal is acknowledging.

### B.12.3	Logical Channel Rate Reject

This message is sent to reject a request for a change in bit rate of a logical channel.

The sequenceNumber shall be the same as the sequenceNumber in the LogicalChannelRateRequest for which this is the response.

logicalChannelNumber indicates the logical channel that the bit rate change request applies to.

rejectReason indicates the reason why the request has been denied. Current defined reasons are undefined reason and insufficient resources.

currentMaximumBitRate indicates, in units of 100 bit/s, the maximum bit rate at which the terminal is going to transmit on the the logical channel.

### B.12.4	Logical Channel Rate Release

This is sent in the case of a timeout.

## B.13	Commands

A command message requires action but no explicit response.

### B.13.1	Send Terminal Capability Set

specificRequest commands the far end terminal to indicate its transmit and receive capabilities by sending one or more TerminalCapabilitySets that contain the information requested, as specified below. This command may be sent at any time to elicit the capabilities of the remote terminal, for example, following an interruption or other cause for uncertainty; however, such messages should not be sent repetitively without strong cause.

A terminal shall only request the transmission of capabilityTableEntryNumbers and capabilityDescriptorNumbers that it has previously received. A terminal shall ignore any requests to transmit capabilityTableEntryNumbers and capabilityDescriptorNumbers that it has not previously transmitted and no fault shall be considered to have occurred.

The boolean multiplexCapability, when true, requests the transmission of the MultiplexCapability.

capabilityTableEntryNumbers is a set of the CapabilityTableEntryNumbers that indicate the CapabilityTableEntrys that the terminal requests to be transmitted.

capabilityDescriptorNumbers is a set of the capabilityDescriptorNumbers that indicate the CapabilityDescriptors that the terminal requests to be transmitted.

genericRequest commands the far end terminal to send its entire terminal capability set.

### B.13.2	Encryption

This command is used to exchange encryption capabilities and to command the transmission of an initialization vector (IV), refer to H.233 and H.234 [126][127].

encryptionSE is an H.233 Session Exchange (SE) message, except that the error protection bits described in H.233 shall not be applied.

encryptionIVRequest commands the far-end encryptor to transmit a new IV in a logical channel opened for encryptionData.

encryptionAlgorithmID indicates to the receiver that the sending terminal will associate the given h233AlgorithmIdentifier value with the non-standard encryption algorithm associatedAlgorithm.

### B.13.3	Flow Control

This command is used to specify the upper limit of bit rate of either a single logical channel or the whole multiplex. A terminal may send this command to restrict the bit rate that the far-end terminal sends. A terminal that receives this command shall comply with it.

When scope is of type logicalChannelNumber the limit applies to the given logical channel, when scope is of type resourceID the limit applies to the given ATM virtual channel, and when scope is of type wholeMultiplex the limit applies to the whole multiplex.

maximumBitRate is measured in units of 100 bit/s averaged over non-overlapping consecutive periods of one second. When this is present, the specified limit supersedes any previous limit, whether higher or lower. When it is not present any previous restriction on the bit rate for the channel is no longer applicable.

The point at which the bit rate limit is applied, and the specification of which bits are included in the calculation of bit rate is not specified in this Recommendation, but should be specified by recommendations that use this Recommendation.

Each transmission of this command affects a specific logical channel or the entire multiplex. More than one such command may be in effect at the same time, up to the number of open logical channels plus one, for the overall multiplex limitation.

NOTE - When the bit rate that can be transmitted on a logical channel is constrained to particular values, for example G.723.1 audio, and the request is to transmit at a rate lower than the lowest rate at which it would normally operate, it shall respond by stopping transmission on the logical channel.

### B.13.4	End session

This command indicates the end of the H.245 session. After transmitting EndSessionCommand, the terminal shall not send any more of the messages defined in this Recommendation.

disconnect indicates that the connection will be dropped.

gstnOptions: is a choice of alternatives that will occur after ending the H.245 session when a V-series modem is used over the GSTN.

The possible options are given in Table B.30.

TABLE B.31/H.245

Options after EndSessionCommand when using an V-series modem over the GSTN

| ASN.1 codepoint | Option |
| --- | --- |
| telephonyMode | The terminal shall initiate the cleardown procedures defined in the V-series modem Recommendation, except that it shall not physically disconnect the GSTN connection. |
| v8bis | The terminal shall initiate the cleardown procedures defined in the V-series modem Recommendation and enter a V.8bis session. |
| v34DSVD | The terminal shall preserve the V.34 modem connection, but use it to support V.DSVD. |
| v34DuplexFAX | The terminal shall preserve the V.34 modem connection, but use it to support T.30 FAX [128]. |
| v34H324 | The terminal shall preserve the V.34 modem connection, but use it to support H.324 [129]. |

isdnOptions: is a choice of alternatives that will occur after ending the H.245 session when a digital communications terminal is used over a digital network.

The possible options are given in Table B.32

TABLE B.33/H.245

Options after EndSessionCommand when using a digital communications terminal over a digital network

| ASN.1 codepoint | Option |
| --- | --- |
| telephonyMode | The terminal shall initiate the cleardown procedures defined in the recommendation governing communication on the particular digital channel to which the terminal is connected, except that it shall not physically disconnect the digital connection. |
| v140 | The terminal shall initiate the cleardown procedures defined in the recommendation governing communication on the particular digital channel to which the terminal is connected and enter a V.140 session [130]. |
| terminalOnHold | The terminal shall initiate the ‘terminal on hold’ procedures defined in the recommendation governing communication on the particular digital channel to which the terminal is connected. |

### B.13.5	Miscellaneous Command

This is used for a variety of commands, some of which are present in H.221 and H.230 [131][132].

logicalChannelNumber indicates the logical channel number to which the command applies. It shall indicate a logical channel opened for video data when the type is one of videoFreezePicture, videoFastUpdatePicture, videoFastUpdateGOB, videoTemporalSpatialTradeOff, videoSendSyncEveryGOB, videoFastUpdateMB, and videoSendSyncEveryGOBCancel. When the type is one of equaliseDelay, zeroDelay, multipointModeCommand or cancelMultipointModeCommand where multiple logical channels are involved, the logicalChannelNumber shall be arbitrary, but shall be a valid LogicalChannelNumber (i.e. in the range 1-65535) and the receiver shall ignore the value.

equaliseDelay and zeroDelay shall have the same meaning as the commands ACE and ACZ defined in H.230[133].

multipointModeCommand commands that a terminal in receipt shall comply with all requestMode requests issued by the MCU. An example of a mode change is an audio coding change from G.711 to G.728.

cancelMultipointModeCommand cancels a previously sent multipointModeCommand command.

videoFreezePicture commands the video decoder to complete updating the current video frame and subsequently display the frozen picture until receipt of the appropriate freeze-picture release control signal.

videoFastUpdatePicture commands the video encoder to enter the fast-update mode at its earliest opportunity.

videoFastUpdateGOB commands the far-end video encoder to perform a fast update of one or more GOBs. firstGOB indicates the number of the first GOB to be updated, and numberOfGOBs indicates the number of GOBs to be updated. It shall only be used with video compression algorithms that define GOBs, for example, H.261 and H.263.

videoTemporalSpatialTradeOff commands the far-end video encoder to change its trade-off between temporal and spatial resolution. A value of 0 commands a high spatial resolution and a value of 31 commands a high frame rate. The values from 0 to 31 indicate monotonically a desire for higher frame rate. Actual values do not correspond to precise values of spatial resolution or frame rate.

videoSendSyncEveryGOB commands the far-end video encoder to use sync for every GOB as defined in H.263 [134], until the command videoSendSyncEveryGOBCancel is received, from which time the far-end video encoder may decide the frequency of GOB syncs. These commands shall only be used with video encoded according to H.263.

videoFastUpdateMB commands the far-end video encoder to perform a fast update of one or more MBs. firstGOB indicates the number of the first GOB to be updated and is only relative to H.263, firstMB indicates the number of the first MB to be updated and is only relative to H.261 and numberOfMBs indicates the number of MBs to be updated. It shall only be used with video compression algorithms that define MBs, for example, H.261 and H.263. Terminals may respond to this command with a GOB update which includes the MBs requested.

maxH223MUXPDUsize commands the transmitter to restrict the size of the H223 MUX-PDUs that it transmits to a maximum of the specified number of octets.

encryptionUpdate and EncryptionUpdateRequest are used to initiate and distribute new keying material to be used in the encryption of the indicated media channels.

Switch receive media on and off can be used by an MC to command an endpoint to switch between a unicast and multicast channel when the MC+MP is mixing audio. In this case, when the MC stream includes the terminal audio, the MC+MP can switch the endpoint to a unicast stream which would contain a special mix for the terminal with its audio removed.

switchReceiveMediaOff is used by an MC to indicate to an endpoint that a particular logical channel should not be used for receive media.

switchReceiveMediaOn is used by an MC to indicate to an endpoint that a particular logical channel should be used for receive media.

doOneProgression commands the video encoder to begin producing a progressive refinement sequence. In this mode, the encoder produces video data consisting of one picture followed by a sequence of zero or more frames of refinement of the quality of the same picture. The encoder stays in this mode until either the encoder decides an acceptable fidelity level has been reached or the progressiveRefinementAbortOne command is received. In addition, the encoder shall insert the Progressive Refinement Segment Start Tag and the Progressive Refinement Segment End Tag to mark the beginning and end of the progressive refinement as defined in the Supplemental Enhancement Information Specification (Annex L) of Recommendation H.263.

doContinuousProgressions commands the video encoder to begin producing progressive refinement sequences. In this mode, the encoder produces video data consisting of one picture followed by a sequence of zero or more frames of refinement of the quality of the same picture. When the encoder decides an acceptable fidelity level has been reached or the progressiveRefinementAbortOne command is received, the encoder stops refining the current progression and begins another progressive refinement for a different picture. The sequence of progressive refinements continues until the progressiveRefinementAbortContinuous command is received. In addition, the encoder shall insert Progressive Refinement Segment Start Tags and Progressive Refinement Segment End Tags to mark the start and end of each progressive refinement as defined in the Supplemental Enhancement Information Specification (Annex L) of Recommendation H.263.

doOneIndependentProgression commands the video encoder to begin an independent progressive refinement sequence. In this mode, the encoder produces video data consisting of one Intra picture followed by a sequence of zero or more frames of refinement of the quality of the same picture. The encoder stays in this mode until either the encoder decides an acceptable fidelity level has been reached or the progressiveRefinementAbortOne command is received. In addition, the encoder shall insert the Progressive Refinement Segment Start Tag and the Progressive Refinement Segment End Tag to mark the beginning and end of the progressive refinement as defined in the Supplemental Enhancement Information Specification (Annex L) of Recommendation H.263.

doContinuousIndependentProgressions commands the video encoder to begin producing independent progressive refinement sequences. In this mode, the encoder produces video data consisting of one Intra picture followed by a sequence of zero or more frames of refinement of the quality of the same picture. When the encoder decides an acceptable fidelity level has been reached or the progressiveRefinementAbortOne command is received, the encoder stops refining the current progression and begins another independent progressive refinement for a different picture. The sequence of independent progressive refinements continues until the progressiveRefinementAbortContinuous command is received. In addition, the terminal shall insert Progressive Refinement Segment Start Tags and Progressive Refinement Segment End Tags to mark the start and end of each independent progressive refinement as defined in the Supplemental Enhancement Information Specification (Annex L) of Recommendation H.263.

progressiveRefinementAbortOne commands the video encoder to terminate doOneProgression, doOneIndependentProgression, or the current progressive refinement in the sequence of progressive refinements in either doContinuousProgressions or doContinuousIndependentProgressions.

progressiveRefinementAbortContinuous commands the video encoder to terminate either doContinuousProgressions or doContinuousIndependentProgressions.

### B.13.6	Conference Command

BroadcastMyLogicalChannel shall be similar to H.230 MCV but shall only refer to a single logical channel.

CancelBroadcastMyLogicalChannel shall be similar to as H.230 Cancel-MCV but shall only refer to a single logical channel.

MakeTerminalBroadcaster shall be defined as H.230 VCB.

CancelMakeTerminalBroadcaster shall be defined as Cancel-VCB.

SendThisSource shall be defined as H.230 VCS.

CancelSendThisSource shall be defined as H.230 Cancel-VCS.

DropConference shall be defined as H.230 CCK.

Substitute CID Command allows an active MC to change the Conference Identifier (CID), effectively moving the recipient of this command into another conference. The recipient of this command shall use the newly assigned CID in all future call signalling messages.

### B.13.7	H.223 Multiplex Reconfiguration

h223ModeChange commands the transmitter to change the level of multiplex mode as described in Annex C/H.324, to level 0, level 1, level 2, or level 2 with AnnexB/H.223 optional header.

h223AnnexADoubleFlag commands the transmitter to start or stop the use of double-flag mode of Annex A/H.223.

### B.13.8	New ATM Virtual Channel Command

This is used to command the remote terminal to open an ATM virtual channel with the given parameters.

resourceID is used to identify the ATM virtual channel. The means by which this parameter is associated with an ATM virtual channel is not specified in this Recommendation.

bitRate indicates the bit rate, measured at the AAL-SAP, of the virtual channel, and is measured in units of 64 kbit/s.

bitRateLockedToPCRClock indicates that the bit rate of the virtual channel is clocked to the clock used to produce H.222.0 clock reference values (Program clock reference or System clock reference).

bitRateLockedToNetworkClock indicates that the bit rate of the virtual channel is clocked to the local network clock. This does not guarantee that the bit rate clock will be locked to the local network at the receiver, as common network clocks may not be available.

aal indicates which ATM Adaptation Layer will be used, and its parameters.

The sequence aal1 indicates which of the options for ATM adaptation layer 1, as specified in I.363 [135], are supported. The codepoints are defined in Table B.34.

The sequence aal5 indicates which of the options for ATM adaptation layer 5, as specified in I.363 [136], are supported. forwardMaximumSDUSize and backwardMaximumSDUSize indicate the maximum CPCS-SDU size in the forward and reverse directions, measured in octets.

multiplex indicates the type of multiplex that will be used on the ATM virtual channel. The options are noMultiplex (No H.222.0 multiplex), H.222.0 Transport Stream and H.222.0 Program Stream.

## B.14	Indications

An indication contains information that does not require action or response.

### B.14.1	Function Not Understood

This is used to return requests, responses and commands that are not understood to the transmitter.

If a terminal receives a request, response or command that it does not understand, either because it is non-standard or has been defined in a subsequent revision of this Recommendation, it should respond by sending FunctionNotSupported or FunctionNotUnderstood.

NOTE - FunctionNotUnderstood was named FunctionNotSupported in version 1 of H.245. The name of this function was changed to allow for the addition of a more powerful FunctionNotSupported command without breaking backward compatibility with version 1 syntax.

### B.14.2	Miscellaneous Indication

This is used for a variety of indications, some of which are present in H.221 and H.230 [137][138].

logicalChannelNumber indicates the logical channel number to which the indication applies. It shall indicate a logical channel opened for video data when the type is videoIndicateReadyToActivate, and videoTemporalSpatialTradeOff. When the type is one of multipointConference, cancelMultipointConference, multipointZeroComm, cancelMultipointZeroComm, multipointSecondaryStatus, or cancelMultipointSecondaryStatus where multiple logical channels are involved, the logicalChannelNumber shall be arbitrary, but shall be a valid LogicalChannelNumber (i.e. in the range 1-65535) and the receiver shall ignore the value.

logicalChannelInactive is used to indicate that the content of the logical channel does not represent a normal signal. It is analogous to AIM and VIS defined in H.230.

logicalChannelActive is complementary to logicalChannelInactive. It is analogous to AIA and VIA defined in H.230. MultipointZeroComm, cancelMultipointZeroComm, multipointSecondaryStatus, and cancelMultipointSecondaryStatus shall have the same meaning as MIZ, cancelMIZ, MIS and cancelMIS respectively, as defined in H.230.

multipointConference indicates that the terminal is joined to an H.243 multipoint conference, and the terminal is expected to obey bit rate symmeterization. However, bit rate symmeterization will be enforced via FlowControlCommand messages. Note that multipointConference has exactly the same meaning has MCC in H.230. Note that multipointConference, like MCC, does not require mode symmetry.

videoIndicateReadyToActivate shall have the same meaning as VIR defined in H.230, that is, it is transmitted by a terminal whose user has decided not to send video unless he will also receive video from the other end.

videoTemporalSpatialTradeOff indicates to the far-end video decoder its current trade-off between temporal and spatial resolution. A value of 0 indicates a high spatial resolution and a value of 31 indicates a high frame rate. The values from 0 to 31 indicate monotonically a higher frame rate. Actual values do not correspond to precise values of spatial resolution or frame rate. A terminal that has indicated temporalSpatialTradeOffCapability shall transmit this indication whenever it changes its trade-off and when a video logical channel is initially opened.

videoNotDecodedMBs indicates to the far-end video encoder that a set of MBs has been received erroneously and that any MB in the specified set has been treated as not coded. The encoder may use this information to compensate transmission errors, as illustrated in Appendix I of H.263. firstMB indicates the number of the first MB treated as not coded, and numberOfMBs indicates the number of MBs treated as not coded. The MB numbering is done according to H.263. The temporal reference of the picture containing not decoded MBs is indicated in temporalReference. This indication shall only be used with the H.263 video compression algorithm.

### B.14.3	Jitter Indication

This is used to indicate the amount of jitter, as estimated by the receive terminal, of a logical channel. It may be useful for choice of bit-rate and buffer control in video channels, or to determine an appropriate rate of transmission of timing information, etc. The video encoder will then have the option of using this information to restrict the video bit-rate or the video decoder buffer fluctuations to help prevent decoder buffer underflow or overflow, given the occurring jitter. If the encoder takes this option, it will enable correct operation for existing designs of video decoder buffers, regardless of the amplitude of received jitter, as well as allow correct operation with minimum delay.

When scope is of type logicalChannelNumber the information applies to the given logical channel, when scope is of type resourceID the information applies to the given ATM virtual channel, and when scope is of type wholeMultiplex the information applies to the whole multiplex.

estimatedReceivedJitterMantissa and estimatedReceivedJitterExponent provide an estimate of the jitter that has been received by the terminal that has sent the message.

estimatedReceivedJitterMantissa indicates the mantissa of the jitter estimate as given in Table B.35.

TABLE B.36/H.245

Mantissa of estimatedReceivedJitterMantissa in JitterIndication

| estimatedReceivedJitterMantissa | Mantissa |
| --- | --- |
| 0 | 1 |
| 1 | 2.5 |
| 2 | 5 |
| 3 | 7.5 |

estimatedReceivedJitterExponent indicates the exponent of the jitter estimate as given in Table B.37.

TABLE B.38/H.245

Exponent of estimatedReceivedJitterExponent in JitterIndication

| estimatedReceivedJitterExponent | Exponent |
| --- | --- |
| 0 | Out of Range |
| 1 | 1 ms |
| 2 | 10 ms |
| 3 | 100 ms |
| 4 | 1 ms |
| 5 | 10 ms |
| 6 | 100 ms |
| 7 | 1 s |

The jitter estimate is obtained by multiplying the mantissa by the exponent, unless estimatedReceivedJitterExponent is equal to zero, in which case the estimate is just known to be more than 7.5 seconds.

skippedFrameCount indicates how many frames have been skipped by the decoder since the last JitterIndication message was received. Since the maximum value that can be encoded is 15, if this option is implemented, this information must be transmitted before more than 15 frames have been skipped.

NOTE - Since frames are skipped when the decoder buffer underflows, additional jitter may cause the decoder buffer to underflow more or less often than the encoder expects frame skips to happen.

additionalDecoderBuffer indicates the additional size of the video decoder buffer over and above that required by the indicated profile and level. This is defined in the same way as vbv_buffer_size H.262 [139].

### B.14.4	H.223 Skew Indication

This is used to indicate to the far-end terminal the average amount of time skew between two logical channels.

logicalChannelNumber1 and logicalChannelNumber2 are logical channel numbers of opened logical channels.

skew is measured in milliseconds, and indicates the delay that must be applied to data belonging to logicalChannelNumber2 as measured at the output of the multiplex, to achieve synchronization with logicalChannelNumber1 as measured at the output of the multiplex. The skew includes differences in: sample time, encoder delay, and transmitter buffer delay, and is measured relative to the transmission time of the first bit of data representing a given sample point. The actual delay necessary for synchronization is dependent on decoder implementation, and is a local matter for the receiver.

### B.14.5	New ATM Virtual Channel Indication

This is used to indicate the parameters of an ATM virtual channel that the terminal intends to open.

resourceID is used to identify the ATM virtual channel. The means by which this parameter is associated with an ATM virtual channel is not specified in this Recommendation.

bitRate indicates the bit rate, measured at the AAL-SAP, of the virtual channel, and is measured in units of 64 kbit/s.

bitRateLockedToPCRClock indicates that the bit rate of the virtual channel is clocked to the clock used to produce H.222.0 clock reference values (Program clock reference or System clock reference).

bitRateLockedToNetworkClock indicates that the bit rate of the virtual channel is clocked to the local network clock. This does not guarantee that the bit rate clock will be locked to the local network at the receiver, as common network clocks may not be available.

aal indicates which ATM Adaptation Layer will be used, and its parameters.

The sequence aal1 indicates which of the options for ATM adaptation layer 1, as specified in I.363 [140], are supported. The codepoints are defined in Table B.39.

The sequence aal5 indicates which of the options for ATM adaptation layer 5, as specified in I.363 [141], are supported. forwardMaximumSDUSize and backwardMaximumSDUSize indicate the maximum CPCS-SDU size in the forward and reverse directions, measured in octets.

multiplex indicates the type of multiplex that will be used on the ATM virtual channel. The options are noMultiplex (No H.222.0 multiplex), H.222.0 Transport Stream and H.222.0 Program Stream.

### B.14.6	User Input

This is used for User Input messages.

alphanumeric is a string of characters coded according to T.51 [142]. This could be used for key-pad input, an equivalent to DTMF.

userInputSupportIndication: indicates to the remote terminal which GENERALSTRING types the terminal supports.

NOTE - It is expected that most implementations of PER decoders will not be capable of decoding other strings than IA5. This indication should be used to "warn" the remote terminal not to attempt fancy variable length coding schemes.

nonStandard is a NonStandardParameter indicating a non-standard use of the UserInput indication message.

The boolean basicString, when true, indicates that the characters 0-9, * and # are supported.

The boolean iA5String, when true, indicates that the complete IA5String character set is supported.

The boolean generalString, when true, indicates that the complete GeneralString character set is supported.

NOTE - Any data that is carried in H.245, including user input messages, may not be encrypted.

The signal and signalUpdate indications may be used when precise control is desired over the alignment of DTMF or hookflash with audio in the associated logical channel and when control or indication of the duration of DTMF is needed.

signal indicates the signalling element to be produced when sent to a PSTN gateway, that was detected in the audio stream when sent from a PSTN gateway, or to be signalled between other endpoint combinations. When received by a gateway to the PSTN, signal causes the gateway to inject the specified signalling element into the PSTN channel; when received by a gateway to another H-series terminal, signal will be translated to the appropriate message in the protocol of the connected terminal. Gateways produce signal (and signalUpdate) messages to indicate detection of signalling elements in the audio received from a PSTN endpoint, or by translation of corresponding messages from other protocols.

signalType is set to “!” (exclamation point) to indicate a hookflash, or to one of “0123456789*#ABCD” to indicate a DTMF tone.

NOTE - Hookflash is a momentary on-hook condition (typically one-half second in duration), commonly used to control features in the attached switching equipment. It may not be possible for a gateway to produce or detect a hookflash due to characteristics of the PSTN channel or due to local configuration (to prevent undesired activation of features in attached equipment). Therefore, the ability to transmit or receive hookflash indications is separately declared in UserInputCapability.

duration indicates the total duration of the tone if known, or an initial estimate of the tone duration if the tone continues to be in progress at the time signal is transmitted. If duration is omitted, the receiver shall use an appropriate default based on local configuration and network requirements. duration shall be ignored in the case of a hookflash (“!”) indication.

signalUpdate revises the estimate of the total duration or declares the actual measured duration of the tone detected or to be generated. It should be transmitted so as to arrive well before the estimate that was previously sent in signal or signalUpdate expires; otherwise the revised duration will be ignored as the tone will have already been terminated by the receiver. Note that it is not necessary to send signalUpdate if the total duration was indicated in signal.

rtp contains parameters needed to align the tone or hookflash with an RTP/UDP stream (H.323). In signalUpdate, this element need be included only if multiple signal messages have been issued specifying different LogicalChannelNumbers and it is necessary to indicate which signal is to be updated.

timestamp specifies, in terms of the RTP timestamp of the primary encoder on the associated audio channel, the time at which the tone or hookflash should be generated (delivered or injected into the audio stream). The tone or hookflash shall not be generated before audio with the same timestamp is played; it should be generated as soon as possible after this time, but not later than the expirationTime timestamp. The sender of an indication shall not set timestamp to a time that is “in the future”; timestamp is normally set equal to the timestamp of audio currently being sent or most recently sent on the associated audio channel. If timestamp is not specified, the signal shall be delivered or injected upon receipt.

expirationTime specifies, in terms of the RTP timestamp of the primary encoder on the associated audio channel, the time after which the tone or hookflash shall be considered “stale” and discarded by the receiver. Endpoints that receive signal and are unable to act on it before the expirationTime timestamp on the associated channel shall discard the message. If an expirationTime time is not specified by the sender, the message may nevertheless be discarded as a result of local configuration of the recipient.

logicalChannelNumber shall specify the LogicalChannelNumber of the associated audio channel, the context in which timestamp and expirationTime are meaningful.

An MC shall convert the timestamps and logical channel number from the received indication into the correct logical channel number and timestamps for each output channel when it forwards the indication to each receiving endpoint (the timestamps might change if the audio is being transcoded or mixed in an MP). An MC receiving an indication after the expirationTime time may discard the message immediately without forwarding it; otherwise, the MC shall forward all requests immediately without waiting for timestamp time to occur.

Endpoints shall use the alphanumeric indication to convey DTMF user input if the other endpoint has not indicated the ability to receive DTMF using UserInputCapability.

An endpoint which has the capability to receive DTMF indications using signal shall also be able to accept alphanumeric indications for compatibility with older terminals. An alphanumeric indication may be treated as a sequence of one or more signal indications with duration, timestamp, and expirationTime elements omitted, and characters not valid in signalType being discarded.

In typical usage, a gateway detecting DTMF in the audio stream from a PSTN channel will send signal immediately upon detection of a tone, using a relatively high estimate of duration, and begin measuring the tone duration. When the tone ends, signalUpdate is sent to indicate the total measured duration. If the tone has not ended but the measured duration is approaching the previous estimate (such that the estimate might be exceeded by the measured duration before a signalUpdate could be received), a signalUpdate is sent increasing the estimate. The frequency of sending signalUpdate, the initial duration estimate sent in signal, and the amount by which subsequent estimates are increased, are left to the implementer, but caution should be exercised so as to not burden the network with large numbers of signalUpdate messages and to avoid premature expiration of previous estimates.

In typical usage from a non-gateway endpoint, the signal element will contain the total duration of the tone to be produced by the gateway. In some applications, however, it may be desirable to provide real-time interactive control of tone duration to the user. In this case, signal and signalUpdate would be used in a manner similar to that described for gateways in the preceding paragraph, with signal being sent upon the activation of the user input (e.g., depression of a key or on-screen control) using an estimated tone duration, and signalUpdate used to send updated estimates so long as the input continues to be activated and to indicate the total duration when the input is deactivated.

### B.14.7	Conference Indications

sbeNumber shall be defined as H.230 SBE Number.

terminalNumberAssign shall be defined as H.230 TIA.

terminalJoinedConference shall be defined as H.230 TIN.

terminalLeftConference shall be defined as H.230 TID.

seenByAtLeastOneOther shall be defined as H.230 MIV.

cancelSeenByAtLeastOneOther shall be defined as H.230 cancel-MIV.

seenByAll shall be defined as H.230 MIV.

cancelSeenByall shall be defined as H.230 MIV.

terminalYouAreSeeing shall be defined as H.230 VIN.

requestForFloor shall be defined as H.230 TIF and be sent from a terminal to the MC.

WithdrawChairToken shall be defined as H.230 CCR and is sent from the MC to the Chair Token holder.

FloorRequested shall be defined as H.230 TIF when sent from the MC to the Chair Token holder. This request included the TerminalLabel of the requesting terminal.

terminalYouAreSeeingInSubPictureNumber shall be defined as H.230 VIN2. subPictureNumber is defined as N as indicated in Figures 2-4/H.243.

videoIndicateCompose shall be defined as H.230 VIC. compositionNumber is defined as M in Table 4/H.243.

### B.14.8	H2250 Maximum Logical Channel Skew

H2250MaximumSkewIndication indicates the maximum skew between logical channels.

skew is measured in milliseconds, and indicates the maximum number of milliseconds that the data on logicalChannelNumber2 is delayed from the data on logicalChannelNumber1 as delivered to the network transport. The skew is measured relative to the time of delivery to the network transport of the first bit of data representing a given sample point. Lip synchronization, if desired, is a local matter for the receiver and shall be achieved via use of timestamps.

### B.14.9	MC Location Indication

This indication is sent by the MC to indicate to other terminals the signalling address that should be used to reach the MC.

### B.14.10	Vendor Identification Indication

vendorIdentification indication should be sent at the start of each call to identify the manufacturer, product, and product version number.

### B.14.11	Function Not Supported

This is used to return requests, responses and commands that are not understood back to the transmitter.

The whole of the RequestMessage, ResponseMessage or CommandMessage is returned.

If a terminal receives a request, response or command that it does not understand, either because it is non-standard or has been defined in a subsequent revision of this Recommendation, it shall respond by sending FunctionNotSupported.

If a terminal receives a request, response or command that has incorrect encoding, it shall set cause to the value syntaxError. If it has correct encoding, but the encoded values are semantically incorrect, it shall set cause to the value semanticError. If the message is an unrecognized extension to MultimediaSystemControlMessage, RequestMessage, ResponseMessage or CommandMessage, it shall set cause to the value unknownFunction.

In each case, the whole MultimediaSystemControlMessage should be returned as an octet string in returnedFunction.

FunctionNotSupported shall not be used at any other time. In particular, when an unrecognized extension is present at other points in the syntax, FunctionNotSupported shall not be used: the terminal shall respond to the message in the normal way, as if no extension were present. FunctionNotSupported shall never be sent in response to a received indication.

### B.14.12	Flow Control Indication

This is used to signal to the remote terminal that the terminal has adjusted its outgoing maximum bit-rate either in response to an incoming FlowControlCommand or because the terminal wishes to adjust its outgoing rate. This allows a terminal to signal any change in the outgoing maximum bit-rate within the constraint of the upper limits set by the open logical channel and the terminal capabilities.

Any terminal receiving a FlowControlCommand should respond with a FlowControl Indication to indicate the new maximum bit-rate it has been set to.

The fields of FlowControlIndication have the same meaning as the fields of the same name in FlowControlCommand.

ANNEX C

# Annex C	Procedures

A==

(This annex forms an integral part of this Recommendation)

## C.1	Introduction

This annex defines generic multimedia system control procedures that use the messages defined in this Recommendation. Recommendations using this Recommendation shall indicate which of these procedures are applicable, as well as defining any specific requirements.

Procedures to perform the following functions are described in this section:

·	master-slave determination

·	terminal capability exchange

·	uni-directional logical channel signalling

·	bi-directional logical channel signalling

·	receive terminal close logical channel request

·	H.223 multiplex table entry modification

·	request multiplex entry

·	receiver to transmitter transmit mode request

·	round trip delay determination

·	maintenance loop

### C.1.1	Method of specification

Procedures are generally specified in this section using SDLs. The SDL provides a graphical specification of the procedures, and includes specification of actions in the event of exception conditions.

### C.1.2	Communication between protocol entity and protocol user

The interaction with the user of a particular function is specified in terms of primitives transferred at the interface between the protocol entity and the protocol user. Primitives are for the purpose of defining protocol procedures and are not intended to specify or constrain implementation. There may be a number of parameters associated with each primitive.

To assist in the specification, protocol states are defined. These states are conceptual and reflect general conditions of the protocol entity in the sequences of primitives exchanged between the protocol entity and the user, and the exchange of messages between the protocol entity and its peer.

For each protocol entity the allowed sequence of primitives between the user and the protocol entity is defined using a state transition diagram. The allowed sequence constrains the actions of the user, and defines the possible responses from the protocol entity.

A primitive parameter described as being null, is equivalent to the parameter not being present.

### C.1.3	Peer-to-peer communication

Protocol information is transferred to the peer protocol entity via the relevant messages defined in Annex C. Some protocol entities described have state variables associated with them. A number of protocol entities described also have timers associated with them.

A timer is identified by the notation Tn, where n is a number. In the SDL diagrams setting a timer means that a timer is loaded with a specified value and the timer is started. Resetting a timer means that a timer is stopped with its value at the time of reset being retained. Timer expiry means that a timer has run for its specified time and has reached the value of zero.

A protocol entity may also have associated parameters. A parameter is identified by the notation Nn, where n is a number.

These timers and counters are listed in Appendix III.

Some protocol entities define an error primitive to report protocol error conditions to a management entity.

### C.1.4	SDL Diagrams

The SDL diagrams show actions to the allowed interactions with the protocol user, and to reception of messages from the peer protocol entity. Primitives which are not allowed for a given state, as specified by the state transition diagrams, are not shown in the SDL diagrams. However the responses to the reception of inappropriate messages are described in the SDL diagrams.

### C.1.5	SDL Key

The SDL key is shown in Figure C.1.

µ §

FIGURE C.2/H.245

SDL key

## C.2	Master slave determination procedures

### C.2.1	Introduction

Conflicts may arise when two or more terminals involved in a call initiate similar events simultaneously, for which resources are available for only one occurrence of the event e.g. opening of logical channels. To resolve such conflicts, one terminal may act as a master and the other terminals(s) may act as slave terminal(s). The procedures described here allow terminals in the call to determine which is the master terminal and which is the slave terminal(s).

The protocol described here is referred to as the Master Slave Determination Signalling Entity (MSDSE). There is one instance of the MSDSE in each terminal involved in a call.

Either terminal may initiate the master slave determination process by issuing the DETERMINE.request primitive to its MSDSE. The result of the procedure is returned by the DETERMINE.indication and DETERMINE.confirm primitives. While the DETERMINE.indication primitive indicates the result, it does not indicate that the result is known at the remote terminal. The DETERMINE.confirm primitive indicates the result and confirms that it is also known at the remote terminal. A terminal may only initiate the master slave determination process if no procedure which depends upon its result is locally active.

A terminal shall respond to procedures that rely on knowledge of the result and are initiated by the remote terminal any time after the status determination result is known at the local terminal. This may be before the local terminal has received confirmation that the remote terminal also has knowledge of the result. A terminal shall not initiate procedures that rely on knowledge of the result until it has received confirmation that the remote terminal also has knowledge of the result of the current instance of the determination procedure.

The following text provides an overview of the operation of the protocol. In the case of any discrepancy with the formal specification of the protocol that follows, the formal specification will supersede.

#### C.2.1.1	Protocol overview - initiation by local user

A master slave determination procedure is initiated when the DETERMINE.request primitive is issued by the MSDSE user. A MasterSlaveDetermination message is sent to the peer MSDSE, and timer T106 is started. If a MasterSlaveDeterminationAck message is received in response to the MasterSlaveDetermination message then timer T106 is stopped and the user is informed with the DETERMINE.confirm primitive that the master slave determination procedure was successful and a MasterSlaveDeterminationAck message is sent to the peer MSDSE. If however a MasterSlaveDeterminationReject message is received in response to the MasterSlaveDetermination message, then a new status determination number is generated, timer T106 is restarted, and another MasterSlaveDetermination message is sent. If after sending a MasterSlaveDetermination message N100 times, a MasterSlaveDeterminationAck still has not been received, then timer T106 is stopped and the user is informed with the REJECT.indication primitive that the master slave determination procedure has failed to produce a result.

If timer T106 expires then the MSDSE user is informed with the REJECT.indication primitive and a MasterSlaveDeterminationRelease message is sent to the peer MSDSE.

#### C.2.1.2	Protocol overview - initiation by remote user

When a MasterSlaveDetermination message is received at the MSDSE, a status determination procedure is initiated. If the status determination procedure returns a determinate result, then the user is informed of the master slave determination result with the DETERMINE.indication primitive, a MasterSlaveDeterminationAck message is sent to the peer MSDSE, and timer T106 is started. If a MasterSlaveDeterminationAck message is received in response to the MasterSlaveDeterminationAck message, then timer T106 is stopped and the user is informed with the DETERMINE.confirm primitive that the master slave determination procedure was successful.

If timer T106 expires then the MSDSE user is informed with the REJECT.indication primitive.

If however the status determination procedure returns an indeterminate result, then the MasterSlaveDeterminationReject message is sent to the peer MSDSE.

#### C.2.1.3	Protocol overview - simultaneous initiation

When a MasterSlaveDetermination message is received at the MSDSE that itself has already initiated a status determination procedure, and is awaiting a MasterSlaveDeterminationAck or MasterSlaveDeterminationReject message, then a status determination procedure is initiated. If the status determination procedure returns a determinate result, the MSDSE responds as if the procedure was initiated by the remote user, and the procedures described above for this condition apply.

If however the status determination procedure returns an indeterminate result, then a new status determination number is generated, and the MSDSE responds as if the procedure was again initiated by the local MSDSE user as described above.

#### C.2.1.4	Status determination procedure

The following procedure is used to determine which terminal is the master from the terminalType and statusDeterminationNumber values. Firstly, the terminalType values are compared and the terminal with the larger terminal type number is determined as the master. If the terminal type numbers are the same, the statusDeterminationNumbers are compared using modulo arithmetic to determine which is master.

If both terminals have equal terminalType field values and the difference between statusDeterminationNumber field values modulo 224 is 0 or 223, an indeterminate result is obtained

### C.2.2	Communication between the MSDSE and the MSDSE user

#### C.2.2.1	Primitives between the MSDSE and the MSDSE user

Communication between the MSDSE, and MSDSE user, is performed using the primitives shown in Table C.1.

TABLE C.2/H.245

Primitives and parameters

|  | type |  |  |  |
| --- | --- | --- | --- | --- |
| generic name | request | indication | response | confirm |
| DETERMINE | - 1 | TYPE | not defined 2 | TYPE |
| REJECT | not defined | - | not defined | not defined |
| ERROR | not defined | ERRCODE | not defined | not defined |

Notes:

1.	“-” means no parameters

2.	“not defined” means that this primitive is not defined.

#### C.2.2.2	Primitive definition

The definition of these primitives is as follows:

a)	The DETERMINE primitive is used to initiate, and to return the result from, the master slave determination procedure.

The DETERMINE.request primitive is used to initiate the master slave determination procedure.

The DETERMINE.indication primitive is used to indicate the result of the master slave determination procedure. As the result of the procedure may not be known at the remote terminal, the terminal shall not initiate any procedures that rely on knowledge of the result, although it shall respond to any procedures that rely on knowledge of the result.

The DETERMINE.confirm primitive is used to indicate the result of the master slave determination procedure and that the result of the procedure is known at both terminals. The terminal may initiate, and shall respond to, any procedures that rely on knowledge of the result.

b)	The REJECT primitive indicates that the master slave determination procedure was unsuccessful.

c)	The ERROR primitive reports MSDSE errors to a management entity.

#### C.2.2.3	Parameter definition

The definition of the primitive parameters shown in Table C.3 are as follows:

a)	The TYPE parameter indicates the terminal status. It has the value of "MASTER" or "SLAVE".

b)	The ERRCODE value indicates the type of MSDSE error Table C.4 indicates the values that the ERRCODE parameter may take.

#### C.2.2.4	MSDSE states

The following states are used to specify the allowed sequence of primitives between the MSDSE and the MSDSE user.

State 0:  IDLE

No master slave determination procedure has been initiated.

State 1:  OUTGOING AWAITING RESPONSE

The local MSDSE user has requested a master slave determination procedure. A response from the remote MSDSE is awaited.

State 2:  INCOMING AWAITING RESPONSE

The remote MSDSE has initiated a master slave determination procedure in the local MSDSE. An acknowledgement was sent to the remote MSDSE and a response from the remote MSDSE is awaited.

#### C.2.2.5	State transition diagram

The allowed sequence of primitives between the MSDSE and the MSDSE user is defined here. The allowed sequences are shown in Figure C.3.

FIGURE C.4/H.245

State transition diagram for sequence of primitives at MSDSE

### C.2.3	Peer to peer MSDSE communication

#### C.2.3.1	MSDSE messages

Table C.5 shows the MSDSE messages and fields, defined in Annex C, which are relevant to the MSDSE protocol.

TABLE C.6/H.245

MSDSE message names and fields

| function | message | field |
| --- | --- | --- |
| determination | MasterSlaveDetermination | terminalType statusDeterminationNumber |
|  | MasterSlaveDeterminationAck | decision |
|  | MasterSlaveDeterminationReject | cause |
| error recovery | MasterSlaveDeterminationRelease | - |

#### C.2.3.2	MSDSE state variables

The following MSDSE state variables are defined:

sv_TT

This state variable holds the terminal type number for this terminal.

sv_SDNUM

This state variable holds the status determination number for this terminal.

sv_STATUS

This state variable is used to store the result of the latest master slave determination procedure. It has values of "master", "slave", and "indeterminate".

sv_NCOUNT

This state variable is used to count the number of MasterSlaveDetermination messages that have been sent during the OUTGOING AWAITING RESPONSE state.

#### C.2.3.3	MSDSE timers

The following timer is specified for the outgoing MSDSE:

T106

This timer is used during the OUTGOING AWAITING RESPONSE state and during the INCOMING AWAITING RESPONSE state. It specifies the maximum allowed time during which no acknowledgement message may be received.

#### C.2.3.4	MSDSE counters

The following parameter is specified for the MSDSE:

N100

This parameter specifies the maximum value of sv_NCOUNT.

### C.2.4	MSDSE procedures

#### C.2.4.1	Introduction

Figure C.5 summarizes the MSDSE primitives and their parameters, and messages.

µ §

FIGURE C.6/H.245

Primitives and messages in the MSDSE.

#### C.2.4.2	Primitive parameter default values

Where not explicitly stated in the SDL diagrams the parameters of the indication and confirm primitives assume values as shown in Table C.7.

TABLE C.8/H.245

Default primitive parameter values

| primitive | parameter | default value |
| --- | --- | --- |
| DETERMINE.confirm | TYPE | MasterSlaveDeterminationAck.decision |
| DETERMINE.indication | TYPE | sv_STATUS |

#### C.2.4.3	Message field default values

Where not explicitly stated in the SDL diagrams the message fields assume values as shown in Table C.9.

TABLE C.10/H.245

Default message field values

| message | field | default value |
| --- | --- | --- |
| MasterSlaveDetermination | terminalType statusDeterminationNumber | sv_TT sv_SDNUM |
| MasterSlaveDeterminationAck | decision | Opposite of sv_STATUS i.e if(sv_STATUS == master) decision = slave if(sv_STATUS == slave) decision = master |
| MasterSlaveDeterminationReject | cause | identicalNumbers |

#### C.2.4.4	ERRCODE parameter values

Table C.11 shows the values that the ERRCODE parameter of the ERROR.indication primitive may take for the MSDSE.

TABLE C.12/H.245

ERRCODE parameter values at MSDSE

| error type | error code | error condition | state |
| --- | --- | --- | --- |
| no response from remote MSDSE | A | local timer T106 expiry | OUTGOING AWAITING RESPONSE INCOMING AWAITING RESPONSE |
| remote sees no response from local MSDSE | B | remote timer T106 expiry | OUTGOING AWAITING RESPONSE INCOMING AWAITING RESPONSE |
| inappropriate message | C | MasterSlaveDetermination | INCOMING AWAITING RESPONSE |
|  | D | MasterSlaveDeterminationReject | INCOMING AWAITING RESPONSE |
| inconsistent field value | E | MasterSlaveDeterminationAck.decision != sv_STATUS | INCOMING AWAITING RESPONSE |
| maximum number of retries | F | sv_NCOUNT == N100 | OUTGOING AWAITING RESPONSE |

#### C.2.4.5	SDLs

The MSDSE procedures are expressed in SDL form in Figure C.7.

terminalTypeProcess is process that returns a number that identifies different types of terminal, such as, terminals, MCUs and gateways.

randomNumber is process that returns a random number in the range 0 ... 224-1.

µ §

FIGURE C.8(i)/H.245

MSDSE SDL

FIGURE C.8(ii)/H.245

MSDSE SDL (continued)

FIGURE C.8(iii)/H.245

MSDSE SDL (continued)

FIGURE C.8(iv)/H.245

MSDSE SDL (continued)

µ §

FIGURE C.8(v)/H.245

MSDSE SDL (concluded)

## C.3	Capability exchange procedures

### C.3.1	Introduction

These procedures are used by terminals to communicate their capabilities, and are referred to as the Capability Exchange Signalling Entity (CESE). Procedures are specified in terms of primitives and states at the interface between the CESE and the CESE user. Protocol information is transferred to the peer CESE via relevant messages defined in Annex C. There is an outgoing CESE and an incoming CESE. At each of the outgoing and incoming ends there is one instance of the CESE for each call.

All terminals intended for use in point-to-point applications or those connected to an MCU shall be able to identify a TerminalCapabilitySet and its structure, and such capability values therein that are mandatory for those applications; any unrecognized capability values shall be ignored, and no fault shall be implied.

The capability exchange may be performed at any time. The capability exchange may signal both changed and unchanged capabilities. Unchanged capabilities should not be sent repetitively without strong cause.

The following text provides an overview of the operation of the protocol. In the case of any discrepancy with the formal specification of the protocol that follows, the formal specification will supersede.

#### C.3.1.1	Protocol overview - outgoing CESE

A capability exchange is initiated when the TRANSFER.request primitive is issued by the user at the outgoing CESE. A TerminalCapabilitySet message is sent to the peer incoming CESE, and timer T101 is started. If a TerminalCapabilitySetAck message is received in response to the TerminalCapabilitySet message then timer T101 is stopped and the user is informed with the TRANSFER.confirm primitive that the capability exchange was successful. If however a TerminalCapabilitySetReject message is received in response to the TerminalCapabilitySet message then timer T101 is stopped and the user is informed with the REJECT.indication primitive that the peer CESE user has refused the capability exchange.

If timer T101 expires then the outgoing CESE user is informed with the REJECT.indication primitive and a TerminalCapabilitySetRelease message is sent.

#### C.3.1.2	Protocol overview - incoming CESE

When a TerminalCapabilitySet message is received at the incoming CESE, the user is informed of the capability exchange request with the TRANSFER.indication primitive. The incoming CESE user signals acceptance of the capability exchange request by issuing the TRANSFER.response primitive, and a TerminalCapabilitySetAck message is sent to the peer outgoing CESE. The incoming CESE user signals rejection of the capability exchange request by issuing the REJECT.request primitive, and a TerminalCapabilitySetReject message is sent to the peer outgoing CESE.

### C.3.2	Communication between CESE and CESE user

#### C.3.2.1	Primitives between CESE and CESE user

Communication between the CESE and CESE user, is performed using the primitives shown in Table C.13.

TABLE C.14/H.245

Primitives and parameters

|  | type |  |  |  |
| --- | --- | --- | --- | --- |
| generic name | request | indication | response | confirm |
| TRANSFER | PROTOID MUXCAP CAPTABLE CAPDESCRIPTORS | PROTOID MUXCAP CAPTABLE CAPDESCRIPTORS | - 1 | - |
| REJECT | CAUSE | SOURCE CAUSE | not defined 2 | not defined |

Notes:

1.	“-” means no parameters

2.	“not defined” means that this primitive is not defined.

#### C.3.2.2	Primitive definition

The definition of these primitives is as follows:

a)	The TRANSFER primitives are used for transfer of the capability exchange.

b)	The REJECT primitives are used to reject a capability descriptor entry, and to terminate a current capability transfer.

#### C.3.2.3	Parameter definition

The definition of the primitive parameters shown in Table C.15 are as follows:

a)	The PROTOID parameter is the protocol identifier parameter. This parameter is mapped to the protocolIdentifier field of the TerminalCapabilitySet message and carried transparently to the peer CESE user. This parameter is mandatory.

b)	The MUXCAP parameter is the multiplex capability parameter. This parameter is mapped to the multiplexCapability field of the TerminalCapabilitySet message and carried transparently to the peer CESE user. This parameter is optional.

c)	The CAPTABLE parameter is the capability table parameter. There may be one or more capability table entries described within this parameter. This parameter is mapped to the capabilityTable field of the TerminalCapabilitySet message and carried transparently to the peer CESE user. This parameter is optional.

d)	The CAPDESCRIPTORS parameter is the capability descriptors parameter. There may be one or more capability descriptors described within in this parameter. This parameter is mapped to the capabilityDescriptors field of the TerminalCapabilitySet message and carried transparently to the peer CESE user. this parameter is optional.

e)	The SOURCE parameter indicates the source of the REJECT.indication primitive. The SOURCE parameter has the value of "USER" or "PROTOCOL". The latter case may occur as the result of a timer expiry.

f)	The CAUSE parameter indicates the reason for rejection of a CAPTABLE or CAPDESCRIPTORS parameter. The CAUSE parameter is not present when the SOURCE parameter indicates "PROTOCOL".

#### C.3.2.4	CESE states

The following states are used to specify the allowed sequence of primitives between the CESE and the CESE user.

The states for an outgoing CESE are:

State 0:  IDLE

The CESE is idle.

State 1:  AWAITING RESPONSE

The CESE is waiting for a response from the remote CESE.

The states for an incoming CESE are:

State 0:  IDLE

The CESE is idle.

State 1:  AWAITING RESPONSE

The CESE is waiting for a response from the CESE user.

#### C.3.2.5	State transition diagram

The allowed sequence of primitives between the CESE and the CESE user is defined here. The allowed sequence of primitives relates to states of the CESE as viewed from the CESE user. The allowed sequences are specified separately for each of an outgoing CESE and an incoming CESE, as shown in Figure C.9 and Figure C.10 respectively.

µ §

FIGURE C.11/H.245

State transition diagram for sequence of primitives at CESE outgoing

FIGURE C.12/H.245

State transition diagram for sequence of primitives at CESE incoming

### C.3.3	Peer to peer CESE communication

#### C.3.3.1	Messages

Table C.16 shows the CESE messages and fields, defined in Annex C, which are relevant to the CESE protocol.

TABLE C.17/H.245

CESE message names and fields

| function | message | direction | field |
| --- | --- | --- | --- |
| transfer | TerminalCapabilitySet | O -> I 1 | sequenceNumber |
|  |  |  | protocolIdentifier |
|  |  |  | multiplexCapability |
|  |  |  | capabilityTable |
|  |  |  | capabilityDescriptors |
|  | TerminalCapabilitySetAck | O <- I | sequenceNumber |
| reject | TerminalCapabilitySetReject | O <- I | sequenceNumber |
|  |  |  | cause |
| reset | TerminalCapabilitySetRelease | O -> I | - |

Notes:

1.	Direction:  O - outgoing, I - incoming.

#### C.3.3.2	CESE state variables

The following state variables are defined at the outgoing CESE:

out_SQ

This state variable is used to indicate the most recent TerminalCapabilitySet message. It is incremented by one and mapped to the TerminalCapabilitySet message sequenceNumber field before transmission of the TerminalCapabilitySet message. Arithmetic performed on out_SQ is modulo 256.

The following state variables are defined at the incoming CESE:

in_SQ

This state variable is used to store the value of the sequenceNumber field of the most recently received TerminalCapabilitySet message. The TerminalCapabilitySetAck and TerminalCapabilitySetReject messages have their sequenceNumber fields set to the value of in_SQ, before being sent to the peer CESE.

#### C.3.3.3	CESE timers

The following timer is specified for the outgoing CESE:

T101

This timer is used during the AWAITING RESPONSE state. It specifies the maximum time during which no TerminalCapabilitySetAck or TerminalCapabilitySetReject message may be received.

### C.3.4	CESE procedures

Figure C.13 summarizes the CESE primitives and their parameters, and messages, for each of the outgoing and incoming CESE.

µ §

FIGURE C.14/H.245

Primitives and messages in the Capability Exchange Signalling Entity

#### C.3.4.1	Primitive parameter default values

Where not explicitly stated in the SDL diagrams the parameters of the indication and confirm primitives assume values as shown in Table C.18.

TABLE C.19/H.245

Default primitive parameter values

| primitive | parameter | default value |
| --- | --- | --- |
| TRANSFER.indication | PROTOID | TerminalCapabilitySet.protocolIdentifier |
|  | MUXCAP | TerminalCapabilitySet.multiplexCapability |
|  | CAPTABLE | TerminalCapabilitySet.capabilityTable |
|  | CAPDESCRIPTORS | TerminalCapabilitySet.capabilityDescriptors |
| REJECT.indication | SOURCE | USER |
|  | CAUSE | null |

#### C.3.4.2	Message field default values

Where not explicitly stated in the SDL diagrams the message fields assume values as shown in Table C.20.

TABLE C.21/H.245

Default message field values

| message | field | default value 1 |
| --- | --- | --- |
| TerminalCapabilitySet | sequenceNumber | out_SQ |
|  | protocolIdentifier | TRANSFER.request(PROTOID) |
|  | multiplexCapability | TRANSFER.request(MUXCAP) |
|  | capabilityTable | TRANSFER.request(CAPTABLE) |
|  | capabilityDescriptors | TRANSFER.request(CAPDESCRIPTORS) |
| TerminalCapabilitySetAck | sequenceNumber | in_SQ |
| TerminalCapabilitySetReject | sequenceNumber | in_SQ |
|  | cause | REJECT.request(CAUSE) |
| TerminalCapabilitySetRelease | - | - |

Notes:

1.	A message field shall not be coded, if the corresponding primitive parameter is null i.e. not present.

#### C.3.4.3	SDLs

The outgoing CESE and the incoming CESE procedures are expressed in SDL form in Figure C.15 and Figure C.16, respectively.

FIGURE C.17(i)/H.245

Outgoing CESE SDL

µ §

FIGURE C.17(ii)/H.245

Outgoing CESE SDL (concluded)

FIGURE C.18(i)/H.245

Incoming CESE SDL

µ §

FIGURE C.18(ii)/H.245

Incoming CESE SDL (concluded)

## C.4	Uni-directional Logical Channel signalling procedures

### C.4.1	Introduction

The protocol specified here provides reliable opening and closing of uni-directional logical channels using acknowledged procedures.

The protocol specified here is referred to as the Logical Channel Signalling Entity (LCSE). Procedures are specified in terms of primitives at the interface between the LCSE and the LCSE user, and LCSE states. Protocol information is transferred to the peer LCSE via relevant messages defined in Annex C.

There is an outgoing LCSE and an incoming LCSE. At each of the outgoing and incoming sides there is one instance of the LCSE for each uni-directional logical channel. There is no connection between an incoming LCSE and an outgoing LCSE at one side, other than via primitives to and from the LCSE user. LCSE error conditions are reported.

Data shall only be sent on a logical channel in the ESTABLISHED state. If data is received on a logical channel that is not in the ESTABLISHED state the data shall be discarded and no fault shall be considered to have occurred.

Mode switching should be performed by closing and opening existing logical channels, or by opening new logical channels.

NOTE - Some recommendations that use this Recommendation may define some default logical channels. These shall be considered ESTABLISHED from the start of communication and shall not be opened using these procedures. They may, however, be closed by these procedures, and subsequently be re-opened for the same or a different purpose.

A terminal that is no longer capability of processing the signals on a logical channel should take appropriate action: this should include closing the logical channel and transmitting the relevant (changed) capability information to the remote terminal.

The following text provides an overview of the operation of the LCSE protocol. In the case of discrepancy between this and the formal specification, the formal specification will supersede.

#### C.4.1.1	Protocol overview

The opening of a logical channel is initiated when the ESTABLISH.request primitive is issued by the user at the outgoing LCSE. An OpenLogicalChannel message, containing forward logical channel parameters but not including reverse logical channel parameters, is sent to the peer incoming LCSE, and timer T103 is started. If an OpenLogicalChannelAck message is received in response to the OpenLogicalChannel message then timer T103 is stopped and the user is informed with the ESTABLISH.confirm primitive that the logical channel has been successfully opened. The logical channel may now be used to transmit user information. If however an OpenLogicalChannelReject message is received in response to the OpenLogicalChannel message then timer T103 is stopped and the user is informed with the RELEASE.indication primitive that the peer LCSE user has refused establishment of the logical channel.

If timer T103 expires in this period then the user is informed with the RELEASE.indication primitive, and a CloseLogicalChannel message is sent to the peer incoming LCSE.

A logical channel which has been successfully established may be closed when the RELEASE.request primitive is issued by the user at the outgoing LCSE. A CloseLogicalChannel message is sent to the peer incoming LCSE, and the timer T103 is started. When a CloseLogicalChannelAck message is received, timer T103 is stopped and the user is informed that the logical channel has been successfully closed with the RELEASE.confirm primitive.

If timer T103 expires in this period then the user is informed with the RELEASE.indication primitive.

Before either of the OpenLogicalChannelAck or OpenLogicalChannelReject messages have been received in response to a previously sent OpenLogicalChannel message, the user at the outgoing LCSE may close the logical channel using the RELEASE.request primitive.

Before the CloseLogicalChannelAck message is received in response to a previously sent CloseLogicalChannel message, the user at the outgoing LCSE may establish a new logical channel by issuing the ESTABLISH.request primitive.

#### C.4.1.2	Protocol overview - incoming LCSE

When an OpenLogicalChannel message is received at the incoming LCSE, the user is informed of the request to open a new logical channel with the ESTABLISH.indication primitive. The incoming LCSE user signals acceptance of the request to establish the logical channel by issuing the ESTABLISH.response primitive, and an OpenLogicalChannelAck message is sent to the peer outgoing LCSE. The logical channel may now be used to receive user information. The incoming LCSE user signals rejection of the request to establish the logical channel by issuing the RELEASE.request primitive, and an OpenLogicalChannelReject message is sent to the peer outgoing LCSE.

A logical channel which has been successfully established may be closed when the CloseLogicalChannel message is received at the incoming LCSE. The incoming LCSE user is informed with the RELEASE.indication primitive, and the CloseLogicalChannelAck message is sent to the peer outgoing LCSE.

#### C.4.1.3	Conflict resolution

Conflicts may arise when requests to open logical channels are initiated at the same time. It may be possible to determine that there is a conflict from knowledge of exchanged capabilities.

Terminals shall be capable of detecting when conflict has arisen, or might arise, and shall act as follows.

Before logical channels can be opened, one terminal must be determined as the master terminal, and the other as the slave. The protocol defined in C.2 provides one means to make this decision. The master terminal shall reject immediately any request from the slave that it identifies as a conflicting request. The slave terminal may identify such conflicts, but shall respond to the request from the master terminal, with the knowledge that its earlier request will be rejected.

NOTE - Such conflicts might be caused by limited terminal resources, for example, when receive and transmission capabilities are dependent, as in the case of a terminal that can support a number of audio algorithms, but can only decode the same algorithm as it is encoding.

### C.4.2	Communication between the LCSE and the LCSE user

#### C.4.2.1	Primitives between the LCSE and the LCSE user

Communication between the LCSE and the LCSE user is performed using the primitives shown in Table C.22.

TABLE C.23/H.245

Primitives and parameters

|  | type |  |  |  |
| --- | --- | --- | --- | --- |
| generic name | request | indication | response | confirm |
| ESTABLISH | FORWARD_PARAM | FORWARD_PARAM | - 1 | - |
| RELEASE | CAUSE | SOURCE CAUSE | not defined 2 | - |
| ERROR | not defined | ERRCODE | not defined | not defined |

Notes:

1.	“-” means no parameters.

2.	“not defined” means that this primitive does not exist.

#### C.4.2.2	Primitive definition

The definition of these primitives is as follows:

a)	The ESTABLISH primitives are used to establish a logical channel for audiovisual and data communication.

b)	The RELEASE primitives are used to release a logical channel.

c)	The ERROR primitive reports LCSE errors to a management entity.

#### C.4.2.3	Parameter definition

The definition of the primitive parameters shown in Table C.24 are as follows:

a)	The FORWARD_PARAM parameter specifies the parameters associated with the logical channel. This parameter is mapped to the forwardLogicalChannelParameters field of the OpenLogicalChannel message and is carried transparently to the peer LCSE user.

b)	The SOURCE parameter indicates to the LCSE user the source of the logical channel release. The SOURCE parameter has the value of "USER" or "LCSE", indicating either the LCSE user, or the LCSE. The latter may occur as the result of a protocol error.

c)	The CAUSE parameter indicates the reason as to why the peer LCSE user rejected a request to establish a logical channel. The CAUSE parameter is not present when the SOURCE parameter indicates "LCSE".

d)	The ERRCODE parameter indicates the type of LCSE error. Table C.25 shows the allowed values of the ERRCODE parameter.

#### C.4.2.4	LCSE states

The following states are used to specify the allowed sequence of primitives between the LCSE and the LCSE user, and the exchange of messages between peer LCSEs. The states are specified separately for each of an outgoing LCSE and an incoming LCSE. The states for an outgoing LCSE are:

State 0:  RELEASED

The logical channel is released. The logical channel shall not be used to send outgoing data.

State 1:  AWAITING ESTABLISHMENT

The outgoing LCSE is waiting to establish a logical channel with a peer incoming LCSE. The logical channel shall not be used to send outgoing data.

State 2:  ESTABLISHED

The LCSE peer-to-peer logical channel connection has been established. The logical channel may be used to send outgoing data.

State 3:  AWAITING RELEASE

The outgoing LCSE is waiting to release a logical channel with the peer incoming LCSE. The logical channel shall not be used to send outgoing data.

The states for an incoming LCSE are:

State 0:  RELEASED

The logical channel is released. The logical channel shall not be used to receive incoming data.

State 1:  AWAITING ESTABLISHMENT

The incoming LCSE is waiting to establish a logical channel with a peer outgoing LCSE. The logical channel shall not be used to receive incoming data.

State 2:  ESTABLISHED

An LCSE peer-to-peer logical channel connection has been established. The logical channel may be used to receive incoming data.

#### C.4.2.5	State transition diagram

The allowed sequence of primitives between the LCSE and the LCSE user is defined here. The allowed sequence of primitives relates to states of the LCSE as viewed from the LCSE user. The allowed sequences are specified separately for each of an outgoing LCSE and an incoming LCSE, as shown in Figure C.19 and Figure C.20 respectively.

FIGURE C.21/H.245

State transition diagram for sequence of primitives at outgoing LCSE

µ §

FIGURE C.22/H.245

State transition diagram for sequence of primitives at incoming LCSE

### C.4.3	Peer to peer LCSE communication

#### C.4.3.1	LCSE messages

Table C.26 shows the LCSE messages and fields, defined in Annex C, which are relevant to the LCSE protocol.

TABLE C.27/H.245

LCSE message names and fields

| function | message | direction | field |
| --- | --- | --- | --- |
| establishment | OpenLogicalChannel | O -> I 1 | forwardLogicalChannelNumber |
|  |  |  | forwardLogicalChannelParameters |
|  | OpenLogicalChannelAck | O <- I | forwardLogicalChannelNumber |
|  | OpenLogicalChannelReject | O <- I | forwardLogicalChannelNumber |
|  |  |  | cause |
| release | CloseLogicalChannel | O -> I | forwardLogicalChannelNumber |
|  |  |  | source |
|  | CloseLogicalChannelAck | O <- I | forwardLogicalChannelNumber |

Notes:

1.	Direction:  O - outgoing, I - incoming.

#### C.4.3.2	LCSE state variables

The following state variable is defined at the outgoing LCSE:

out_LCN

This state variable distinguishes between outgoing LCSEs. It is initialised at outgoing LCSE initialization. The value of out_LCN is used to set the forwardLogicalChannelNumber field of LCSE messages sent from an outgoing LCSE. For LCSE messages received at an outgoing LCSE, the message forwardLogicalChannelNumber field value is identical to the value of out_LCN.

The following state variable is defined at the incoming LCSE:

in_LCN

This state variable distinguishes between incoming LCSEs. It is initialised at incoming LCSE initialization. The value of in_LCN is used to set the forwardLogicalChannelNumber field of LCSE messages sent from an incoming LCSE. For LCSE messages received at an incoming LCSE, the message forwardLogicalChannelNumber field value is identical to the value of in_LCN.

#### C.4.3.3	LCSE timers

The following timer is specified for the outgoing LCSE:

T103

This timer is used during the AWAITING ESTABLISHMENT and AWAITING RELEASE states. It specifies the maximum allowed time during which no OpenLogicalChannelAck, OpenLogicalChannelReject or CloseLogicalChannelAck message may be received.

### C.4.4	LCSE procedures

#### C.4.4.1	Introduction

Figure C.23 summarizes the primitives and their parameters, and the messages, for each of the outgoing and incoming LCSE.

µ §

FIGURE C.24/H.245

Primitives and messages in the Logical Channel Signalling Entity

#### C.4.4.2	Primitive parameter default values

Where not explicitly stated in the SDL diagrams the parameters of the indication and confirm primitives assume values as shown in Table C.28.

TABLE C.29/H.245

Default primitive parameter values

| primitive | parameter | default value 1 |
| --- | --- | --- |
| ESTABLISH.indication | FORWARD_PARAM | OpenLogicalChannel.forwardLogicalChannelParameters |
| RELEASE.indication | SOURCE | CloseLogicalChannel.source |
|  | CAUSE | null |

Notes:

1.	A primitive parameter shall be coded as null, if an indicated message field is not present in the message.

#### C.4.4.3	Message field default values

Where not explicitly stated in the SDL diagrams the message fields assume values as shown in Table C.30.

TABLE C.31/H.245

Default message field values

| message | field | default value 1 |
| --- | --- | --- |
| OpenLogicalChannel2 | forwardLogicalChannelNumber | out_LCN |
|  | forwardLogicalChannelParameters | ESTABLISH.request(FORWARD_PARAM) |
| OpenLogicalChannelAck | forwardLogicalChannelNumber | in_LCN |
| OpenLogicalChannelReject | forwardLogicalChannelNumber | in_LCN |
|  | cause | RELEASE.request(CAUSE) |
| CloseLogicalChannel | forwardLogicalChannelNumber | out_LCN |
|  | source | user |
| CloseLogicalChannelAck | forwardLogicalChannelNumber | in_LCN |

Notes:

1.	A message field shall not be coded, if the corresponding primitive parameter is null i.e. not present.

2.	reverseLogicalChannelParameters are not coded in uni-directional logical channel signalling procedures.

#### C.4.4.4	ERRCODE parameter values

The ERRCODE parameter of the ERROR.indication primitive indicates a particular error condition. Table C.32 shows the values that the ERRCODE parameter may take at the outgoing LCSE. There is no ERROR.indication primitive associated with the incoming LCSE.

TABLE C.33/H.245

ERRCODE parameter values at outgoing LCSE

| error type | error code | error condition | state |
| --- | --- | --- | --- |
| inappropriate message | A | OpenLogicalChannelAck | RELEASED |
|  | B | OpenLogicalChannelReject | RELEASED ESTABLISHED |
|  | C | CloseLogicalChannelAck | ESTABLISHED |
| no response from peer LCSE | D | timer T103 expiry | AWAITING ESTABLISHMENT AWAITING RELEASE |

#### C.4.4.5	SDLs

The outgoing LCSE and the incoming LCSE procedures are expressed in SDL form in Figure C.25 and Figure C.26 respectively.

µ §

FIGURE C.27(i)/H.245

Outgoing LCSE SDL

µ §

FIGURE C.27(ii)/H.245

Outgoing LCSE SDL (continued)

µ §

FIGURE C.27(iii)/H.245

Outgoing LCSE SDL (continued)

µ §

FIGURE C.27(iv)/H.245

Outgoing LCSE SDL (concluded)

µ §

FIGURE C.28(i)/H.245

Incoming LCSE SDL

µ §

FIGURE C.28(ii)/H.245

Incoming LCSE SDL (continued)

µ §

FIGURE C.28(iii)/H.245

Incoming LCSE SDL (concluded)

## C.5	Bi-directional Logical Channel signalling procedures

### C.5.1	Introduction

The protocol specified here provides reliable opening and closing of bi-directional logical channels using acknowledged procedures.

The protocol specified here is referred to as the Bi-directional Logical Channel Signalling Entity (B-LCSE). Procedures are specified in terms of primitives at the interface between the B-LCSE and the B-LCSE user, and B-LCSE states. Protocol information is transferred to the peer B-LCSE via relevant messages defined in Annex C.

There is an outgoing B-LCSE and an incoming B-LCSE. At each of the outgoing and incoming sides there is one instance of the B-LCSE for each bi-directional logical channel. There is no connection between an incoming B-LCSE and an outgoing B-LCSE at one side, other than via primitives to and from the B-LCSE user. B-LCSE error conditions are reported.

A bi-directional logical channel consists of a pair of associated uni-directional channels. "Forward"  (Outgoing side) is used to refer to transmission in the direction from the terminal making the request for a bi-directional logical channel to the other terminal, and "reverse"  (Incoming side) is used to refer to the opposite direction of transmission.

Data shall only be sent on a bi-directional logical channel in the ESTABLISHED state. However, data may be received on the forward channel when the incoming B-LCSE is in the AWAITING CONFIRMATION state. Data that is received while in other states than the ESTABLISHED state and the AWAITING CONFIRMATION state shall be discarded and no fault shall be considered to have occurred.

A terminal may reject a request to open a bi-directional logical channel solely because it can not support the requested reverse channel parameters. In this case it shall reject the request with cause equal to unsuitableReverseParameters, and shall immediately initiate procedures to establish a bi-directional logical channel as requested by the remote terminal, in which the reverse parameters are identical to the forward parameters of the remote terminal's failed request, and with forward parameters that the terminal can support and which the remote terminal is known to be able to support.

Mode switching should be performed by closing and opening existing logical channels, or by opening new logical channels.

NOTE - Some recommendations that use this Recommendation may define some default logical channels. These shall be considered ESTABLISHED from the start of communication and shall not be opened using these procedures. They may, however, be closed by these procedures, and subsequently be re-opened for the same or a different purpose.

A terminal that is no longer capable of processing the signals on a logical channel should take appropriate action: this should include closing the logical channel and transmitting the relevant (changed) capability information to the remote terminal.

The following text provides an overview of the operation of the B-LCSE protocol. In the case of discrepancy between this and the formal specification, the formal specification will supersede.

#### C.5.1.1	Protocol overview

The opening of a logical channel is initiated when the ESTABLISH.request primitive is issued by the user at the outgoing B-LCSE. An OpenLogicalChannel message, containing both forward and reverse logical channel parameters, is sent to the peer incoming B-LCSE, and timer T103 is started. If an OpenLogicalChannelAck message is received in response to the OpenLogicalChannel message then timer T103 is stopped, an OpenLogicalChannelConfirm message is sent to the peer incoming B-LCSE, and the user is informed with the ESTABLISH.confirm primitive that the logical channel has been successfully opened. The logical channel may now be used to transmit and receive user information. If however an OpenLogicalChannelReject message is received in response to the OpenLogicalChannel message then timer T103 is stopped and the user is informed with the RELEASE.indication primitive that the peer B-LCSE user has refused establishment of the logical channel.

If timer T103 expires in this period then the user is informed with the RELEASE.indication primitive, and a CloseLogicalChannel message is sent to the peer incoming B-LCSE.

A logical channel which has been successfully established may be closed when the RELEASE.request primitive is issued by the user at the outgoing B-LCSE. A CloseLogicalChannel message is sent to the peer incoming B-LCSE, and the timer T103 is started. When a CloseLogicalChannelAck message is received, timer T103 is stopped and the user is informed that the logical channel has been successfully closed with the RELEASE.confirm primitive.

If timer T103 expires in this period then the user is informed with the RELEASE.indication primitive.

Before either of the OpenLogicalChannelAck or OpenLogicalChannelReject messages have been received in response to a previously sent OpenLogicalChannel message, the user at the outgoing B-LCSE may close the logical channel using the RELEASE.request primitive.

Before the CloseLogicalChannelAck message is received in response to a previously sent CloseLogicalChannel message, the user at the outgoing B-LCSE may establish a new logical channel by issuing the ESTABLISH.request primitive.

#### C.5.1.2	Protocol overview - incoming B-LCSE

When an OpenLogicalChannel message is received at the incoming B-LCSE, the user is informed of the request to open a new logical channel with the ESTABLISH.indication primitive. The incoming B-LCSE user signals acceptance of the request to establish the logical channel by issuing the ESTABLISH.response primitive, and an OpenLogicalChannelAck message is sent to the peer outgoing B-LCSE. The forward channel of the bi-directional logical channel may now be used to receive user information. The incoming B-LCSE user signals rejection of the request to establish the logical channel by issuing the RELEASE.request primitive, and an OpenLogicalChannelReject message is sent to the peer outgoing B-LCSE.

When an OpenLogicalChannelConfirm message is received at the incoming B-LCSE, the user is informed that the bi-directional logical channel is established with the ESTABLISH.confirm primitive. The reverse channel of the bi-directional logical channel may now be used to transmit user information.

A logical channel which has been successfully established may be closed when the CloseLogicalChannel message is received at the incoming B-LCSE. The incoming B-LCSE user is informed with the RELEASE.indication primitive, and the CloseLogicalChannelAck message is sent to the peer outgoing B-LCSE.

#### C.5.1.3	Conflict resolution

Conflicts may arise when requests to open logical channels are initiated at the same time. It may be possible to determine that there is conflict from knowledge of exchanged capabilities. On other occasions, both terminals may initiate the opening of a bi-directional logical channel for the same purpose, even though the exact parameters requested may be different, and both terminals have sufficient capability for both requests. Terminals shall be capable of detecting when both of these situations have arisen, any shall act as follows.

Before logical channels can be opened, one terminal must be determined as the master terminal, and the other as the slave. The protocol defined in C.2 provides one means to make this decision. The master terminal shall reject immediately any request from the slave that it identifies as a conflicting request. The slave terminal may identify such conflicts, but shall respond to the request from the master terminal, with the knowledge that its earlier request will be rejected.

In the second type of conflict defined above, it is impossible to distinguish when two bi-directional channels are actually wanted from the case when only one is wanted. Terminals shall respond assuming that only one is wanted, but a terminal may subsequently repeat its request if the assumption was incorrect.

### C.5.2	Communication between the B-LCSE and the B-LCSE user

#### C.5.2.1	Primitives between the B-LCSE and the B-LCSE user

Communication between the B-LCSE and the B-LCSE user is performed using the primitives shown in Table C.34.

TABLE C.35/H.245

Primitives and parameters

|  | type |  |  |  |
| --- | --- | --- | --- | --- |
| generic name | request | indication | response | confirm |
| ESTABLISH | FORWARD_PARAM REVERSE_PARAM | FORWARD_PARAM REVERSE_PARAM | REVERSE_DATA | REVERSE_DATA |
| RELEASE | CAUSE | SOURCE CAUSE | not defined 2 | - 1 |
| ERROR | not defined | ERRCODE | not defined | not defined |

Notes:

1.	“-” means no parameters.

2.	“not defined” means that this primitive does not exist.

#### C.5.2.2	Primitive definition

The definition of these primitives is as follows:

a)	The ESTABLISH primitives are used to establish a logical channel for audiovisual and data communication.

b)	The RELEASE primitives are used to release a logical channel.

c)	The ERROR primitive reports B-LCSE errors to a management entity.

#### C.5.2.3	Parameter definition

The definition of the primitive parameters shown in Table C.36 are as follows:

a)	The FORWARD_PARAM parameter specifies the parameters associated with the forward channel, that is, from the terminal containing the outgoing B-LCSE to the terminal containing the incoming B-LCSE. This parameter is mapped to the forwardLogicalChannelParameters field of the OpenLogicalChannel message and is carried transparently to the peer LCSE user.

b)	The REVERSE_PARAM parameter specifies the parameters associated with the reverse channel, that is, from the terminal containing the incoming B-LCSE to the terminal containing the outgoing B-LCSE. This parameter is mapped to the reverseLogicalChannelParameters field of the OpenLogicalChannel message and is carried transparently to the peer LCSE user.

c)	The REVERSE_DATA parameter specifies some parameters associated with the reverse channel, that is, from the terminal containing the incoming B-LCSE to the terminal containing the outgoing B-LCSE. This parameter is mapped to the reverseLogicalChannelParameters field of the OpenLogicalChannelAck message and is carried transparently to the peer B-LCSE user.

d)	The SOURCE parameter indicates to the B-LCSE user the source of the logical channel release. The SOURCE parameter has the value of "USER" or "B-LCSE", indicating either the B-LCSE user, or the B-LCSE. The latter may occur as the result of a protocol error.

e)	The CAUSE parameter indicates the reason as to why the peer B-LCSE user rejected a request to establish a logical channel. The CAUSE parameter is not present when the SOURCE parameter indicates "B-LCSE".

f)	The ERRCODE parameter indicates the type of B-LCSE error. Table C.37 shows the allowed values of the ERRCODE parameter.

#### C.5.2.4	B-LCSE states

The following states are used to specify the allowed sequence of primitives between the B-LCSE and the B-LCSE user, and the exchange of messages between peer B-LCSEs. The states are specified separately for each of an outgoing B-LCSE and an incoming B-LCSE. The states for an outgoing B-LCSE are:

State 0:  RELEASED

The logical channel is released. The logical channel shall not be used to send or receive data.

State 1:  AWAITING ESTABLISHMENT

The outgoing B-LCSE is waiting to establish a logical channel with a peer incoming B-LCSE. The logical channel shall not be used to send or receive data.

State 2:  ESTABLISHED

The B-LCSE peer-to-peer logical channel connection has been established. The logical channel may be used to send and receive data.

State 3:  AWAITING RELEASE

The outgoing B-LCSE is waiting to release a logical channel with the peer incoming B-LCSE. The logical channel shall not be used to send data, but data may continue to be received.

The states for an incoming B-LCSE are:

State 0:  RELEASED

The logical channel is released. The logical channel shall not be used to receive or send data.

State 1:  AWAITING ESTABLISHMENT

The incoming B-LCSE is waiting to establish a logical channel with a peer outgoing B-LCSE. The logical channel shall not be used to receive or send data.

State 2:  AWAITING CONFIRMATION

The incoming B-LCSE is awaiting confirmation that the logical channel is established with a peer outgoing B-LCSE. The logical channel shall not be used to send data, but data may be received.

State 3:  ESTABLISHED

An B-LCSE peer-to-peer logical channel connection has been established. The logical channel may be used to receive and send data.

#### C.5.2.5	State transition diagram

The allowed sequence of primitives between the B-LCSE and the B-LCSE user is defined here. The allowed sequence of primitives relates to states of the B-LCSE as viewed from the B-LCSE user. The allowed sequences are specified separately for each of an outgoing B-LCSE and an incoming B-LCSE, as shown in Figure C.29 and Figure C.30 respectively.

FIGURE C.31/H.245

State transition diagram for sequence of primitives at outgoing B-LCSE

µ §

FIGURE C.32/H.245

State transition diagram for sequence of primitives at incoming B-LCSE

### C.5.3	Peer to peer B-LCSE communication

#### C.5.3.1	B-LCSE messages

Table C.38 shows the B-LCSE messages and fields, defined in Annex C, which are relevant to the B-LCSE protocol.

TABLE C.39/H.245

B-LCSE message names and fields

| function | message | direction | field |
| --- | --- | --- | --- |
| establishment | OpenLogicalChannel | O -> I 1 | forwardLogicalChannelNumber |
|  |  |  | forwardLogicalChannelParameters |
|  |  |  | reverseLogicalChannelParameters |
|  | OpenLogicalChannelAck | O <- I | forwardLogicalChannelNumber |
|  |  |  | reverseLogicalChannelParameters |
|  | OpenLogicalChannelReject | O <- I | forwardLogicalChannelNumber |
|  |  |  | cause |
|  | OpenLogicalChannelConfirm | O->I | forwardLogicalChannelNumber |
| release | CloseLogicalChannel | O -> I | forwardLogicalChannelNumber |
|  |  |  | source |
|  | CloseLogicalChannelAck | O <- I | forwardLogicalChannelNumber |

Notes:

1.	Direction:  O - outgoing, I - incoming.

#### C.5.3.2	B-LCSE state variables

The following state variable is defined at the outgoing B-LCSE:

out_LCN

This state variable distinguishes between outgoing B-LCSEs. It is initialised at outgoing B-LCSE initialization. The value of out_LCN is used to set the forwardLogicalChannelNumber field of B-LCSE messages sent from an outgoing B-LCSE. For B-LCSE messages received at an outgoing B-LCSE, the message forwardLogicalChannelNumber field value is identical to the value of out_LCN.

The following state variable is defined at the incoming B-LCSE:

in_LCN

This state variable distinguishes between incoming B-LCSEs. It is initialised at incoming B-LCSE initialization. The value of in_LCN is used to set the forwardLogicalChannelNumber field of B-LCSE messages sent from an incoming B-LCSE. For B-LCSE messages received at an incoming B-LCSE, the message forwardLogicalChannelNumber field value is identical to the value of in_LCN.

#### C.5.3.3	B-LCSE timers

The following timer is specified for the outgoing and incoming B-LCSE:

T103

At the outgoing B-LCSE this timer is used during the AWAITING ESTABLISHMENT and AWAITING RELEASE states. It specifies the maximum time during which no OpenLogicalChannelAck, OpenLogicalChannelReject or CloseLogicalChannelAck message may be received.

At the incoming B-LCSE, this timer is used during the AWAITING CONFIRMATION state. It specifies the maximum time during which no OpenLogicalChannelConfirm message may be received.

### C.5.4	B-LCSE procedures

#### C.5.4.1	Introduction

Figure C.33 summarizes the primitives and their parameters, and the messages, for each of the outgoing and incoming B-LCSE.

µ §

FIGURE C.34/H.245

Primitives and messages in the Bi-directional Logical Channel Signalling Entity

#### C.5.4.2	Primitive parameter default values

Where not explicitly stated in the SDL diagrams the parameters of the indication and confirm primitives assume values as shown in Table C.40.

TABLE C.41/H.245

Default primitive parameter values

| primitive | parameter | default value 1 |
| --- | --- | --- |
| ESTABLISH.indication | FORWARD_PARAM | OpenLogicalChannel.forwardLogicalChannelParameters |
|  | REVERSE_PARAM | OpenLogicalChannel.reverseLogicalChannelParameters |
| ESTABLISH.confirm | REVERSE_DATA | OpenLogicalChannelAck.reverseLogicalChannelParameters |
| RELEASE.indication | SOURCE | CloseLogicalChannel.source |
|  | CAUSE | null |

Notes:

1.	A primitive parameter shall be coded as null, if an indicated message field is not present in the message.

#### C.5.4.3	Message field default values

Where not explicitly stated in the SDL diagrams the message fields assume values as shown in Table C.42.

TABLE C.43/H.245

Default message field values

| message | field | default value 1 |
| --- | --- | --- |
| OpenLogicalChannel | forwardLogicalChannelNumber | out_LCN |
|  | forwardLogicalChannelParameters | ESTABLISH.request(FORWARD_PARAM) |
|  | reverseLogicalChannelParameters | ESTABLISH.request(REVERSE_PARAM) |
| OpenLogicalChannelAck | forwardLogicalChannelNumber | in_LCN |
|  | reverseLogicalChannelParameters | ESTABLISH.response(REVERSE_DATA) |
| OpenLogicalChannelReject | forwardLogicalChannelNumber | in_LCN |
|  | cause | RELEASE.request(CAUSE) |
| OpenLogicalChannelConfirm | forwardLogicalChannelNumber | out_LCN |
| CloseLogicalChannel | forwardLogicalChannelNumber | out_LCN |
|  | source | user |
| CloseLogicalChannelAck | forwardLogicalChannelNumber | in_LCN |

Notes:

1.	A message field shall not be coded, if the corresponding primitive parameter is null i.e. not present.

#### C.5.4.4	ERRCODE parameter values

The ERRCODE parameter of the ERROR.indication primitive indicates a particular error condition. Table C.44 shows the values that the ERRCODE parameter may take at the outgoing B-LCSE and Table C.45 shows the values that the ERRCODE parameter may take at the incoming B-LCSE.

TABLE C.46/H.245

ERRCODE parameter values at outgoing B-LCSE

| error type | error code | error condition | state |
| --- | --- | --- | --- |
| inappropriate message | A | OpenLogicalChannelAck | RELEASED |
|  | B | OpenLogicalChannelReject | RELEASED ESTABLISHED |
|  | C | CloseLogicalChannelAck | ESTABLISHED |
| no response from peer B-LCSE | D | timer T103 expiry | AWAITING ESTABLISHMENT AWAITING RELEASE |

TABLE C.47/H.245

ERRCODE parameter values at incoming B-LCSE

| error type | error code | error condition | state |
| --- | --- | --- | --- |
| inappropriate message | E | OpenLogicalChannelConfirm | AWAITING ESTABLISHMENT |
| no response from peer B-LCSE | F | timer T103 expiry | AWAITING CONFIRMATION |

#### C.5.4.5	SDLs

The outgoing B-LCSE and the incoming B-LCSE procedures are expressed in SDL form in Figure C.35 and Figure C.36 respectively.

µ §

FIGURE C.37(i)/H.245

Outgoing B-LCSE SDL

µ §

FIGURE C.37(ii)/H.245

Outgoing B-LCSE SDL (continued)

µ §

FIGURE C.37(iii)/H.245

Outgoing B-LCSE SDL (continued)

µ §

FIGURE C.37(iv)/H.245

Outgoing B-LCSE SDL (concluded)

µ §

FIGURE C.38(i)/H.245

Incoming B-LCSE SDL

µ §

FIGURE C.38(ii)/H.245

Incoming B-LCSE SDL (continued)

µ §

FIGURE C.38(iii)/H.245

Incoming B-LCSE SDL (continued)

µ §

FIGURE C.38(iv)/H.245

Incoming B-LCSE SDL (concluded)

## C.6	Close Logical Channel procedures

### C.6.1	Introduction

These procedures are used by a terminal to request the remote terminal to close a logical channel. Note that these are only close request procedures; the actual logical channel close occurs using the LCSE and B-LCSE procedures. The procedures are referred to here as the Close Logical Channel Signalling Entity (CLCSE). Procedures are specified in terms of primitives and states at the interface between the CLCSE and the CLCSE user. Protocol information is transferred to the peer CLCSE via relevant messages defined in Annex C. There is an outgoing CLCSE and an incoming CLCSE. At each of the outgoing and incoming ends there is one instance of the CLCSE for each logical channel.

If a terminal is incapable of processing the incoming signals, it may use these procedures to request the closing of the relevant logical channels.

A terminal that answers such a response positively, that is, by issuing the CLOSE.response primitive, shall initiate the closing of the logical channel by sending the RELEASE.request primitive to the appropriate LCSE or B-LCSE as soon as possible.

The following text provides an overview of the operation of the protocol. In the case of any discrepancy with the formal specification of the protocol that follows, the formal specification will supersede.

#### C.6.1.1	Protocol overview - outgoing CLCSE

A close logical channel request procedure is initiated when the CLOSE.request primitive is issued by the user at the outgoing CLCSE. A RequestChannelClose message is sent to the peer incoming CLCSE, and timer T108 is started. If a RequestChannelCloseAck message is received in response to the RequestChannelClose message then timer T108 is stopped and the user is informed with the CLOSE.confirm primitive that the close logical channel request procedure was successful. If however a RequestChannelCloseReject message is received in response to the RequestChannelClose message then timer T108 is stopped and the user is informed with the REJECT.indication primitive that the peer CLCSE user has refused to close the logical channel.

If timer T108 expires then the outgoing CLCSE user is informed with the REJECT.indication primitive and a RequestChannelCloseRelease message is sent.

#### C.6.1.2	Protocol overview - incoming CLCSE

When a RequestChannelClose message is received at the incoming CLCSE, the user is informed of the close logical channel request with the CLOSE.indication primitive. The incoming CLCSE user signals acceptance of the close logical channel request by issuing the CLOSE.response primitive, and a RequestChannelCloseAck message is sent to the peer outgoing CLCSE. The incoming CLCSE user signals rejection of the close logical channel request by issuing the REJECT.request primitive, and a RequestChannelCloseReject message is sent to the peer outgoing CLCSE.

### C.6.2	Communication between CLCSE and CLCSE user

#### C.6.2.1	Primitives between CLCSE and CLCSE user

Communication between the CLCSE and CLCSE user, is performed using the primitives shown in Table C.48.

TABLE C.49/H.245

Primitives and parameters

|  | type |  |  |  |
| --- | --- | --- | --- | --- |
| generic name | request | indication | response | confirm |
| CLOSE | - 1 | - | - | - |
| REJECT | CAUSE | SOURCE CAUSE | not defined 2 | not defined |

Notes:

1.	"-" means no parameters.

2.	“not defined” means that this primitive is not defined.

#### C.6.2.2	Primitive definition

The definition of these primitives is as follows:

a)	The CLOSE primitives are used to request closure of a logical channel.

b)	The REJECT primitives are used to reject the closing of a logical channel.

#### C.6.2.3	Parameter definition

The definition of the primitive parameters shown in Table C.50 are as follows:

a)	The SOURCE parameter indicates the source of the REJECT.indication primitive. The SOURCE parameter has the value of "USER" or "PROTOCOL". The latter case may occur as the result of a timer expiry.

b)	The CAUSE parameter indicates the reason for refusal to close a logical channel. The CAUSE parameter is not present when the SOURCE parameter indicates "PROTOCOL".

#### C.6.2.4	CLCSE states

The following states are used to specify the allowed sequence of primitives between the CLCSE and the CLCSE user.

The states for an outgoing CLCSE are:

State 0:  IDLE

The CLCSE is idle.

State 1:  AWAITING RESPONSE

The CLCSE is waiting for a response from the remote CLCSE.

The states for an incoming CLCSE are:

State 0:  IDLE

The CLCSE is idle.

State 1:  AWAITING RESPONSE

The CLCSE is waiting for a response from the CLCSE user.

#### C.6.2.5	State transition diagram

The allowed sequence of primitives between the CLCSE and the CLCSE user is defined here. The allowed sequences are specified separately for each of an outgoing CLCSE and an incoming CLCSE, as shown in Figure C.39 and Figure C.40 respectively.

FIGURE C.41/H.245

State transition diagram for sequence of primitives at CLCSE outgoing

µ §

FIGURE C.42/H.245

State transition diagram for sequence of primitives at CLCSE incoming

### C.6.3	Peer to peer CLCSE communication

#### C.6.3.1	Messages

Table C.51 shows the CLCSE messages and fields, defined in Annex C, which are relevant to the CLCSE protocol.

TABLE C.52/H.245

CLCSE message names and fields

| function | message | direction | field |
| --- | --- | --- | --- |
| transfer | RequestChannelClose | O -> I 1 | forwardLogicalChannelNumber |
|  | RequestChannelCloseAck | O <- I | forwardLogicalChannelNumber |
|  | RequestChannelCloseReject | O <- I | forwardLogicalChannelNumber |
| reset | RequestChannelCloseRelease | O -> I | forwardLogicalChannelNumber |

Notes:

1.	Direction:  O - outgoing, I - incoming.

#### C.6.3.2	CLCSE state variables

The following state variable is defined at the outgoing CLCSE:

out_LCN

This state variable distinguishes between outgoing CLCSEs. It is initialised at outgoing CLCSE initialization. The value of out_LCN is used to set the forwardLogicalChannelNumber field of CLCSE messages sent from an outgoing CLCSE. For CLCSE messages received at an outgoing CLCSE, the message forwardLogicalChannelNumber field value is identical to the value of out_LCN.

The following state variable is defined at the incoming CLCSE:

in_LCN

This state variable distinguishes between incoming CLCSEs. It is initialised at incoming CLCSE initialization. The value of in_LCN is used to set the forwardLogicalChannelNumber field of CLCSE messages sent from an incoming CLCSE. For CLCSE messages received at an incoming CLCSE, the message forwardLogicalChannelNumber field value is identical to the value of in_LCN.

#### C.6.3.3	CLCSE timers

The following timer is specified for the outgoing CLCSE:

T108

This timer is used during the AWAITING RESPONSE state. It specifies the maximum time during which no RequestChannelCloseAck or RequestChannelCloseReject message may be received.

### C.6.4	CLCSE procedures

Figure C.43 summarizes the CLCSE primitives and their parameters, and messages, for each of the outgoing and incoming CLCSE.

µ §

FIGURE C.44/H.245

Primitives and messages in the Close Logical Channel Signalling Entity

#### C.6.4.1	Primitive parameter default values

Where not explicitly stated in the SDL diagrams the parameters of the indication and confirm primitives assume values as shown in Table C.53.

TABLE C.54/H.245

Default primitive parameter values

| primitive | parameter | default value |
| --- | --- | --- |
| REJECT.indication | SOURCE | USER |
|  | CAUSE | null |

#### C.6.4.2	Message field default values

Where not explicitly stated in the SDL diagrams the message fields assume values as shown in Table C.55.

TABLE C.56/H.245

Default message field values

| message | field | default value |
| --- | --- | --- |
| RequestChannelClose | forwardLogicalChannelNumber | out_LCN |
| RequestChannelCloseAck | forwardLogicalChannelNumber | in_LCN |
| RequestChannelCloseReject | forwardLogicalChannelNumber | in_LCN |
|  | cause | REJECT.request(CAUSE) |
| RequestChannelCloseRelease | forwardLogicalChannelNumber | out_LCN |

#### C.6.4.3	SDLs

The outgoing CLCSE and the incoming CLCSE procedures are expressed in SDL form in Figure C.45 and Figure C.46 respectively.

FIGURE C.47(i)/H.245

Outgoing CLCSE SDL

FIGURE C.47(ii)/H.245

Outgoing CLCSE SDL (concluded)

FIGURE C.48(i)/H.245

Incoming CLCSE SDL

FIGURE C.48(ii)/H.245

Incoming CLCSE SDL (concluded)

## C.7	H.223 Multiplex Table Procedures

### C.7.1	Introduction

The multiplex table serves to associate each octet within a H.223 MUX-PDU [8] with a particular logical channel number. The H.223 multiplex table may have up to 16 entries, numbered from 0 to 15. Table entries 1 to 15 shall be sent from transmitters to receivers as specified in the following procedures.

The procedures described here are referred to as the Multiplex Table Signalling Entity (MTSE). Procedures are specified in terms of primitives and states at the interface between the MTSE and the MTSE user. Protocol information is transferred to the peer MTSE via relevant messages defined in Annex C.

There is an outgoing MTSE and an incoming MTSE. There is one instance of the MTSE for each multiplex table entry.

A transmit terminal uses this protocol to signal to a remote terminal one or more new multiplex table entries. The remote terminal may accept or reject the new multiplex table entries. If the remote terminal accepts a multiplex table entry, the previous entry at the given entry number is replaced with the new entry.

The transmitter may deactivate a multiplex table entry by sending a MultiplexEntryDescriptor with no elementList. The transmitter shall at no time use a multiplex table entry that is deactivated. Before transmitting a MultiplexEntrySend, the transmitter shall stop using the entries that are described by it. It shall not restart using those entries until it has received a MultiplexEntrySendAck. This procedure is used because if the use of these multiplex table entries is not stopped before sending the MultiplexEntrySend, errors may cause an ambiguity in the receiver.

The transmitter shall stop using deactivated entries before sending the MultiplexEntrySend indicating that they have been deactivated. Deactivated entries may be used again at any time by transmitting a MultiplexEntrySend message for activating that entry. Deactivating entries that are no longer required by the transmitter may increase the probability of detecting errors in the H.223 Multiplex Code field.

NOTE - While some multiplex table entries are being updated, other (active) entries may continue to be used. Also, a multiplex table entry may be deleted in the same MultiplexEntrySend that is used to modify other multiplex table entries.

At the start of communication, unless specified otherwise in an appropriate Recommendation, only table entry 0 is available for transmission, and table entries 1 to 15 are deactivated.

A Request Multiplex Entry procedure may be used at any time to elicit retransmission of specified multiplex table entries from the remote terminal, for example, following an interruption or other cause for uncertainty.

The following text provides an overview of the operation of the protocol. In the case of any discrepancy with the formal specification of the protocol that follows, the formal specification will supersede.

#### C.7.1.1	Protocol overview - outgoing MTSE

A multiplex table entry send request procedure is initiated when the TRANSFER.request primitive is issued by the user at the outgoing MTSE. A MultiplexEntrySend message is sent to the peer incoming MTSE, and timer T104 is started. If a MultiplexEntrySendAck message is received in response to the MultiplexEntrySend message then timer T104 is stopped and the user is informed with the TRANSFER.confirm primitive that the multiplex table entry send request was successful. If however a MultiplexEntrySendReject message is received in response to the MultiplexEntrySend message then timer T104 is stopped and the user is informed with the REJECT.indication primitive that the peer MTSE user has refused to accept the multiplex table entry.

If timer T104 expires then the outgoing MTSE user is informed with the REJECT.indication primitive and a MultiplexEntrySendRelease message is sent.

Only MultiplexEntrySendAck and MultiplexEntrySendReject messages which are in response to the most recent MultiplexEntrySend message are accepted. Responses to earlier MultiplexEntrySend messages are ignored.

A new multiplex table entry send request procedure may be initiated with the TRANSFER.request primitive by the user at the outgoing MTSE before a MultiplexEntrySendAck or a MultiplexEntrySendReject message has been received.

#### C.7.1.2	Protocol overview - incoming MTSE

When a MultiplexEntrySend message is received at the incoming MTSE, the user is informed of the multiplex table entry send request with the TRANSFER.indication primitive. The incoming MTSE user signals acceptance of the multiplex table entry by issuing the TRANSFER.response primitive, and a MultiplexEntrySendAck message is sent to the peer outgoing MTSE. The incoming MTSE user signals rejection of the multiplex table entry by issuing the REJECT.request primitive, and a MultiplexEntrySendReject message is sent to the peer outgoing MTSE.

A new MultiplexEntrySend message may be received before the incoming MTSE user has responded to an earlier MultiplexEntrySend message. The incoming MTSE user is informed with the REJECT.indication primitive, followed by the TRANSFER.indication primitive, and the incoming MTSE user responds to the new multiplex table entry.

If a MultiplexEntrySendRelease message is received before the incoming MTSE user has responded to an earlier MultiplexEntrySend message, then the incoming MTSE user is informed with the REJECT.indication, and the earlier multiplex table entry is discarded.

### C.7.2	Communication between the MTSE and MTSE user

#### C.7.2.1	Primitives between MTSE and MTSE user

Communication between the MTSE, and MTSE user, is performed using the primitives shown in Table C.57.

TABLE C.58/H.245

Primitives and parameters

|  | type |  |  |  |
| --- | --- | --- | --- | --- |
| generic name | request | indication | response | confirm |
| TRANSFER | MUX-DESCRIPTOR | MUX-DESCRIPTOR | - 1 | - |
| REJECT | CAUSE | SOURCE CAUSE | not defined 2 | not defined |

Notes:

1.	“-” means no parameters

2.	“not defined” means that this primitive is not defined.

#### C.7.2.2	Primitive definition

The definition of these primitives is as follows:

a)	The TRANSFER primitives are used to transfer multiplex table entries.

b)	The REJECT primitives are used to reject a multiplex table entry, and to terminate a multiplex table entry transfer.

#### C.7.2.3	Parameter definition

The definition of the primitive parameters shown in Table C.59 are as follows:

a)	The MUX-DESCRIPTOR parameter is a multiplex table entry. This parameter is mapped to the MultiplexEntryDescriptor field of the multiplexEntrySend message and carried transparently from the MTSE user at the outgoing MTSE to the MTSE user at the incoming MTSE. There may be multiple MUX-DESCRIPTORs associated with the TRANSFER primitive.

b)	The SOURCE parameter indicates the source of the REJECT.indication primitive. The SOURCE parameter has the value of "USER" or "PROTOCOL". The latter case may occur as the result of a timer expiry.

c)	The CAUSE parameter indicates the reason for rejection of a multiplex table entry. The CAUSE parameter is not present when the SOURCE parameter indicates "PROTOCOL".

#### C.7.2.4	MTSE states

The following states are used to specify the allowed sequence of primitives between the MTSE and the MTSE user. The states are specified separately for each of an outgoing MTSE and an incoming MTSE. The states for an outgoing MTSE are:

State 0:  IDLE

There is no MTSE transfer is in progress. The multiplex table entry may be used by the transmitter.

State 1:  AWAITING RESPONSE

The MTSE user has requested the transfer of a multiplex table entry, and a response from the peer MTSE is awaited. The multiplex table entry shall not be used by the transmitter.

The states for an incoming MTSE are:

State 0:  IDLE

There is no MTSE transfer in progress. The multiplex table entry may be in use by the transmitter.

State 1:  AWAITING RESPONSE

The peer MTSE has transferred a multiplex table entry, and a response from the MTSE user is awaited. The multiplex table entry may not be in use by the transmitter.

#### C.7.2.5	State transition diagram

The allowed sequence of primitives between the MTSE and the MTSE user is defined here. The allowed sequences are specified separately for each of an outgoing MTSE and an incoming MTSE, as shown in Figure C.49 and Figure C.50 respectively.

FIGURE C.51/H.245

State transition diagram for sequence of primitives at outgoing MTSE

FIGURE C.52/H.245

State transition diagram for sequence of primitives at incoming MTSE

### C.7.3	Peer to peer MTSE communication

#### C.7.3.1	Messages

Table C.60 shows the MTSE messages and fields, defined in Annex C, which are relevant to the MTSE protocol.

TABLE C.61/H.245

MTSE message names and fields

| function | message | direction | field |
| --- | --- | --- | --- |
| transfer | MultiplexEntrySend | O -> I 1 | sequenceNumber |
|  |  |  | multiplexEntryDescriptors.multiplexTableEntryNumber |
|  |  |  | multiplexEntryDescriptors.elementList |
|  | MultiplexEntrySendAck | O <- I | sequenceNumber |
|  |  |  | multiplexTableEntryNumber |
| reject | MultiplexEntrySendReject | O <- I | sequenceNumber |
|  |  |  | multiplexTableEntryNumber |
|  |  |  | rejectionDescriptions.cause |
| reset | MultiplexEntrySendRelease | O -> I | multiplexTableEntryNumber |

Notes:

1.	Direction:  O - outgoing, I - incoming.

#### C.7.3.2	MTSE state variables

The following state variables are defined at the outgoing MTSE:

out_ENUM

This state variable distinguishes between outgoing MTSEs. It is initialised at outgoing MTSE initialization. The value of out_ENUM is used to set the multiplexTableEntryNumber field of MTSE messages sent from an outgoing MTSE. For MTSE messages received at an outgoing MTSE, the message multiplexTableEntryNumber field value is identical to the value of out_ENUM.

out_SQ

This state variable is used to indicate the most recently sent MultiplexEntrySend message. It is incremented by one and mapped to the MultiplexEntrySend message sequenceNumber field before transmission of a MultiplexEntrySend message. Arithmetic performed on out_SQ is modulo 256.

The following state variables are defined at the incoming MTSE:

in_ENUM

This state variable distinguishes between incoming MTSEs. It is initialised at incoming MTSE initialization. The value of in_ENUM is used to set the multiplexTableEntryNumber field of MTSE messages sent from an incoming MTSE. For MTSE messages received at an incoming MTSE, the message multiplexTableEntryNumber field value is identical to the value of in_ENUM.

in_SQ

This state variable is used to store the value of the sequenceNumber field of the most recently received MultiplexEntrySend message. The MultiplexEntrySendAck and MultiplexEntrySendReject messages have their sequenceNumber fields set to the value of in_SQ, before being sent to the peer MTSE.

#### C.7.3.3	MTSE timers

The following timer is specified for the outgoing MTSE:

T104

This timer is used during the AWAITING RESPONSE state. It specifies the maximum time during which no MultiplexEntrySendAck or MultiplexEntrySendReject message may be received.

### C.7.4	MTSE procedures

#### C.7.4.1	Introduction

Figure C.53 summarizes the primitives and their parameters, and the messages and relevant fields, for each of the outgoing and incoming MTSE.

µ §

FIGURE C.54/H.245

Primitives and messages in the Multiplex Table Signalling Entity

#### C.7.4.2	Primitive parameter default values

Where not explicitly stated in the SDL diagrams the parameters of the indication and confirm primitives assume values as shown in Table C.62.

TABLE C.63/H.245

Default primitive parameter values

| primitive | parameter | default value |
| --- | --- | --- |
| TRANSFER.indication | MUX-DESCRIPTOR | MultiplexEntrySend.multiplexEntryDescriptors.elementList |
| REJECT.indication | SOURCE | USER |
|  | CAUSE | null |

#### C.7.4.3	Message field default values

Where not explicitly stated in the SDL diagrams the message fields assume values as shown in Table C.64.

TABLE C.65/H.245

Default message field values

| message | field | default value 1 |
| --- | --- | --- |
| MultiplexEntrySend | sequenceNumber | out_SQ |
|  | multiplexEntryDescriptors.multiplexTableEntryNumber | out_ENUM |
|  | multiplexEntryDescriptors.elementList | TRANSFER.request(MUX-DESCRIPTOR) |
| MultiplexEntrySendAck | sequenceNumber | in_SQ |
|  | multiplexTableEntryNumber | in_ENUM |
| MultiplexEntrySendReject | sequenceNumber | in_SQ |
|  | rejectionDescriptions.multiplexTableEntryNumber | in_ENUM |
|  | rejectionDescriptions.cause | REJECT.request(CAUSE) |
| MultiplexEntrySendRelease | multiplexTableEntryNumber | out_ENUM |

Notes:

1.	A message field shall not be coded, if the corresponding primitive parameter is null i.e. not present.

#### C.7.4.4	SDLs

The outgoing MTSE and the incoming MTSE procedures are expressed in SDL form in Figure C.55 and Figure C.56 respectively.

FIGURE C.57(i)/H.245

Outgoing MTSE SDL

FIGURE C.57(ii)/H.245

Outgoing MTSE SDL (continued)

FIGURE C.57(iii)/H.245

Outgoing MTSE SDL (concluded)

FIGURE C.58(i)/H.245

Incoming MTSE SDL

FIGURE C.58(ii)/H.245

Incoming MTSE SDL (concluded)

## C.8	Request Multiplex Entry procedures

### C.8.1	Introduction

These procedures are used by a terminal to request the retransmission of one or more MultiplexEntryDescriptors. The procedures are referred to here as the Request Multiplex Entry Signalling Entity (RMESE). Procedures are specified in terms of primitives and states at the interface between the RMESE and the RMESE user. Protocol information is transferred to the peer RMESE via relevant messages defined in Annex C. There is an outgoing RMESE and an incoming RMESE. There is one instance of the RMESE for each multiplex table entry.

A terminal that answers such a response positively, that is, by issuing the SEND.response primitive, shall initiate the Multiplex Table procedures to send the multiplex table entry as soon as possible.

The following text provides an overview of the operation of the protocol. In the case of any discrepancy with the formal specification of the protocol that follows, the formal specification will supersede.

NOTE - This protocol has been defined so that there is an independent RMESE for each multiplex table entry, and the syntax has been defined to allow a single message to carry information relating to one or more multiplex table entries. The way that messages are constructed is an implementation decision: for example, a terminal may respond to a RequestMultiplexEntry message requesting three entries to be sent with one, two or three response messages.

#### C.8.1.1	Protocol overview - outgoing RMESE

A request multiplex entry procedure is initiated when the SEND.request primitive is issued by the user at the outgoing RMESE. A RequestMultiplexEntry message is sent to the peer incoming RMESE, and timer T107 is started. If a RequestMultiplexEntryAck message is received in response to the RequestMultiplexEntry message then timer T107 is stopped and the user is informed with the SEND.confirm primitive that the request multiplex entry procedure was successful. If however a RequestMultiplexEntryReject message is received in response to the RequestMultiplexEntry message then timer T107 is stopped and the user is informed with the REJECT.indication primitive that the peer RMESE user has refused to send the multiplex entry.

If timer T107 expires then the outgoing RMESE user is informed with the REJECT.indication primitive and a RequestMultiplexEntryRelease message is sent.

#### C.8.1.2	Protocol overview - incoming RMESE

When a RequestMultiplexEntry message is received at the incoming RMESE, the user is informed of the multiplex entry request with the SEND.indication primitive. The incoming RMESE user signals acceptance of the multiplex entry request by issuing the SEND.response primitive, and a RequestMultiplexEntryAck message is sent to the peer outgoing RMESE. The incoming RMESE user signals rejection of the multiplex entry request by issuing the REJECT.request primitive, and a RequestMultiplexEntryReject message is sent to the peer outgoing RMESE.

### C.8.2	Communication between RMESE and RMESE user

#### C.8.2.1	Primitives between RMESE and RMESE user

Communication between the RMESE and RMESE user, is performed using the primitives shown in Table C.66.

TABLE C.67/H.245

Primitives and parameters

|  | type |  |  |  |
| --- | --- | --- | --- | --- |
| generic name | request | indication | response | confirm |
| SEND | - 1 | - | - | - |
| REJECT | CAUSE | SOURCE CAUSE | not defined 2 | not defined |

Notes:

1.	"-" means no parameters.

2.	“not defined” means that this primitive is not defined.

#### C.8.2.2	Primitive definition

The definition of these primitives is as follows:

a)	The SEND primitives are used to request the transmission of a multiplex entry.

b)	The REJECT primitives are used to reject the request for transmission of a multiplex entry.

#### C.8.2.3	Parameter definition

The definition of the primitive parameters shown in Table C.68 are as follows:

a)	The SOURCE parameter indicates the source of the REJECT.indication primitive. The SOURCE parameter has the value of "USER" or "PROTOCOL". The latter case may occur as the result of a timer expiry.

b)	The CAUSE parameter indicates the reason for refusal to send a multiplex table entry. The CAUSE parameter is not present when the SOURCE parameter indicates "PROTOCOL".

#### C.8.2.4	RMESE states

The following states are used to specify the allowed sequence of primitives between the RMESE and the RMESE user.

The states for an outgoing RMESE are:

State 0:  IDLE

The RMESE is idle.

State 1:  AWAITING RESPONSE

The RMESE is waiting for a response from the remote RMESE.

The states for an incoming RMESE are:

State 0:  IDLE

The RMESE is idle.

State 1:  AWAITING RESPONSE

The RMESE is waiting for a response from the RMESE user.

#### C.8.2.5	State transition diagram

The allowed sequence of primitives between the RMESE and the RMESE user is defined here. The allowed sequences are specified separately for each of an outgoing RMESE and an incoming RMESE, as shown in Figure C.59 and Figure C.60 respectively.

µ §

FIGURE C.61/H.245

State transition diagram for sequence of primitives at RMESE outgoing

µ §

FIGURE C.62/H.245

State transition diagram for sequence of primitives at RMESE incoming

### C.8.3	Peer to peer RMESE communication

#### C.8.3.1	Messages

Table C.69 shows the RMESE messages and fields, defined in Annex C, which are relevant to the RMESE protocol.

TABLE C.70/H.245

RMESE message names and fields

| function | message | direction | field |
| --- | --- | --- | --- |
| transfer | RequestMultiplexEntry | O -> I 1 | multiplexTableEntryNumber |
|  | RequestMultiplexEntryAck | O <- I | multiplexTableEntryNumber |
|  | RequestMultiplexEntryReject | O <- I | multiplexTableEntryNumber |
|  |  |  | rejectionDescriptions.cause |
| reset | RequestMultiplexEntryRelease | O -> I |  |

Notes:

1.	Direction:  O - outgoing, I - incoming.

#### C.8.3.2	RMESE state variables

The following state variable is defined at the outgoing RMESE:

out_ENUM

This state variable distinguishes between outgoing RMESEs. It is initialised at outgoing RMESE initialization. The value of out_ENUM is used to set the multiplexTableEntryNumber field of RMESE messages sent from an outgoing RMESE. For RMESE messages received at an outgoing RMESE, the message multiplexTableEntryNumber field value is identical to the value of out_ENUM.

The following state variable is defined at the incoming RMESE:

in_ENUM

This state variable distinguishes between incoming RMESEs. It is initialised at incoming RMESE initialization. The value of in_ENUM is used to set the multiplexTableEntryNumber field of RMESE messages sent from an incoming RMESE. For RMESE messages received at an incoming RMESE, the message multiplexTableEntryNumber field value is identical to the value of in_ENUM.

#### C.8.3.3	RMESE timers

The following timer is specified for the outgoing RMESE:

T107

This timer is used during the AWAITING RESPONSE state. It specifies the maximum time during which no RequestMultiplexEntryAck or RequestMultiplexEntryReject message may be received.

### C.8.4	RMESE procedures

Figure C.63 summarizes the RMESE primitives and their parameters, and messages, for each of the outgoing and incoming RMESE.

µ §

FIGURE C.64/H.245

Primitives and messages in the Request Multiplex Entry Signalling Entity

#### C.8.4.1	Primitive parameter default values

Where not explicitly stated in the SDL diagrams the parameters of the indication and confirm primitives assume values as shown in Table C.71.

TABLE C.72/H.245

Default primitive parameter values

| primitive | parameter | default value |
| --- | --- | --- |
| REJECT.indication | SOURCE | USER |
|  | CAUSE | null |

#### C.8.4.2	Message field default values

Where not explicitly stated in the SDL diagrams the message fields assume values as shown in Table C.73.

TABLE C.74/H.245

Default message field values

| message | field | default value |
| --- | --- | --- |
| RequestMultiplexEntry | multiplexTableEntryNumber | out_ENUM |
| RequestMultiplexEntryAck | multiplexTableEntryNumber | in_ENUM |
| RequestMultiplexEntryReject | multiplexTableEntryNumber | in_ENUM |
|  | cause | REJECT.request(CAUSE) |
| RequestMultiplexEntryRelease | multiplexTableEntryNumber | out_ENUM |

#### C.8.4.3	SDLs

The outgoing RMESE and the incoming RMESE procedures are expressed in SDL form in Figure C.65 and Figure C.66 respectively.

µ §

FIGURE C.67(i)/H.245

Outgoing RMESE SDL

µ §

FIGURE C.67(ii)/H.245

Outgoing RMESE SDL

µ §

FIGURE C.68(i)/H.245

Incoming RMESE SDL

µ §

FIGURE C.68(ii)/H.245

Incoming RMESE SDL (concluded)

## C.9	Mode Request procedures

### C.9.1	Introduction

The procedures described here allow a terminal to request a remote terminal to use a particular mode of operation in its transmit direction. The procedures are referred to here as the Mode Request Signalling Entity (MRSE). Procedures are specified in terms of primitives and states at the interface between the MRSE and the MRSE user. Protocol information is transferred to the peer MRSE via relevant messages defined in Annex C. There is an outgoing MRSE and an incoming MRSE. At each of the outgoing and incoming ends there is one instance of the MRSE per call.

A terminal that answers such a response positively, that is, by issuing the TRANSFER.response primitive, shall initiate the logical channel signalling procedures to establish the appropriate mode of transmission as soon as possible.

If the currently valid capabilities received from the remote terminal contain one or more transmission capabilities, a terminal may select a mode that it prefers to have transmitted to it by performing the Mode Request procedures. A terminal whose currently valid capabilities contain one or more transmission capabilities and which is in receipt of such a request, should comply with the request.

A mode request shall not be sent to a terminal whose currently valid capabilities contain no transmission capabilities, that is, the terminal does not wish to, and shall not, be remotely controlled. If such a terminal does however receive a mode request, it may comply.

A terminal that receives multipointModeCommand shall comply with all received mode requests, until the command is cancelled by receipt of cancelMultipointModeCommand. A mode request may be sent to a terminal whose currently valid capabilities contain no transmission capabilities when multipointModeCommand has previously been sent.

The requested mode may include channels which are already open. For example, if a channel for G.723.1 was currently open and a terminal wished to receive an additional G.728 channel, it would send a mode request containing both the G.723.1 and the G.728 channel. If the G.723.1 channel request were absent, this would indicate that G.723.1 was no longer desired.

NOTE - The request mode description specifies a complete mode. If, for example, video is currently being transmitted and a mode request is received that does not include any specification for video, then this requests video transmission to stop.

Where one source is feeding several receivers it may be unable to respond to any received signals such as requests to transmit in a particular mode.

The following text provides an overview of the operation of the MRSE protocol. In the case of any discrepancy between this and the formal specification, the formal specification will supersede.

#### C.9.1.1	Protocol overview - outgoing MRSE

A mode request procedure is initiated when the TRANSFER.request primitive is issued by the user at the outgoing MRSE. A RequestMode message is sent to the peer incoming MRSE, and timer T109 is started. If a RequestModeAck message is received in response to the RequestMode message then timer T109 is stopped and the user is informed with the TRANSFER.confirm primitive that the mode request was successful. If however a RequestModeReject message is received in response to the RequestMode message then timer T109 is stopped and the user is informed with the REJECT.indication primitive that the peer MRSE user has refused to accept the mode request.

If timer T109 expires then the outgoing MRSE user is informed with the REJECT.indication primitive and a RequestModeRelease message is sent.

Only RequestModeAck and RequestModeReject messages which are in response to the most recent RequestMode message are accepted. Messages in response to earlier RequestMode messages are ignored.

A new mode request procedure may be initiated with the TRANSFER.request primitive by the user at the outgoing MRSE before a RequestModeAck or a RequestModeReject message has been received.

#### C.9.1.2	Protocol overview - incoming MRSE

When a RequestMode message is received at the incoming MRSE, the user is informed of the mode request with the TRANSFER.indication primitive. The incoming MRSE user signals acceptance of the mode request by issuing the TRANSFER.response primitive, and a RequestModeAck message is sent to the peer outgoing MRSE. The incoming MRSE user signals rejection of the mode request by issuing the REJECT.request primitive, and a RequestModeReject message is sent to the peer outgoing MRSE.

A new RequestMode message may be received before the incoming MRSE user has responded to an earlier RequestMode message. The incoming MRSE user is informed with the REJECT.indication primitive, followed by the TRANSFER.indication primitive, and the incoming MRSE user responds to the new multiplex table entry.

If a RequestModeRelease message is received before the incoming MRSE user has responded to an earlier RequestMode message, then the incoming MRSE user is informed with the REJECT.indication, and the earlier mode request is discarded.

### C.9.2	Communication between MRSE and MRSE user

#### C.9.2.1	Primitives between MRSE and MRSE user

Communication between the MRSE and MRSE user, is performed using the primitives shown in Table C.75.

TABLE C.76/H.245

Primitives and parameters

|  | type |  |  |  |
| --- | --- | --- | --- | --- |
| generic name | request | indication | response | confirm |
| TRANSFER | MODE-ELEMENT | MODE-ELEMENT | MODE-PREF | MODE-PREF |
| REJECT | CAUSE | SOURCE CAUSE | not defined 1 | not defined |

Notes:

1.	“not defined” means that this primitive is not defined.

#### C.9.2.2	Primitive definition

The definition of these primitives is as follows:

a)	The TRANSFER primitives are used for the transfer of the mode request.

b)	The REJECT primitives are used to reject a mode request.

#### C.9.2.3	Parameter definition

The definition of the primitive parameters shown in Table C.77 are as follows:

a)	The MODE-ELEMENT parameter specifies a mode element. This parameter is mapped to the requestedModes field of the RequestMode message and is carried transparently from the outgoing MRSE user to the incoming MRSE user. This parameter is mandatory. There may be multiple MODE-ELEMENTS associated with the TRANSFER primitives.

b)	The MODE-PREF parameter informs the user as to whether the most preferred mode requested will be used or not. This parameter is mapped to the response field of the RequestModeAck message and carried transparently from the incoming RMSE user to the outgoing RMSE user. It has two values being "MOST-PREFERRED" and "LESS-PREFERRED".

c)	The SOURCE parameter indicates the source of the REJECT.indication primitive. The SOURCE parameter has the value of "USER" or "PROTOCOL". The latter case may occur as the result of a timer expiry.

d)	The CAUSE parameter indicates the reason for refusal to reject a mode request. The CAUSE parameter is not present when the SOURCE parameter indicates "PROTOCOL".

#### C.9.2.4	MRSE states

The following states are used to specify the allowed sequence of primitives between the MRSE and the MRSE user. The states for an outgoing MRSE are:

State 0:  IDLE

The MRSE is idle.

State 1:  AWAITING RESPONSE

The MRSE is waiting for a response from the remote MRSE.

The states for an incoming MRSE are:

State 0:  IDLE

The MRSE is idle.

State 1:  AWAITING RESPONSE

The MRSE is waiting for a response from the MRSE user.

#### C.9.2.5	State transition diagram

The allowed sequence of primitives between the MRSE and the MRSE user is defined here. The allowed sequences are specified separately for each of an outgoing MRSE and an incoming MRSE, as shown in Figure C.69 and Figure C.70 respectively.

FIGURE C.71/H.245

State transition diagram for sequence of primitives at MRSE outgoing

FIGURE C.72/H.245

State transition diagram for sequence of primitives at MRSE incoming

### C.9.3	Peer to peer MRSE communication

#### C.9.3.1	Messages

Table C.78 shows the MRSE messages and fields, defined in Annex C, which are relevant to the MRSE protocol.

TABLE C.79/H.245

MRSE message names and fields

| function | message | direction | field |
| --- | --- | --- | --- |
| mode request | RequestMode | O -> I 1 | sequenceNumber |
|  |  |  | requestedModes |
|  | RequestModeAck | O <- I | sequenceNumber |
|  |  |  | response |
|  | RequestModeReject | O <- I | sequenceNumber |
|  |  |  | cause |
| reset | RequestModeRelease | O -> I | - |

Notes:

1.	Direction:  O - outgoing, I - incoming.

#### C.9.3.2	MRSE state variables

The following state variables are defined at the outgoing MRSE:

out_SQ

This state variable is used to indicate the most recent RequestMode message. It is incremented by one and mapped to the RequestMode message sequenceNumber field before transmission of the RequestMode message. Arithmetic performed on out_SQ is modulo 256.

The following state variables are defined at the incoming MRSE:

in_SQ

This state variable is used to store the value of the sequenceNumber field of the most recently received RequestMode message. The RequestModeAck and RequestModeReject messages have their sequenceNumber fields set to the value of in_SQ, before being sent to the peer MRSE.

#### C.9.3.3	MRSE timers

The following timer is specified for the outgoing MRSE:

T109

This timer is used during the AWAITING RESPONSE state. It specifies the maximum time during which no RequestModeAck or RequestModeReject message may be received.

### C.9.4	MRSE procedures

Figure C.73 summarizes the MRSE primitives and their parameters, and messages, for each of the outgoing and incoming MRSE.

µ §

FIGURE C.74/H.245

Primitives and messages in the Mode Request Signalling Entity

#### C.9.4.1	Primitive parameter default values

Where not explicitly stated in the SDL diagrams the parameters of the indication and confirm primitives assume values as shown in Table C.80.

TABLE C.81/H.245

Default primitive parameter values

| primitive | parameter | default value |
| --- | --- | --- |
| TRANSFER.indication | MODE-ELEMENT | RequestMode.requestedModes |
| TRANSFER.confirm | MODE-PREF | RequestModeAck.response |
| REJECT.indication | SOURCE | USER |
|  | CAUSE | null |

#### C.9.4.2	Message field default values

Where not explicitly stated in the SDL diagrams the message fields assume values as shown in Table C.82.

TABLE C.83/H.245

Default message field values

| message | field | default value |
| --- | --- | --- |
| RequestMode | sequenceNumber | out_SQ |
|  | requestedModes | TRANSFER.request(MODE-ELEMENT) |
| RequestModeAck | sequenceNumber | in_SQ |
|  | response | TRANSFER.response(MODE-PREF) |
| RequestModeReject | sequenceNumber | in_SQ |
|  | cause | REJECT.request(CAUSE) |
| RequestModeRelease | - | - |

#### C.9.4.3	SDLs

The outgoing MRSE and the incoming MRSE procedures are expressed in SDL form in Figure C.75 and Figure C.76 respectively.

FIGURE C.77(i)/H.245

Outgoing MRSE SDL

FIGURE C.77(ii)/H.245

Outgoing MRSE SDL (continued)

FIGURE C.77(iii)/H.245

Outgoing MRSE SDL (concluded)

FIGURE C.78(i)/H.245

Incoming MRSE SDL

FIGURE C.78(ii)/H.245

Incoming MRSE SDL (concluded)

## C.10	Round trip delay procedures

### C.10.1	Introduction

Procedures are described here that allow the determination of the round trip delay between two communicating terminals. This function also enables a H.245 user to determine if the peer H.245 protocol entity is still alive.

The function described here is referred to as the Round Trip Delay Signalling Entity (RTDSE). Procedures are specified in terms of primitives and states at the interface between the RTDSE and the RTDSE user. There is one instance of the RTDSE in each terminal. Any terminal may perform the round trip delay determination.

The following text provides an overview of the operation of the RTDSE protocol. In the case of any discrepancy between this and the formal specification, the formal specification will supersede.

#### C.10.1.1	Protocol overview - RTDSE

A round trip delay determination procedure is initiated when the TRANSFER.request primitive is issued by the RTDSE user. A RoundTripDelayRequest message is sent to the peer RTDSE, and timer T105 is started. If a RoundTripDelayResponse message is received in response to the RoundTripDelayRequest message then timer T105 is stopped and the user is informed with the TRANSFER.confirm primitive of the round trip delay, which is the value of timer T105.

If a RoundTripDelayRequest message is at any time received from the peer RTDSE, a RoundTripDelayResponse message is immediately sent to the peer RTDSE.

If timer T105 expires then the RTDSE user is informed with the EXPIRY.indication primitive

Only the RoundTripDelayResponse message which is in response to the most recent RoundTripDelayRequest message is accepted. Messages in response to earlier RoundTripDelayRequest messages are ignored.

A new round trip delay determination procedure may be initiated with the TRANSFER.request primitive by the RTDSE user before a RoundTripDelayResponse message has been received.

### C.10.2	Communication between the RTDSE and the RTDSE user

#### C.10.2.1	Primitives between the RTDSE and the RTDSE user

Communication between the RTDSE, and RTDSE user, is performed using the primitives shown in Table C.84. These primitives are for the purpose of defining RTDSE procedures and are not meant to specify or constrain implementation.

TABLE C.85/H.245

Primitives and parameters

|  | type |  |  |  |
| --- | --- | --- | --- | --- |
| generic name | request | indication | response | confirm |
| TRANSFER | - 1 | not defined 2 | not defined | DELAY |
| EXPIRY | not defined | - | not defined | not defined |

Notes:

1.	“-” means no parameters

2.	“not defined” means that this primitive is not defined.

#### C.10.2.2	Primitive definition

The definition of these primitives is as follows:

a)	The TRANSFER primitive is used to request, and report upon, the round trip delay determination.

b)	The EXPIRY primitive indicates that no response has been received from the peer terminal.

#### C.10.2.3	Parameter definition

The definition of the primitive parameters shown in Table C.86 are as follows:

a)	The DELAY parameter returns the measured round trip delay.

#### C.10.2.4	RTDSE states

The following states are used to specify the allowed sequence of primitives between the RTDSE and the RTDSE user.

State 0:  IDLE

There is no RTDSE transfer in progress.

State 1:  AWAITING RESPONSE

The RTDSE user has requested the measurement of the round trip delay. A response from the peer RTDSE is awaited.

#### C.10.2.5	State transition diagram

The allowed sequence of primitives between the RTDSE and the RTDSE user is defined here. The allowed sequences are shown in Figure C.79.

FIGURE C.80/H.245

State transition diagram for sequence of primitives at RTDSE

### C.10.3	Peer to peer RTDSE communication

#### C.10.3.1	Messages

Table C.87 shows the RTDSE messages and fields, defined in Annex C, which are relevant to the RTDSE protocol.

TABLE C.88/H.245

RTDSE Message names and fields

| function | Message | field |
| --- | --- | --- |
| transfer | RoundTripDelayRequest | sequenceNumber |
|  | RoundTripDelayResponse | sequenceNumber |

#### C.10.3.2	RTDSE state variables

The following RTDSE state variables are defined:

out_SQ

This state variable is used to indicate the most recent RoundTripDelayRequest message. It is incremented by one and mapped to the RoundTripDelayRequest message sequenceNumber field before transmission of an RoundTripDelayRequest message. Arithmetic performed on out_SQ is modulo 256.

#### C.10.3.3	RTDSE timers

The following timer is specified for the RTDSE:

T105

This timer is used during the AWAITING RESPONSE state. It specifies the maximum time during which no RoundTripDelayResponse message may be received.

### C.10.4	RTDSE procedures

#### C.10.4.1	Introduction

Figure C.81 summarizes the RTDSE primitives and their parameters, and messages.

µ §

FIGURE C.82/H.245

Primitives and messages in the RTDSE.

#### C.10.4.2	Primitive parameter default values

Where not explicitly stated in the SDL diagrams the parameters of the indication and confirm primitives assume values as shown in Table C.89.

TABLE C.90/H.245

Default primitive parameter values

| primitive | parameter | default value |
| --- | --- | --- |
| TRANSFER.confirm | DELAY | initial value of timer T105 minus value of timer T105 |
| EXPIRY.indication | - | - |

NOTE - Timers are defined to count down to zero. The DELAY parameter indicates the time that the timer has been running, and so has the value of the difference between the initial setting and the retained value of the timer.

#### C.10.4.3	Message field default values

Where not explicitly stated in the SDL diagrams the message fields assume values as shown in Table C.91.

TABLE C.92/H.245

Default message field values

| message | field | default value |
| --- | --- | --- |
| RoundTripDelayRequest | sequenceNumber | out_SQ |
| RoundTripDelayResponse | sequenceNumber | RoundTripDelayRequest.sequenceNumber |

#### C.10.4.4	SDLs

The RTDSE procedures are expressed in SDL form in Figure C.83.

FIGURE C.84(i)/H.245

RTDSE SDL

FIGURE C.84(ii)/H.245

RTDSE SDL (concluded)

## C.11	Maintenance Loop procedures

### C.11.1	Introduction

The protocol specified here provides reliable operation of maintenance loops using acknowledged procedures.

The protocol specified here is referred to as the Maintenance Loop Signalling Entity (MLSE). Procedures are specified in terms of primitives at the interface between the MLSE and the MLSE user, and MLSE states. Protocol information is transferred to the peer MLSE via relevant messages defined in Annex C.

There is an outgoing MLSE and an incoming MLSE. At each of the outgoing and incoming sides there is one instance of the MLSE for each bi-directional logical channel, and one for the system loop. There is no connection between an incoming MLSE and an outgoing MLSE at one side, other than via primitives to and from the MLSE user. MLSE error conditions are reported.

The terminal that contains the incoming MLSE shall loop the appropriate data while it is in the LOOPED state, and not at any other time. The terminal that contains the outgoing MLSE shall be capable of receiving looped data while in any state, but while in the LOOPED state, should receive looped data only.

NOTE - The MaintenanceLoopOffCommand message applies to all MLSEs. It is always used to stop all maintenance loops.

The following text provides an overview of the operation of the MLSE protocol. In the case of discrepancy between this and the formal specification, the formal specification will supersede.

#### C.11.1.1	Protocol overview - outgoing

The establishment of a maintenance loop is initiated when the LOOP.request primitive is issued by the user at the outgoing MLSE. An MaintenanceLoopRequest message is sent to the peer incoming MLSE, and timer T102 is started. If an MaintenanceLoopAck message is received in response to the MaintenanceLoopRequest message then timer T102 is stopped and the user is informed with the LOOP.confirm primitive that the maintenance loop has been successfully established. If however an MaintenanceLoopReject message is received in response to the MaintenanceLoopRequest message then timer T102 is stopped and the user is informed with the RELEASE.indication primitive that the peer MLSE user has refused establishment of the maintenance loop.

If timer T102 expires in this period then the user is informed with the RELEASE.indication primitive, and a MaintenanceLoopOffCommand message is sent to the peer incoming MLSE. This will cancel all maintenance loops, and not just the one concerned with the particular MLSE.

A maintenance loop that has been successfully established may be cancelled when the RELEASE.request primitive is issued by the user at the outgoing MLSE. A MaintenanceLoopOffCommand message is sent to the peer incoming MLSE.

Before either of the MaintenanceLoopAck or MaintenanceLoopReject messages have been received in response to a previously sent MaintenanceLoopRequest message, the user at the outgoing MLSE may cancel the maintenance loop using the RELEASE.request primitive.

#### C.11.1.2	Protocol overview - incoming

When an MaintenanceLoopRequest message is received at the incoming MLSE, the user is informed of the request to establish a maintenance loop with the LOOP.indication primitive. The incoming MLSE user signals acceptance of the request to establish the maintenance loop by issuing the LOOP.response primitive, and an MaintenanceLoopAck message is sent to the peer outgoing MLSE. The maintenance loop shall now be performed. The incoming MLSE user signals rejection of the request to establish the maintenance loop by issuing the RELEASE.request primitive, and an MaintenanceLoopReject message is sent to the peer outgoing MLSE.

A maintenance loop that has been successfully established may be cancelled when the MaintenanceLoopOffCommand message is received at the incoming MLSE. The incoming MLSE user is informed with the RELEASE.indication primitive.

### C.11.2	Communication between the MLSE and the MLSE user

#### C.11.2.1	Primitives between the MLSE and the MLSE user

Communication between the MLSE and the MLSE user is performed using the primitives shown in Table C.93.

TABLE C.94/H.245

Primitives and parameters

|  | type |  |  |  |
| --- | --- | --- | --- | --- |
| generic name | request | indication | response | confirm |
| LOOP | LOOP_TYPE | LOOP_TYPE | - 1 | - |
| RELEASE | CAUSE | SOURCE CAUSE | not defined 2 | not defined |
| ERROR | not defined | ERRCODE | not defined | not defined |

Notes:

1.	“-” means no parameters.

2.	“not defined” means that this primitive does not exist.

#### C.11.2.2	Primitive definition

The definition of these primitives is as follows:

a)	The LOOP primitives are used to establish a maintenance loop.

b)	The RELEASE primitives are used to cancel a maintenance loop.

c)	The ERROR primitive reports MLSE errors to a management entity.

#### C.11.2.3	Parameter definition

The definition of the primitive parameters shown in Table C.95 are as follows:

a)	The LOOP_TYPE parameter specifies the parameters associated with the maintenance loop. It has values of “SYSTEM”, “MEDIA”, and “LOGICAL_CHANNEL”. This parameter, and the logical channel number, determine the value of   the type field of the MaintenanceLoopRequest message which is then carried transparently to the peer MLSE user.

b)	The SOURCE parameter indicates to the MLSE user the source of the maintenance loop release. The SOURCE parameter has the value of "USER" or "MLSE", indicating either the MLSE user, or the MLSE. The latter may occur as the result of a protocol error.

c)	The CAUSE parameter indicates the reason as to why the peer MLSE user rejected a request to establish a maintenance loop. The CAUSE parameter is not present when the SOURCE parameter indicates "MLSE".

d)	The ERRCODE parameter indicates the type of MLSE error. Table C.96 shows the allowed values of the ERRCODE parameter.

#### C.11.2.4	MLSE states

The following states are used to specify the allowed sequence of primitives between the MLSE and the MLSE user, and the exchange of messages between peer MLSEs. The states are specified separately for each of an outgoing MLSE and an incoming MLSE. The states for an outgoing MLSE are:

State 0:. NOT LOOPED

There is no maintenance loop.

State 1:. AWAITING RESPONSE

The outgoing MLSE is waiting to establish a maintenance loop with a peer incoming MLSE.

State 2:. LOOPED

The MLSE peer-to-peer maintenance loop has been established. All data received on the appropriate channel should be looped data.

The states for an incoming MLSE are:

State 0:. NOT LOOPED

There is no maintenance loop.

State 1:. AWAITING RESPONSE

The incoming MLSE is waiting to establish a maintenance loop with a peer outgoing MLSE. The appropriate data shall not be looped.

State 2:. LOOPED

An MLSE peer-to-peer maintenance loop has been established. All data received on the appropriate channel shall be looped.

#### C.11.2.5	State transition diagram

The allowed sequence of primitives between the MLSE and the MLSE user is defined here. The allowed sequence of primitives relates to states of the MLSE as viewed from the MLSE user. The allowed sequences are specified separately for each of an outgoing MLSE and an incoming MLSE, as shown in Figure C.85 and Figure C.86 respectively.

µ §

FIGURE C.87/H.245

State transition diagram for sequence of primitives at outgoing MLSE

µ §

FIGURE C.88/H.245

State transition diagram for sequence of primitives at incoming MLSE

### C.11.3	Peer to peer MLSE communication

#### C.11.3.1	MLSE messages

Table C.97 shows the MLSE messages and fields, defined in Annex C, which are relevant to the MLSE protocol.

TABLE C.98/H.245

MLSE message names and fields

| function | message | direction | field |
| --- | --- | --- | --- |
| establish | MaintenanceLoopRequest | O -> I 1 | type |
|  | MaintenanceLoopAck | O <- I | type |
|  | MaintenanceLoopReject | O <- I | type |
|  |  |  | cause |
| release | MaintenanceLoopOffCommand | O -> I | - |

Notes:

1.	Direction:. O - outgoing, I - incoming.

#### C.11.3.2	MLSE state variables

The following state variable is defined at the outgoing MLSE:

out_MLN

This state variable distinguishes between outgoing MLSEs. It is initialised at outgoing MLSE initialization. The value of out_MLN is used to set the type field of MaintenanceLoopRequest messages sent from an outgoing MLSE.

The following state variable is defined at the incoming MLSE:

in_MLN

This state variable distinguishes between incoming MLSEs. It is initialised at incoming MLSE initialization. For MaintenanceLoopRequest messages received at an incoming MLSE, the message type field value is consistent with the value of in_MLN.

in_TYPE

This state variable stores the value of LOOP_TYPE when the MaintenanceLoopRequest is received. This state variable assists in setting the value of the type field in the MaintenanceLoopAck message.

#### C.11.3.3	MLSE timers

The following timer is specified for the outgoing MLSE:

T102

This timer is used during the AWAITING RESPONSE state. It specifies the maximum allowed time during which no MaintenanceLoopAck or MaintenanceLoopReject message may be received.

### C.11.4	MLSE procedures

#### C.11.4.1	Introduction

Figure C.89 summarizes the primitives and their parameters, and the messages, for each of the outgoing and incoming MLSE.

µ §

FIGURE C.90/H.245

Primitives and messages in the Maintenance loop Signalling Entity

#### C.11.4.2	Primitive parameter default values

Where not explicitly stated in the SDL diagrams the parameters of the indication and confirm primitives assume values as shown in Table C.99.

TABLE C.100/H.245

Default primitive parameter values

| primitive | parameter | default value 1 |
| --- | --- | --- |
| LOOP.indication | LOOP_TYPE | MaintenanceLoopRequest.type |
| RELEASE.indication | SOURCE | USER |
|  | CAUSE | MaintenanceLoopReject.cause |

Notes:

1.	A primitive parameter shall be coded as null, if an indicated message field is not present in the message.

#### C.11.4.3	Message field default values

Where not explicitly stated in the SDL diagrams the message fields assume values as shown in Table C.101.

TABLE C.102/H.245

Default message field values

| message | field | default value 1 |
| --- | --- | --- |
| MaintenanceLoopRequest | type | LOOP.request(LOOP_TYPE) and out_MLN2 |
| MaintenanceLoopAck | type | in_LOOP and in_MLN3 |
| MaintenanceLoopReject | type | in_LOOP and in_MLN3 |
|  | cause | RELEASE.request(CAUSE) |
| MaintenanceLoopOffCommand | - | - |

Notes:

1.	A message field shall not be coded, if the corresponding primitive parameter is null i.e. not present.

2.	The value of the type field is derived from the LOOP_TYPE parameter and the logical channel number.

3.	The value of the type field is derived from the in_LOOP and in_MLN state variables.

#### C.11.4.4	ERRCODE parameter values

The ERRCODE parameter of the ERROR.indication primitive indicates a particular error condition. Table C.103 shows the values that the ERRCODE parameter may take at the outgoing MLSE. There is no ERROR.indication primitive associated with the incoming MLSE.

TABLE C.104/H.245

ERRCODE parameter values at outgoing MLSE

| error type | error code | error condition | state |
| --- | --- | --- | --- |
| inappropriate message | A | MaintenanceLoopAck | LOOPED |
| no response from peer MLSE | B | timer T102 expiry | AWAITING RESPONSE |

#### C.11.4.5	SDLs

The outgoing MLSE and the incoming MLSE procedures are expressed in SDL form in Figure C.91 and Figure C.92 respectively.

µ §

FIGURE C.93(i)/H.245

Outgoing MLSE SDL

µ §

FIGURE C.93(ii)/H.245

Outgoing MLSE SDL (continued)

µ §

FIGURE C.93(iii)/H.245

Outgoing MLSE SDL (concluded)

µ §

FIGURE C.94(i)/H.245

Incoming MLSE SDL

µ §

FIGURE C.94(ii)/H.245

Incoming MLSE SDL (continued)

µ §

FIGURE C.94(iii)/H.245

Incoming MLSE SDL (concluded)

ANNEX D

# Annex D	Object identifier assignments

A==

(This annex forms an integral part of this Recommendation)

Table D.1 lists the assignment of Object Identifiers defined for use by this Recommendation.

TABLE D.1/H.245

| Object Identifier Value | Description |
| --- | --- |
| {itu-t (0) recommendation (0) h (8) 245 version (0) 1} | This Object Identifier is used to indicate the version of this Recommendation in use as a multimedia system control protocol. This indicates the first version of this Recommendation. |
| {itu-t (0) recommendation (0) h (8) 245 version (0) 2} | This Object Identifier is used to indicate the version of this Recommendation in use as a multimedia system control protocol. At this time there are six standardized versions defined. This indicates the second version of this Recommendation. |
| {itu-t (0) recommendation (0) h (8) 245 version (0) 3} | This Object Identifier is used to indicate the version of this Recommendation in use as a multimedia system control protocol. At this time there are six standardized versions defined. This indicates the third version of this Recommendation. |
| {itu-t (0) recommendation (0) h (8) 245 version (0) 4} | This Object Identifier is used to indicate the version of this Recommendation in use as a multimedia system control protocol. At this time there are six standardized versions defined. This indicates the fourth version of this Recommendation. |
| {itu-t (0) recommendation (0) h (8) 245 version (0) 5} | This Object Identifier is used to indicate the version of this Recommendation in use as a multimedia system control protocol. At this time there are six standardized versions defined. This indicates the fifth version of this Recommendation. |
| {itu-t (0) recommendation (0) h (8) 245 version (0) 6} | This Object Identifier is used to indicate the version of this Recommendation in use as a multimedia system control protocol. At this time there are six standardized versions defined. This indicates the sixth version of this Recommendation. |
| {itu-t (0) recommendation (0) h (8) 245 generic-capabilities (1) video (0) ISO/IEC 14496-2 (0)} | This Object Identifier is used to indicate the generic capability for ISO/IEC 14496-2. This capability is defined in Annex E of this Recommendation. |
| {itu-t (0) recommendation (0) h (8) 245 generic-capabilities (1) audio (1) ISO/IEC 14496-3 (0)} | This Object Identifier is used to indicate the generic capability for ISO/IEC 14496-3. This capability is defined in Annex H of this Recommendation. |
| {itu-t (0) recommendation (0) h (8) 245 generic-capabilities (1) audio (1) amr (1)} | This Object Identifier is used to indicate the generic capability for the GSM Adaptive Multi rate speech codec. This capability is defined in Annex I of this Recommendation. |
| {itu-t (0) recommendation (0) h (8) 245 generic-capabilities (1) audio (1) acelp (2)} | This Object Identifier is used to indicate the generic capability for the TIA/EIA/ANSI IS-136 ACELP voice codec. This capability is defined in Annex J of this Recommendation. |
| {itu-t (0) recommendation (0) h (8) 245 generic-capabilities (1) audio (1) us1 (3)} | This Object Identifier is used to indicate the generic capability for the TIA/EIA /ANSI IS-136 US1 voice codec. This capability is defined in Annex K of this Recommendation. |
| {itu-t (0) recommendation (0) h (8) 245 generic-capabilities (1) audio (1) is127evrc (4)} | This Object Identifier is used to indicate the generic capability for the TIA/EIA IS-127 Enhanced Variable Rate Codec. This capability is defined in Annex L of this Recommendation. |
| {itu-t (0) recommendation (0) h (8) 245 generic-capabilities (1) data (2) ISO/IEC 14496-1 (0)} | This Object Identifier is used to indicate the generic capability for ISO/IEC 14496-1. This capability is defined in Annex G of this Recommendation. |
| {itu-t (0) recommendaton (0) h (8) 245 generic-capabilities (1) control (3) logical-channel-bit-rate-management (0)} | This Object Identifier is used to indicate the generic capability for logical channel bit rate management. This capability is defined in Annex F of this Recommendation. |

ANNEX E

# Annex E	ISO/IEC 14496-2 Capability Definitions

A==

(This annex forms an integral part of this Recommendation)

Table E.1 defines the capability identifier for ISO/IEC 14496-2 Capabilities [143]. These parameters shall only be included as genericVideoCapability within the VideoCapability structure and as genericVideoMode within the VideoMode structure. Tables E.2 to E.5 define the associated capability parameters.

A single profileAndLevel instantiation of ISO/IEC 14496-2 may support multiple visual objects. Each such visual object is carried as an elementary stream in its own separate logical channel. Since it is possible for multiple ISO/IEC 14496-2 visual environments to be actively transmitted at the same time, and since each of these may be constructed from multiple object streams, it is necessary to have a mechanism for indicating which object streams are associated together in a single ISO/IEC 14496-2 visual environment. This association shall be performed by use of the forwardLogicalChannelDependency mechanism in OpenLogicalChannel whenever multiple visual objects are in use for the same ISO/IEC 14496-2 visual environment. All visual objects associated together to an ISO/IEC 14496-2 visual environment shall have the same profile and level by indicating the same profileAndLevel value when opening logical channels. If a logical channel was opened with an indication of a dependency on some other logical channel in this fashion, and the logical channel on which its dependency was indicated is closed, the remaining open logical channels which had previously been grouped together by some chain of forwardLogicalChannelDependency links shall remain logically grouped as a single ISO/IEC 14496-2 visual environment.

TABLE E.1/H.245

Capability Identifier for ISO/IEC 14496-2 Capability

| Capability name: | ISO/IEC 14496-2 |
| --- | --- |
| Capability class: | Video codec |
| Capability identifier type: | Standard. |
| Capability identifier value: | itu-t (0) recommendation (0) h (8) 245 generic-capabilities (1) video (0) ISO/IEC 14496-2 (0) |
| maxBitRate: | The maxBitRate field shall always be included. |
| nonCollapsingRaw: | This field shall not be included. |
| transport: | This field shall not be included. |

TABLE E.2/H.245

Profile and Level for ISO/IEC 14496-2 Capability

| Parameter name: | profileAndLevel |
| --- | --- |
| Parameter description: | This is a nonCollapsing GenericParameter. profileAndLevel indicates the capability of processing the particular profiles in combination with the level as given in Table G-1 'FLC table for profile_and_level_indication' of ISO/IEC 14496-2. |
| Parameter identifier value: | 0 |
| Parameter status: | Mandatory |
| Parameter type: | unsignedMax. Shall be in the range 0..255. |
| Supersedes: | - |

TABLE E.3/H.245

Object Parameter for ISO/IEC 14496-2 Capability

| Parameter name: | object |
| --- | --- |
| Parameter description: | This is a nonCollapsing GenericParameter. object indicates the set of tools to be used by the decoder of the bitstream contained in the logical channel as given in Table 6-10 'FLC table for video_object_type indication' of ISO/IEC 14496-2. |
| Parameter identifier value: | 1 |
| Parameter status: | Optional. Shall not be present for Capability Exchange. Shall be present for Logical Channel Signalling. May be present for Mode Request |
| Parameter type: | unsignedMax. Shall be in the range 0..255. |
| Supersedes: | - |

TABLE E.4/H.245

Decoder Configuration Information for ISO/IEC 14496-2 Capability

| Parameter name: | decoderConfigurationInformation |
| --- | --- |
| Parameter description: | This is a nonCollapsing GenericParameter. decoderConfigurationInformation indicates how to configure the decoder for a particular object (bitstream) (refer to Subclause 6.2.1 'Start Codes' and subclause K.3.1 'VideoObject' to K.3.4 'FaceObject' of ISO/IEC 14496-2). |
| Parameter identifier value: | 2 |
| Parameter status: | Optional. Shall not be present for Capability Exchange and Mode Request. May be present for Logical Channel Signalling. |
| Parameter type: | octetString. |
| Supersedes: | - |

TABLE E.5/H.245

Drawing Order for ISO/IEC 14496-2 Capability

| Parameter name: | drawingOrder |
| --- | --- |
| Parameter description: | This is a nonCollapsing GenericParameter. drawingOrder indicates the drawing order of a visual object within a composition of (possibly overlaid) visual objects. The visual object having the lowest drawingOrder shall be drawn first. If visual objects have the same drawingOrder, the object corresponding to the logical channel with the lowest logical channel number shall be drawn first. If drawingOrder is not present during logical channel signalling, it is assumed to have the value 32768. |
| Parameter identifier value: | 3 |
| Parameter status: | Optional. Shall not be present for Capability Exchange and Mode Request. May be present for Logical Channel Signalling. |
| Parameter type: | unsignedMax. Shall be in the range 0..65535. |
| Supersedes: | - |

ANNEX F

# Annex F	Logical Channel Bit Rate Management Capability Definitions

A==

(This annex forms an integral part of this Recommendation)

Table F.1 defines the capability identifier for Bit Rate Management. These parameters, which provide information about which bit rate management messages the terminal supports, shall only be included as genericControlCapability within the Capability structure. Tables F.2 to F.4 define the associated capability parameters.

TABLE F.6/H.245

Capability Identifier for Logical Channel Bit Rate Management

| Capability name: | H.245 Logical Channel Bit Rate Management |
| --- | --- |
| Capability class: | Control. |
| Capability identifier type: | Standard. |
| Capability identifier value: | itu-t (0) recommendation (0) h (8) 245 generic-capabilities (1) control (3) logical-channel-bitrate-management (0) |
| maxBitRate: | The maxBitRate field shall not be included. |
| nonCollapsingRaw: | This field shall not be included. |
| transport: | This field shall not be included. |

TABLE F.7/H.245

Flow Control Capability Parameter for Bit Rate Management

| Parameter name: | Flow Control Capability |
| --- | --- |
| Parameter description: | The presence of this parameter indicates the capability to support the FlowcontrolIndication message. |
| Parameter identifier value: | 0 |
| Parameter status: | Optional |
| Parameter type: | Logical. |
| Supersedes: | - |

TABLE F.8/H.245

Logical Channel Bit Rate Change Capability Parameter for Bit Rate Management

| Parameter name: | Logical Channel Bit Rate Change Capability |
| --- | --- |
| Parameter description: | The presence of this parameter indicates the capability to support the Logical Channel Rate Change Procedure, which uses the messages LogicalChannelRateRequest, LogicalChannelRateAcknowledge, LogicalChannelRateReject and LogicalChannelRateRelease. |
| Parameter identifier value: | 1 |
| Parameter status: | Optional |
| Parameter type: | Logical. |
| Supersedes: | - |

TABLE F.9/H.245

RTCP Frequency Parameter for Bit Rate Management

| Parameter name: | RTCP Frequency Capability |
| --- | --- |
| Parameter description: | This indicates the frequency at which the terminal can send RTCP reports. |
| Parameter identifier value: | 2 |
| Parameter status: | Optional |
| Parameter type: | Unsigned32Min. |
| Supersedes: | - |

ANNEX G

# Annex G	ISO/IEC 14496-1 Capability Definitions

A==

(This annex forms an integral part of this Recommendation)

Table G.1 below defines the capability identifier for ISO/IEC 14496-1 [144] capabilities. Tables G.2 to G.6 define the associated capability parameters for ISO/IEC 14496-1. These parameters shall only be included as genericDataCapability within the DataCapability structure and as genericDataMode within the DataMode structure. For capability exchange, only streamType and profileAndLevel and objectType shall be specified. When opening a logical channel (forward or reverse) either ES_ID or objectDescriptor shall be specified.

Further information about the usage of the ISO/IEC 14496-1 Generic Capability is included in Annex F to H.324 version 1998.

## G.1	Capability Identifier

TABLE G.1/H.245

Capability Identifier for ISO/IEC 14496-1

| Capability name: | ISO/IEC 14496-1 Capability |
| --- | --- |
| Capability class: | Data application |
| Capability identifier type: | Standard |
| Capability identifier value: | itu-t (0) recommendation (0) h (8) 245 generic-capabilities (1) data (2) ISO/IEC 14496-1 (0) |
| maxBitRate: | This field shall be included to indicate the maximum bitrate of one single ISO/IEC 14496 data stream. |
| nonCollapsingRaw: | This field shall not be included. |
| transport: | This field shall be included to indicate the error protection protocol for a specific ISO/IEC 14496 data stream carried in one logical channel. |

## G.2	Capability parameters used for capability negotiations and logical channel signalling

TABLE G.2/H.245

Capability Parameter streamType

| Parameter name: | streamType |
| --- | --- |
| Parameter description: | This is a nonCollapsing GenericParameter. StreamType indicates the type of the ISO/IEC 14496 stream that is referred to by a specific instance of ISO/IEC 14496-1 Generic Capability as given in table 9 (“streamType Values”) of ISO/IEC 14496-1. |
| Parameter identifier value: | 0 |
| Parameter status | Optional. Shall be present for Capability Exchange. Shall not be present for Logical Channel Signalling and Mode Request. |
| Parameter type: | unsignedMax. Shall be in the range 0..255. |
| Supersedes: | - |

TABLE G.3/H.245

Capability Parameter profileAndLevel

| Parameter name: | ProfileAndLevel |
| --- | --- |
| Parameter description: | This is a nonCollapsing GenericParameter. ProfileAndLevel indicates the capability of processing the particular profiles in combination with the level as given in · Table 3 of ISO/IEC 14496-1 ("ODProfileLevelIndication Values") for streamType = 0x01 · Table 4 of ISO/IEC 14496-1 ("sceneProfileLevelIndication Values") for streamType = 0x03 · Table 5 of ISO/IEC 14496-1 ("audioProfileLevelIndication Values") for streamType = 0x05 · Table 6 of ISO/IEC 14496-1 ("visualProfileLevelIndication Values") for streamType = 0x04 |
| Parameter identifier value: | 1 |
| Parameter status | Optional. Shall be present for Capability Exchange. Shall not be present for Logical Channel Signalling and Mode Request. |
| Parameter type: | unsignedMax. Shall be in the range 0..255. |
| Supersedes: | - |

TABLE G.4/H.245

Capability Parameter objectType

| Parameter name: | objectType |
| --- | --- |
| Parameter description: | This is a nonCollapsing GenericParameter. objectType indicates the set of tools to be used by the decoder of the bitstream contained in one logical channel as given in · Table 8 of ISO/IEC 14496-1 ("objectTypeIndication Values ") for streamType = 0x04 or 0x05 · Table 7 of ISO/IEC 14496-1 ("graphicsProfileLevelIndication Values") for streamType = 0x03 For all other values of streamType, objectType is not defined and shall therefore not be used. |
| Parameter identifier value: | 2 |
| Parameter status | Optional. Shall not be present for Capability Exchange. Shall be present for Logical Channel Signalling. May be present for Mode Request. |
| Parameter type: | unsignedMax. Shall be in the range 0..255. |
| Supersedes: | - |

## G.3	Capability parameters used for logical channel signalling only

TABLE G.5/H.245

Capability Parameter objectDescriptor

| Parameter name: | ObjectDescriptor |
| --- | --- |
| Parameter description: | This is a nonCollapsing GenericParameter. ObjectDescriptor contains an octet string which provides all the necessary information to configure the decoder for a particular bitstream in one logical channel (refer to ISO/IEC 14496-1). It shall contain information for only one Elementary Stream. |
| Parameter identifier value: | 3 |
| Parameter status | Optional. Shall not be present for Capability Exchange and Mode Request. May be present for Logical Channel Signalling. |
| Parameter type: | octetString. |
| Supersedes: | - |

TABLE G.6/H.245

Capability Parameter ES_ID

| Parameter name: | ES_ID |
| --- | --- |
| Parameter description: | This is a nonCollapsing GenericParameter. ES_ID indicates the ID of the elementary stream that is contained in one specific logical channel and by which it may be referred by other ISO/IEC 14496 data streams. For the InitialObjectDescriptor, ES_ID shall be set to ‘0’ (zero). |
| Parameter identifier value: | 4 |
| Parameter status | Optional. Shall not be present for Capability Exchange. May be present for Logical Channel Signalling. Shall be present for Mode Request. |
| Parameter type: | unsignedMax. Shall be in the range 0..65535. |
| Supersedes: | - |

ANNEX H

# Annex H	ISO/IEC 14496-3 Capability Definitions

A==

(This annex forms an integral part of this Recommendation)

Table H.1 defines the capability identifier for ISO/IEC 14496-3 [145] and ISO/IEC 14496-3/AMD1 [146] Capabilities. Tables H.2 to H.11 define the associated capability parameters for ISO/IEC 14496-3. These parameters shall only be included as genericAudioCapability within the AudioCapability structure and as genericAudioMode within the AudioMode structure. For capability exchange, profileAndLevel, formatType and maxAl-sduAudioFrames shall be present, audioObjectType and maxAudioObjects may be present, and all other parameters shall be absent. If formatType indicates ISO/IEC 14496-3 Transport Stream format, maxAudioObjects shall be present for capability exchange. When opening a logical channel (forward or reverse), profileAndLevel, formatType and audioObjectType shall be present and all other parameters  may be specified. For mode request, profileAndLevel and formatType shall be present and audioObjectType may be specified.

profileAndLevel of ISO/IEC1446-3 and ISO/IEC 14496-3/AMD1 may support several types of audio objects. The audio object shall be carried as one of two bitstream formats which are the raw data format and the ISO/IEC 14496-3 Transport Stream format. formatType indicate the choice of the bitstream format type. In applications using multi-rate or scalable transmission, it is useful to allow changes in the structure of the audio objects in one logical channel. This can be realized with the MPEG-4/Audio format which allows changing the configuration of the stream frame by frame. For low bit-rate transmission, the raw data format may be used to reduce redundancy of transmitting the configuration of the stream every frame.

TABLE H.1/H.245

Capability Identifier for ISO/IEC 14496-3 Capability

| Capability name: | ISO/IEC 14496-3 Capability |
| --- | --- |
| Capability class: | Audio Codec |
| Capability identifier type: | Standard |
| Capability identifier value: | itu-t (0) recommendation (0) h (8) 245 generic-capabilities (1) audio (1) ISO/IEC 14496-3 (0) |
| maxBitRate: | This field shall always be included. |
| nonCollapsingRaw: | This field shall not be included. |
| transport: | This field shall not be included. |

TABLE H.2/H.245

Profile and Level for ISO/IEC 14496-3 Capability

| Parameter name: | profileAndLevel |
| --- | --- |
| Parameter description: | This is a nonCollapsing GenericParameter. profileAndLevel indicates the capability of processing the particular profiles in combination with the level as given in ISO/IEC 14496-1 and ISO/IEC 14496-1/AMD1. |
| Parameter identifier value: | 0 |
| Parameter status | Mandatory |
| Parameter type: | unsignedMax. Shall be in the range 0..255. |
| Supersedes: | - |

TABLE H.3/H.245

formatType for ISO/IEC 14496-3 Capability

| Parameter name: | formatType |
| --- | --- |
| Parameter description: | This is a nonCollapsing GenericParameter. FormatType indicates the choice of the bitstream format type of a audio object between the raw data format and the audio format as follows: · 0 : the raw data format (ISO/IEC14496-3 and ISO/IEC 14496-3/AMD1) · 1 : the format defined as Low-overhead MPEG-4 Audio Transport Multiplex (LATM) in ISO/IEC14496-3/AMD1. |
| Parameter identifier value: | 1 |
| Parameter status | Mandatory |
| Parameter type: | logical |
| Supersedes: | - |

TABLE H.4/H.245

maxAl-sduAudioFrames for ISO/IEC 14496-3 Capability

| Parameter name: | MaxAl-sduFrames |
| --- | --- |
| Parameter description: | This is a collapsing GenericParameter. It specifies what is the maximum number of audio frames per AL-SDU |
| Parameter identifier value: | 2 |
| Parameter status: | Shall be present for capability exchange and logical channel signalling. Shall not be present for mode request. |
| Parameter type: | UnsignedMin. Shall be in the range 1..256. |
| Supersedes: | - |

TABLE H.5/H.245

audioObjectType for ISO/IEC 14496-3 Capability

| Parameter name: | audioObjectType |
| --- | --- |
| Parameter description: | This is a nonCollapsing GenericParameter. audioObjectType indicates the set of tools to be used by the decoder of the bitstream contained in the logical channel as given in ISO/IEC14496-3/AMD1. It can be used to limit the capability within the specified profileAndLevel in capability exchange. |
| Parameter identifier value: | 3 |
| Parameter status | Optional. May be present for Capability Exchange. Shall be present for Logical Channel Signalling. May be present for Mode Request. |
| Parameter type: | unsignedMax. Shall be in the range 0..31. |
| Supersedes: | - |

TABLE H.6/H.245

audioSpecificConfig for ISO/IEC 14496-3 Capability

| Parameter name: | audioSpecificConfig |
| --- | --- |
| Parameter description: | This is a nonCollapsing GenericParameter. audioSpecificConfig indicates how to configure the decoder for a particular object (refer to ISO/IEC14496-3/AMD1). |
| Parameter identifier value: | 4 |
| Parameter status | Optional. Shall not be present for Capability Exchange and Mode Request. Shall be present for Logical Channel Signalling if formatType equals 0 (raw data format). If not, shall not be present for Logical Channel Signalling. |
| Parameter type: | octetString |
| Supersedes: | - |

TABLE H.7/H.245

maxAudioObjects for ISO/IEC 14496-3 Capability

| Parameter name: | MaxAudioObjects |
| --- | --- |
| Parameter description: | This is a Collapsing GenericParameter. It specifies what is the maximum number of multiplexed audio objects in the audio payload. |
| Parameter identifier value: | 5 |
| Parameter status | Optional. If formatType equals 0 (raw data format), shall not be present for Capability Exchange and Logical Channel Signalling. If not, shall be present for Capability Exchange and Logical Channel Signalling. Shall not be present for Mode Request. |
| Parameter type: | UnsignedMin. Shall be in the range 1..16. |
| Supersedes: | - |

TABLE H.8/H.245

muxConfigPresent for ISO/IEC 14496-3 Capability

| Parameter name: | MuxConfigPresent |
| --- | --- |
| Parameter description: | This is a nonCollapsing GenericParameter. muxConfigPresent indicates whether audio payload configuration is multiplexed into the audio payload itself as given in ISO/IEC 14496-3/AMD1: 0: audio payload configuration (streamMuxConfig) is not multiplexed into the audio payload. 1: streamMuxConfig is multiplexed into the audio payload. |
| Parameter identifier value: | 6 |
| Parameter status | Optional. Shall not be present for Capability Exchange and Mode Request. Shall be present for Logical Channel Signalling if formatType equals 1 (LATM format). If not, shall not be present for Logical Channel Signalling. |
| Parameter type: | Logical |
| Supersedes: | - |

TABLE H.9/H.245

EP_DataPresent for ISO/IEC 14496-3 Capability

| Parameter name: | EP_DataPresent |
| --- | --- |
| Parameter description: | This is a nonCollapsing GenericParameter. EP_DataPresent indicates whether the audio payload has error resiliency for bit error (not packet loss) as given in ISO/IEC 14496-3/AMD1: 0: The audio payload has not error resiliency. 1: The audio payload has error resiliency. The configuration for the ISO/IEC 14496-3/AMD1 EP tool (ErrorProtection_SpecificConfig) may be present for Logical Channel Signaling. |
| Parameter identifier value: | 7 |
| Parameter status | Optional. Shall not be present for Capability Exchange and Mode Request. Shall be present for Logical Channel Signalling if formatType equals 1 (LATM format). If not, shall not be present for Logical Channel Signalling. |
| Parameter type: | Logical |
| Supersedes: | - |

TABLE H.10/H.245

streamMuxConfig for ISO/IEC 14496-3 Capability

| Parameter name: | StreamMuxConfig |
| --- | --- |
| Parameter description: | This is a nonCollapsing GenericParameter. streamMuxConfig indicates coniguration of the audio payload as given in ISO/IEC 14496-3/AMD1. |
| Parameter identifier value: | 8 |
| Parameter status | Optional. Shall not be present for Capability Exchange and Mode Request. Shall be present for Logical Channel Signalling if formatType equals 1 (LATM format). If not, shall not be present for Logical Channel Signalling. |
| Parameter type: | OctetString |
| Supersedes: | - |

TABLE H.11/H.245

ErrorProtection_SpecificConfig for ISO/IEC 14496-3 Capability

| Parameter name: | ErrorProtection_SpecificConfig |
| --- | --- |
| Parameter description: | This is a nonCollapsing GenericParameter. ErrorProtection_SpecificConfig indicates how to configure the ISO/IEC 14496-3/Amd1 EP tool as given in the LATM EP_MuxElement() description in ISO/IEC 14496-3/AMD1. |
| Parameter identifier value: | 9 |
| Parameter status | Optional. Shall not be present for Capability Exchange and Mode Request. Shall be present for Logical Channel Signalling if formatType equals 1 (LATM format). If not, shall not be present for Logical Channel Signalling. |
| Parameter type: | OctetString |
| Supersedes: | - |

ANNEX I

# Annex I	GSM Adaptive Multi Rate Capability Definitions

A==

(This annex forms an integral part of this Recommendation)

Table I.1 defines the capability identifier for GSM Adaptive Multi Rate (AMR) Capabilities. Tables I.2 to I.7 define the associated capability parameters. The relevant specifications are [147], [148], [149], [150], [151], [152], [153] and [154].

The section I.1 defines the mode signaling and the packetization of speech frames to the octet structure.

TABLE I.1/H.245

GSM AMR Capability Identifier

| Capability name: | AMR |
| --- | --- |
| Capability class: | Audio codec. |
| Capability identifier type: | Standard. |
| Capability identifier value: | {itu-t (0) recommendation (0) h (8) 245 generic-capabilities (1) audio (1) amr (1)} |
| maxBitRate: | Shall be 122 |
| NonCollapsingRaw: | This field shall not be included |
| transport: | This field shall not be included |

TABLE I.2/H.245

GSM AMR Capability Parameter - maxAl-sduFrames

| Parameter name: | MaxAl-sduFrames |
| --- | --- |
| Parameter description: | This is a collapsing GenericParameter. It specifies what is the maximum number of audio frames per AL-SDU |
| Parameter identifier value: | 0 |
| Parameter status: | Shall be present for capability exchange and logical channel signalling. Shall not be present for mode request. |
| Parameter type: | UnsignedMin |
| Supersedes: | - |

TABLE I.3/H.245

GSM AMR Capability Parameter - bitRate

| Parameter name: | bitRate |
| --- | --- |
| Parameter description: | This is a nonCollapsing GenericParameter. It specifies the AMR bitrate. This parameter shall be used only in mode requests. 0 = 4.75, 1 = 5.15, 2 = 5.90, 3 = 6.70, 4 = 7.40, 5 = 7.95, 6 = 10.2, 7 = 12.2 |
| Parameter identifier value: | 1 |
| Parameter status: | Optional |
| Parameter type: | UnsignedMin |
| Supersedes: | - |

TABLE I.4/H.245

GSM AMR Capability Parameter - GSM AMR Comfort Noise

| Parameter name: | gsmAmrComfortNoise |
| --- | --- |
| Parameter description: | This is a collapsing GenericParameter. It specifies that GSM AMR comfort noise is to be used in mode request. This parameter shall be used only in mode requests but not in capabilities because this capability is mandatory. |
| Parameter identifier value: | 2 |
| Parameter status: | Optional |
| Parameter type: | Logical |
| Supersedes: | - |

TABLE I.5/H.245

GSM AMR Capability Parameter - GSM EFR comfort noise

| Parameter name: | gsmEfrComfortNoise |
| --- | --- |
| Parameter description: | This is a collapsing GenericParameter. In a capability, the parameter specifies whether there is a GSM EFR comfort noise capability or not. In a mode request, the parameter specifies that GSM EFR comfort noise is requested. |
| Parameter identifier value: | 3 |
| Parameter status: | Optional |
| Parameter type: | Logical |
| Supersedes: | - |

TABLE I.6/H.245

GSM AMR Capability Parameter - IS-641 comfort noise

| Parameter name: | is-641ComfortNoise |
| --- | --- |
| Parameter description: | This is a collapsing GenericParameter. In a capability, the parameter specifies whether there is a IS-641 comfort noise capability or not. In a mode request, the parameter specifies that IS-641 comfort noise is requested. |
| Parameter identifier value: | 4 |
| Parameter status: | Optional |
| Parameter type: | Logical |
| Supersedes: | - |

TABLE I.7/H.245

GSM AMR Capability Parameter - PDC EFR confort noise

| Parameter name: | pdcEFRComfortNoise |
| --- | --- |
| Parameter description: | This is a collapsing GenericParameter. In a capability, the parameter specifies whether there is a PDC EFR comfort noise capability or not. PDC EFR is a 6.7 kbit/s ACELP codec specified in section 5.4 of [155]. In a mode request, the parameter specifies that PDC EFR comfort noise is requested. |
| Parameter identifier value: | 5 |
| Parameter status: | Optional |
| Parameter type: | Logical |
| Supersedes: | - |

## I.1	Definition of mode signaling and bit stuffing to achieve octet alignment

The AMR mode signaling in mobile systems is partly based on external signaling not defined within the AMR speech codec specification. For compatibility with mobile systems this section defines the mode signaling needed for AMR use in the ITU-T H series of recommendations. The size of the speech frames of the AMR codec in the different modes is not a multiple of eight. For that reason bit stuffing is needed to achieve an octet structure.

Note, in future the content of the chapter "Definition of mode signaling and bit stuffing to achieve octet alignment" may be changed to reference to ETSI documentation or other appropriate standardization documentation.

Table I.10 maps all the AMR modes into specific mode indicies mi(k). Mode indicies are also reserved for silence suppression frames used in different systems. Tables I.11 to I.14 specify the formats for these. Table I.15 specifies a no transmission frame.

The bits delivered by the AMR speech encoder, {s(1),s(2),...,s(Ks)}, shall be rearranged according to subjective importance before they are octet aligned. Tables I.16 to I.23 define the correct rearrangement for the speech codec modes 12.2 kbit/s, 10.2 kbit/s, 7.95 kbit/s, 7.40 kbit/s, 6.70 kbit/s, 5.90 kbit/s, 5.15 kbit/s and 4.75 kbit/s, respectively. In the tables speech codec parameters are numbered in the order they are delivered by the corresponding speech encoder according to GSM 06.90 and the rearranged bits are labelled {d(0),d(1),...,d(Kd-1)}, defined in the order of decreasing importance. Index Kd refers to the number of bits delivered by the speech encoder, see Table I.8 below.

TABLE I.8/H.245

Number of speech bits in different AMR modes

| Codec Mode | Number of speech bits delivered per block (Kd) |
| --- | --- |
| AMR12.2 | 244 |
| AMR10.2 | 204 |
| AMR7.95 | 159 |
| AMR7.4 | 148 |
| AMR6.7 | 134 |
| AMR5.9 | 118 |
| AMR5.15 | 103 |
| AMR4.75 | 95 |

The ordering algorithm is in pseudo code as:

for j = 0 to Kd-1

d(j) := s(table(j)+1); 		where table(j) is read line by line left to right

Hence the octet structure bn(k) for each AMR codec mode is defined as follows:

Number of stuffing bits: 	Ks = 8*N – Kd – Ki, 	where Ki is the number of mode index bits

Octet[0]: 			b0(k) = mi(k),				for k = 0, 1, 2 ,3 (mode index)

b0(k) = d(k - 4), 			for k = 4, 5, 6, 7

Octet[n]: 			bn(k) = d( 8 * n  -  4 + k),  		for k = 0, 1, 2, 3, 4, 5, 6, 7

Octet[N-1]:		bN-1(k) = d( 8 * (N - 1) -  4 + k),		for k = 0, …,  7 - Ks

If Ks > 0

bN-1(k) = UB,			for k = 8 - Ks, …, 7

TABLE I.9/H.245

Example, mapping of the AMR speech coding mode 6.7 kbit/s.

| Octet | MSB | Octet Structure |  |  |  |  |  | LSB |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| b0 | d(3) | d(2) | d(1) | d(0) | 0 | 0 | 1 | 1 |
| b1 | d(11) | d(10) | d(9) | d(8) | d(7) | d(6) | d(5) | d(4) |
| b2 | ... | ... | ... | ... | ... | ... | ... | d(12) |
| b17 | UB | UB | UB | UB | UB | UB | d(133) | d(132) |

TABLE I.10/H.245

Mapping of the AMR speech coding modes defined in GSM 06.90 to mode index

bits in transmitted octets

| Mode_index (4 bits) | Naming in GSM 06.90 and GSM 06.92 |
| --- | --- |
| 0 (Amr4-75k) | 4.75 kbit/s mode |
| 1 (Amr5-15k) | 5.15 kbit/s mode |
| 2 (Amr5-90k) | 5.90 kbit/s mode |
| 3 (Amr6-70k) | 6.70 kbit/s mode (PDC-EFR) |
| 4 (Amr7-40k) | 7.40 kbit/s mode (IS-641) |
| 5 (Amr7-95k) | 7.95 kbit/s mode |
| 6 (Amr10-2k) | 10.2 kbit/s mode |
| 7 (Amr12-2k) | 12.2 kbit/s mode (GSM EFR) |
| 8 | GsmAmr Comfort Noise Frame (mandatory) |
| 9 | Gsm-Efr Comfort Noise Frame (optional) |
| 10 | IS-641 Comfort Noise frame (optional) |
| 11 | Pdc-Efr Comfort Noise Frame (optional) |
| 12-14 | For future use |
| 15 | No transmission |

TABLE I.11/H.245

Mapping of Comfort Noise descriptor bits from GSM 06.92 to octets for the mode index 8

(Bits from s1 to s35 refer to GSM 06.92).

| Transmitted octets | MSB | Mapping of bits |  |  |  |  |  | LSB |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Index of 1st LSF subvector | index of LSF reference vector |  |  | Mode_Index |  |  |  |
|  | s4 | s3 | s2 | s1 | mi(3) | mi(2) | mi(1) | mi(0) |
| 2 | Index of 2nd LSF subvector | index of 1st LSF subvector |  |  |  |  |  |  |
|  | s12 | s11 | s10 | s9 | s8 | s7 | s6 | s5 |
| 3 | index of 2nd LSF subvector |  |  |  |  |  |  |  |
|  | s20 | s19 | s18 | s17 | s16 | s15 | s14 | s13 |
| 4 | index of 3rd LSF subvector |  |  |  |  |  |  |  |
|  | s28 | s27 | s26 | s25 | s24 | s23 | s22 | s21 |
| 5 | SID type bit | frame energy |  |  |  |  |  | index of 3rd LSF subvector |
|  | t1 | s35 | s34 | s33 | s32 | s31 | s30 | s29 |
| 6 | Stuffing bits |  |  |  |  | Speech_Mode_Indication |  |  |
|  | UB | UB | UB | UB | UB | smi(2) | smi(1) | smi(0) |

Definitions of additional descriptor bits needed for the silence descriptor in Table I.11:

SID-type (t1) is {0=SID_FIRST, 1=SID_UPDATE }

Speech Mode Indication (smi(0)- smi(2)) is the Speech Mode according to the first eight entries in the Mode_Index table.

TABLE I.12/H.245

Mapping of silence insertion descriptor bits from GSM 06.60 (parameters also described in GSM 06.62)

to octets for the Mode Index 9 (Bits from s1 to s91 refer to GSM 06.60).

| Transmitted octets | MSB | Mapping of bits |  |  |  |  |  |  | LSB |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Index of 1st LSF subMatrix |  |  |  |  | Mode_Index |  |  |  |
|  | s4 | s3 | s2 | s1 |  | mi(3) | mi(2) | mi(1) | mi(0) |
| 2 | Index of 2nd LSF submatrix |  |  |  |  |  | index of 1st LSF subMatrix |  |  |
|  | s12 | s11 | s10 | s9 |  | s8 | s7 | s6 | s5 |
| 3 | Index of 3rd LSF submatrix |  |  |  |  |  | Index of 2nd LSF submatrix |  |  |
|  | s20 | s19 | s18 | s17 |  | s16 | s15 | s14 | s13 |
| 4 | index of 4th LSF submatrix |  |  |  | sign of 3rd LSF submatrix |  | index of 3rd LSF submatrix |  |  |
|  | s28 | s27 | s26 | s25 |  | s24 | s23 | s22 | s21 |
| 5 | index of 5th LSF submatrix |  |  |  |  | index of 4th LSF submatrix |  |  |  |
|  | s36 | s35 | s34 | s33 |  | s32 | s31 | s30 | s29 |
| 6 | Stuffing bits | fixed codebook gain |  |  |  |  |  | index of 5th LSF submatrix |  |
|  | UB | S91 | s90 | s89 |  | s88 | s87 | s38 | s37 |

TABLE I.13/H.245

Mapping of silence insertion descriptor bits from TIA IS641-A to octets, for the mode index 10

(Bits from cn0 to cn37 refer to TIA IS-641-A).

| Transmitted octets | MSB | Mapping of bits |  |  |  |  |  | LSB |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Index of 1st LSF subvector |  |  |  | Mode_Index |  |  |  |
|  | cn3 | cn2 | cn1 | cn0 | mi(3) | mi(2) | mi(1) | mi(0) |
| 2 | Index of 2nd LSF subvector |  |  |  | index of 1st LSF subvector |  |  |  |
|  | cn11 | Cn10 | cn9 | cn8 | cn7 | cn6 | cn5 | cn4 |
| 3 | Index of 3rd LSF subvector |  |  | Index of 2nd LSF subvector |  |  |  |  |
|  | cn19 | cn18 | cn17 | cn16 | cn15 | cn14 | cn13 | cn12 |
| 4 | Random Excitation Gain |  | index of 3rd LSF subvector |  |  |  |  |  |
|  | cn27 | cn26 | cn25 | cn24 | cn23 | cn22 | cn21 | cn20 |
| 5 | Index of 1st RESC parameter |  | Random Excitation Gain |  |  |  |  |  |
|  | cn35 | cn34 | cn33 | cn32 | cn31 | cn30 | cn29 | cn28 |
| 6 | Stuffing bits |  |  |  |  |  | Index of 2nd RESC parameter |  |
|  | UB | UB | UB | UB | UB | UB | cn37 | cn36 |

TABLE I.14/H.245

Mapping of silence insertion descriptor bits from RCR STD-27H to octets for the mode index 11

(Bits from s1 to s35 refer to RCR STD-27H).

| TransmittedOctets | MSB | Mapping of bits |  |  |  |  |  | LSB |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | index of 1st LSF subvector | index of LSF reference vector |  |  | Mode_Index |  |  |  |
|  | s4 | s3 | s2 | s1 | mi(3) | mi(2) | mi(1) | mi(0) |
| 2 | index of 2nd LSF subvector | index of 1st LSF subvector |  |  |  |  |  |  |
|  | s12 | s11 | s10 | s9 | s8 | s7 | s6 | s5 |
| 3 | index of 2nd LSF subvector |  |  |  |  |  |  |  |
|  | s20 | s19 | s18 | s17 | s16 | s15 | s14 | s13 |
| 4 | index of 3rd LSF subvector |  |  |  |  |  |  |  |
|  | s28 | s27 | s26 | s25 | s24 | s23 | s22 | s21 |
| 5 | SID type | frame energy |  |  |  |  |  | Index of 3rd LSF subvector |
|  | t1 | s35 | s34 | S33 | s32 | s31 | s30 | s29 |
| 6 | Stuffing bits |  |  |  |  |  |  | SID type |
|  | UB | UB | UB | UB | UB | UB | UB | t2 |

Definition of additional descriptor bits needed for PDC-EFR Table I.14:

SID-type is {0=POST0, 1=POST1(SID_UPDATE), 2=PRE, 3=POST1_BAD }, where LSB of SID_type is t1, and MSB of SID-type is t2.

TABLE I.15/H.245

The definition of the no transmission frame for the mode index 15.

| Transmitted Octets | MSB | Frame content |  |  |  |  |  | LSB |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Stuffing bits |  |  |  | Mode_Index |  |  |  |
|  | UB | UB | UB | UB | mi(3) | mi(2) | mi(1) | mi(0) |

TABLE I.16/H.245

Subjective importance of the speech encoded bits for AMR12.2.

| 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 11 | 12 | 13 | 14 | 23 | 15 | 16 | 17 | 18 |
| 19 | 20 | 21 | 22 | 24 | 25 | 26 | 27 | 28 | 38 |
| 141 | 39 | 142 | 40 | 143 | 41 | 144 | 42 | 145 | 43 |
| 146 | 44 | 147 | 45 | 148 | 46 | 149 | 47 | 97 | 150 |
| 200 | 48 | 98 | 151 | 201 | 49 | 99 | 152 | 202 | 86 |
| 136 | 189 | 239 | 87 | 137 | 190 | 240 | 88 | 138 | 191 |
| 241 | 91 | 194 | 92 | 195 | 93 | 196 | 94 | 197 | 95 |
| 198 | 29 | 30 | 31 | 32 | 33 | 34 | 35 | 50 | 100 |
| 153 | 203 | 89 | 139 | 192 | 242 | 51 | 101 | 154 | 204 |
| 55 | 105 | 158 | 208 | 90 | 140 | 193 | 243 | 59 | 109 |
| 162 | 212 | 63 | 113 | 166 | 216 | 67 | 117 | 170 | 220 |
| 36 | 37 | 54 | 53 | 52 | 58 | 57 | 56 | 62 | 61 |
| 60 | 66 | 65 | 64 | 70 | 69 | 68 | 104 | 103 | 102 |
| 108 | 107 | 106 | 112 | 111 | 110 | 116 | 115 | 114 | 120 |
| 119 | 118 | 157 | 156 | 155 | 161 | 160 | 159 | 165 | 164 |
| 163 | 169 | 168 | 167 | 173 | 172 | 171 | 207 | 206 | 205 |
| 211 | 210 | 209 | 215 | 214 | 213 | 219 | 218 | 217 | 223 |
| 222 | 221 | 73 | 72 | 71 | 76 | 75 | 74 | 79 | 78 |
| 77 | 82 | 81 | 80 | 85 | 84 | 83 | 123 | 122 | 121 |
| 126 | 125 | 124 | 129 | 128 | 127 | 132 | 131 | 130 | 135 |
| 134 | 133 | 176 | 175 | 174 | 179 | 178 | 177 | 182 | 181 |
| 180 | 185 | 184 | 183 | 188 | 187 | 186 | 226 | 225 | 224 |
| 229 | 228 | 227 | 232 | 231 | 230 | 235 | 234 | 233 | 238 |
| 237 | 236 | 96 | 199 |  |  |  |  |  |  |

TABLE I.17/H.245

Subjective importance of the speech encoded bits for AMR10.2.

| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 | 16 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 14 | 13 | 12 | 11 | 10 | 9 | 8 | 26 | 27 | 28 |
| 29 | 30 | 31 | 115 | 116 | 117 | 118 | 119 | 120 | 72 |
| 73 | 161 | 162 | 65 | 68 | 69 | 108 | 111 | 112 | 154 |
| 157 | 158 | 197 | 200 | 201 | 32 | 33 | 121 | 122 | 74 |
| 75 | 163 | 164 | 66 | 109 | 155 | 198 | 19 | 23 | 21 |
| 22 | 18 | 17 | 20 | 24 | 25 | 37 | 36 | 35 | 34 |
| 80 | 79 | 78 | 77 | 126 | 125 | 124 | 123 | 169 | 168 |
| 167 | 166 | 70 | 67 | 71 | 113 | 110 | 114 | 159 | 156 |
| 160 | 202 | 199 | 203 | 76 | 165 | 81 | 82 | 92 | 91 |
| 93 | 83 | 95 | 85 | 84 | 94 | 101 | 102 | 96 | 104 |
| 86 | 103 | 87 | 97 | 127 | 128 | 138 | 137 | 139 | 129 |
| 141 | 131 | 130 | 140 | 147 | 148 | 142 | 150 | 132 | 149 |
| 133 | 143 | 170 | 171 | 181 | 180 | 182 | 172 | 184 | 174 |
| 173 | 183 | 190 | 191 | 185 | 193 | 175 | 192 | 176 | 186 |
| 38 | 39 | 49 | 48 | 50 | 40 | 52 | 42 | 41 | 51 |
| 58 | 59 | 53 | 61 | 43 | 60 | 44 | 54 | 194 | 179 |
| 189 | 196 | 177 | 195 | 178 | 187 | 188 | 151 | 136 | 146 |
| 153 | 134 | 152 | 135 | 144 | 145 | 105 | 90 | 100 | 107 |
| 88 | 106 | 89 | 98 | 99 | 62 | 47 | 57 | 64 | 45 |
| 63 | 46 | 55 | 56 |  |  |  |  |  |  |

TABLE I.18/H.245

Subjective importance of the speech encoded bits for AMR7.95.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 14 | 16 | 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 12 | 13 | 15 | 11 | 17 | 20 | 22 | 24 | 23 |
| 19 | 18 | 21 | 56 | 88 | 122 | 154 | 57 | 89 | 123 |
| 155 | 58 | 90 | 124 | 156 | 52 | 84 | 118 | 150 | 53 |
| 85 | 119 | 151 | 27 | 93 | 28 | 94 | 29 | 95 | 30 |
| 96 | 31 | 97 | 61 | 127 | 62 | 128 | 63 | 129 | 59 |
| 91 | 125 | 157 | 32 | 98 | 64 | 130 | 1 | 0 | 25 |
| 26 | 33 | 99 | 34 | 100 | 65 | 131 | 66 | 132 | 54 |
| 86 | 120 | 152 | 60 | 92 | 126 | 158 | 55 | 87 | 121 |
| 153 | 117 | 116 | 115 | 46 | 78 | 112 | 144 | 43 | 75 |
| 109 | 141 | 40 | 72 | 106 | 138 | 36 | 68 | 102 | 134 |
| 114 | 149 | 148 | 147 | 146 | 83 | 82 | 81 | 80 | 51 |
| 50 | 49 | 48 | 47 | 45 | 44 | 42 | 39 | 35 | 79 |
| 77 | 76 | 74 | 71 | 67 | 113 | 111 | 110 | 108 | 105 |
| 101 | 145 | 143 | 142 | 140 | 137 | 133 | 41 | 73 | 107 |
| 139 | 37 | 69 | 103 | 135 | 38 | 70 | 104 | 136 |  |

TABLE I.19/H.245

Subjective importance of the speech encoded bits for AMR7.4.

| 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 11 | 12 | 13 | 14 | 15 | 16 | 26 | 87 | 27 |
| 88 | 28 | 89 | 29 | 90 | 30 | 91 | 51 | 80 | 112 |
| 141 | 52 | 81 | 113 | 142 | 54 | 83 | 115 | 144 | 55 |
| 84 | 116 | 145 | 58 | 119 | 59 | 120 | 21 | 22 | 23 |
| 17 | 18 | 19 | 31 | 60 | 92 | 121 | 56 | 85 | 117 |
| 146 | 20 | 24 | 25 | 50 | 79 | 111 | 140 | 57 | 86 |
| 118 | 147 | 49 | 78 | 110 | 139 | 48 | 77 | 53 | 82 |
| 114 | 143 | 109 | 138 | 47 | 76 | 108 | 137 | 32 | 33 |
| 61 | 62 | 93 | 94 | 122 | 123 | 41 | 42 | 43 | 44 |
| 45 | 46 | 70 | 71 | 72 | 73 | 74 | 75 | 102 | 103 |
| 104 | 105 | 106 | 107 | 131 | 132 | 133 | 134 | 135 | 136 |
| 34 | 63 | 95 | 124 | 35 | 64 | 96 | 125 | 36 | 65 |
| 97 | 126 | 37 | 66 | 98 | 127 | 38 | 67 | 99 | 128 |
| 39 | 68 | 100 | 129 | 40 | 69 | 101 | 130 |  |  |

TABLE I.20/H.245

Subjective importance of the speech encoded bits for AMR6.7.

| 0 | 1 | 4 | 3 | 5 | 6 | 13 | 7 | 2 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 9 | 11 | 15 | 12 | 14 | 10 | 28 | 82 | 29 | 83 |
| 27 | 81 | 26 | 80 | 30 | 84 | 16 | 55 | 109 | 56 |
| 110 | 31 | 85 | 57 | 111 | 48 | 73 | 102 | 127 | 32 |
| 86 | 51 | 76 | 105 | 130 | 52 | 77 | 106 | 131 | 58 |
| 112 | 33 | 87 | 19 | 23 | 53 | 78 | 107 | 132 | 21 |
| 22 | 18 | 17 | 20 | 24 | 25 | 50 | 75 | 104 | 129 |
| 47 | 72 | 101 | 126 | 54 | 79 | 108 | 133 | 46 | 71 |
| 100 | 125 | 128 | 103 | 74 | 49 | 45 | 70 | 99 | 124 |
| 42 | 67 | 96 | 121 | 39 | 64 | 93 | 118 | 38 | 63 |
| 92 | 117 | 35 | 60 | 89 | 114 | 34 | 59 | 88 | 113 |
| 44 | 69 | 98 | 123 | 43 | 68 | 97 | 122 | 41 | 66 |
| 95 | 120 | 40 | 65 | 94 | 119 | 37 | 62 | 91 | 116 |
| 36 | 61 | 90 | 115 |  |  |  |  |  |  |

TABLE I.21/H.245

Subjective importance of the speech encoded bits for AMR5.9.

| 0 | 1 | 4 | 5 | 3 | 6 | 7 | 2 | 13 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 9 | 11 | 12 | 14 | 10 | 16 | 28 | 74 | 29 |
| 75 | 27 | 73 | 26 | 72 | 30 | 76 | 51 | 97 | 50 |
| 71 | 96 | 117 | 31 | 77 | 52 | 98 | 49 | 70 | 95 |
| 116 | 53 | 99 | 32 | 78 | 33 | 79 | 48 | 69 | 94 |
| 115 | 47 | 68 | 93 | 114 | 46 | 67 | 92 | 113 | 19 |
| 21 | 23 | 22 | 18 | 17 | 20 | 24 | 111 | 43 | 89 |
| 110 | 64 | 65 | 44 | 90 | 25 | 45 | 66 | 91 | 112 |
| 54 | 100 | 40 | 61 | 86 | 107 | 39 | 60 | 85 | 106 |
| 36 | 57 | 82 | 103 | 35 | 56 | 81 | 102 | 34 | 55 |
| 80 | 101 | 42 | 63 | 88 | 109 | 41 | 62 | 87 | 108 |
| 38 | 59 | 84 | 105 | 37 | 58 | 83 | 104 |  |  |

TABLE I.22/H.245

Table A.14: Subjective importance of the speech encoded bits for AMR5.15.

| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 | 15 | 14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 13 | 12 | 11 | 10 | 9 | 8 | 23 | 24 | 25 | 26 |
| 27 | 46 | 65 | 84 | 45 | 44 | 43 | 64 | 63 | 62 |
| 83 | 82 | 81 | 102 | 101 | 100 | 42 | 61 | 80 | 99 |
| 28 | 47 | 66 | 85 | 18 | 41 | 60 | 79 | 98 | 29 |
| 48 | 67 | 17 | 20 | 22 | 40 | 59 | 78 | 97 | 21 |
| 30 | 49 | 68 | 86 | 19 | 16 | 87 | 39 | 38 | 58 |
| 57 | 77 | 35 | 54 | 73 | 92 | 76 | 96 | 95 | 36 |
| 55 | 74 | 93 | 32 | 51 | 33 | 52 | 70 | 71 | 89 |
| 90 | 31 | 50 | 69 | 88 | 37 | 56 | 75 | 94 | 34 |
| 53 | 72 | 91 |  |  |  |  |  |  |  |

TABLE I.23/H.245

Subjective importance of the speech encoded bits for AMR4.75.

| 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 11 | 12 | 13 | 14 | 15 | 23 | 24 | 25 | 26 |
| 27 | 28 | 48 | 49 | 61 | 62 | 82 | 83 | 47 | 46 |
| 45 | 44 | 81 | 80 | 79 | 78 | 17 | 18 | 20 | 22 |
| 77 | 76 | 75 | 74 | 29 | 30 | 43 | 42 | 41 | 40 |
| 38 | 39 | 16 | 19 | 21 | 50 | 51 | 59 | 60 | 63 |
| 64 | 72 | 73 | 84 | 85 | 93 | 94 | 32 | 33 | 35 |
| 36 | 53 | 54 | 56 | 57 | 66 | 67 | 69 | 70 | 87 |
| 88 | 90 | 91 | 34 | 55 | 68 | 89 | 37 | 58 | 71 |
| 92 | 31 | 52 | 65 | 86 |  |  |  |  |  |

ANNEX J

# Annex J	TDMA ACELP Voice Codec Definitions

A==

(This annex forms an integral part of this Recommendation)

Table I.1 defines the capability identifier for TIA/EIA 136 ACELP voice codec Capabilities [156]. Tables I.2 to I.4 define the associated capability parameters. This codec is used in TDMA Cellular and PCS base stations and mobile phones. The technical specifications of this codec are provided in TIA/EIA standard 136 Part 410. This standard is published by the TIA (North American) Telecommunications Industry Association and endorsed by ANSI, the American National Standards Institute.

TABLE J.1/H.245

136 ACELP Capability Identifier

| Capability name: | 136 ACELP Vocoder |
| --- | --- |
| Capability class: | Audio codec. |
| Capability identifier type: | Standard. |
| Capability identifier value: | {itu-t (0) recommendation (0) h (8) 245 generic-capabilities (1) audio (1) acelp (2)} |
| maxBitRate: | This field shall be included |
| NonCollapsingRaw: | This field shall not be included |
| transport: | This field shall not be included |

TABLE J.2/H.245

136 ACELP Capability Parameter - maxAl-sduFrames

| Parameter name: | MaxAl-sduFrames |
| --- | --- |
| Parameter description: | Specifies the maximum number of audio frames per AL-SDU |
| Parameter identifier value: | 0 |
| Parameter status: | Shall be present for capability exchange and logical channel signalling. Shall not be present for mode request. |
| Parameter type: | UnsignedMin |
| Supersedes: | - |

TABLE J.3/H.245

136 ACELP Capability Parameter - Comfort Noise

| Parameter name: | ComfortNoise |
| --- | --- |
| Parameter description: | Specifies that TIA/EIA 136 (IS-641) comfort noise is to be used in mode request. This parameter shall be used only in mode requests but not in capabilities because this capability is mandatory. |
| Parameter identifier value: | 1 |
| Parameter status: | Optional |
| Parameter type: | Logical |
| Supersedes: | - |

TABLE J.4/H.245

136 ACELP Capability Parameter - Scrambled

| Parameter name: | Scrambled |
| --- | --- |
| Parameter description: | Specifies that scrambling is to be used in mode request. This parameter shall be used only in mode requests but not in capabilities because this capability is mandatory. |
| Parameter identifier value: | 2 |
| Parameter status: | Optional |
| Parameter type: | Logical |
| Supersedes: | - |

ANNEX K

# Annex K	TDMA US1 Voice Codec Definitions

A==

(This annex forms an integral part of this Recommendation)

Table K.1 defines the capability identifier for TIA/EIA 136 US1 voice codec Capabilities [157]. Tables K.2 to K.4 define the associated capability parameters. This codec is used in TDMA Cellular and PCS base stations and mobile phones. The technical specifications of this codec are provided in TIA/EIA standard 136 Part 430. Thi standard is published by TIA - the (North American) Telecommunications Industry Association and endorsed by ANSI, The American National Standards Institute.

TABLE K.1/H.245

136 US1 Capability Identifier

| Capability name: | 136 US1 Vocoder |
| --- | --- |
| Capability class: | Audio codec. |
| Capability identifier type: | Standard. |
| Capability identifier value: | {itu-t (0) recommendation (0) h (8) 245 generic-capabilities (1) audio (1) us1 (3)} |
| maxBitRate: | This field shall be included |
| NonCollapsingRaw: | This field shall not be included |
| transport: | This field shall not be included |

TABLE K.2/H.245

136 US1 Capability Parameter - maxAl-sduFrames

| Parameter name: | MaxAl-sduFrames |
| --- | --- |
| Parameter description: | Specifies the maximum number of audio frames per AL-SDU |
| Parameter identifier value: | 0 |
| Parameter status: | Shall be present for capability exchange and logical channel signalling. Shall not be present for mode request. |
| Parameter type: | UnsignedMin |
| Supersedes: | - |

TABLE K.3/H.245

136 US1Capability Parameter - Comfort Noise

| Parameter name: | ComfortNoise |
| --- | --- |
| Parameter description: | Specifies that comfort noise is to be used in mode request. This parameter shall be used only in mode requests but not in capabilities because this capability is mandatory. |
| Parameter identifier value: | 1 |
| Parameter status: | Optional |
| Parameter type: | Logical |
| Supersedes: | - |

TABLE K.4/H.245

136 US1 Capability Parameter - Scrambled

| Parameter name: | Scrambled |
| --- | --- |
| Parameter description: | Specifies that scrambling is to be used in mode request. This parameter shall be used only in mode requests but not in capabilities because this capability is mandatory. |
| Parameter identifier value: | 2 |
| Parameter status: | Optional |
| Parameter type: | Logical |
| Supersedes: | - |

ANNEX L

# Annex L	CDMA  EVRC Voice Codec Definitions

A==

(This annex forms an integral part of this Recommendation)

Table L.1 defines the capability identifier for TIA/EIA IS-127 Enhanced Variable Rate Codec (EVRC)  which is used in TIA/EIA IS-95 CDMA Cellular and PCS base stations and mobile phones. The full technical description and detailed specifications of this codec are provided in TIA/EIA IS-127 standard which is published by the North American Telecommunications Industry Association (TIA). Tables L.2 to L.4 define the associated capability parameters.

TABLE L.1/H.245

CDMA EVRC Capability Identifier

| Capability name: | IS-127 CDMA EVRC |
| --- | --- |
| Capability class: | Audio codec. |
| Capability identifier type: | Standard. |
| Capability identifier value: | {itu-t (0) recommendation (0) h (8) 245 generic-capabilities (1) audio (1) is127evrc (4)} |
| maxBitRate: | This field shall be included |
| NonCollapsingRaw: | This field shall not be included |
| transport: | This field shall not be included |

TABLE L.2/H.245

IS-127 CDMA EVRC Capability Parameter - maxAl-sduFrames

| Parameter name: | MaxAl-sduFrames |
| --- | --- |
| Parameter description: | Specifies the maximum number of audio frames per AL-SDU |
| Parameter identifier value: | 0 |
| Parameter status: | Shall be present for capability exchange and logical channel signalling. Shall not be present for mode request. |
| Parameter type: | UnsignedMin |
| Supersedes: | - |

TABLE L.3/H.245

CDMA EVRC Capability Parameter - EVRC Bit-Rate

| Parameter name: | EVRCRate |
| --- | --- |
| Parameter description: | This is a non-collapsing GenericParameter. It specifies the vocoder output bit-rate mode. This parameter shall be used in mode requests: 1=full-rate; 2=half-rate; 3=eighth-rate; 4=blanked mode. |
| Parameter identifier value: | 1 |
| Parameter status: | Optional |
| Parameter type: | UnsignedMin |
| Supersedes: | - |

TABLE L.4/H.245

CDMA EVRC Capability Parameter - Scrambled

| Parameter name: | Scrambled |
| --- | --- |
| Parameter description: | Specifies that scrambling is to be used in mode request. |
| Parameter identifier value: | 2 |
| Parameter status: | Optional |
| Parameter type: | Logical |
| Supersedes: | - |

APPENDIX I

# Appendix I	Overview of ASN.1 syntax

(This appendix does not form an integral part of this Recommendation)

## I.1	Introduction to ASN.1

Abstract Syntax Notation One (ASN.1) is a data specification language. It was originally standardized as part of the X.400 electronic mail series as X.409. This evolved to X.208 and most recently X.680. ASN.1 allows unambiguous specification of complex data structures including those with variable-length fields, optional fields and recursion.

The above Recommendations deal only with the syntax and semantics of ASN.1 specifications. The binary encoding of data structures is covered in other Recommendations, notably X.690 (basic encoding rules or BER) and X.691 (packed encoding rules or PER). BER allows data to be deciphered by systems that have general knowledge of ASN.1 but do not know the details of the specification used to form the data. In other words, the data types are encoded along with the data values. PER is much more efficient since only data values are encoded and the coding is designed with very little redundancy. This method can be used when both the transmitter and the receiver expect data to adhere to a known structure.

H.245 is implemented using the packed encoding rules. Since both sides of a call know that messages will conform to the H.245 specification it is not necessary to encode that specification into the messages. For decoding simplicity, the aligned variant of PER is used. This forces fields that require eight or more bits to be aligned on octet boundaries and to consume an integral number of octets. Alignment is done by padding the data with zeros before large fields.

## I.2	Basic ASN.1 data types

The simplest data type is BOOLEAN, which represents the values FALSE and TRUE. These are encoded in a single bit as 0 and 1 respectively. For example, segmentableFlag BOOLEAN is coded

| Value | Encoding |
| --- | --- |
| FALSE | 0 |
| TRUE | 1 |

The most fundamental data type is INTEGER, which represents whole number values. Integers can be unconstrained as in:

bitRate	INTEGER

or they can be constrained to a range of values, for example:

maximumAl2SDUSize	INTEGER (0..65535)

Constrained integers are encoded differently depending on the size of the range. Suppose N is the number of integers in the range, i.e. the upper limit minus the lower limit plus one. Depending on N, the constrained integer will be encoded in one of five ways:

| N | Encoding |
| --- | --- |
| 1 | no bits needed |
| 2-255 | an unaligned field of 1 to 8 bits |
| 256 | an aligned 8-bit field |
| 257-65536 | an aligned 16-bit field |
| larger | as the minimum number of aligned octets preceded by the above encoding of the number of octets |

In all cases, the number that is actually used is the value to be encoded minus the lower limit of the range. In these examples "pad" represents zero to seven 0 bits that are added to the encoding so that the following field will start on a 8-bit boundary.

firstGOB	INTEGER (0..17)

| Value | Encoding |
| --- | --- |
| 0 | 00000 |
| 3 | 00011 |

h233IVResponseTime	INTEGER (0..255)

| Value | Encoding |
| --- | --- |
| 3 | pad 00000011 |
| 254 | pad 11111110 |

skew	INTEGER (0..4095)

| Value | Encoding |
| --- | --- |
| 3 | pad 00000000 00000011 |
| 4095 | pad 00001111 11111111 |

Unconstrained (2's complement) integer values that can be represented in 127 octets or less are encoded in the minimum number of octets needed. The number of octets (the length) is encoded as an aligned octet that precedes the number itself. For example,

| -1 | pad 00000001 11111111 |
| --- | --- |
| 0 | pad 00000001 00000000 |
| 128 | pad 00000010 00000000 10000000 |
| 1000000 | pad 00000011 00001111 01000010 01000000 |

ASN.1 supports a variety of string data types. These are variable-length lists of bits, octets or other short data types. They are typically encoded as a length followed by the data. The length can be encoded as an unconstrained integer or as a constrained integer if the SIZE of the string is specified. For example,

data		OCTET STRING

Since the length of the octet string is not bounded, it will have to be encoded as a semi-constrained whole number (has a lower bound, but no upper bound). First, the data is padded so that the encoding will be aligned. The rest of the code is as follows:

| Length | Encoding |
| --- | --- |
| 0 to 127 | 8-bit length followed by the data |
| 128 to 16K-1 | 16-bit length with the MSB set, then the data |
| 16K to 32K-1 | 11000001, 16K octets of data, then code the rest |
| 32K to 48K-1 | 11000010, 32K octets of data, then code the rest |
| 48K to 64K-1 | 11000011, 48K octets of data, then code the rest |
| 64K or more | 11000100, 64K octets of data, then code the rest |

This method is called "fragmentation". Note that if the length is a multiple of 16K, then the representation will end with an octet of zero indicating a zero-length string.

## I.3	Aggregate data types

ASN.1 includes several aggregate or container data types that are similar in concept to C's union, struct and array types. These are, respectively, CHOICE, SEQUENCE and SEQUENCE OF. In all cases they are encoded with some bits specific to the container followed by the normal encodings of the contents.

CHOICE is used to select exactly one of a group of data types. For example,

VideoCapability	::= CHOICE

{

nonStandard	NonStandardParameter,

h261VideoCapability	H261VideoCapability,

h262VideoCapability	H262VideoCapability,

h263VideoCapability	H263VideoCapability,

is11172VideoCapability	IS11172VideoCapability,

...

}

An index number is assigned to each choice, starting with zero. The index of the actual choice is encoded as a constrained integer. The index is followed by the encoding of the actual selection or nothing if the selection is NULL. If the extension marker is present (as above), the index is preceded by a bit that is zero if the actual choice is from the original list.

SEQUENCE is simply a grouping of dissimilar data types. Individual elements of the sequence may be OPTIONAL. The encoding is very simple. If there is an extension marker the first bit indicates the presence of additional elements. This is followed by a series of bits, one for each optional element that indicates if that data is present. This is followed by the encodings of the components of the sequence. For example,

H261VideoCapability	::= SEQUENCE

{

qcifMPI		INTEGER (1..4) OPTIONAL,	-- units 1/29.97 Hz

cifMPI		INTEGER (1..4) OPTIONAL, 	-- units 1/29.97 Hz

temporalSpatialTradeOffCapability	BOOLEAN,

...

}

The encoding has one bit for the extension marker, two bits for the optional fields, two bits each for any optional field that is present, one bit for the boolean and then any extension data. Note that in this sequence has no padding for octet alignment.

The SEQUENCE OF and SET OF types describe a collection of similar components (an array). SEQUENCE OF implies that the order of the elements is significant, with SET OF the element order is arbitrary. The PER encoding is the same for both types.

These types can have a SIZE constraint or an unconstrained number of elements. If the number is known a priori and is less than 64K, it is not encoded. Otherwise the actual number of components is encoded as a constrained or semi-constrained length. This is followed by the encoding of the data. If the length is at least 16K and is encoded then the list of data will be broken into fragments like the octet string. In this case the fragments are broken after some number of component fields (16K, 32K, etc.), not after some number of octets.

## I.4	Object identifier type

Normally the type of a value is given in the ASN.1 specification so that the only information that needs to be coded and transmitted is the data itself. Occasionally, however, it is desirable to encode the data type as well as the data value. For example, protocolIdentifier contains

protocolIdentifier	OBJECT IDENTIFIER,

-- shall be set to the value

-- {itu-t (0) recommendation (0) h (8) 245 version (0) 1}

This is encoded as the data encoded with the BER (X.690) preceded by the length of that encoding in octets. The length is encoded as a semi-constrained whole number (see the OCTET STRING example above). The following illustrates how this is encoded.

The first octet indicates the length of the encoding that follows.

The first two components of the object identifier are combined together as 40*first one + second one, in this case 40*0+0 = 0. The others are encoded as they are. Each is encoded into a series of octets, the first bit of which indicates whether there is any more. So

0 -> 0000 0000,

8 -> 0000 0100,

while 245, being more than 127 becomes 1000 0001 0111 0101,

So the entire encoding in hexadecimal consists of the seven octets 06000881 750001

APPENDIX II

# Appendix II	Examples of H.245 procedures

(This appendix does not form an integral part of this Recommendation)

## II.1	Introduction

This appendix illustrates examples of the procedures defined in 8/H.245. Figure II.1-1 shows the key to diagrams used in this appendix.

FIGURE II.1-1

Key to figures

## II.2	Master Slave Determination Signalling Entity

In the following figures messages are represented by the shortened names given in Table 2-1.

Table 2-1/H.245

Master slave determination shortened names

| message | name in examples |
| --- | --- |
| MasterSlaveDetermination | MSD |
| MasterSlaveDeterminationAck | MSDAck |
| MasterSlaveDeterminationReject | MSDReject |
| MasterSlaveDeterminationRelease | MSDRelease |

In the following figures IDLE, OUTGOING AWAITING RESPONSE, and INCOMING AWAITING RESPONSE states are labelled as "0", "1", and "2" respectively.

In the following figures the parameter value associated with the DETERMINE.indication and DETERMINE.confirm primitives is that of the TYPE parameter. The field value associated with the MasterSlaveDeterminationAck message is that of the decision field.

FIGURE II.2-1/H.245

Master slave determination - master at remote MSDSE

FIGURE II.2-2/H.245

Master slave determination - slave at remote MSDSE

FIGURE II.2-3/H.245

Master slave determination - first attempt produced an indeterminate result. The second attempt was successful.

FIGURE II.2-4/H.245

Master slave determination - simultaneous determination

FIGURE II.2-5/H.245

Master slave determination - simultaneous determination but with the first attempt returning an indeterminate result.

In Figure II.2-6 local timer T106 has expired. Only the terminal on the right knows its status. The terminal on the right is able to receive new commands but may not request anything of the other terminal that relies on knowledge of the status determination result. The terminal on the left can neither accept nor initiate new procedures. A second status determination procedure should be initiated.

FIGURE II.2-6/H.245

Master slave determination - local timer T106 expiry with slave at remote end.

In Figure II.2-7 remote timer T106 has expired during the INCOMING AWAITING ACKNOWLEDGEMENT state. Both terminals know their status. The terminal on the left may receive and issue commands. However the remote terminal does not know if the local terminal is ready to receive, and can not issue commands that rely on knowledge of the status determination result. A second status determination procedure should be initiated.

FIGURE II.2-7/H.245

Master slave determination - remote timer T106 expiry with master at remote end.

In Figure II.2-8 remote timer T106 has expired during the OUTGOING AWAITING ACKNOWLEDGEMENT state during a simultaneous determination procedure. Both terminals know their status. The terminal on the right can receive and issue commands. However the terminal on the left does not know if the other terminal is ready to receive, and can not issue commands that rely on knowledge of the status determination result. It may receive such commands. A second status determination procedure should be initiated.

FIGURE II.2-8/H.245

Master slave determination - simultaneous determination procedures with timer T106 expiry at slave.

In Figure II.2-9 remote timer T106 has expired during the INCOMING AWAITING ACKNOWLEDGEMENT state, during a simultaneous determination procedure. Both terminals know their status. The terminal on the left can receive and issue commands. However the terminal on the right does not know if the other terminal is ready to receive, and can not issue commands that rely on knowledge of the status determination result. It may receive such commands. A second status determination procedure should be initiated.

FIGURE II.2-9/H.245

Master slave determination - simultaneous determination procedures with timer T106 expiry during INCOMING AWAITING ACKNOWLEDGEMENT.

In Figure II.2-10 an indeterminate result was obtained N100 times. In this case N100 = 3.

FIGURE II.2-10/H.245

Master slave determination - indeterminate result with N100 = 3.

## II.3	Capability Exchange Signalling Entity

The following figures illustrate CESE procedures. The IDLE and AWAITING RESPONSE states are labelled as "0" and "1" respectively.

FIGURE II.3-1/H.245

Capability exchange with acceptance from the peer incoming CESE user

FIGURE II.3-2/H.245

Capability exchange with rejection from peer incoming CESE user.

FIGURE II.3-3/H.245

Capability exchange with timer T101 expiry. The TerminalCapabilitySetRelease message arrives at the incoming CESE before response from the incoming CESE user.

FIGURE II.3-4/H.245

Capability exchange with timer T101 expiry followed by a second capability exchange. The TerminalCapabilitySetRelease message arrives at the incoming CESE after response from the incoming CESE user. At the outgoing CESE the TerminalCapabilitySetAck message in response to the first TerminalCapabilitySet message is ignored. Only the second capability exchange is successful.

## II.4	Logical Channel Signalling Entity

The following figures illustrate LCSE procedures. The outgoing LCSE states of RELEASED, AWAITING ESTABLISHMENT, ESTABLISHED, and AWAITING RELEASE are labelled as "0", "1", "2", and "3" respectively. The incoming LCSE states of RELEASED, AWAITING ESTABLISHMENT, and ESTABLISHED, are labelled as "0", "1", and "2" respectively.

FIGURE II.4-1/H.245

Logical channel establishment

FIGURE II.4-2/H.245

Logical channel release

FIGURE II.4-3/H.245

Logical channel establishment rejection by peer LCSE user

FIGURE II.4-4/H.245

Logical channel release followed by immediate re establishment

FIGURE II.4-5/H.245

Logical channel establishment request with expiry of timer T103 due to slow response from peer incoming LCSE user.

FIGURE II.4-6/H.245

Logical channel establishment request with expiry of timer T103. Timer T103 has expired after transmission of the OpenLogicalChannelAck message at the incoming LCSE, but before reception of the OpenLogicalChannelAck message at the outgoing LCSE.

FIGURE II.4-7/H.245

Logical channel release request with expiry of timer T103.

## II.5	Close Logical Channel Signalling Entity

The following figures illustrate CLCSE procedures. The IDLE and AWAITING RESPONSE states are labelled as "0" and "1" respectively.

FIGURE II.5-1/H.245

Close logical channel request

FIGURE II.5-2/H.245

Close logical channel request with rejection from peer incoming CLCSE user.

FIGURE II.5-3/H.245

Close logical channel request with timer T108 expiry. The RequestChannelCloseRelease message arrives at the incoming CLCSE before response from the incoming CLCSE user.

FIGURE II.5-4/H.245

Close logical channel request with timer T108 expiry followed by a second close logical channel request. The close channel request is confirmed on reception of the first RequestChannelClose message.

## II.6	Multiplex Table Signalling Entity

The following figures illustrate MTSE procedures. The IDLE and AWAITING RESPONSE states are labelled as "0" and "1" respectively.

FIGURE II.6-1/H.245

Successful multiplex table send request

FIGURE II.6-2/H.245

Multiplex table send request with rejection from the peer MTSE user.

FIGURE II.6-3/H.245

Multiplex table send request with a second multiplex table send request before acknowledgement of the first request. The first request was unsuccessful.

FIGURE II.6-4/H.245

Multiplex table send request with timer T104 expiry.

FIGURE II.6-5/H.245

Multiplex table send request with timer T104 expiry followed by a second multiplex table send request. The first MultiplexEntrySendAck message is ignored at the outgoing MTSE. Only the second request was successful.

## II.7	Mode Request Signalling Entity

The following figures illustrate MTSE exchanges. The IDLE and AWAITING RESPONSE states are labelled as "0" and "1" respectively.

FIGURE II.7-1/H.245

Successful mode request.

FIGURE II.7-2/H.245

Mode request with rejection from the peer MTSE user.

FIGURE II.7-3/H.245

Mode request with a second mode request before acknowledgement of the first request. The first request was unsuccessful.

FIGURE II.7-4/H.245

Mode request with timer T109 expiry. The mode request was unsuccessful.

FIGURE II.7-5/H.245

Mode request with timer T109 expiry followed by a second mode request. The first RequestModeAck message is ignored at the outgoing MRSE. Only the second request was successful.

## II.8	Round Trip Delay Signalling Entity

The following figures illustrate RTDSE procedures. The RTDSE states of IDLE and AWAITING RESPONSE are labelled as "0" and "1" respectively.

FIGURE II.8-1/H.245

Round trip delay determination procedure

FIGURE II.8-2/H.245

Round trip delay determination procedure with an earlier unacknowledged round trip delay procedure outstanding.

FIGURE II.8-3/H.245

Round trip delay determination procedure with timer T105 expiry.

FIGURE II.8-4/H.245

Round trip delay determination procedure with timer T105 expiry, followed by a second round trip delay determination procedure. The RoundTripDelayResponse message from the first procedure arrives during the second procedure and is ignored.

## II.9.	Bi-directional Logical Channel Signalling Entity

The following figures illustrate B-LCSE procedures.  The outgoing B-LCSE states of RELEASED, AWAITING ESTABLISHMENT, ESTABLISHED, and AWAITING RELEASE are labelled as "0", "1", "2", and "3" respectively.  The incoming B-LCSE states of RELEASED, AWAITING ESTABLISHMENT, AWAITING CONFIRMATION, and ESTABLISHED, are labelled as "0", "1", "2", and "3" respectively.

FIGURE II.9-1/H.245

Bi-directional logical channel establishment

FIGURE II.9-2/H.245

Bi-directional logical channel release

FIGURE II.9-3/H.245

Bi-directional logical channel establishment rejection by peer B-LCSE user

FIGURE II.9-4/H.245

Bi-directional logical channel release followed by immediate re establishment

FIGURE II.9-5/H.245

Bi-directional logical channel establishment request with expiry of timer T103 at the outgoing side due to slow response from peer incoming B-LCSE user.

FIGURE II.9-6/H.245

Bi-directional logical channel establishment request with expiry of timer T103 at the outgoing side.  Timer T013 at the outgoing side has expired after transmission of the OpenLogicalChannelAck message at the incoming B-LCSE, but before reception of the OpenLogicalChannelAck message at the outgoing B-LCSE.

FIGURE II.9-7/H.245

Bi-directional logical channel release request with expiry of timer T103 at the outgoing side.

APPENDIX III

(This appendix does not form an integral part of this Recommendation)

# Appendix III	Summary of procedure timers and counters

This appendix provides a list of the timers and counters specified in section C.

This Recommendation does not define the values loaded into these timers. The values may be defined in other Recommendations such as H.310, H.323 and H.324.

## III.1	Timers

Table III.1 shows the timers specified in this Recommendation

TABLE III.1/ H.245

Procedure Timers

| Timer | Procedure | Definition |
| --- | --- | --- |
| T106 | Master Slave Determination | This timer is used in the OUTGOING AWAITING RESPONSE state and during the INCOMING AWAITING RESPONSE state. It specifies the maximum time during which no acknowledgement message may be received. |
| T101 | Capability Exchange | This timer is used in the AWAITING RESPONSE state. It specifies the maximum time during which no TerminalCapabilitySetAck or TerminalCapabilitySetReject message may be received. |
| T103 | Uni-directional and Bi-directional Logical Channel Signalling | This timer is used in the AWAITING ESTABLISHMENT and AWAITING RELEASE states. It specifies the maximum time during which no OpenLogicalChannelAck or OpenLogicalChannelReject or CloseLogicalChannelAck message may be received. |
| T108 | Close Logical Channel | This timer is used in the AWAITING RESPONSE state. It specifies the maximum time during which no RequestChannelCloseAck or RequestChannelCloseReject message may be received. |
| T104 | H.223 Multiplex Table | This timer is used in the AWAITING RESPONSE state. It specifies the maximum time during which no MultiplexEntrySendAck or MultiplexEntrySendReject message may be received. |
| T109 | Mode Request | This timer is used in the AWAITING RESPONSE state. It specifies the maximum time during which no RequestModeAck or RequestModeReject message may be received. |
| T105 | Round Trip Delay | This timer is used in the AWAITING RESPONSE state. It specifies the maximum time during which no RoundTripDelayResponse message may be received. |
| T107 | Request Multiplex Entry | This timer is used during the AWAITING RESPONSE state. It specifies the maximum time during which no RequestMultiplexEntryAck or RequestMultiplexEntryReject message may be received. |
| T102 | Maintenance Loop | This timer is used during the AWAITING RESPONSE state. It specifies the maximum allowed time during which no MaintenanceLoopAck or MaintenanceLoopReject message may be received. |

## III.2	Counters

Table III.2 shows the counters specified in this Recommendation

TABLE III.2/ H.245

Procedure Counters

| Timer | Procedure | Definition |
| --- | --- | --- |
| N100 | Master slave determination | This counter specifies the maximum number of times that MasterSlaveDetermination messages will be sent during the OUTGOING AWAITING RESPONSE state. |

APPENDIX IV

# Appendix IV	H.245 Extension Procedure

(This appendix does not form an integral part of this Recommendation)

H.245 is a “living document” used by a number of systems Recommendations including H.310, H.323, H.324, and V.70, which is expected to be extended, in a backward-compatible way, likely at each meeting of ITU-T Study Group 16. This appendix explains the procedure that should be used to add extensions to H.245.

At a given point in time there is only one H.245 syntax in force. No other ITU-T Recommendation should include other variants of H.245 syntax in their Recommendations in a normative manner.

Requests for extensions to H.245 should be submitted as a White Contribution or formal liaison to Study Group 16, with a copy sent as early as possible to H.245 Rapporteur and editor. Such requests should include:

1.	functional requirements for syntax to be drafted by H.245 editor or proposed syntax based on the current approved version of H.245; and

2.	proposed semantics for section 7/H.245; and

3.	proposed procedures for section 8/H.245 if new procedures are requested.

All extensions to H.245 must be backwards compatible with all previous versions of H.245. Pre-existing syntax, semantics, and procedures cannot be changed.  The meaning of pre-existing syntax cannot be changed. Specifically, when an H.245 Capability is extended, the extension shall not change the meaning of the original capability in such a way that a terminal which does not understand the extension would need to modify its operation to use the capability without the extension. All ASN.1 extension components should be constrained.

Requests should be submitted as early as possible to allow time for review of extensions by H.245 experts in Study group 16. It must be understood that the exact requested syntax may be modified because of:

1.	Verification of correct ASN.1 syntax

2.	Harmonization with other, conflicting, requests for H.245 extensions

3.	Backward compatibility with pre-existing versions of H.245

4.	Expert review of placement of new functions relative to the existing H.245 structure

5.	Naming that is inconsistent with pre-existing syntax

6.	Unconstrained or ambiguous ASN.1 components

Abbreviations and acronyms should be avoided, especially if a word or phrase is not abbreviated or expressed as an acronym in pre-existing syntax. For example, the word, Parameters, should not be abbreviated as Params. If a word has been used in pre-existing syntax, do not use another word with the same meaning. For example, call components of an aggregate type, Entry, instead of Item, because Entry has consistently been used to describe this. Be consistent.

Although all ASN.1 components should be constrained, how to constrain the most common types is described below.

Constrain SET OF and SEQUENCE OF ASN.1 components by providing either a minimum and maximum or a fixed size. If there is no inherent maximum based upon a component's semantics, choose a reasonable, although arbitrary, maximum such as 256. If a SET OF or SEQUENCE OF component is OPTIONAL, specify a non-zero minimum unless there is a semantic difference between the cases, present-but-empty and not present, in which case the semantic difference should be described. If a request for extensions contains SET OF or SEQUENCE OF components that are not constrained, the editor may use SIZE (1..256) as a default constraint.

Constrain ASN.1 character string components by providing a size, either a minimum and maximum or a fixed size. If a request for extensions contains character string components that are not constrained, the editor may use SIZE (0..255) as a default constraint.

Constrain INTEGER components by providing a range of values. If there is no inherent range based upon a component's semantics, choose a reasonable, although arbitrary, range whose maximum value is chosen from the following:

255	(28 - 1)

65535	(216 - 1)

16777215	(224 - 1)

4294967295	(232 - 1)

If a request for extensions contains INTEGER components that are not constrained, the editor may use INTEGER (0..4294967295) as a default.

H.245 editor will review all extension requests and propose final text for extended versions of H.245 for Study Group 16 approval by the Resolution 1 process. Upon Study Group approval of each new version of H.245, H.245 version number in protocolIdentifier will be incremented to identify the new version.

Please note that it is the intention of Study Group 16 to accept only harmonized H.245 extensions originating from H.245 editor.

APPENDIX V

# Appendix V	The ReplacementFor Procedure

(This appendix does not form an integral part of this Recommendation)

The H.245 replacementFor procedure allows seamless changing of modes from one codec to another without the need for two media decoders. This procedure may be used only if the receiving terminal has indicated the maxPendingReplacementFor capability.

Since opening and closing of H.245 logical channels is not synchronized with media content, media dropout can occur between the time of closing a logical channel and the opening of its replacement. The replacementFor parameter allows the avoidance of such media dropout.

Example

Suppose logical channel 723 is open, carrying G.723.1 audio, and it is desired to switch to G.711 (on logical channel 711), but the receiver has a capability for only one audio channel. The replacementFor procedure may be used by the transmitter to effect a seamless mode change as follows:

1.	Only for the case of H.323 using RSVP, since the new channel will require more bandwidth (64 kbps) than the existing channel (6.4 kbps), the transmitter and receiver establish a larger RSVP bandwidth reservation.

2.	The transmitter sends OpenLogicalchannel for the new logical channel 711, including the replacementFor parameter, referring to the existing logical channel 723.

This tells the receiver that logical channel 711 is a replacement for logical channel 723, and that logical channel 711 will never carry traffic simultaneously with logical channel 723.

3.	While continuing to decode G.723.1 from logical channel 723, the receiver prepares for a seamless switch to decoding G.711.

Such preparation might include loading appropriate decoder software.

When the receiver has completed preparations to accept the G.711 audio stream, it responds with OpenLogicalChannelAck for logical channel 711. For H.323, the media and media control transport addresses returned are the same as those already used for logical channel 723.

4.	The transmitter stops sending G.723 audio on logical channel 723 and seamlessly begins sending G.711 audio on logical channel 711.

5.	The transmitter immediately sends CloseLogicalChannel for logical channel 723, as this logical channel is no longer carrying any traffic, and is no longer needed.

6.	Only for the case of H.323 using RSVP, if the new channel requires less bandwidth than the original channel, the transmitter and receiver establish a smaller RSVP bandwidth reservation (does not apply in this example).

In all cases, LCSE and B-LCSE operations conform to normal procedures. The replacementFor parameter merely informs the receiver of the pending mode change and that the two logical channels will not be used simultaneously, and therefore that the second logical channel can (in some implementations) be accepted in cases where it would otherwise be rejected (for lack of the capability to receive another independent logical channel).

Note that in some cases the receiver may reject the attempt to open the logical channel using the replacementFor mechanism (for example, if a receiver can accept the replacementFor mechanism for audio channels, but not for video channels). In that case transmitters should re-try the mode change without replacementFor, for example by closing the channel, then opening a new one, accepting any temporary media dropout.

Note also that in H.323 systems, receivers are required to re-use the existing media and media control transport addresses. The switch over point to the new logical channel is marked by the RTP header.

APPENDIX VI

# Appendix VI	Examples of H.263 Capability Structure Settings

(This appendix does not form an integral part of this Recommendation)

To clarify the usage of the H.263 Capability Structure, a number of examples are given in this appendix.

## VI.1	Examples of Enhancement Layer H.245 parameter setting

Table VI.1 shows the following example settings of parameters of enhancement layer parameters.

Example # 1: This signals a simple H.263 base video capability at 10 frames/s, maximum bitrate of 20k bps with no options.

Example # 2: These parameters settings signal the capability of a logical channel stream with a spatial enhancement layer at QCIF resolution, 10 frames/s at a maximum bitrate of 5kbps and no other options set.

Example # 3: These parameters settings signal the capability of a logical channel stream with a SNR enhancement layer at SQCIF resolution, 10 frames/s at a maximum bitrate of 5kbps and no other options set.

Example # 4: These parameters settings signal the capability of a logical channel stream with a three enhancment layers. Two SNR enhancement layers, one at SQCIF the other a QCIF, at 10 frames/s and no other options set and the other a spatial enhancement layer at CIF resolution, 10 frames/s and no other options set; all three combined at a maximum bitrate of 15kbps.

Example # 5: These parameters settings signal the capability of a logical channel stream with a three enhancement layers and a base layer at a maximum bitrate of 25kbps. The base layer in QCIF with no options. In addition the terminal is capable of one snr enhancement layer at QCIF, 10 frames/s and no other options set, one SNR enhancement layer at cif resolution, 10 frames/s with no other options set and a spatial enhancement layer at CIF resolution, 10 frames/s and no other options set.

TABLE VI.2/H.245

Enhancement Layer H.245 parameter setting examples.

|  |  | Examples |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| H263Capability parameter |  | 1 | 2 | 3 | 4 |  |  | 5 |  |  |
| sqcifMPI |  | 3 | NP | NP | NP |  |  | NP |  |  |
| qcifMPI |  | NP | NP | NP | NP |  |  | 3 |  |  |
| cifMPI |  | NP | NP | NP | NP |  |  | NP |  |  |
| cif4MPI |  | NP | NP | NP | NP |  |  | NP |  |  |
| cif16MPI |  | NP | NP | NP | NP |  |  | NP |  |  |
| maxBitRate |  | 200 | 50 | 50 | 150 |  |  | 250 |  |  |
| unrestrictedVector |  | F | F | F | F |  |  | F |  |  |
| arithmeticCoding |  | F | F | F | F |  |  | F |  |  |
| advancedPrediction |  | F | F | F | F |  |  | F |  |  |
| pbFrames |  | F | F | F | F |  |  | F |  |  |
| temporalSpatialTradeOffCap |  | F | F | F | F |  |  | F |  |  |
| hrd-B |  | NP | NP | NP | NP |  |  | NP |  |  |
| bppMaxKb |  | NP | NP | NP | NP |  |  | NP |  |  |
| slowSqcifMPI |  | NP | NP | NP | NP |  |  | NP |  |  |
| slowQcifMPI |  | NP | NP | NP | NP |  |  | NP |  |  |
| slowCifMPI |  | NP | NP | NP | NP |  |  | NP |  |  |
| slowCif4MPI |  | NP | NP | NP | NP |  |  | NP |  |  |
| slowCif16MPI |  | NP | NP | NP | NP |  |  | NP |  |  |
| errorCompensation |  | NP | NP | NP | NP |  |  | NP |  |  |
| SET OF (EnhancementOptions*) = |  | NP | NP | 1 | 1 2 |  |  | 1 2 |  |  |
| snrEnhancement | sqcifMPI |  |  | 3 | 3 | NP |  | NP | NP |  |
|  | qcifMPI |  |  | NP | NP | 3 |  | 3 | NP |  |
|  | cifMPI |  |  | NP | NP | NP |  | NP | 3 |  |
|  | cif4MPI |  |  | NP | NP | NP |  | NP | NP |  |
|  | cif16MPI |  |  | NP | NP | NP |  | NP | NP |  |
|  | maxbitrate |  |  | 50 | 50 | 50 |  | 50 | 50 |  |
| SET OF (EnhancementOptions*) = |  | NP | 1 | NP | NP | NP | 1 | NP | NP | 1 |
| spatialEnhancement | sqcifMPI |  | NP |  |  |  | NP |  |  | NP |
|  | qcifMPI |  | 3 |  |  |  | NP |  |  | NP |
|  | cifMPI |  | NP |  |  |  | 3 |  |  | 3 |
|  | cif4MPI |  | NP |  |  |  | NP |  |  | NP |
|  | cif16MPI |  | NP |  |  |  | NP |  |  | NP |
|  | maxbitrate |  | 50 |  |  |  | 50 |  |  | 50 |
| SET OF (EnhancementOptions*) = |  | NP | NP | NP | NP | NP | NP |  |  |  |
| bframeEnhancement | sqcifMPI |  |  |  |  |  |  |  |  |  |
|  | qcifMPI |  |  |  |  |  |  |  |  |  |
|  | cifMPI |  |  |  |  |  |  |  |  |  |
|  | cif4MPI |  |  |  |  |  |  |  |  |  |
|  | cif16MPI |  |  |  |  |  |  |  |  |  |
|  | maxbitrate |  |  |  |  |  |  |  |  |  |

NP = Not Present, T =TRUE, F = FALSE

*other options below “maxbitrate” in the EnhancementOptions structure not shown

## VI.2	Examples of Video Back Channel H.245 parameter setting

This section provides examples of H263Capability and H263Options settings for video back channel operation.

Example 1: Separate Logical Channel mode

In this mode, an extra bi-directional logical channel is opened for video back channel messages. The dependency between a forward video channel and the video back channel is described by forwardLogicalChannelDependency and reverseLogicalChannelDependency in the OpenLogicalChannel message.

The logical channel for video back channel messages shall only be established after the forward video channel is established. If an OpenLogicalChannel message is received with a dependency reference to a non-existing channel, the terminal shall respond with an OpenLogicalChannelReject with the reason code invalidDependentChannel. An example follows.

1. 	A bi-directional logical channel for video data is opened between terminal A and terminal B as shown in Figure VI.1/H.245. The OpenLogicalChannel message for the bi-directional logical channel includes RefPictureSelectionCapability in H263VideoCapability.

µ §

FIGURE VI.2/H.245

Logical Channels for Video Data

2. 	Next, a bi-directional logical channel for video back channel messages is opened, as shown in Figure VI.3/H.245. In this example, we assume that terminal A requests to open the bi-directional logical channel. (If the terminal B request to open the channel, forwardLogicalChannelDependency is replaced by reverseLogicalChannelDependency and vice versa.) The OpenLogicalChannel message of this logical channel includes forwardLogicalChannelDependency in forwardLogicalChannelParameters indicating LCN of LC #a in Figure VI.4/H.245 and reverseLogicalChannelDependency in reverseLogicalChannelParameters indicating LCN of LC #b, as well as separateVideoBackChannel.

µ §

FIGURE VI.5/H.245

Logical Channels for Separate Logical Channel Mode

2. 	After the logical channel for video back channel messages is established, terminal A sends the video data to LC #a and receives from LC #d, the video back channel messages that correspond to the video data sent to LC #a. In the same manner, terminal A receives the video data from LC #b and sends to LC #c, the video back channel messages that correspond to the video data from terminal B.

An example of setting H263Capability parameters in each OpenLogicalChannel messages is summarized in Table VI.3/H.245. Only a part of capabilities of H263Capability is shown for simplicity.

TABLE VI.4/H.245

H263Capability setting examples for OpenLogicalChannel messages

|  |  | H263Capability in OpenLogicalChannel message |  |  |
| --- | --- | --- | --- | --- |
| H263Capability parameter |  | #a, #b | #c, #d | #e, #f |
| sqcifMPI |  | NP | NP | NP |
| qcifMPI |  | 3 | NP | 3 |
| cifMPI |  | NP | NP | NP |
| cif4MPI |  | NP | NP | NP |
| cif16MPI |  | NP | NP | NP |
| maxBitRate |  | 240 | 10 | 240 |
| refPictureSelection |  |  | NP |  |
|  | additionalPictureMemory | unspecified | - | Unspecified |
|  | videoMuxCapability | False | - | (shall be) True |
|  | videoBackCuhannelSendCapability | ackAndNackMessage | - | AckAndNackMessage |
| separateVideoBackChannel |  | False | True | False |

NP: Not Present

Example 2: VideoMux mode

When a terminal indicates the videoMuxCapability in RefPictureSelectionCapability during capability exchange, another terminal may use this mode to send video back channel messages. Because video back channel messages are multiplexed into the coded video bitstream, the terminals do not need to establish an extra logical channel for video back channel messages. An example follows.

1. 	A bi-directional logical channel for video is opened by the OpenLogicalChannel message including refPictureSelectionCapability with the true values VideoMux mode in their H263VideoCapability. (See Figure VI.6/H.245.)

2. 	After the logical channel for video is established, terminal A sends the video data to LC #e and receives from LC #f, the video back channel messages that correspond to the video data sent to LC #e are multiplexed into the video data from terminal B.

µ §

FIGURE VI.7/H.245

Logical Channels for VideoMux Mode

A example of setting H263Capability parameters in each OpenLogicalChannel messages is summarized in Table VI.5/H.245.

Example 3: Separate Logical Channel mode in uni-directional video communication

This example shows the case when only terminal A sends video data and terminal B sends only video back channel messages. (Figure VI.8/H.245) The parameter settings of logical channel #g and #h are shown in Table VI.6/H.245.

µ §

FIGURE VI.9/H.245

Separate Logical Channel mode in uni-directional video communication

Example 4: Coexistence of Separate Logical Channel mode with VideoMux mode

The example illustrated in Figure VI.10/H.245 shows the case when only terminal A uses the Separate Logical Channel mode to receive video back channel messages from terminal B via LC #l and terminal B uses the VideoMux mode to receive video back channel message via LC #i. This example may not be realistic but is a possible configuration. The parameter settings of each logical channels are shown in Table VI.7/H.245.

µ §

FIGURE VI.11/H.245

Coexistence of Separate Logical Channel mode with VideoMux mode

TABLE VI.8/H.245

H263Capability setting examples for OpenLogicalChannel messages

|  |  | H263Capability in OpenLogicalChannel message |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| H263Capability parameter |  | #g | #h | #i | #j | #k | #l |
| sqcifMPI |  | NP | NP | NP | NP | NP | NP |
| qcifMPI |  | 3 | NP | 3 | 3 | NP | NP |
| cifMPI |  | NP | NP | NP | NP | NP | NP |
| cif4MPI |  | NP | NP | NP | NP | NP | NP |
| cif16MPI |  | NP | NP | NP | NP | NP | NP |
| maxBitRate |  | 240 | 10 | 240 | 240 | 10 | 10 |
| refPictureSelection |  |  | NP |  |  | NP | NP |
|  | additionalPictureMemory | unspecified | - | unspecified | unspecified | - | - |
|  | videoMuxCapability | F | - | F | (shall be) T | - | - |
|  | videoBackChannelSendCapability | ackAndNackMessage | - | ackAndNackMessage | ackAndNackMessage | - | - |
| separateVideoBackChannel |  | F | T | F | F | F | T |

NP: Not Present, T: True, F: False

APPENDIX VII

# Appendix VIIProcedure and template for defining new capabilities with H.245 generic capabilities

(This appendix does not form an integral part of this Recommendation)

This appendix defines the procedure and a template for defining new capabilities that are expressed in the form of H.245 generic capabilities. It also provides an example of how this template could have been used to describe the H.261 codec, instead of the ASN.1 syntax that has been used in H.245. This new mechanism for defining capabilities in H.245 is intended to be used for all new capabilities that are added to H.245; it is not intended to be used to redefine existing capabilities.

Capability descriptions relating to ITU Recommendations shall be defined in Annexes to either H.245 or the Recommendation itself (e.g. H.283).

Other capability descriptions may be defined in Annexes to H.245, or elsewhere.

## VII.1	Procedure

### VII.1.1	Definition of generic capabilities in H.245

In case the definition is to be included in Annexes to H.245, the following procedure should be performed

1) Define a OBJECT IDENTIFIER for this capability, and list it in Annex D/H.245.

2) Define the capability with Generic Capability in a new Annex to H.245.

OBJECT IDENTIFIER has the form: {itu-t (0) recommendation (0) h (8) 245 generic-capabilities (1) capability-class capability-name}.

capability-class is one of video(0), audio(1), data(2), control(3) or multiplex(4). The value for capability-name is assigned in numerical order for each capability-class.

### VII.1.2	Definition of generic capabilities in other ITU Recommendations

In case the definition is to be included in other ITU Recommendations, the following procedure should be performed

1) Define a OBJECT IDENTIFIER for this capability in the Recommendation itself, and list it in Appendix VIII /H.245.

2) Define the capability with Generic Capability in a new Annex to the appropriate recommendation.

### VII.1.3	Definition of generic capabilities in non-ITU Standards

In case the definition is to be included in non-ITU standards, the following procedure should be performed

1) Define a OBJECT IDENTIFIER for this capability in appropriate standard, and list it in Appendix VIII /H.245.

2) Define the capability with Generic Capability in appropriate standard.

## VII.2	Template

### VII.2.1	Capability Identifier

A single instance of this table shall be defined for each GenericCapability description.

TABLE VII.1/H.245

Capability Identifier Template

| Capability name: | The name of the codec, e.g. H.261 |
| --- | --- |
| Capability class: | The class of the capability, e.g. video, audio etc. |
| Capability identifier type: | The type of the identifier that defines the codec: standard, h221NonStandard,or uuid. |
| Capability identifier value: | The value of the codec tag, e.g. { itu-t (0) recommendation (0) h (8) 261 generic-capabilities (1) 0}. The value of generic-capabilities identifies a type or set of parameters associated with the capability. Note that the actual format of this object identifier is the responsibility of those defining the capabilities, but should be defined considering possible extensions. |
| maxBitRate: | Whether the maxBitRate field shall be included, shall not be included. or is optional. |
| nonCollapsingRaw: | The specification of the format of the OCTET STRING, and whether it shall or shall not be included. |
| transport: | Whether the transport field shall be included, shall not be included. or is optional. |

### VII.2.2	Capability parameters

This section is applicable to collapsing and nonCollapsing GenericParameters. An instance of this table shall be defined for each GenericParameter.  The template should be divided into sections to distinguish between which parameters are for capability negotionation and which are specific to logical channel signalling.

Note. This table does not allow the ParameterTag type (standard, h221NonStandard, or uuid) to be specified as it is only to be used for standard capability descriptions.

TABLE VII.2/H.245

Capability Parameter Template

| Parameter name: | The name of the parameter, e.g. cifMPI |
| --- | --- |
| Parameter description: | A descriptive name of the parameter, e.g. Specifies the minimum picture interval at CIF resolution |
| Parameter identifier value: | An integer that identifies this "standard" parameter |
| Parameter status: | Whether the parameter is mandatory, conditionally mandatory (e.g. dependent on another parameter) or optional. |
| Parameter type: | The type of the parameter: logical, booleanArray, unsignedMin, unsignedMax, unsigned32Min, unsigned32Max, octetString [or genericParameter]. |
| Supersedes: | The parameters that this parameter supersedes. This table element shall specify zero, 1 or more parameters that this parameter supersedes. The format shall be: parameter-name "(" parameter-identifier-value ")", e.g. qcifMPI (0) |

## VII.3	Example Template - H.261

### VII.3.1	H.261 Capability Identifier

TABLE VII.3/H.245

Example H.261 Capability Identifier

| Capability name: | ITU-T Recommendation H.261 |
| --- | --- |
| Capability class: | Video codec. |
| Capability identifier type: | Standard. |
| Capability identifier value: | itu-t (0) recommendation (0) h (8) 261 generic-capabilities (1) 0. This is the first (and only) set of parameters defined for H.261. |
| maxBitRate: | The maxBitRate field shall always be included. |
| nonCollapsingRaw: | This field shall not be included. |
| transport: | This field shall not be included. |

### VII.3.2	H.261 Capability parameters

Note that there is no table for the maximum bit rate field that is found in the ASN.1 syntax of H.245 for H.261. This is because the maximum bit rate is given at the top level of the GeneriCapability structure. Note also that temporalSpatialTradeOffCapability and stillImageTransmission could have been combined into one GenericParameter of type booleanArray.

TABLE VII.4/H.245

Example H.261 Capability Parameter - qcifMPI

| Parameter name: | qcifMPI |
| --- | --- |
| Parameter description: | If present, this indicates the minimum picture interval in units of 1/29.97 for the encoding and/or decoding of QCIF pictures, and if not present, no capability for QCIF pictures is indicated. |
| Parameter identifier value: | 0 |
| Parameter status: | Optional |
| Parameter type: | unsignedMax. |
| Supersedes: | - |

TABLE VII.5/H.245

Example H.261 Capability Parameter - cifMPI

| Parameter name: | cifMPI |
| --- | --- |
| Parameter description: | If present, this indicates the minimum picture interval in units of 1/29.97 for the encoding and/or decoding of CIF pictures, and if not present, no capability for CIF pictures is indicated. |
| Parameter identifier value: | 1 |
| Parameter status: | Optional |
| Parameter type: | unsignedMax. |
| Supersedes: | qcifMPI (0) |

TABLE VII.6/H.245

Example H.261 Capability Parameter - temporalSpatialTradeOffCapability

| Parameter name: | temporalSpatialTradeOffCapability |
| --- | --- |
| Parameter description: | The presence of this parameter indicates that the encoder is able to vary its trade-off between temporal and spatial resolution as commanded by the remote terminal. It has no meaning when part of a receive capability. |
| Parameter identifier value: | 2 |
| Parameter status: | Optional |
| Parameter type: | logical. |
| Supersedes: | - |

TABLE VII.7/H.245

Example H.261 Capability Parameter - stillImageTransmision

| Parameter name: | stillImageTransmission |
| --- | --- |
| Parameter description: | The presence of this parameter indicates the capability for still images as specified in Annex D of H.261. |
| Parameter identifier value: | 3 |
| Parameter status: | Optional |
| Parameter type: | logical. |
| Supersedes: | - |

APPENDIX VIII

# Appendix VIII	List of generic capabilities defined in other Recommendations/Standards than this Recommendation

(This appendix does not form an integral part of this Recommendation)

Table VIII.1 lists the generic capabilities defined in other Recommendations or Standards than this Recommendation.

TABLE VIII.2/H.245

List of generic capabilies defined in other standardisation orginasations

| Capability name | Capability class | Capability identifier | Name of Recommendation or Standards that defines this capability |
| --- | --- | --- | --- |
| ITU-T Recommendation H.283 | Data protocol | itu-t (0) recommendation (0) h (8) 283 generic-capabilities (1) 0} | ITU-T Recommendation H.283 |
| ITU-T Recommendation G.722.1 | Audio protocol | itu-t (0) recommendation (0) g (7) 7221 generic-capabilities (1) 0} | ITU-T Recommendation G.722.1 |

*	 Contact:	Mike Nilsson	Tel.: +44 1473 645413

BT Labs	Fax: +44 1473 643791

Ipswich, UK	E-mail: mike.nilsson@bt-sys.bt.co.uk