| INTERNATIONAL TELECOMMUNICATION UNION |  |
| --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 1997 - 2000 | TD- -E(PLEN) October 1999 Original: English |

Question 3, 11, 12, 13, 14, 15/16

STUDY GROUP 16 - CONTRIBUTION

SOURCE* :		RAPPORTEUR for Question 14 (G.Freundlich)

TITLE:			RECOMMENDATION H.245 - VERSION 6

__________

This contribution, known as H245DELK, provides delta text for version 6 of Recommendation H.245 "Control protocol for multimedia communication". Changes are shown relative to the decided version 5, known as H245NCMI. It has been created from H245NCMI, the changes determined in the May 1999 meeting of SG16 in TDs 14, 22, 24, and 69 (PLEN), and the changes agreed at the Berlin meeting of Q.11/12/13/14 (APC-1644). The version number in the syntax has been changed and Annex D has been updated to reference the new version.

The changes from version 5 are summarised below.

1) TD14 : Generic capability support of MPEG-4 Systems.

2) TD22 : Generic capability support of MPEG-4 Audio.

3) TD24 : Generic capability support of GSM Adaptive Multi Rate Speech Codec.

4) TD69 : Add atm-AAL5-compressed to the ASN.1 syntax for MediaTransportType.

5) APC-1644 : APC-1568 : Generic Capability support for ACELP and US1.

6) APC-1644 : APC-1575 : Addition of more ASN.1 syntax and semantics for G.729 .

7) APC-1644 : APC-1596 : Add Generic Capability OID definition to Appendix VIII.

Summary

This Recommendation specifies syntax and semantics of terminal information messages as well as procedures to use them for in-band negotiation at the start of or during communication. The messages cover receiving and transmitting capabilities as well as mode preference from the receiving end, logical channel signalling, and Control & Indication. Acknowledged signalling procedures are specified to ensure reliable audiovisual and data communication.

# 1	References

[1]	ISO/IEC 14496-1 (1999): Information Technology - Generic Coding of Audio-Visual Objects - Part 1: Systems.

[2]	ISO/IEC 14496-2 (1999): Information Technology - Generic Coding of Audio-Visual Objects - Part 2: Visual.

[3]	ISO/IEC 14496-3 (1999): Information Technology - Generic Coding of Audio-Visual Objects - Part 3: Audio.

[4]	ISO/IEC TR9577 (1990): Information Technology - Telecommunications information exchange between systems - protocol identification in the network layer.

[5] I-ETS Recommendation 300 036 (GSM 06.10) - Full-rate speech transcoding.

[6] I-ETS Recommendation 300 969 (GSM 06.20) - Half-rate speech transcoding.

[7] I-ETS Recommendation 300 726 (GSM 06.60) - Enhanced Full-rate speech transcoding.

[8] I-ETS Recommendation 300 038 (GSM 06.12) - Comfort noise aspects for full rate speech traffic channels.

[9] I-ETS Recommendation 300 971 (GSM 06.22) - Comfort noise aspects for half rate speech traffic channels.

[10] I-ETS Recommendation 300 728 (GSM 06.62) - Comfort noise aspects for enhanced full rate speech traffic channels.

[11] I-ETS Recommendation 300 039 (GSM 06.31) - Discontinuous transmission for full rate speech traffic channels.

[12] I-ETS Recommendation 300 972 (GSM 06.41) - Discontinuous transmission for half rate speech traffic channels.

[13] I-ETS Recommendation 300 729 (GSM 06.81) - Discontinuous transmission for enhanced full rate speech traffic channels.

[14]	I-ETS Recommendation 300 037 (GSM 06.11) - Substitution and muting of lost frames for full rate speech traffic channels.

[15]	I-ETS Recommendation 300 970 (GSM 06.21) - Substitution and muting of lost frames for half rate speech traffic channels.

[16]	I-ETS Recommendation 300 727 (GSM 06.61) - Substitution and muting of lost frames for enhanced full rate speech traffic channels.

[17]	I-ETS Recommendation 300 040 (GSM 06.32) - Voice activity detection for full rate speech traffic channels.

[18]	I-ETS Recommendation 300 973 (GSM 06.42) - Voice activity detection for half rate speech traffic channels.

[19]	I-ETS Recommendation 300 730 (GSM 06.82) - Voice activity detection for enhanced full rate speech traffic channels.

[20]	 I-ETS Recommendation 300 724 (GSM 06.53) - ANSI-C code for the GSM Enhanced Full Rate speech.

[21]	 GSM 06.73 - ANSI-C code for the GSM Adaptive Multi Rate speech codec.

[22]	 GSM 06.90 - Adaptive Multi Rate speech transcoding.

[23]	 GSM 06.91 - Substitution and muting of lost frames for Adaptive Multi Rate speech traffic channels.

[24]	 GSM 06.92 - Comfort noise aspects for Adaptive Multi Rate speech traffic channels

[25]	 GSM 06.94 - Voice Activity Detector  for Adaptive Multi Rate speech traffic channels.

[26]	 RCR STD-27H Section 5.4 ACELP Speech Codec.

[27]	 TIA/EIA IS-641-A, TDMA Cellular/PCS - Radio Interface, Enhanced Full-Rate Voice Codec, Revision A.

ANNEX A

-- ===================================================================================

-- Capability exchange definitions

-- ===================================================================================

TerminalCapabilitySet	::=SEQUENCE

{

sequenceNumber	SequenceNumber,

protocolIdentifier	OBJECT IDENTIFIER,

-- shall be set to the value

-- {itu-t (0) recommendation (0) h (8) 245 version (0) 65}

multiplexCapability	MultiplexCapability OPTIONAL,

capabilityTable	SET SIZE (1..256) OF CapabilityTableEntry OPTIONAL,

capabilityDescriptors	SET SIZE (1..256) OF CapabilityDescriptor OPTIONAL,

...

}

MediaTransportType	::=CHOICE

{

ip-UDP		NULL,

ip-TCP		NULL,

atm-AAL5-UNIDIR	NULL, -- virtual circuits used as unidirectional

atm-AAL5-BIDIR	NULL, -- virtual circuits used as bidirectional

...,

atm-AAL5-compressed	SEQUENCE

{

variable-delta	BOOLEAN,

...

}

}

-- ===================================================================================

-- Capability exchange definitions: Audio capabilities

-- ===================================================================================

-- For an H.222 multiplex, the integers indicate the size of the STD buffer in units of 256 octets

-- For an H.223 multiplex, the integers indicate the maximum number of audio frames per AL-SDU

-- For an H.225.0 multiplex, the integers indicate the maximum number of audio frames per packet

AudioCapability	::=CHOICE

{

nonStandard	NonStandardParameter,

g711Alaw64k	INTEGER (1..256),

g711Alaw56k	INTEGER (1..256),

g711Ulaw64k	INTEGER (1..256),

g711Ulaw56k	INTEGER (1..256),

g722-64k	INTEGER (1..256),

g722-56k	INTEGER (1..256),

g722-48k	INTEGER (1..256),

g7231		SEQUENCE

{

maxAl-sduAudioFrames	INTEGER (1..256),

silenceSuppression	BOOLEAN

},

g728		INTEGER (1..256),

g729		INTEGER (1..256),

g729AnnexA	INTEGER (1..256),

is11172AudioCapability	IS11172AudioCapability,

is13818AudioCapability	IS13818AudioCapability,

...,

g729wAnnexB	INTEGER(1..256),

g729AnnexAwAnnexB	INTEGER(1..256),

g7231AnnexCCapability	G7231AnnexCCapability,

gsmFullRate	GSMAudioCapability,

gsmHalfRate	GSMAudioCapability,

gsmEnhancedFullRate	GSMAudioCapability,

genericAudioCapability	GenericCapability,

g729wAnnexD	INTEGER (1..256),

g729wAnnexE	INTEGER (1..256),

g729wAnnexDwAnnexE	INTEGER (1..256),

g729wAnnexDwAnnexB	INTEGER (1..256),

g729wAnnexEwAnnexB	INTEGER (1..256),

g729wAnnexDwAnnexEwAnnexB	INTEGER (1..256),

g729AnnexAwAnnexD	INTEGER (1..256),

g729AnnexAwAnnexE	INTEGER (1..256),

g729AnnexAwAnnexDwAnnexE	INTEGER (1..256),

g729AnnexAwAnnexDwAnnexB	INTEGER (1..256),

g729AnnexAwAnnexEwAnnexB	INTEGER (1..256),

g729AnnexAwAnnexDwAnnexEwAnnexB	INTEGER (1..256)

}

-- ===================================================================================

-- Request mode definitions: Mode description

-- ===================================================================================

ModeDescription	::=SET SIZE (1..256) OF ModeElement

ModeElement	::= SEQUENCE

{

type		CHOICE

{

nonStandard	NonStandardParameter,

videoMode	VideoMode,

audioMode	AudioMode,

dataMode	DataMode,

encryptionMode	EncryptionMode,

...,

h235Mode	H235Mode

},

h223ModeParameters	H223ModeParameters OPTIONAL,

...,

v76ModeParameters	V76ModeParameters OPTIONAL,

h2250ModeParameters	H2250ModeParameters OPTIONAL,

genericModeParameters	GenericCapability OPTIONAL

}

-- ===================================================================================

-- Request mode definitions: Audio modes

-- ===================================================================================

AudioMode	::=CHOICE

{

nonStandard	NonStandardParameter,

g711Alaw64k	NULL,

g711Alaw56k	NULL,

g711Ulaw64k	NULL,

g711Ulaw56k	NULL,

g722-64k	NULL,

g722-56k	NULL,

g722-48k	NULL,

g728		NULL,

g729		NULL,

g729AnnexA	NULL,

g7231		CHOICE

{

noSilenceSuppressionLowRate	NULL,

noSilenceSuppressionHighRate	NULL,

silenceSuppressionLowRate	NULL,

silenceSuppressionHighRate	NULL

},

is11172AudioMode	IS11172AudioMode,

is13818AudioMode	IS13818AudioMode,

...,

g729wAnnexB	INTEGER(1..256),

g729AnnexAwAnnexB	INTEGER(1..256),

g7231AnnexCMode	G7231AnnexCMode,

gsmFullRate	GSMAudioCapability,

gsmHalfRate	GSMAudioCapability,

gsmEnhancedFullRate	GSMAudioCapability,

genericAudioMode	GenericCapability,

g729wAnnexD	NULL,

g729wAnnexE	NULL,

g729wAnnexDwAnnexE	NULL,

g729wAnnexDwAnnexB	NULL,

g729wAnnexEwAnnexB	NULL,

g729wAnnexDwAnnexEwAnnexB	NULL,

g729AnnexAwAnnexD	NULL,

g729AnnexAwAnnexE	NULL,

g729AnnexAwAnnexDwAnnexE	NULL,

g729AnnexAwAnnexDwAnnexB	NULL,

g729AnnexAwAnnexEwAnnexB	NULL,

g729AnnexAwAnnexDwAnnexEwAnnexB	NULL

}

ANNEX B

Annex B

#### B.1.0.1	Audio Capabilities

This indicates audio capabilities. The indication of more than a single capability within a single AudioCapability does not indicate simultaneous processing capability. Simultaneous processing capability can be indicated by instances of AudioCapability in different AlternativeCapabilitySets in a single CapabilityDescriptor.

The capability to transmit and/or receive G-series audio is indicated by a choice of integers. When an H.222.1 multiplex is used, these numbers refer to the available STD buffer size in units of 256 octets. When an H.223 multiplex is used, these numbers refer to the maximum number of audio frames per AL-SDU. When an H.225.0 multiplex is used, these numbers indicate the maximum number of audio frames per packet. The exact meaning of the codepoints is given in Table B.1.

TABLE B.2/H.245

G-series audio codepoints

| ASN.1 Codepoint | Semantic meaning of codepoint |
| --- | --- |
| g711Alaw64k | G.711 audio at 64 kbit/s, A-law |
| g711Alaw56k | G.711 audio at 56 kbit/s, A-law, truncated to 7 bits |
| g711Ulaw64k | G.711 audio at 64 kbit/s, m-law |
| g711Ulaw56k | G.711 audio at 56 kbit/s, m-law, truncated to 7 bits |
| g722-64k | G.722 7 kHz audio at 64 kbit/s |
| g722-56k | G.722 7 kHz audio at 56 kbit/s |
| g722-48k | G.722 7 kHz audio at 48 kbit/s |
| g7231 | G.723.1 at either 5.3 or 6.3 kbit/s |
| g728 | G.728 audio at 16 kbit/s |
| g729 | G.729 audio at 8 kbit/s |
| g729AnnexA | G.729AnnexA audio at 8 kbit/s |
| g729wAnnexB | G.729 audio at 8 kbit/s with silence suppression as in Annex B |
| g729AnnexAwAnnexB | G.729AnnexA audio at 8 kbit/s with silence suppression as in Annex B |
| g7231AnnexCCapability | G.723.1 with Annex C |
| gsmFullRate | Full-rate speech transcoding (GSM 06.10) |
| gsmHalfRate | Half-rate speech transcoding (GSM 06.20) |
| gsmEnhancedFullRate | Enhanced Full Rate (EFR) speech transoding (GSM 06.60) |
| g729wAnnexD | G.729 audio at 6.4 and 8 kbit/s |
| g729wAnnexE | G.729 audio at 8 and 11.8 kbit/s |
| g729wAnnexDwAnnexE | G.729 audio at 6.4, 8 and 11.8 kbit/s |
| g729wAnnexDwAnnexB | G.729 audio at 6.4 and 8 kbit/s with silence suppression |
| g729wAnnexEwAnnexB | G.729 audio at 8 and 11.8 kbit/s with silence suppression |
| g729wAnnexDwAnnexEw AnnexB | G.729 audio at 6.4, 8 and 11.8 kbit/s with silence suppression |
| g729AnnexAwAnnexD | G.729AnnexA audio at 6.4 and 8 kbit/s |
| g729AnnexAwAnnexE | G.729AnnexA audio at 8 and 11.8 kbit/s |
| g729AnnexAwAnnexDw AnnexE | G.729AnnexA audio at 6.4, 8 and 11.8 kbit/s |
| g729AnnexAwAnnexDw AnnexB | G.729AnnexA audio at 6.4 and 8 kbit/s with silence suppression |
| g729AnnexAwAnnexEw AnnexB | G.729AnnexA audio at 8 and 11.8 kbit/s with silence suppression |
| g729AnnexAwAnnexDw AnnexEwAnnexB | G.729AnnexA audio at 6.4, 8 and 11.8 kbit/s with silence suppression |

ANNEX C

### C.1.1	Peer-to-peer communication

Protocol information is transferred to the peer protocol entity via the relevant messages defined in Annex Cin section 6. Some protocol entities described have state variables associated with them. A number of protocol entities described also have timers associated with them.

#### C.1.1.1	MSDSE messages

Table C.3 shows the MSDSE messages and fields, defined in Annex Cin section 6, which are relevant to the MSDSE protocol.

### C.1.2	Introduction

These procedures are used by terminals to communicate their capabilities, and are referred to as the Capability Exchange Signalling Entity (CESE). Procedures are specified in terms of primitives and states at the interface between the CESE and the CESE user. Protocol information is transferred to the peer CESE via relevant messages defined in Annex Cin section 6. There is an outgoing CESE and an incoming CESE. At each of the outgoing and incoming ends there is one instance of the CESE for each call.

#### C.1.2.1	Messages

Table C.4 shows the CESE messages and fields, defined in Annex Cin section 6, which are relevant to the CESE protocol.

### C.1.3	Introduction

The protocol specified here provides reliable opening and closing of uni-directional logical channels using acknowledged procedures.

The protocol specified here is referred to as the Logical Channel Signalling Entity (LCSE). Procedures are specified in terms of primitives at the interface between the LCSE and the LCSE user, and LCSE states. Protocol information is transferred to the peer LCSE via relevant messages defined in Annex Cin section 6.

#### C.1.3.1	LCSE messages

Table C.5 shows the LCSE messages and fields, defined in Annex Cin section 6, which are relevant to the LCSE protocol.

### C.1.4	Introduction

The protocol specified here provides reliable opening and closing of bi-directional logical channels using acknowledged procedures.

The protocol specified here is referred to as the Bi-directional Logical Channel Signalling Entity (B-LCSE). Procedures are specified in terms of primitives at the interface between the B-LCSE and the B-LCSE user, and B-LCSE states. Protocol information is transferred to the peer B-LCSE via relevant messages defined in Annex Cin section 6.

#### C.1.4.1	B-LCSE messages

Table C.6 shows the B-LCSE messages and fields, defined in Annex Cin section 6, which are relevant to the B-LCSE protocol.

### C.1.5	Introduction

These procedures are used by a terminal to request the remote terminal to close a logical channel. Note that these are only close request procedures; the actual logical channel close occurs using the LCSE and B-LCSE procedures. The procedures are referred to here as the Close Logical Channel Signalling Entity (CLCSE). Procedures are specified in terms of primitives and states at the interface between the CLCSE and the CLCSE user. Protocol information is transferred to the peer CLCSE via relevant messages defined in Annex Cin section 6. There is an outgoing CLCSE and an incoming CLCSE. At each of the outgoing and incoming ends there is one instance of the CLCSE for each logical channel.

#### C.1.5.1	Messages

Table C.7 shows the CLCSE messages and fields, defined in Annex Cin section 6, which are relevant to the CLCSE protocol.

### C.1.6	Introduction

The multiplex table serves to associate each octet within a H.223 MUX-PDU [8] with a particular logical channel number. The H.223 multiplex table may have up to 16 entries, numbered from 0 to 15. Table entries 1 to 15 shall be sent from transmitters to receivers as specified in the following procedures.

The procedures described here are referred to as the Multiplex Table Signalling Entity (MTSE). Procedures are specified in terms of primitives and states at the interface between the MTSE and the MTSE user. Protocol information is transferred to the peer MTSE via relevant messages defined in Annex Cin section 6.

#### C.1.6.1	Messages

Table C.8 shows the MTSE messages and fields, defined in Annex Cin section 6, which are relevant to the MTSE protocol.

### C.1.7	Introduction

These procedures are used by a terminal to request the retransmission of one or more MultiplexEntryDescriptors. The procedures are referred to here as the Request Multiplex Entry Signalling Entity (RMESE). Procedures are specified in terms of primitives and states at the interface between the RMESE and the RMESE user. Protocol information is transferred to the peer RMESE via relevant messages defined in Annex Cin section 6. There is an outgoing RMESE and an incoming RMESE. There is one instance of the RMESE for each multiplex table entry.

#### C.1.7.1	Messages

Table C.9 shows the RMESE messages and fields, defined in Annex Cin section 6, which are relevant to the RMESE protocol.

### C.1.8	Introduction

The procedures described here allow a terminal to request a remote terminal to use a particular mode of operation in its transmit direction. The procedures are referred to here as the Mode Request Signalling Entity (MRSE). Procedures are specified in terms of primitives and states at the interface between the MRSE and the MRSE user. Protocol information is transferred to the peer MRSE via relevant messages defined in Annex Cin section 6. There is an outgoing MRSE and an incoming MRSE. At each of the outgoing and incoming ends there is one instance of the MRSE per call.

#### C.1.8.1	Messages

Table C.10 shows the MRSE messages and fields, defined in Annex Cin section 6, which are relevant to the MRSE protocol.

#### C.1.8.2	Messages

Table C.11 shows the RTDSE messages and fields, defined in Annex Cin section 6, which are relevant to the RTDSE protocol.

### C.1.9	Introduction

The protocol specified here provides reliable operation of maintenance loops using acknowledged procedures.

The protocol specified here is referred to as the Maintenance Loop Signalling Entity (MLSE). Procedures are specified in terms of primitives at the interface between the MLSE and the MLSE user, and MLSE states. Protocol information is transferred to the peer MLSE via relevant messages defined in Annex Cin section 6.

#### C.1.9.1	MLSE messages

Table C.12 shows the MLSE messages and fields, defined in Annex Cin section 6, which are relevant to the MLSE protocol.

ANNEX D

# Annex D	Object identifier assignments

A==

(This annex forms an integral part of this Recommendation)

Table D.1 lists the assignment of Object Identifiers defined for use by this Recommendation.

TABLE D.1/H.245

| Object Identifier Value | Description |
| --- | --- |
| {itu-t (0) recommendation (0) h (8) 245 version (0) 1} | This Object Identifier is used to indicate the version of this Recommendation in use as a multimedia system control protocol. This indicates the first version of this Recommendation. |
| {itu-t (0) recommendation (0) h (8) 245 version (0) 2} | This Object Identifier is used to indicate the version of this Recommendation in use as a multimedia system control protocol. At this time there are sixfive standardized versions defined. This indicates the second version of this Recommendation. |
| {itu-t (0) recommendation (0) h (8) 245 version (0) 3} | This Object Identifier is used to indicate the version of this Recommendation in use as a multimedia system control protocol. At this time there are sixfive standardized versions defined. This indicates the third version of this Recommendation. |
| {itu-t (0) recommendation (0) h (8) 245 version (0) 4} | This Object Identifier is used to indicate the version of this Recommendation in use as a multimedia system control protocol. At this time there are sixfive standardized versions defined. This indicates the fourth version of this Recommendation. |
| {itu-t (0) recommendation (0) h (8) 245 version (0) 5} | This Object Identifier is used to indicate the version of this Recommendation in use as a multimedia system control protocol. At this time there are sixfive standardized versions defined. This indicates the fifth version of this Recommendation. |
| {itu-t (0) recommendation (0) h (8) 245 version (0) 6} | This Object Identifier is used to indicate the version of this Recommendation in use as a multimedia system control protocol. At this time there are six standardized versions defined. This indicates the sixth version of this Recommendation. |
| {itu-t (0) recommendation (0) h (8) 245 generic-capabilities (1) video (0) is14496-2 (0)} | This Object Identifier is used to indicate the generic capability for ISO/IEC 14496-2. This capability is defined in Annex E of this Recommendation. |
| {itu-t (0) recommendation (0) h (8) 245 generic-capabilities (1) audio (1) is14496-3 (0)} | This Object Identifier is used to indicate the generic capability for ISO/IEC 14496-3. This capability is defined in Annex H of this Recommendation. |
| {itu-t (0) recommendation (0) h (8) 245 generic-capabilities (1) audio (1) amr (1)} | This Object Identifier is used to indicate the generic capability for the GSM Adaptive Multi rate speech codec. This capability is defined in Annex I of this Recommendation. |
| {itu-t (0) recommendation (0) h (8) 245 generic-capabilities (1) audio (1) acelp (2)} | This Object Identifier is used to indicate the generic capability for the TIA/EIA/ANSI IS-136 ACELP voice codec. This capability is defined in Annex J of this Recommendation. |
| {itu-t (0) recommendation (0) h (8) 245 generic-capabilities (1) audio (1) us1 (3)} | This Object Identifier is used to indicate the generic capability for the TIA/EIA /ANSI IS-136 US1 voice codec. This capability is defined in Annex K of this Recommendation. |
| {itu-t (0) recommendation (0) h (8) 245 generic-capabilities (1) data (2) is14496-1 (0)} | This Object Identifier is used to indicate the generic capability for ISO/IEC 14496-1. This capability is defined in Annex G of this Recommendation. |
| {itu-t (0) recommendaton (0) h (8) 245 generic-capabilities (1) control (3) logical-channel-bit-rate-management (0)} | This Object Identifier is used to indicate the generic capability for logical channel bit rate management. This capability is defined in Annex F of this Recommendation. |

ANNEX G

# Annex G	ISO/IEC 14496-1 Capability Definitions

A==

(This annex forms an integral part of this Recommendation)

Table G.1 below defines the capability identifier for ISO/IEC 14496-1 [28] capabilities. Tables G.2 to G.6 define the associated capability parameters for ISO/IEC 14496-1. These parameters shall only be included as genericDataCapability within the DataCapability structure and as genericDataMode within the DataMode structure. For capability exchange, only streamType and profileAndLevel and objectType shall be specified. When opening a logical channel (forward or reverse) either ES_ID or objectDescriptor shall be specified.

Further information about the usage of the IS14496-1 Generic Capability is included in Annex F to H.324 version 1998.

## G.1	Capability Identifier

TABLE G.1/H.245

Capability Identifier for ISO/IEC 14496-1

| Capability name: | ISO/IEC 14496-1 Capability |
| --- | --- |
| Capability class: | Data application |
| Capability identifier type: | Standard |
| Capability identifier value: | itu-t (0) recommendation (0) h (8) 245 generic-capabilities (1) data (2) is14496-1 (0) |
| maxBitRate: | This field shall be included to indicate the maximum bitrate of one single ISO/IEC 14496 data stream. |
| nonCollapsingRaw: | This field shall not be included. |
| transport: | This field shall be included to indicate the error protection protocol for a specific ISO/IEC 14496 data stream carried in one logical channel. |

## G.2	Capability parameters used for capability negotiations and logical channel signalling

TABLE G.2/H.245

Capability Parameter streamType

| Parameter name: | streamType |
| --- | --- |
| Parameter description: | This is a nonCollapsing GenericParameter. StreamType indicates the type of the IS14496 stream that is referred to by a specific instance of IS14496-1 Generic Capability as given in table 9 (“streamType Values”) of ISO/IEC 14496-1. |
| Parameter identifier value: | 0 |
| Parameter status | Optional. Shall be present for Capability Exchange. Shall not be present for Logical Channel Signalling and Mode Request. |
| Parameter type: | unsignedMax. Shall be in the range 0..255. |
| Supersedes: | - |

TABLE G.3/H.245

Capability Parameter profileAndLevel

| Parameter name: | ProfileAndLevel |
| --- | --- |
| Parameter description: | This is a nonCollapsing GenericParameter. ProfileAndLevel indicates the capability of processing the particular profiles in combination with the level as given in · Table 3 of ISO/IEC 14496-1 ("ODProfileLevelIndication Values") for streamType = 0x01 · Table 4 of ISO/IEC 14496-1 ("sceneProfileLevelIndication Values") for streamType = 0x03 · Table 5 of ISO/IEC 14496-1 ("audioProfileLevelIndication Values") for streamType = 0x05 · Table 6 of ISO/IEC 14496-1 ("visualProfileLevelIndication Values") for streamType = 0x04 |
| Parameter identifier value: | 1 |
| Parameter status | Optional. Shall be present for Capability Exchange. Shall not be present for Logical Channel Signalling and Mode Request. |
| Parameter type: | unsignedMax. Shall be in the range 0..255. |
| Supersedes: | - |

TABLE G.4/H.245

Capability Parameter objectType

| Parameter name: | objectType |
| --- | --- |
| Parameter description: | This is a nonCollapsing GenericParameter. objectType indicates the set of tools to be used by the decoder of the bitstream contained in one logical channel as given in · Table 8 of ISO/IEC 14496-1 ("objectTypeIndication Values ") for streamType = 0x04 or 0x05 · Table 7 of ISO/IEC 14496-1 ("graphicsProfileLevelIndication Values") for streamType = 0x03 For all other values of streamType, objectType is not defined and shall therefore not be used. |
| Parameter identifier value: | 2 |
| Parameter status | Optional. Shall not be present for Capability Exchange. Shall be present for Logical Channel Signalling. May be present for Mode Request. |
| Parameter type: | unsignedMax. Shall be in the range 0..255. |
| Supersedes: | - |

## G.3	Capability parameters used for logical channel signalling only

TABLE G.5/H.245

Capability Parameter objectDescriptor

| Parameter name: | ObjectDescriptor |
| --- | --- |
| Parameter description: | This is a nonCollapsing GenericParameter. ObjectDescriptor contains an octet string which provides all the necessary information to configure the decoder for a particular bitstream in one logical channel (refer to ISO/IEC14496-1). It shall contain information for only one Elementary Stream. |
| Parameter identifier value: | 3 |
| Parameter status | Optional. Shall not be present for Capability Exchange and Mode Request. May be present for Logical Channel Signalling. |
| Parameter type: | octetString. |
| Supersedes: | - |

TABLE G.6/H.245

Capability Parameter ES_ID

| Parameter name: | ES_ID |
| --- | --- |
| Parameter description: | This is a nonCollapsing GenericParameter. ES_ID indicates the ID of the elementary stream that is contained in one specific logical channel and by which it may be referred by other ISO/IEC 14496 data streams. For the InitialObjectDescriptor, ES_ID shall be set to ‘0’ (zero). |
| Parameter identifier value: | 4 |
| Parameter status | Optional. Shall not be present for Capability Exchange. May be present for Logical Channel Signalling. Shall be present for Mode Request. |
| Parameter type: | unsignedMax. Shall be in the range 0..65535. |
| Supersedes: | - |

ANNEX H

# Annex H	ISO/IEC 14496-3 Capability Definitions

A==

(This annex forms an integral part of this Recommendation)

Table H.1 defines the capability identifier for ISO/IEC 14496-3 [29] Capabilities. Tables H.2 to H.5 define the associated capability parameters for ISO/IEC 14496-3. These parameters shall only be included as genericAudioCapability within the AudioCapability structure and as genericAudioMode within the AudioMode structure. For capability exchange, profileAndLevel and formatType shall be present and all other parameters shall be absent. When opening a logical channel (forward or reverse), profileAndLevel, formatType and audioObjectType shall be present and audioSpecificConfig may be specified. For mode request, profileAndLevel and formatType shall be present and audioObjectType may be specified.

profileAndLevel of ISO/IEC1446-3 may support several types of audio objects. The audio object shall be carried as one of two bitstream formats which are the raw data format and the ISO/IEC 14496-3 Transport Stream format. formatType indicate the choice of the bitstream format type. In applications using multi-rate or scalable transmission, it is useful to allow changes in the structure of the audio objects in one logical channel. This can be realized with the MPEG-4/Audio format which allows changing the configuration of the stream frame by frame. For low bit-rate transmission, the raw data format may be used to reduce redundancy of transmitting the configuration of the stream every frame.

TABLE H.1/H.245

Capability Identifier for ISO/IEC 14496-3 Capability

| Capability name: | ISO/IEC 14496-3 Capability |
| --- | --- |
| Capability class: | Audio Codec |
| Capability identifier type: | Standard |
| Capability identifier value: | itu-t (0) recommendation (0) h (8) 245 generic-capabilities (1) audio (1) is14496-3 (0) |
| maxBitRate: | This field shall always be included. |
| nonCollapsingRaw: | This field shall not be included. |
| transport: | This field shall not be included. |

TABLE H.2/H.245

Profile and Level for ISO/IEC 14496-3 Capability

| Parameter name: | profileAndLevel |
| --- | --- |
| Parameter description: | This is a nonCollapsing GenericParameter. profileAndLevel indicates the capability of processing the particular profiles in combination with the level as given in Table A.13 in section 1 of ISO/IEC 14496-3(*). |
| Parameter identifier value: | 0 |
| Parameter status | Mandatory |
| Parameter type: | unsignedMax. Shall be in the range 0..15. |
| Supersedes: | - |

TABLE H.3/H.245

formatType for ISO/IEC 14496-3 Capability

| Parameter name: | formatType |
| --- | --- |
| Parameter description: | This is a nonCollapsing GenericParameter. formatType indicates the choice of the bitstream format type of a audio object between the raw data format and the audio format as follows: · 0 : the raw data format (subclause 6.2.7 in section 1 of ISO/IEC14496-3(*)) · 1 : the format defined at subclause 13 of ISO/IEC14496-3/AMD1(*). |
| Parameter identifier value: | 1 |
| Parameter status | Mandatory |
| Parameter type: | logical |
| Supersedes: | - |

TABLE H.4/H.245

audioObjectType for ISO/IEC 14496-3 Capability

| Parameter name: | audioObjectType |
| --- | --- |
| Parameter description: | This is a nonCollapsing GenericParameter. audioObjectType indicates the set of tools to be used by the decoder of the bitstream contained in the logical channel as given in Table 5.1 in section 1 of ISO/IEC14496-3(*). |
| Parameter identifier value: | 2 |
| Parameter status | Optional. Shall not be present for Capability Exchange. Shall be present for Logical Channel Signalling. May be present for Mode Request. |
| Parameter type: | unsignedMax. Shall be in the range 0..31. |
| Supersedes: | - |

TABLE H.5/H.245

audioSpecificConfig for ISO/IEC 14496-3 Capability

| Parameter name: | audioSpecificConfig |
| --- | --- |
| Parameter description: | This is a nonCollapsing GenericParameter. AudioSpecificConfig indicates how to configure the decoder for a particular object (refer to subclause 6.2.1 in section 1 of ISO/IEC14496-3(*)). |
| Parameter identifier value: | 3 |
| Parameter status | Optional. Shall not be present for Capability Exchange and Mode Request. May be present for Logical Channel Signalling. |
| Parameter type: | octetString |
| Supersedes: | - |

(*) References may be changed according to the standardization process of MPEG-4 Version 2.

ANNEX I

# Annex I	GSM Adaptive Multi Rate Capability Definitions

A==

(This annex forms an integral part of this Recommendation)

Table I.1 defines the capability identifier for GSM Adaptive Multi Rate (AMR) Capabilities. Tables I.2 to I.7 define the associated capability parameters. The relevant specifications are [30], [31], [32], [33], [34], [35], [36] and [37].

The section I.1 defines the mode signaling and the packetization of speech frames to the octet structure.

TABLE I.1/H.245

GSM AMR Capability Identifier

| Capability name: | AMR |
| --- | --- |
| Capability class: | Audio codec. |
| Capability identifier type: | Standard. |
| Capability identifier value: | {itu-t (0) recommendation (0) h (8) 245 generic-capabilities (1) audio (1) amr (1)} |
| maxBitRate: | Shall be 122 |
| NonCollapsingRaw: | This field shall not be included |
| transport: | This field shall not be included |

TABLE I.2/H.245

GSM AMR Capability Parameter - maxAl-sduFrames

| Parameter name: | MaxAl-sduFrames |
| --- | --- |
| Parameter description: | This is a collapsing GenericParameter. It specifies what is the maximum number of audio frames per AL-SDU |
| Parameter identifier value: | 0 |
| Parameter status: | Optional |
| Parameter type: | UnsignedMin |
| Supersedes: | - |

TABLE I.3/H.245

GSM AMR Capability Parameter - bitRate

| Parameter name: | bitRate |
| --- | --- |
| Parameter description: | This is a nonCollapsing GenericParameter. It specifies the AMR bitrate. This parameter shall be used only in mode requests. 0 = 4.75, 1 = 5.15, 2 = 5.90, 3 = 6.70, 4 = 7.40, 5 = 7.95, 6 = 10.2, 7 = 12.2 |
| Parameter identifier value: | 1 |
| Parameter status: | Optional |
| Parameter type: | UnsignedMin |
| Supersedes: | - |

TABLE I.4/H.245

GSM AMR Capability Parameter - GSM AMR Comfort Noise

| Parameter name: | gsmAmrComfortNoise |
| --- | --- |
| Parameter description: | This is a collapsing GenericParameter. It specifies that GSM AMR comfort noise is to be used in mode request. This parameter shall be used only in mode requests but not in capabilities because this capability is mandatory. |
| Parameter identifier value: | 2 |
| Parameter status: | Optional |
| Parameter type: | Logical |
| Supersedes: | - |

TABLE I.5/H.245

GSM AMR Capability Parameter - GSM EFR comfort noise

| Parameter name: | gsmEfrComfortNoise |
| --- | --- |
| Parameter description: | This is a collapsing GenericParameter. In a capability, the parameter specifies whether there is a GSM EFR comfort noise capability or not. In a mode request, the parameter specifies that GSM EFR comfort noise is requested. |
| Parameter identifier value: | 3 |
| Parameter status: | Optional |
| Parameter type: | Logical |
| Supersedes: | - |

TABLE I.6/H.245

GSM AMR Capability Parameter - IS-641 comfort noise

| Parameter name: | is-641ComfortNoise |
| --- | --- |
| Parameter description: | This is a collapsing GenericParameter. In a capability, the parameter specifies whether there is a IS-641 comfort noise capability or not. In a mode request, the parameter specifies that IS-641 comfort noise is requested. |
| Parameter identifier value: | 4 |
| Parameter status: | Optional |
| Parameter type: | Logical |
| Supersedes: | - |

TABLE I.7/H.245

GSM AMR Capability Parameter - PDC EFR confort noise

| Parameter name: | pdcEFRComfortNoise |
| --- | --- |
| Parameter description: | This is a collapsing GenericParameter. In a capability, the parameter specifies whether there is a PDC EFR comfort noise capability or not. In a mode request, the parameter specifies that PDC EFR comfort noise is requested. |
| Parameter identifier value: | 5 |
| Parameter status: | Optional |
| Parameter type: | Logical |
| Supersedes: | - |

## I.1	Definition of mode signaling and bit stuffing to achieve octet alignment

The AMR mode signaling in mobile systems is partly based on external signaling not defined within the AMR speech codec specification. For compatibility with mobile systems this section defines the mode signaling needed for AMR use in the ITU-T H series of recommendations. The size of the speech frames of the AMR codec in the different modes is not a multiple of eight. For that reason bit stuffing is needed to achieve an octet structure.

Note, in future the content of the chapter "Definition of mode signaling and bit stuffing to achieve octet alignment" may be changed to reference to ETSI documentation or other appropriate standardization documentation.

Table I.10 maps all the AMR modes into specific mode indicies mi(k). Mode indicies are also reserved for silence suppression frames used in different systems. Tables I.11 to I.14 specify the formats for these. Table I.15 specifies a no transmission frame.

The bits delivered by the AMR speech encoder, {s(1),s(2),...,s(Ks)}, shall be rearranged according to subjective importance before they are octet aligned. Tables I.16 to I.23 define the correct rearrangement for the speech codec modes 12.2 kbit/s, 10.2 kbit/s, 7.95 kbit/s, 7.40 kbit/s, 6.70 kbit/s, 5.90 kbit/s, 5.15 kbit/s and 4.75 kbit/s, respectively. In the tables speech codec parameters are numbered in the order they are delivered by the corresponding speech encoder according to GSM 06.90 and the rearranged bits are labelled {d(0),d(1),...,d(Kd-1)}, defined in the order of decreasing importance. Index Kd refers to the number of bits delivered by the speech encoder, see Table I.8 below.

TABLE I.8/H.245

Number of speech bits in different AMR modes

| Codec Mode | Number of speech bits delivered per block (Kd) |
| --- | --- |
| AMR12.2 | 244 |
| AMR10.2 | 204 |
| AMR7.95 | 159 |
| AMR7.4 | 148 |
| AMR6.7 | 134 |
| AMR5.9 | 118 |
| AMR5.15 | 103 |
| AMR4.75 | 95 |

The ordering algorithm is in pseudo code as:

for j = 0 to Kd-1

d(j) := s(table(j)+1); 		where table(j) is read line by line left to right

Hence the octet structure bn(k) for each AMR codec mode is defined as follows:

Number of stuffing bits: 	Ks = 8*N – Kd – Ki, 	where Ki is the number of mode index bits

Octet[0]: 			b0(k) = mi(k),				for k = 0, 1, 2 ,3 (mode index)

b0(k) = d(k - 4), 			for k = 4, 5, 6, 7

Octet[n]: 			bn(k) = d( 8 * n  -  4 + k),  		for k = 0, 1, 2, 3, 4, 5, 6, 7

Octet[N-1]:		bN-1(k) = d( 8 * (N - 1) -  4 + k),		for k = 0, …,  7 - Ks

If Ks > 0

bN-1(k) = UB,			for k = 8 - Ks, …, 7

TABLE I.9/H.245

Example, mapping of the AMR speech coding mode 6.7 kbit/s.

| Octet | MSB | Octet Structure |  |  |  |  |  | LSB |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| b0 | d(3) | d(2) | d(1) | d(0) | 0 | 0 | 1 | 1 |
| b1 | d(11) | d(10) | d(9) | d(8) | d(7) | d(6) | d(5) | d(4) |
| b2 | ... | ... | ... | ... | ... | ... | ... | d(12) |
| b17 | UB | UB | UB | UB | UB | UB | d(133) | d(132) |

TABLE I.10/H.245

Mapping of the AMR speech coding modes defined in GSM 06.90 to mode index

bits in transmitted octets

| Mode_index (4 bits) | Naming in GSM 06.90 and GSM 06.92 |
| --- | --- |
| 0 (Amr4-75k) | 4.75 kbit/s mode |
| 1 (Amr5-15k) | 5.15 kbit/s mode |
| 2 (Amr5-90k) | 5.90 kbit/s mode |
| 3 (Amr6-70k) | 6.70 kbit/s mode (PDC-EFR) |
| 4 (Amr7-40k) | 7.40 kbit/s mode (IS-641) |
| 5 (Amr7-95k) | 7.95 kbit/s mode |
| 6 (Amr10-2k) | 10.2 kbit/s mode |
| 7 (Amr12-2k) | 12.2 kbit/s mode (GSM EFR) |
| 8 | GsmAmr Comfort Noise Frame (mandatory) |
| 9 | Gsm-Efr Comfort Noise Frame (optional) |
| 10 | IS-641 Comfort Noise frame (optional) |
| 11 | Pdc-Efr Comfort Noise Frame (optional) |
| 12-14 | For future use |
| 15 | No transmission |

TABLE I.11/H.245

Mapping of Comfort Noise descriptor bits from GSM 06.92 to octets for the mode index 8

(Bits from s1 to s35 refer to GSM 06.92).

| Transmitted octets | MSB | Mapping of bits |  |  |  |  |  | LSB |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Index of 1st LSF subvector | index of LSF reference vector |  |  | Mode_Index |  |  |  |
|  | s4 | s3 | s2 | s1 | mi(3) | mi(2) | mi(1) | mi(0) |
| 2 | Index of 2nd LSF subvector | index of 1st LSF subvector |  |  |  |  |  |  |
|  | s12 | s11 | s10 | s9 | s8 | s7 | s6 | s5 |
| 3 | index of 2nd LSF subvector |  |  |  |  |  |  |  |
|  | s20 | s19 | s18 | s17 | s16 | s15 | s14 | s13 |
| 4 | index of 3rd LSF subvector |  |  |  |  |  |  |  |
|  | s28 | s27 | s26 | s25 | s24 | s23 | s22 | s21 |
| 5 | SID type bit | frame energy |  |  |  |  |  | index of 3rd LSF subvector |
|  | t1 | s35 | s34 | s33 | s32 | s31 | s30 | s29 |
| 6 | Stuffing bits |  |  |  |  | Speech_Mode_Indication |  |  |
|  | UB | UB | UB | UB | UB | smi(2) | smi(1) | smi(0) |

Definitions of additional descriptor bits needed for the silence descriptor in Table I.11:

SID-type (t1) is {0=SID_FIRST, 1=SID_UPDATE }

Speech Mode Indication (smi(0)- smi(2)) is the Speech Mode according to the first eight entries in the Mode_Index table.

TABLE I.12/H.245

Mapping of silence insertion descriptor bits from GSM 06.60 (parameters also described in GSM 06.62)

to octets for the Mode Index 9 (Bits from s1 to s91 refer to GSM 06.60).

| Transmitted octets | MSB | Mapping of bits |  |  |  |  |  |  | LSB |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Index of 1st LSF subMatrix |  |  |  |  | Mode_Index |  |  |  |
|  | s4 | s3 | s2 | s1 |  | mi(3) | mi(2) | mi(1) | mi(0) |
| 2 | Index of 2nd LSF submatrix |  |  |  |  |  | index of 1st LSF subMatrix |  |  |
|  | s12 | s11 | s10 | s9 |  | s8 | s7 | s6 | s5 |
| 3 | Index of 3rd LSF submatrix |  |  |  |  |  | Index of 2nd LSF submatrix |  |  |
|  | s20 | s19 | s18 | s17 |  | s16 | s15 | s14 | s13 |
| 4 | index of 4th LSF submatrix |  |  |  | sign of 3rd LSF submatrix |  | index of 3rd LSF submatrix |  |  |
|  | s28 | s27 | s26 | s25 |  | s24 | s23 | s22 | s21 |
| 5 | index of 5th LSF submatrix |  |  |  |  | index of 4th LSF submatrix |  |  |  |
|  | s36 | s35 | s34 | s33 |  | s32 | s31 | s30 | s29 |
| 6 | Stuffing bits | fixed codebook gain |  |  |  |  |  | index of 5th LSF submatrix |  |
|  | UB | S91 | s90 | s89 |  | s88 | s87 | s38 | s37 |

TABLE I.13/H.245

Mapping of silence insertion descriptor bits from TIA IS641-A to octets, for the mode index 10

(Bits from cn0 to cn37 refer to TIA IS-641-A).

| Transmitted octets | MSB | Mapping of bits |  |  |  |  |  | LSB |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Index of 1st LSF subvector |  |  |  | Mode_Index |  |  |  |
|  | cn3 | cn2 | cn1 | cn0 | mi(3) | mi(2) | mi(1) | mi(0) |
| 2 | Index of 2nd LSF subvector |  |  |  | index of 1st LSF subvector |  |  |  |
|  | cn11 | Cn10 | cn9 | cn8 | cn7 | cn6 | cn5 | cn4 |
| 3 | Index of 3rd LSF subvector |  |  | Index of 2nd LSF subvector |  |  |  |  |
|  | cn19 | cn18 | cn17 | cn16 | cn15 | cn14 | cn13 | cn12 |
| 4 | Random Excitation Gain |  | index of 3rd LSF subvector |  |  |  |  |  |
|  | cn27 | cn26 | cn25 | cn24 | cn23 | cn22 | cn21 | cn20 |
| 5 | Index of 1st RESC parameter |  | Random Excitation Gain |  |  |  |  |  |
|  | cn35 | cn34 | cn33 | cn32 | cn31 | cn30 | cn29 | cn28 |
| 6 | Stuffing bits |  |  |  |  |  | Index of 2nd RESC parameter |  |
|  | UB | UB | UB | UB | UB | UB | cn37 | cn36 |

TABLE I.14/H.245

Mapping of silence insertion descriptor bits from RCR STD-27H to octets for the mode index 11

(Bits from s1 to s35 refer to RCR STD-27H).

| TransmittedOctets | MSB | Mapping of bits |  |  |  |  |  | LSB |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | index of 1st LSF subvector | index of LSF reference vector |  |  | Mode_Index |  |  |  |
|  | s4 | s3 | s2 | s1 | mi(3) | mi(2) | mi(1) | mi(0) |
| 2 | index of 2nd LSF subvector | index of 1st LSF subvector |  |  |  |  |  |  |
|  | s12 | s11 | s10 | s9 | s8 | s7 | s6 | s5 |
| 3 | index of 2nd LSF subvector |  |  |  |  |  |  |  |
|  | s20 | s19 | s18 | s17 | s16 | s15 | s14 | s13 |
| 4 | index of 3rd LSF subvector |  |  |  |  |  |  |  |
|  | s28 | s27 | s26 | s25 | s24 | s23 | s22 | s21 |
| 5 | SID type | frame energy |  |  |  |  |  | Index of 3rd LSF subvector |
|  | t1 | s35 | s34 | S33 | s32 | s31 | s30 | s29 |
| 6 | Stuffing bits |  |  |  |  |  |  | SID type |
|  | UB | UB | UB | UB | UB | UB | UB | t2 |

Definition of additional descriptor bits needed for PDC-EFR Table I.14:

SID-type is {0=POST0, 1=POST1(SID_UPDATE), 2=PRE, 3=POST1_BAD }, where LSB of SID_type is t1, and MSB of SID-type is t2.

TABLE I.15/H.245

The definition of the no transmission frame for the mode index 15.

| Transmitted Octets | MSB | Frame content |  |  |  |  |  | LSB |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Stuffing bits |  |  |  | Mode_Index |  |  |  |
|  | UB | UB | UB | UB | mi(3) | mi(2) | mi(1) | mi(0) |

TABLE I.16/H.245

Subjective importance of the speech encoded bits for AMR12.2.

| 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 11 | 12 | 13 | 14 | 23 | 15 | 16 | 17 | 18 |
| 19 | 20 | 21 | 22 | 24 | 25 | 26 | 27 | 28 | 38 |
| 141 | 39 | 142 | 40 | 143 | 41 | 144 | 42 | 145 | 43 |
| 146 | 44 | 147 | 45 | 148 | 46 | 149 | 47 | 97 | 150 |
| 200 | 48 | 98 | 151 | 201 | 49 | 99 | 152 | 202 | 86 |
| 136 | 189 | 239 | 87 | 137 | 190 | 240 | 88 | 138 | 191 |
| 241 | 91 | 194 | 92 | 195 | 93 | 196 | 94 | 197 | 95 |
| 198 | 29 | 30 | 31 | 32 | 33 | 34 | 35 | 50 | 100 |
| 153 | 203 | 89 | 139 | 192 | 242 | 51 | 101 | 154 | 204 |
| 55 | 105 | 158 | 208 | 90 | 140 | 193 | 243 | 59 | 109 |
| 162 | 212 | 63 | 113 | 166 | 216 | 67 | 117 | 170 | 220 |
| 36 | 37 | 54 | 53 | 52 | 58 | 57 | 56 | 62 | 61 |
| 60 | 66 | 65 | 64 | 70 | 69 | 68 | 104 | 103 | 102 |
| 108 | 107 | 106 | 112 | 111 | 110 | 116 | 115 | 114 | 120 |
| 119 | 118 | 157 | 156 | 155 | 161 | 160 | 159 | 165 | 164 |
| 163 | 169 | 168 | 167 | 173 | 172 | 171 | 207 | 206 | 205 |
| 211 | 210 | 209 | 215 | 214 | 213 | 219 | 218 | 217 | 223 |
| 222 | 221 | 73 | 72 | 71 | 76 | 75 | 74 | 79 | 78 |
| 77 | 82 | 81 | 80 | 85 | 84 | 83 | 123 | 122 | 121 |
| 126 | 125 | 124 | 129 | 128 | 127 | 132 | 131 | 130 | 135 |
| 134 | 133 | 176 | 175 | 174 | 179 | 178 | 177 | 182 | 181 |
| 180 | 185 | 184 | 183 | 188 | 187 | 186 | 226 | 225 | 224 |
| 229 | 228 | 227 | 232 | 231 | 230 | 235 | 234 | 233 | 238 |
| 237 | 236 | 96 | 199 |  |  |  |  |  |  |

TABLE I.17/H.245

Subjective importance of the speech encoded bits for AMR10.2.

| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 | 16 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 14 | 13 | 12 | 11 | 10 | 9 | 8 | 26 | 27 | 28 |
| 29 | 30 | 31 | 115 | 116 | 117 | 118 | 119 | 120 | 72 |
| 73 | 161 | 162 | 65 | 68 | 69 | 108 | 111 | 112 | 154 |
| 157 | 158 | 197 | 200 | 201 | 32 | 33 | 121 | 122 | 74 |
| 75 | 163 | 164 | 66 | 109 | 155 | 198 | 19 | 23 | 21 |
| 22 | 18 | 17 | 20 | 24 | 25 | 37 | 36 | 35 | 34 |
| 80 | 79 | 78 | 77 | 126 | 125 | 124 | 123 | 169 | 168 |
| 167 | 166 | 70 | 67 | 71 | 113 | 110 | 114 | 159 | 156 |
| 160 | 202 | 199 | 203 | 76 | 165 | 81 | 82 | 92 | 91 |
| 93 | 83 | 95 | 85 | 84 | 94 | 101 | 102 | 96 | 104 |
| 86 | 103 | 87 | 97 | 127 | 128 | 138 | 137 | 139 | 129 |
| 141 | 131 | 130 | 140 | 147 | 148 | 142 | 150 | 132 | 149 |
| 133 | 143 | 170 | 171 | 181 | 180 | 182 | 172 | 184 | 174 |
| 173 | 183 | 190 | 191 | 185 | 193 | 175 | 192 | 176 | 186 |
| 38 | 39 | 49 | 48 | 50 | 40 | 52 | 42 | 41 | 51 |
| 58 | 59 | 53 | 61 | 43 | 60 | 44 | 54 | 194 | 179 |
| 189 | 196 | 177 | 195 | 178 | 187 | 188 | 151 | 136 | 146 |
| 153 | 134 | 152 | 135 | 144 | 145 | 105 | 90 | 100 | 107 |
| 88 | 106 | 89 | 98 | 99 | 62 | 47 | 57 | 64 | 45 |
| 63 | 46 | 55 | 56 |  |  |  |  |  |  |

TABLE I.18/H.245

Subjective importance of the speech encoded bits for AMR7.95.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 14 | 16 | 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 12 | 13 | 15 | 11 | 17 | 20 | 22 | 24 | 23 |
| 19 | 18 | 21 | 56 | 88 | 122 | 154 | 57 | 89 | 123 |
| 155 | 58 | 90 | 124 | 156 | 52 | 84 | 118 | 150 | 53 |
| 85 | 119 | 151 | 27 | 93 | 28 | 94 | 29 | 95 | 30 |
| 96 | 31 | 97 | 61 | 127 | 62 | 128 | 63 | 129 | 59 |
| 91 | 125 | 157 | 32 | 98 | 64 | 130 | 1 | 0 | 25 |
| 26 | 33 | 99 | 34 | 100 | 65 | 131 | 66 | 132 | 54 |
| 86 | 120 | 152 | 60 | 92 | 126 | 158 | 55 | 87 | 121 |
| 153 | 117 | 116 | 115 | 46 | 78 | 112 | 144 | 43 | 75 |
| 109 | 141 | 40 | 72 | 106 | 138 | 36 | 68 | 102 | 134 |
| 114 | 149 | 148 | 147 | 146 | 83 | 82 | 81 | 80 | 51 |
| 50 | 49 | 48 | 47 | 45 | 44 | 42 | 39 | 35 | 79 |
| 77 | 76 | 74 | 71 | 67 | 113 | 111 | 110 | 108 | 105 |
| 101 | 145 | 143 | 142 | 140 | 137 | 133 | 41 | 73 | 107 |
| 139 | 37 | 69 | 103 | 135 | 38 | 70 | 104 | 136 |  |

TABLE I.19/H.245

Subjective importance of the speech encoded bits for AMR7.4.

| 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 11 | 12 | 13 | 14 | 15 | 16 | 26 | 87 | 27 |
| 88 | 28 | 89 | 29 | 90 | 30 | 91 | 51 | 80 | 112 |
| 141 | 52 | 81 | 113 | 142 | 54 | 83 | 115 | 144 | 55 |
| 84 | 116 | 145 | 58 | 119 | 59 | 120 | 21 | 22 | 23 |
| 17 | 18 | 19 | 31 | 60 | 92 | 121 | 56 | 85 | 117 |
| 146 | 20 | 24 | 25 | 50 | 79 | 111 | 140 | 57 | 86 |
| 118 | 147 | 49 | 78 | 110 | 139 | 48 | 77 | 53 | 82 |
| 114 | 143 | 109 | 138 | 47 | 76 | 108 | 137 | 32 | 33 |
| 61 | 62 | 93 | 94 | 122 | 123 | 41 | 42 | 43 | 44 |
| 45 | 46 | 70 | 71 | 72 | 73 | 74 | 75 | 102 | 103 |
| 104 | 105 | 106 | 107 | 131 | 132 | 133 | 134 | 135 | 136 |
| 34 | 63 | 95 | 124 | 35 | 64 | 96 | 125 | 36 | 65 |
| 97 | 126 | 37 | 66 | 98 | 127 | 38 | 67 | 99 | 128 |
| 39 | 68 | 100 | 129 | 40 | 69 | 101 | 130 |  |  |

TABLE I.20/H.245

Subjective importance of the speech encoded bits for AMR6.7.

| 0 | 1 | 4 | 3 | 5 | 6 | 13 | 7 | 2 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 9 | 11 | 15 | 12 | 14 | 10 | 28 | 82 | 29 | 83 |
| 27 | 81 | 26 | 80 | 30 | 84 | 16 | 55 | 109 | 56 |
| 110 | 31 | 85 | 57 | 111 | 48 | 73 | 102 | 127 | 32 |
| 86 | 51 | 76 | 105 | 130 | 52 | 77 | 106 | 131 | 58 |
| 112 | 33 | 87 | 19 | 23 | 53 | 78 | 107 | 132 | 21 |
| 22 | 18 | 17 | 20 | 24 | 25 | 50 | 75 | 104 | 129 |
| 47 | 72 | 101 | 126 | 54 | 79 | 108 | 133 | 46 | 71 |
| 100 | 125 | 128 | 103 | 74 | 49 | 45 | 70 | 99 | 124 |
| 42 | 67 | 96 | 121 | 39 | 64 | 93 | 118 | 38 | 63 |
| 92 | 117 | 35 | 60 | 89 | 114 | 34 | 59 | 88 | 113 |
| 44 | 69 | 98 | 123 | 43 | 68 | 97 | 122 | 41 | 66 |
| 95 | 120 | 40 | 65 | 94 | 119 | 37 | 62 | 91 | 116 |
| 36 | 61 | 90 | 115 |  |  |  |  |  |  |

TABLE I.21/H.245

Subjective importance of the speech encoded bits for AMR5.9.

| 0 | 1 | 4 | 5 | 3 | 6 | 7 | 2 | 13 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 9 | 11 | 12 | 14 | 10 | 16 | 28 | 74 | 29 |
| 75 | 27 | 73 | 26 | 72 | 30 | 76 | 51 | 97 | 50 |
| 71 | 96 | 117 | 31 | 77 | 52 | 98 | 49 | 70 | 95 |
| 116 | 53 | 99 | 32 | 78 | 33 | 79 | 48 | 69 | 94 |
| 115 | 47 | 68 | 93 | 114 | 46 | 67 | 92 | 113 | 19 |
| 21 | 23 | 22 | 18 | 17 | 20 | 24 | 111 | 43 | 89 |
| 110 | 64 | 65 | 44 | 90 | 25 | 45 | 66 | 91 | 112 |
| 54 | 100 | 40 | 61 | 86 | 107 | 39 | 60 | 85 | 106 |
| 36 | 57 | 82 | 103 | 35 | 56 | 81 | 102 | 34 | 55 |
| 80 | 101 | 42 | 63 | 88 | 109 | 41 | 62 | 87 | 108 |
| 38 | 59 | 84 | 105 | 37 | 58 | 83 | 104 |  |  |

TABLE I.22/H.245

Table A.14: Subjective importance of the speech encoded bits for AMR5.15.

| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 | 15 | 14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 13 | 12 | 11 | 10 | 9 | 8 | 23 | 24 | 25 | 26 |
| 27 | 46 | 65 | 84 | 45 | 44 | 43 | 64 | 63 | 62 |
| 83 | 82 | 81 | 102 | 101 | 100 | 42 | 61 | 80 | 99 |
| 28 | 47 | 66 | 85 | 18 | 41 | 60 | 79 | 98 | 29 |
| 48 | 67 | 17 | 20 | 22 | 40 | 59 | 78 | 97 | 21 |
| 30 | 49 | 68 | 86 | 19 | 16 | 87 | 39 | 38 | 58 |
| 57 | 77 | 35 | 54 | 73 | 92 | 76 | 96 | 95 | 36 |
| 55 | 74 | 93 | 32 | 51 | 33 | 52 | 70 | 71 | 89 |
| 90 | 31 | 50 | 69 | 88 | 37 | 56 | 75 | 94 | 34 |
| 53 | 72 | 91 |  |  |  |  |  |  |  |

TABLE I.23/H.245

Subjective importance of the speech encoded bits for AMR4.75.

| 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 11 | 12 | 13 | 14 | 15 | 23 | 24 | 25 | 26 |
| 27 | 28 | 48 | 49 | 61 | 62 | 82 | 83 | 47 | 46 |
| 45 | 44 | 81 | 80 | 79 | 78 | 17 | 18 | 20 | 22 |
| 77 | 76 | 75 | 74 | 29 | 30 | 43 | 42 | 41 | 40 |
| 38 | 39 | 16 | 19 | 21 | 50 | 51 | 59 | 60 | 63 |
| 64 | 72 | 73 | 84 | 85 | 93 | 94 | 32 | 33 | 35 |
| 36 | 53 | 54 | 56 | 57 | 66 | 67 | 69 | 70 | 87 |
| 88 | 90 | 91 | 34 | 55 | 68 | 89 | 37 | 58 | 71 |
| 92 | 31 | 52 | 65 | 86 |  |  |  |  |  |

ANNEX J

# Annex J	TDMA ACELP Voice Codec Definitions

A==

(This annex forms an integral part of this Recommendation)

Table I.1 defines the capability identifier for TIA/EIA /ANSI IS-136 ACELP voice codec Capabilities. Tables I.2 to I.4 define the associated capability parameters. This codec is used in TDMA Cellular and PCS base stations and mobile phones. The technical specifications of this codec are provided in TIA.EIA standard IS-136 Part 410. This standard is published by the TIA (North American) Telecommunications Industry Association and by ANSI, the American National Standards Institute.

TABLE J.1/H.245

IS-136 ACELP Capability Identifier

| Capability name: | IS-136 ACELP Vocoder |
| --- | --- |
| Capability class: | Audio codec. |
| Capability identifier type: | Standard. |
| Capability identifier value: | {itu-t (0) recommendation (0) h (8) 245 generic-capabilities (1) audio (1) acelp (2)} |
| maxBitRate: | This field shall be included |
| NonCollapsingRaw: | This field shall not be included |
| transport: | This field shall not be included |

TABLE J.2/H.245

IS-136 ACELP Capability Parameter - maxAl-sduFrames

| Parameter name: | MaxAl-sduFrames |
| --- | --- |
| Parameter description: | Specifies the maximum number of audio frames per AL-SDU |
| Parameter identifier value: | 0 |
| Parameter status: | Optional |
| Parameter type: | UnsignedMin |
| Supersedes: | - |

TABLE J.3/H.245

IS-136 ACELP Capability Parameter - Comfort Noise

| Parameter name: | ComfortNoise |
| --- | --- |
| Parameter description: | Specifies that IS-136 (IS-641) comfort noise is to be used in mode request. This parameter shall be used only in mode requests but not in capabilities because this capability is mandatory. |
| Parameter identifier value: | 1 |
| Parameter status: | Optional |
| Parameter type: | Logical |
| Supersedes: | - |

TABLE J.4/H.245

IS-136 ACELP Capability Parameter - Scrambled

| Parameter name: | Scrambled |
| --- | --- |
| Parameter description: | Specifies that scrambling is to be used in mode request. This parameter shall be used only in mode requests but not in capabilities because this capability is mandatory. |
| Parameter identifier value: | 2 |
| Parameter status: | Optional |
| Parameter type: | Logical |
| Supersedes: | - |

ANNEX K

# Annex K	TDMA US1 Voice Codec Definitions

A==

(This annex forms an integral part of this Recommendation)

Table K.1 defines the capability identifier for TIA/EIA /ANSI IS-136 US1 voice codec Capabilities. Tables K.2 to K.4 define the associated capability parameters. This codec is used in TDMA Cellular and PCS base stations and mobile phones. The technical specifications of this codec are provided in ANSI J-STD-007 and channel coding is specified in TIA/EIA standard IS-136 Part 430. These standard are published by ANSI, the American National Standards Institute and the TIA (North American) Telecommunications Industry Association respectively.

TABLE K.1/H.245

IS-136 US1 Capability Identifier

| Capability name: | IS-136 US1 Vocoder |
| --- | --- |
| Capability class: | Audio codec. |
| Capability identifier type: | Standard. |
| Capability identifier value: | {itu-t (0) recommendation (0) h (8) 245 generic-capabilities (1) audio (1) us1 (3)} |
| maxBitRate: | This field shall be included |
| NonCollapsingRaw: | This field shall not be included |
| transport: | This field shall not be included |

TABLE K.2/H.245

IS-136 US1 Capability Parameter - maxAl-sduFrames

| Parameter name: | MaxAl-sduFrames |
| --- | --- |
| Parameter description: | Specifies the maximum number of audio frames per AL-SDU |
| Parameter identifier value: | 0 |
| Parameter status: | Optional |
| Parameter type: | UnsignedMin |
| Supersedes: | - |

TABLE K.3/H.245

IS-136 US1Capability Parameter - Comfort Noise

| Parameter name: | ComfortNoise |
| --- | --- |
| Parameter description: | Specifies that comfort noise is to be used in mode request. This parameter shall be used only in mode requests but not in capabilities because this capability is mandatory. |
| Parameter identifier value: | 1 |
| Parameter status: | Optional |
| Parameter type: | Logical |
| Supersedes: | - |

TABLE K.4/H.245

IS-136 US1 Capability Parameter - Scrambled

| Parameter name: | Scrambled |
| --- | --- |
| Parameter description: | Specifies that scrambling is to be used in mode request. This parameter shall be used only in mode requests but not in capabilities because this capability is mandatory. |
| Parameter identifier value: | 2 |
| Parameter status: | Optional |
| Parameter type: | Logical |
| Supersedes: | - |

APPENDIX VIII

# Appendix VIII	List of generic capabilities defined in other Recommendations/Standards than this Recommendation

(This appendix does not form an integral part of this Recommendation)

Table VIII.1 lists the generic capabilities defined in other Recommendations or Standards than this Recommendation.

TABLE VIII.2/H.245

List of generic capabilies defined in other standardisation orginasations

| Capability name | Capability class | Capability identifier | Name of Recommendation or Standards that defines this capability |
| --- | --- | --- | --- |
| ITU-T Recommendation H.283 | Data protocol | itu-t (0) recommendation (0) h (8) 283 generic-capabilities (1) 0} | ITU-T Recommendation H.283 |
| ITU-T Recommendation G.722.1 | Audio protocol | itu-t (0) recommendation (0) g (7) 2221 generic-capabilities (1) 0} | ITU-T Recommendation G.722.1 |

*	 Contact:	Mike Nilsson	Tel.: +44 1473 645413

BT Labs	Fax: +44 1473 643791

Ipswich, UK	E-mail: mike.nilsson@bt-sys.bt.co.uk