| INTERNATIONAL TELECOMMUNICATION UNION |  |
| --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2000 - 2004 | TD- -E(PLEN) June 2001 Original: English |

Question D, F, G, 1-6/16

STUDY GROUP 16 - CONTRIBUTION

SOURCE* :		RAPPORTEUR for Question 3 (G.Freundlich)

TITLE:			RECOMMENDATION H.245 - VERSION 8

__________

This contribution, known as H245DELP, provides text for version 8 of Recommendation H.245 "Control protocol for multimedia communication". Only changes are shown, and these are shown relative to the decided version 7, known as H245NCMO. It has been created from H245NCMO, TD 95 (WP2) from the November 2000 meeting of SG16, and various AVD documents from the March 2001 Launceton rapporteur meeting of D, F, G, 2-5. The version number in the syntax has been changed and Annex D has been updated to reference the new version.

The changes from version 7 are summarised below.

1) TD 95 (WP2): Generic Capability description for AAC and AC-3.

2) TD 95 (WP2): Appendix VIII generic capability entries for H.324 HTTP Annex I (also AVD-2054 and TD-53) and H.263 Annex X.

3) AVD-2055: Generic capability definitions: added 'This is a collapsing Generic Parameter' at appropriate places in Annexes F, J, K and L, and 'This is a nonCollapsing Generic Parameter' in Annex L.

4) AVD-2040: Appendix VIII generic capability entry for H.281/H.224 based FECC for H.323.

Summary

This Recommendation specifies syntax and semantics of terminal information messages as well as procedures to use them for in-band negotiation at the start of or during communication. The messages cover receiving and transmitting capabilities as well as mode preference from the receiving end, logical channel signalling, and Control & Indication. Acknowledged signalling procedures are specified to ensure reliable audiovisual and data communication.

|  | INTERNATIONAL TELECOMMUNICATION UNION |
| --- | --- |

ITU-T	H.245

TELECOMMUNICATION	(June, 2001)
STANDARDIZATION  SECTOR
OF  ITU

LINE  TRANSMISSION  OF  NON-TELEPHONE
SIGNALS

ITU-T  Recommendation  H.245

SUMMARY

This Recommendation specifies syntax and semantics of terminal information messages as well as procedures to use them for in-band negotiation at the start of or during communication. The messages cover receiving and transmitting capabilities as well as mode preference from the receiving end, logical channel signalling, and Control & Indication. Acknowledged signalling procedures are specified to ensure reliable audiovisual and data communication.

Products claiming compliance with Version 87 of H.245 shall comply with all of the mandatory requirements of this Recommendation. Version 87 products can be identified by H.245 TerminalCapabilitySet messages containing a protocolIdentifier value of {itu-t (0) recommendation (0) h (8) 245 version (0) 87}.

ANNEX A

# Annex A	Messages: syntax

A==

(This annex forms an integral part of this Recommendation)

This annex specifies the syntax of messages using the notation defined in ASN.1 [1]. Messages shall be encoded for transmission by applying the packed encoding rules specified in [2] using the basic aligned variant. The first bit in each octet which is transmitted is the most significant bit of the octet as is specified in X.691.

MULTIMEDIA-SYSTEM-CONTROL DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- Export all symbols

-- ===================================================================================

-- Capability exchange definitions

-- ===================================================================================

TerminalCapabilitySet	::=SEQUENCE

{

sequenceNumber	SequenceNumber,

protocolIdentifier	OBJECT IDENTIFIER,

-- shall be set to the value

-- {itu-t (0) recommendation (0) h (8) 245 version (0) 87}

multiplexCapability	MultiplexCapability OPTIONAL,

capabilityTable	SET SIZE (1..256) OF CapabilityTableEntry OPTIONAL,

capabilityDescriptors	SET SIZE (1..256) OF CapabilityDescriptor OPTIONAL,

...

}

ANNEX B

# Annex B	Messages: semantic definitions

A==

(This annex forms an integral part of this Recommendation)

### B.0.1	Terminal Capability Set

This message contains information about the terminal's capability to transmit and receive. It also indicates the version of this Recommendation that is in use. It is sent from an outgoing CESE to a peer incoming CESE.

sequenceNumber is used to label instances of TerminalCapabilitySet so that the corresponding response can be identified.

protocolIdentifier is used to indicate the version of this Recommendation that is in use. Annex DA lists the object identifiers defined for use by this Recommendation.

multiplexCapability indicates capabilities relating to multiplexing and network adaptation. A terminal shall include multiplexCapability in the first TerminalCapabilitySet sent.

V75Capability indicates the capabilities of the V.75 control entity. The audioHeader indicates the capability of the V.75 audio header.

ANNEX D

# Annex D	Object identifier assignments

A==

(This annex forms an integral part of this Recommendation)

Table D.1 lists the assignment of Object Identifiers defined for use by this Recommendation.

TABLE D.1/H.245

| Object Identifier Value | Description |
| --- | --- |
| {itu-t (0) recommendation (0) h (8) 245 version (0) 1} | This Object Identifier is used to indicate the version of this Recommendation in use as a multimedia system control protocol. This indicates the eightfirst version of this Recommendation. |
| {itu-t (0) recommendation (0) h (8) 245 version (0) 2} | This Object Identifier is used to indicate the version of this Recommendation in use as a multimedia system control protocol. At this time there are eightseven standardized versions defined. This indicates the second version of this Recommendation. |
| {itu-t (0) recommendation (0) h (8) 245 version (0) 3} | This Object Identifier is used to indicate the version of this Recommendation in use as a multimedia system control protocol. At this time there are eightseven standardized versions defined. This indicates the third version of this Recommendation. |
| {itu-t (0) recommendation (0) h (8) 245 version (0) 4} | This Object Identifier is used to indicate the version of this Recommendation in use as a multimedia system control protocol. At this time there are seven standardized versions defined. This indicates the fourth version of this Recommendation. |
| {itu-t (0) recommendation (0) h (8) 245 version (0) 5} | This Object Identifier is used to indicate the version of this Recommendation in use as a multimedia system control protocol. At this time there are eightseven standardized versions defined. This indicates the fifth version of this Recommendation. |
| {itu-t (0) recommendation (0) h (8) 245 version (0) 6} | This Object Identifier is used to indicate the version of this Recommendation in use as a multimedia system control protocol. At this time there are eightseven standardized versions defined. This indicates the sixth version of this Recommendation. |
| {itu-t (0) recommendation (0) h (8) 245 version (0) 7} | This Object Identifier is used to indicate the version of this Recommendation in use as a multimedia system control protocol. At this time there are eightseven standardized versions defined. This indicates the seventh version of this Recommendation. |
| {itu-t (0) recommendation (0) h (8) 245 version (0) 8} | This Object Identifier is used to indicate the version of this Recommendation in use as a multimedia system control protocol. At this time there are eight standardized versions defined. This indicates the eighth version of this Recommendation. |
| {itu-t (0) recommendation (0) h (8) 245 generic-capabilities (1) video (0) ISO/IEC 14496-2 (0)} | This Object Identifier is used to indicate the generic capability for ISO/IEC 14496-2. This capability is defined in Annex E of this Recommendation. |
| {itu-t (0) recommendation (0) h (8) 245 generic-capabilities (1) audio (1) ISO/IEC 14496-3 (0)} | This Object Identifier is used to indicate the generic capability for ISO/IEC 14496-3. This capability is defined in Annex H of this Recommendation. |
| {itu-t (0) recommendation (0) h (8) 245 generic-capabilities (1) audio (1) amr (1)} | This Object Identifier is used to indicate the generic capability for the GSM Adaptive Multi rate speech codec. This capability is defined in Annex I of this Recommendation. |
| {itu-t (0) recommendation (0) h (8) 245 generic-capabilities (1) audio (1) acelp (2)} | This Object Identifier is used to indicate the generic capability for the TIA/EIA/ANSI IS-136 ACELP voice codec. This capability is defined in Annex J of this Recommendation. |
| {itu-t (0) recommendation (0) h (8) 245 generic-capabilities (1) audio (1) us1 (3)} | This Object Identifier is used to indicate the generic capability for the TIA/EIA /ANSI IS-136 US1 voice codec. This capability is defined in Annex K of this Recommendation. |
| {itu-t (0) recommendation (0) h (8) 245 generic-capabilities (1) audio (1) is127evrc (4)} | This Object Identifier is used to indicate the generic capability for the TIA/EIA IS-127 Enhanced Variable Rate Codec. This capability is defined in Annex L of this Recommendation. |
| {itu-t (0) recommendation (0) h (8) 245 generic-capabilities (1) data (2) ISO/IEC 14496-1 (0)} | This Object Identifier is used to indicate the generic capability for ISO/IEC 14496-1. This capability is defined in Annex G of this Recommendation. |
| {itu-t (0) recommendaton (0) h (8) 245 generic-capabilities (1) control (3) logical-channel-bit-rate-management (0)} | This Object Identifier is used to indicate the generic capability for logical channel bit rate management. This capability is defined in Annex F of this Recommendation. |
| {itu-t (0) recommendation (0) h (8) 245 generic-capabilities (1) audio (1) ISO/IEC 13818-7 (5)} | This Object Identifier is used to indicate the generic capability for the ISO/IEC 13818-7. This capability is defined in Annex M of this Recommendation. |
| {itu-t (0) recommendation (0) h (8) 245 generic-capabilities (1) audio (1) ITU-R BS.1196 (6)} | This Object Identifier is used to indicate the generic capability for the ITU-R BS.1196. This capability is defined in Annex M of this Recommendation. |

ANNEX F

# Annex F	Logical Channel Bit Rate Management Capability Definitions

A==

(This annex forms an integral part of this Recommendation)

Table F.1 defines the capability identifier for Bit Rate Management. These parameters, which provide information about which bit rate management messages the terminal supports, shall only be included as genericControlCapability within the Capability structure. Tables F.2 to F.4 define the associated capability parameters.

TABLE F.1/H.245

Capability Identifier for Logical Channel Bit Rate Management

| Capability name: | H.245 Logical Channel Bit Rate Management |
| --- | --- |
| Capability class: | Control. |
| Capability identifier type: | Standard. |
| Capability identifier value: | itu-t (0) recommendation (0) h (8) 245 generic-capabilities (1) control (3) logical-channel-bitrate-management (0) |
| maxBitRate: | The maxBitRate field shall not be included. |
| nonCollapsingRaw: | This field shall not be included. |
| transport: | This field shall not be included. |

TABLE F.2/H.245

Flow Control Capability Parameter for Bit Rate Management

| Parameter name: | Flow Control Capability |
| --- | --- |
| Parameter description: | This is a collapsing Generic Parameter. The presence of this parameter indicates the capability to support the FlowcontrolIndication message. |
| Parameter identifier value: | 0 |
| Parameter status: | Optional |
| Parameter type: | Logical. |
| Supersedes: | - |

TABLE F.3/H.245

Logical Channel Bit Rate Change Capability Parameter for Bit Rate Management

| Parameter name: | Logical Channel Bit Rate Change Capability |
| --- | --- |
| Parameter description: | This is a collapsing Generic Parameter. The presence of this parameter indicates the capability to support the Logical Channel Rate Change Procedure, which uses the messages LogicalChannelRateRequest, LogicalChannelRateAcknowledge, LogicalChannelRateReject and LogicalChannelRateRelease. |
| Parameter identifier value: | 1 |
| Parameter status: | Optional |
| Parameter type: | Logical. |
| Supersedes: | - |

TABLE F.4/H.245

RTCP Frequency Parameter for Bit Rate Management

| Parameter name: | RTCP Frequency Capability |
| --- | --- |
| Parameter description: | This is a collapsing Generic Parameter. This indicates the frequency at which the terminal can send RTCP reports. |
| Parameter identifier value: | 2 |
| Parameter status: | Optional |
| Parameter type: | Unsigned32Min. |
| Supersedes: | - |

ANNEX J

# Annex J	TDMA ACELP Voice Codec Definitions

A==

(This annex forms an integral part of this Recommendation)

Table I.1 defines the capability identifier for TIA/EIA 136 ACELP voice codec Capabilities [3]. Tables I.2 to I.4 define the associated capability parameters. This codec is used in TDMA Cellular and PCS base stations and mobile phones. The technical specifications of this codec are provided in TIA/EIA standard 136 Part 410. This standard is published by the TIA (North American) Telecommunications Industry Association and endorsed by ANSI, the American National Standards Institute.

TABLE J.1/H.245

136 ACELP Capability Identifier

| Capability name: | 136 ACELP Vocoder |
| --- | --- |
| Capability class: | Audio codec. |
| Capability identifier type: | Standard. |
| Capability identifier value: | {itu-t (0) recommendation (0) h (8) 245 generic-capabilities (1) audio (1) acelp (2)} |
| maxBitRate: | This field shall be included |
| NonCollapsingRaw: | This field shall not be included |
| transport: | This field shall not be included |

TABLE J.2/H.245

136 ACELP Capability Parameter - maxAl-sduFrames

| Parameter name: | MaxAl-sduFrames |
| --- | --- |
| Parameter description: | This is a collapsing Generic Parameter. Specifies the maximum number of audio frames per AL-SDU |
| Parameter identifier value: | 0 |
| Parameter status: | Shall be present for capability exchange and logical channel signalling. Shall not be present for mode request. |
| Parameter type: | UnsignedMin |
| Supersedes: | - |

TABLE J.3/H.245

136 ACELP Capability Parameter - Comfort Noise

| Parameter name: | ComfortNoise |
| --- | --- |
| Parameter description: | This is a collapsing Generic Parameter. Specifies that TIA/EIA 136 (IS-641) comfort noise is to be used in mode request. This parameter shall be used only in mode requests but not in capabilities because this capability is mandatory. |
| Parameter identifier value: | 1 |
| Parameter status: | Optional |
| Parameter type: | Logical |
| Supersedes: | - |

TABLE J.4/H.245

136 ACELP Capability Parameter - Scrambled

| Parameter name: | Scrambled |
| --- | --- |
| Parameter description: | This is a collapsing Generic Parameter. Specifies that scrambling is to be used in mode request. This parameter shall be used only in mode requests but not in capabilities because this capability is mandatory. |
| Parameter identifier value: | 2 |
| Parameter status: | Optional |
| Parameter type: | Logical |
| Supersedes: | - |

ANNEX K

# Annex K	TDMA US1 Voice Codec Definitions

A==

(This annex forms an integral part of this Recommendation)

Table K.1 defines the capability identifier for TIA/EIA 136 US1 voice codec Capabilities [4]. Tables K.2 to K.4 define the associated capability parameters. This codec is used in TDMA Cellular and PCS base stations and mobile phones. The technical specifications of this codec are provided in TIA/EIA standard 136 Part 430. Thi standard is published by TIA - the (North American) Telecommunications Industry Association and endorsed by ANSI, The American National Standards Institute.

TABLE K.1/H.245

136 US1 Capability Identifier

| Capability name: | 136 US1 Vocoder |
| --- | --- |
| Capability class: | Audio codec. |
| Capability identifier type: | Standard. |
| Capability identifier value: | {itu-t (0) recommendation (0) h (8) 245 generic-capabilities (1) audio (1) us1 (3)} |
| maxBitRate: | This field shall be included |
| NonCollapsingRaw: | This field shall not be included |
| transport: | This field shall not be included |

TABLE K.2/H.245

136 US1 Capability Parameter - maxAl-sduFrames

| Parameter name: | MaxAl-sduFrames |
| --- | --- |
| Parameter description: | This is a collapsing Generic Parameter. Specifies the maximum number of audio frames per AL-SDU |
| Parameter identifier value: | 0 |
| Parameter status: | Shall be present for capability exchange and logical channel signalling. Shall not be present for mode request. |
| Parameter type: | UnsignedMin |
| Supersedes: | - |

TABLE K.3/H.245

136 US1Capability Parameter - Comfort Noise

| Parameter name: | ComfortNoise |
| --- | --- |
| Parameter description: | This is a collapsing Generic Parameter. Specifies that comfort noise is to be used in mode request. This parameter shall be used only in mode requests but not in capabilities because this capability is mandatory. |
| Parameter identifier value: | 1 |
| Parameter status: | Optional |
| Parameter type: | Logical |
| Supersedes: | - |

TABLE K.4/H.245

136 US1 Capability Parameter - Scrambled

| Parameter name: | Scrambled |
| --- | --- |
| Parameter description: | This is a collapsing Generic Parameter. Specifies that scrambling is to be used in mode request. This parameter shall be used only in mode requests but not in capabilities because this capability is mandatory. |
| Parameter identifier value: | 2 |
| Parameter status: | Optional |
| Parameter type: | Logical |
| Supersedes: | - |

ANNEX L

# Annex L	CDMA  EVRC Voice Codec Definitions

A==

(This annex forms an integral part of this Recommendation)

Table L.1 defines the capability identifier for TIA/EIA IS-127 Enhanced Variable Rate Codec (EVRC)  which is used in TIA/EIA IS-95 CDMA Cellular and PCS base stations and mobile phones. The full technical description and detailed specifications of this codec are provided in TIA/EIA IS-127 standard which is published by the North American Telecommunications Industry Association (TIA). Tables L.2 to L.4 define the associated capability parameters.

TABLE L.1/H.245

CDMA EVRC Capability Identifier

| Capability name: | IS-127 CDMA EVRC |
| --- | --- |
| Capability class: | Audio codec. |
| Capability identifier type: | Standard. |
| Capability identifier value: | {itu-t (0) recommendation (0) h (8) 245 generic-capabilities (1) audio (1) is127evrc (4)} |
| maxBitRate: | This field shall be included |
| NonCollapsingRaw: | This field shall not be included |
| transport: | This field shall not be included |

TABLE L.2/H.245

IS-127 CDMA EVRC Capability Parameter - maxAl-sduFrames

| Parameter name: | MaxAl-sduFrames |
| --- | --- |
| Parameter description: | This is a collapsing Generic Parameter. Specifies the maximum number of audio frames per AL-SDU |
| Parameter identifier value: | 0 |
| Parameter status: | Shall be present for capability exchange and logical channel signalling. Shall not be present for mode request. |
| Parameter type: | UnsignedMin |
| Supersedes: | - |

TABLE L.3/H.245

CDMA EVRC Capability Parameter - EVRC Bit-Rate

| Parameter name: | EVRCRate |
| --- | --- |
| Parameter description: | This is a nonC-collapsing GenericParameter. It specifies the vocoder output bit-rate mode. This parameter shall be used in mode requests: 1=full-rate; 2=half-rate; 3=eighth-rate; 4=blanked mode. |
| Parameter identifier value: | 1 |
| Parameter status: | Optional |
| Parameter type: | UnsignedMin |
| Supersedes: | - |

TABLE L.4/H.245

CDMA EVRC Capability Parameter - Scrambled

| Parameter name: | Scrambled |
| --- | --- |
| Parameter description: | This is a collapsing Generic Parameter. Specifies that scrambling is to be used in mode request. |
| Parameter identifier value: | 2 |
| Parameter status: | Optional |
| Parameter type: | Logical |
| Supersedes: | - |

ANNEX M

# Annex M	ISO/IEC 13818-7 and ITU-R BS.1196 Definitions

A==

(This annex forms an integral part of this Recommendation)

Table M.1 defines the capability identifier for ISO/IEC 13818-7. Table M.2 defines the associated capability parameters.

Table M.1 defines the capability identifier for ITU-R BS.1196. There are no associated capability parameters.

TABLE M.1/H.245

ISO/IEC 13818-7 Capability Identifier

| Capability name: | ISO/IEC 13818-7 Capability |
| --- | --- |
| Capability class: | Audio codec. |
| Capability identifier type: | Standard. |
| Capability identifier value: | {itu-t (0) recommendation (0) h (8) 245 generic-capabilities (1) audio (1) ISO/IEC 13818-7 (5)} |
| maxBitRate: | This field shall be included |
| NonCollapsingRaw: | This field shall not be included |
| transport: | This field shall not be included |

TABLE M.2/H.245

Profile and Level for ISO/IEC 13818-7 Capability

| Parameter name: | profileAndLevel |
| --- | --- |
| Parameter description: | This is a nonCollapsing GenericParameter. profileAndLevel indicates the capability of processing the particular profiles in combination with the level as given in ISO/IEC 13818-7. |
| Parameter identifier value: | 0 |
| Parameter status: | Mandatory. |
| Parameter type: | unsignedMax. Shall be in the range 0..255. |
| Supersedes: | - |

TABLE M.3/H.245

ITU-R BS.1196 Capability Identifier

| Capability name: | ITU-R BS.1196 Capability |
| --- | --- |
| Capability class: | Audio codec. |
| Capability identifier type: | Standard. |
| Capability identifier value: | {itu-t (0) recommendation (0) h (8) 245 generic-capabilities (1) audio (1) ITU-R BS.1196 (6)} |
| maxBitRate: | This field shall be included |
| NonCollapsingRaw: | This field shall not be included |
| transport: | This field shall not be included |

APPENDIX VIII

# Appendix VIII	List of generic capabilities defined in other Recommendations/Standards than this Recommendation

(This appendix does not form an integral part of this Recommendation)

Table VIII.1 lists the generic capabilities defined in other Recommendations or Standards than this Recommendation.

TABLE VIII.2/H.245

List of generic capabilies defined in other standardisation orginasations

| Capability name | Capability class | Capability identifier | Name of Recommendation or Standards that defines this capability |
| --- | --- | --- | --- |
| ITU-T Recommendation H.283 | Data protocol | {itu-t (0) recommendation (0) h (8) 283 generic-capabilities (1) 0} | ITU-T Recommendation H.283 |
| ITU-T Recommendation G.722.1 | Audio protocol | {itu-t (0) recommendation (0) g (7) 7221 generic-capabilities (1) 0} | ITU-T Recommendation G.722.1 |
| ITU-T Recommendation H.324 | Data protocol | itu-t (0) recommendation (0) h (8) 324 generic-capabilities (1) http (0) | ITU-T Recommendation H.324 |
| ITU-T Recommendation H.263 | Video protocol | {itu-t (0) recommendation (0) h (8) 263 generic-capabilities (1) 0} | ITU-T Recommendation H.263 |
| ITU-T Recommendation H.224 | Data protocol | {itu-t (0) recommendation (0) h (8) 224 generic-capabilities (1) 0} | ITU-T Recommendation H.224 |

*	 Contact:	Mike Nilsson	Tel.: +44 1473 645413

BT Labs	Fax: +44 1473 646589

Ipswich, UK	E-mail: mike.nilsson@bt.com